Pacific Project

Pacific Project

Objective

This project aims to substantially improve operational efficiency and environmental outcomes on the major air traffic flow between North America and Asia.

The key to this objective is to enable aircraft to more effectively utilise current onboard technology while flying User Preferred Routes (UPR).

The project will integrate capability with NEXTGEN and SESAR and provide a link to the Asia Pacific “Seamless Skies” initiative launched at this year’s Directors General Conference in Japan.

# Background

The North Pacific is characterised by large geographic volumes of airspace managed by Canada, Japan, Russia and the United States.

Over the last two decades new routes and procedures have increased capacity and improved efficiency.

However, this capacity has been absorbed by air traffic growth, which will continue to outpace capacity increases.

Over the same period airlines have invested heavily in improved aircraft capability, which is now well in advance of ATC capabilities and supporting infrastructure.  Unfortunately this creates a situation where proven technology and procedures cannot be employed to deliver available benefits in safety, capacity and efficiency.

The “Pacific Project” aims to collectively generate improvements in airspace management to more effectively utilize this airborne capability. In so doing this will increase airspace capacity and assist to satisfy future demand without the ongoing escalation of inefficiencies.

Current Situation

The current route structure is based on fixed tracks (NOPAC, RTE, etc) together with flexible tracking (PACOTS) and User Preferred Routes (UPR) in defined areas.

- Many of the fixed tracks are based on terrestrial aids

- NOPAC fixed tracks condense traffic into a confined area

- NOPAC fixed tracks are assigned priority limiting the benefits which could be obtained from more a flexible route structure

- Flights that transit Russian airspace have limited entry/exit points and therefore little track flexibility

- The great circle nature of fixed tracks does not allow best use of prevailing winds and avoidance of unfavourable winds

- Demand for the limited number of tracks frequently exceeds capacity

- The design of fixed tracks does not take advantage of developing navigation capacity such as PBN

PACOTS do generate efficiencies but they are limited in their generic nature, validity periods, lead-time for publishing and operational restrictions against NOPAC.

UPRs are available in some areas but operational restrictions can negate any possible benefit due to the priority allocated to both PACOTS and fixed tracks.

A “seamless” operation is not possible because of the varying separation and navigation requirements and the surveillance and communication capabilities.

Benefits

The greatest benefit will clearly be obtained by the use of UPR.

Benefits include reduced flight times and fuel burn, increased payload capability and significantly reduced environmental emissions.

The long-haul nature of flights between North America and Asia enables enormous gains if aircraft are able to take advantage of upper wind patterns.

Modelling conducted to date between LAX/HKG suggests that a B777 UPR flight time reduces on average by 25 minutes. There are similar savings LAX/BJS.

Of greater significance, B747 aircraft UPR LAX/HKG have potential winter flight time reductions of 70-80 mins and payload increases of 5%.

This is a saving of 8000kg of fuel and CO2 reduction of some 25000Kg.

Extrapolating these savings across the number of aircraft which fly in this area manifestly demonstrates the potential benefit.

Environment

Aviation must reduce CO2 emissions.

The aviation industry has agreed ambitious environmental targets.

Efficient airspace management will be an integral part of attaining these targets.

Initiatives such as the ASPIRE demonstration flights have shown the efficiencies possible if all other aircraft are removed from the trial aircraft’s desired profile.

The Pacific Project will enable ALL aircraft to fly their desired profile and ALL aircraft to obtain the efficiencies, which we know are obtainable.

Whilst this may, on the face of it, seem an insurmountable challenge today the seasonal variation will provide a partial solution. Flights eastbound would look to take advantage of westerly jet streams whereas flights westbound would be looking to avoid these areas. Nature effectively producing a uni-directional flow arrangement.

Next Steps

Whilst various forums, such as IPACG & CPWG, have facilitated significant regional gains there is no single forum for this project.

Therefore, we propose that a specific project be established to consider operations between North America and Asia collectively and from end-to-end.

This project requires the involvement of the key stakeholders, Canada, Japan, Russia, USA and IATA/Airlines. Significant input is also required from China to provide connector routes for aircraft to feed into the North Pacific area. Other stakeholders DPR Korea, the Philippines and South Korea also need to be engaged during the project to facilitate connector routes.

IATA will be promoting this project at both Operational and Political forums to gain endorsement. We are looking to the States to provide the necessary support to this project to begin the quest to reduce airline costs and reduce CO2 emissions.

Summary

We believe that the benefits are undeniable.

Conversely we do not under estimate the challenge.

We acknowledge Industry has worked tirelessly, together, to provide benefits across the current North Pacific track structure.

We applaud current efforts to demonstrate where environmental savings can be obtained in an attempt to coalesce action.

We now look for Leadership, from all parties, to agree that these benefits are worth the effort required and that we should now join together and work out a plan.

And from planning to action – the time has come.

Any comments or questions please contact

Geoff Hounsell

Assistant Director ATM

Safety Operations and Infrastructure

IATA Asia Pacific

hounsellg@iata.org

Example Data of Projected Benefits B772

(Oct 2009-Oct 2010 based on historical winds)

| EASTBOUND |  |  | TIME |  |  | FUEL |  |  | DIST |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NOW | FLEX | Δ mins | NOW | FLEX | Δ kgs | NOW | FLEX | Δ NM |
| PEK | YVR | OCT | 9.28 | 9.16 | 12 | 55,006 | 53,302 | 1,704 | 4,447 | 4,318 | 129 |
|  |  | APR | 9.31 | 9.24 | 7 | 65,869 | 64,693 | 1,176 | 4,427 | 4,350 | 77 |
| PEK | LAX | OCT | 11.16 | 10.48 | 28 | 84,307 | 80,132 | 4,175 | 5,330 | 5,096 | 234 |
|  |  | APR | 11.04 | 10.36 | 28 | 79,847 | 76,034 | 3,813 | 5,107 | 4,910 | 197 |
| NRT | YVR | OCT | 8.20 | 8.10 | 10 | 57,857 | 56,623 | 1,234 | 3,895 | 3,823 | 72 |
|  |  | APR | 8.05 | 8.03 | 2 | 54,607 | 54,469 | 138 | 3,722 | 3,721 | 1 |
| NRT | LAX | OCT | 9.48 | 9.41 | 7 | 70,212 | 68,989 | 1,223 | 4,628 | 4,562 | 66 |
|  |  | APR | 9.05 | 9.05 | 0 | 62,325 | 62,282 | 43 | 4,188 | 4,195 | -7 |
| HKG | YVR | OCT | 11.23 | 10.47 | 36 | 83,802 | 78,249 | 5,553 | 5,412 | 5,098 | 314 |
|  |  | APR | 11.01 | 10.56 | 5 | 80,775 | 80,091 | 684 | 5,164 | 5,116 | 48 |
| HKG | LAX | OCT | 13.07 | 12.26 | 41 | 98,049 | 93,581 | 4,468 | 6,270 | 5,923 | 347 |
|  |  | APR | 11.54 | 11.47 | 7 | 86,327 | 85,265 | 1,062 | 5,554 | 5,507 | 47 |
| SIN | YVR | OCT | 14.15 | 13.46 | 29 | 105,029 | 102,228 | 2,801 | 6,823 | 6,583 | 240 |
|  |  | APR | 13.39 | 13.32 | 7 | 100,099 | 99,313 | 786 | 6,435 | 6,377 | 58 |
| SIN | LAX | OCT | 15.43 | 15.15 | 28 | 113,460 | 110,671 | 2,789 | 7,531 | 7,286 | 245 |
|  |  | APR | 14.32 | 14.25 | 7 | 104,928 | 104,159 | 769 | 7,727 | 7,703 | 24 |

| WESTBOUND |  |  | TIME |  |  | FUEL |  |  | DIST |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NOW | FLEX | Δ mins | NOW | FLEX | Δ kgs | NOW | FLEX | Δ NM |
| YVR | PEK | OCT | 10.35 | 10.13 | 23 | 78,033 | 74,989 | 3,044 | 4,985 | 4,797 | 188 |
|  |  | FEB | 10.33 | 10.07 | 26 | 76,719 | 72,117 | 4,602 | 4,715 | 4,695 | 20 |
|  |  | JUN | 9.57 | 9.47 | 10 | 73,325 | 71,636 | 1,689 | 4,659 | 4,566 | 93 |
| LAX | PEK | OCT | 12.29 | 12.03 | 26 | 93,573 | 91,116 | 2,457 | 4,179 | 4,251 | -72 |
|  |  | FEB | 12.26 | 12.00 | 26 | 91,669 | 88,454 | 3,215 | 4,448 | 4,393 | 55 |
|  |  | JUN | 11.52 | 11.30 | 22 | 88,478 | 86,288 | 2,190 | 5,565 | 5,403 | 162 |
| YVR | NRT | OCT | 9.04 | 8.55 | 9 | 65,202 | 63,989 | 1,213 | 4,179 | 4,251 | -72 |
|  |  | FEB | 9.31 | 9.24 | 7 | 68,599 | 67,553 | 1,046 | 4,448 | 4,393 | 55 |
|  |  | JUN | 9.06 | 9.02 | 4 | 65,810 | 65,024 | 786 | 4,271 | 4,228 | 43 |
| LAX | NRT | OCT | 10.20 | 10.12 | 8 | 76,094 | 74,887 | 1,207 | 4,874 | 4,838 | 36 |
|  |  | FEB | 11.10 | 11.03 | 7 | 82,851 | 81,791 | 1,060 | 5,262 | 5,198 | 64 |
|  |  | JUN | 10.58 | 10.54 | 4 | 81,992 | 81,803 | 189 | 5,190 | 5,174 | 16 |
| YVR | HKG | OCT | 12.16 | 12.01 | 15 | 92,084 | 90,582 | 1,502 | 5,804 | 5,678 | 126 |
|  |  | FEB | 13.22 | 12.46 | 36 | 97,817 | 94,203 | 3,614 | 6,286 | 5,997 | 289 |
|  |  | JUN | 12.11 | 11.54 | 17 | 91,425 | 89,798 | 1,627 | 5,759 | 5,613 | 146 |
| LAX | HKG | OCT'09 | 13.37 | 13.21 | 16 | 100,215 | 98,669 | 1,546 | 6,463 | 6,326 | 137 |
|  |  | NOV | 14.52 | 14.28 | 24 | 106,533 | 104,117 | 2,416 | 6,998 | 6,797 | 201 |
|  |  | DEC | 14.44 | 14.25 | 19 | 105,957 | 104,129 | 1,828 | 6,942 | 6,787 | 155 |
|  |  | JAN | 14.49 | 14.06 | 43 | 105,868 | 102,149 | 3,719 | 6,931 | 6,634 | 297 |
|  |  | FEB | 15.12 | 14.31 | 41 | 108,713 | 104,794 | 3,919 | 7,162 | 6,836 | 326 |
|  |  | MAR | 14.56 | 14.40 | 16 | 107,090 | 105,430 | 1,660 | 7,035 | 6,890 | 145 |
|  |  | APR | 14.29 | 13.47 | 42 | 104,105 | 100,550 | 3,555 | 6,792 | 6,469 | 323 |
|  |  | MAY | 13.36 | 13.21 | 15 | 99,841 | 98,404 | 1,437 | 6,443 | 6,321 | 122 |
|  |  | JUN | 14.15 | 13.55 | 20 | 104,606 | 102,757 | 1,849 | 6,786 | 6,637 | 149 |
|  |  | JUL | 13.53 | 13.27 | 26 | 102,561 | 99,786 | 2,775 | 6,624 | 6,413 | 211 |
|  |  | AUG | 13.49 | 13.38 | 11 | 101,733 | 100,677 | 1,056 | 6,563 | 6,492 | 71 |
|  |  | SEP | 13.58 | 13.37 | 21 | 101,983 | 99,878 | 2,105 | 6,585 | 6,421 | 164 |
|  |  | OCT'10 | 14.17 | 13.59 | 18 | 104,396 | 102,263 | 2,133 | 6,781 | 6,612 | 169 |
| YVR | SIN | NOV | 14.44 | 14.36 | 8 | 107,408 | 106,573 | 835 | 7,034 | 6,976 | 58 |
|  |  | MAR |  |  |  |  |  | 0 |  |  | 0 |
|  |  | JUL |  |  |  |  |  | 0 |  |  | 0 |
| LAX | SIN | OCT | 16.59 | 16.39 | 20 | 120,141 | 118,206 | 1,935 | 8,120 | 7,956 | 164 |
|  |  | FEB | 17.12 | 16.38 | 34 | 120,491 | 117,251 | 3,240 | 8,153 | 7,899 | 254 |
|  |  | JUN | 16.29 | 16.13 | 16 | 117,109 | 115,485 | 1,624 | 7,873 | 7,731 | 142 |

Examples of expected benefits for a B772 detailed at Appendix 1

ICAO HLM on Environment