Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4:   Provide Status on CPWG/8 Actions

MANAGING GATEWAY RESERVATION LIST COMPLIANCE

(Action Item #CP01-18)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper presents information regarding the Air Traffic Flow Management procedures for managing Gateway Reservation List (GRL) compliance for flights track loaded over Cross Polar (CP) fixes ORVIT and north; and Russian Trans East (RTE) fixes PILUN and south.

1.	Introduction

# Operators utilizing the Cross Polar (CP) or Russian Trans-East (RTE) routes will comply with the requirements outlined in the “Track Advisory User’s Guide for Dispatchers” (TAUG) for use of the Dynamic Ocean Tracking System (DOTS) Plus Track Advisory (TA) program.  Flight dispatchers are required to update the information on the Gateway Reservation List (GRL) on all planned flights.  The Anchorage Air Route Traffic Control Center (ZAN) portion of the TAUG is at Appendix A.

# The Air Traffic Control System Command Center (ATCSCC) notifies NAV CANADA Traffic Management Unit (TMU) West located at Vancouver Area Control Centre (ACC) and appropriate airline dispatchers of any identified conflicts on the CP routes.

Note:  TMU West’s area of responsibility encompasses the Vancouver, Edmonton, and Winnipeg Flight Information Regions (FIRs).

# TMU West monitors identified flights filed on the CP/RTE routes and works with the ATCSCC, the affected Canadian ACC and appropriate airline dispatchers to achieve early conflict resolution within Canadian airspace.

# ZAN is responsible for conflict resolution on the RTE routes for boundary fixes PILUN and south.  ZAN works with the ATCSCC, TMU West, and appropriate airline dispatchers to achieve early conflict resolution within Canadian or United States airspace as required.

- Discussion

- Responsibilities are defined as follows:

  - Operators will:

    - Brief dispatchers/flight crews on the new procedures, emphasizing that the GRL is a tool used to assist dispatchers in their flight planning and to provide air traffic facilities early intent information.

    - Brief dispatchers/flight crews that the GRL does not offer or establish priority for any flight operating on the CP routes or via RTE fixes PILUN and LISKI.

    - Brief dispatchers/flight crews that ZAN is currently conducting a 60 day trial to reduce the 20 minute track load requirement for RTE fixes FRENK, KUTAL, VALDA, and ERNIK to 10 minutes, which institutes the same procedures currently used on all CP fixes and RTE fixes PILUN and LISKI.

    - Brief dispatchers/flight crews that flights operating on the RTE routes via RUSOR and south, aircraft must arrive at the gateway fix within the scheduled window (GRL time, plus 10 minutes) to increase the probability of GRL altitude assignment.  It should be noted, however, that the TA program software does not consider aircraft performance characteristics when assigning slots on the GRL. Therefore, even though a flight may arrive at the GRL fix “on-time”, the flight may not be assigned the GRL altitude.

    - Until time of departure, update the information provided on the GRL on all planned flights by submitting a new TKF request as outlined in the TAUG.  To provide for more dynamic system awareness, operators may revise their GRL information including any changes to the wheels-up time, requested altitude, or estimate at the gateway fix, at any time after departure.  If further assistance is required, the customer may contact the ATCSCC or ZAN TMU as outlined below for assistance.

    - Call the ATCSCC International Operations Position at +1 703-925-5312 with any issues pertaining to the CP routes.

    - Call ZAN TMU at +1 907-269-1840 with any issues pertaining to the RTE routes PILUN and south.

  - ATCSCC will:

    - Monitor CP routes to identify potential conflicts at the CP fixes.

    - Implement early conflict resolution on CP route traffic and coordinate all actions with the NOC as appropriate.

    - Assist NOC/TMU West on implementing any TMU West/Canadian ACC initiated conflict resolution plan.

    - Coordinate with the airlines when appropriate.

    - Assist ZAN TMU on implementing any ZAN TMU initiated conflict resolution plan, including the issuance of controlled departure times via the National Traffic Management Log (NTML).

  - TMU West will:

    - Coordinate with the ATCSCC and other ATC facilities to determine an appropriate conflict resolution decision(s).

    - Coordinate with airline dispatchers when appropriate.

  - ZAN TMU will:

    - Operate and maintain the DOTS Plus TA Program.

    - Actively monitor the GRL and RTE route traffic PILUN and south for potential conflicts.  Alert the ATCSCC and TMU West when requiring assistance with conflict resolution on the RTE routes.  When required, issue controlled departure times via NTML through the ATCSCC.

- Currently, priority is given by Edmonton ACC utilizing the GRL in a “tie breaking” manner. Those that are “on time” will normally be given their planned altitude, and will thus be given priority.

- Recommendation

3.1	The meeting is invited to:

  - note the information provided in this working paper; and

  - discuss and support implementation of procedures and processes needed to mitigate compacted demand over Cross Polar and Russian Trans East routes.

ANCHORAGE CENTER TRACK ADVISORY PROCEDURES

This section provides specific procedures for westbound flights planning to enter the Russian Cross Polar and Trans East (CP/RTE) route system.

Dispatchers must transmit daily TKF options to Anchorage ARTCC (ZAN) for all westbound flights planning to enter the CP/RTE tracks according to the following table:

| Trackload Program | File TKF | Compile and Send GRL | Negotiation Period | Archive Time |
| --- | --- | --- | --- | --- |
| 1700Z-0700Z | 0900Z-1430Z | 1430Z | 1430Z-0000Z | 0000Z |
| 0700Z-1700Z | 0015Z-0430Z | 0430Z | 0430Z-0900Z | 0900Z |

1700 UTC – 0700 UTC: GRL is compiled and sent to users at 1430 UTC.

1430 UTC – 0000 UTC: The negotiation period.

0000 UTC – 0700 UTC: All TKF requests must be called in to ZAN - 907-269-1104.

0700 UTC – 1700 UTC: GRL is compiled and sent to users at 0430 UTC.

0430 UTC – 0900 UTC: The negotiation period.

0900 UTC – 1700 UTC:  All TKF requests must be called in to ZAN - 907-269-1104.

TKFs for CP/RTE routes requiring a metric altitude use the following designator:

| FL = Meters | Designator |
| --- | --- |
| FL280 = 8600 Meters | 86 |
| FL310 = 9600 Meters | 96 |
| FL350 = 10600 Meters | 106 |
| FL380/390 = 11600 Meters | 116 |
| FL430 = 13100 Meters | 131 |

Required trackload minima and useable altitudes at the gateway fixes:

| Gateway Fix | In-trail Spacing | Useable Altitudes |
| --- | --- | --- |
| DEVID | 10 minutes | 310, 350, 390 |
| RAMEL | 10 minutes | 320, 340, 360 |
| NIKIN | 10 minutes | 320, 340, 380 |
| ORVIT | 10 minutes | 320, 340, 380 |
| PILUN | 10 minutes | 96, 106, 116 |
| LISKI | 10 minutes | 96, 106, 116 |
| FRENK | 20 minutes | 86, 96, 106, 116, 131 |
| KUTAL | 20 minutes | 86, 96, 106, 116, 131 |
| VALDA | 20 minutes | 86, 96, 106, 116, 131 |
| ERNIK | 20 minutes | 86, 96, 106, 116, 131 |
| RUSOR | 20 minutes | 86, 96, 106, 131 |
| MARCC | 20 minutes | 86, 96, 106, 131 |
| BAMOK | 20 minutes | 86, 96, 106, 131 |

NOTE: For flights transiting ERNIK and north, tactical sequencing may be utilized to ensure required in-trail spacing for aircraft at the same altitude.  This may include speed control and/or vectors to accomplish the required spacing.  For flights transiting via RUSOR and south, aircraft must arrive at the gateway fix at the GRL gateway time or no more than 10 minutes later to increase the probability of being assigned the GRL altitude for the waypoints with 20 minutes-in-trail trackload spacing.

Example of ZAN TKF message format:

TKF

Callsign.2000.KSFO.RJAA.106.VALDA.0130.15.96.VALDA.0130.20.REM. DISPATCHER REMARKS GO HERE FOR THE FLIGHT

After the GRL is published, TKF messages will be processed automatically without TMU intervention during the negotiation period.

Transmit messages to the ZAN TA program via ARINC or NADIN/AFTN using the following addresses:

ARINC:	ANCDPXA

# NADIN/AFTN:	PANCZOZD

## 1430 - 0000 UTC & 0430 - 0900 UTC: Slot availability message function is active.

REVISED RESERVATION FOR UAL1 DEPARTING KSFO HAS BEEN APPROVED

TRACK=VALDA  ALTITUDE=106  DEPARTURE TIME=2040  VALDA TIME=0211

## PROCEDURES FOR COMPUTER OR COMMUNICATION

## LINK FAILURES AT ANCHORAGE CENTER

In the event that the TA system fails, the following procedure will be used until the service is restored:

Users will be notified to forward flight request via alternate AFTN address as listed on the Quick Reference sheet, via FAX, or via phone. Please include your name and phone number with your flight request.

After 1430 UTC ZAN will compile a GRL.  Dispatchers should not expect to receive the GRL until 1530 UTC.

Any changes or additions after 1430 UTC must be made by phone (907-269-1104).

In this manual mode of TA, the TKL function to request additional or updated GRLs is not available.

NOTE: There will not be a GRL compiled for flights between 0700 and 1700 UTC.

QUICK REFERENCE SHEET FOR TRACK ADVISORY

## FOR ANCHORAGE CENTER

Normal ARINC address:        						ANCDPXA

Normal NADIN/AFTN address:              				PANCZOZD

Alternate NADIN/AFTN address if line is down: 			PAZAZQZX

Anchorage Center TRACK ADVISORY PHONE NUMBER:  	907-269-1104

Anchorage Center TMU FAX PHONE NUMBER:          		907-269-1343

DISPATCHERS SHOULD USE THE RELEASE TIMES, ALTITUDES AND ROUTINGS FROM THE GRL FOR FLIGHT PLANNING PURPOSES.

DISPATCHER MESSAGES:

TKF    - Two or more line flight request.

TKC    - Cancel flight request.

TKL    - Request a GRL.

TRACK ADVISORY DAILY PROCEDURES CHART

FOR ANCHORAGE CENTER

ANCHORAGE ARTCC TMU

0015-0430Z

TA Processor Accepts

Gateway Requests

0900-1430Z

TA Processor Accepts

Gateway Requests

0430Z TA Processor Builds Gateway Reservation List

1430Z TA Processor Builds Gateway Reservation List

0430-0900Z and

1430-0000Z

TA Processor Updates List Automatically

Automated requests are coordinated before approval.  Automated messages returned to users.

0000-0700Z and

0900-1700Z

Call Anchorage TMU at (907) 269-1104 for Gateway Reservation.

PLANNING PHASE

Gateway Requests

Two Separate Request Periods

##### AIRLINE

DISPATCH

OFFICES

Gateway Reservation List

NEGOTIATION PHASE

### Change Request

New Reservation

### Change Request

New Reservation