Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4:	Provide Status on CPWG/8 Actions

- OPTIONAL ROUTING FROM ORVIT TO JAPAN

- (Action Item #CP05-42)

(Presented by Continental Airlines on behalf of the International Air Transport Association)

SUMMARY This paper presents a proposal for the Russian Federation to study/implement a new optional routing from ORVIT to Japan.

- Introduction

1.1	Continental Airlines has used Polar 4 or the routing ORVIT.G494.LURET.B933.ODEPI.B223. LUMIN approximately 164 times for our New York to Tokyo flight.  With recent standardisation of the FIRs’ operating hours to 24/7, this allows the possibility of more use of this particular piece of airspace and possibly some new more efficient (not necessarily shorter) routes.

- Discussion

- The routes studied via statistical wind data were the Standard Route (STD), the shortest route (X03) and a variation between the two (X02). Further information is provided below in the spreadsheet and pictorial in paragraphs 2.3 and 2.4, respectively.

- It should be noted that the shortest route is not always the most efficient when the prevailing winds are taken into account.  The Equal Still Air Distance (ESAD) is where the influence of headwind/tailwinds negates the apparent shortness of a route versus a longer one with beneficial winds between the same two points in space.

- STD route =  ORVIT.G494.LURET.B933.ODEPI.B223.LUMIN

Or                  ORVIT.G494.LURET.G806.BANOT.B223.LUMIN

Test Route - X02 =  ORVIT..75N170E..65N153E..BANOT.B223.LUMIN

Test Route - X03 = ORVIT..77N180E..70N161E..65N155E..60N151E..ROMEM.B337.ANIMO

- Quarterly statistical winds were applied to all three routes for the results.

| Route | Distance | Date | Time | B/O | Time Saved | Fuel Saved | Emissions not produced | Winds |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STD | 2943.5Nm | JFM | 6.29 | 87.3 |  |  |  | M015 |
| STD |  | AMJ | 6.19 | 86.1 |  |  |  | M008 |
| STD |  | JAS | 6.18 | 86.0 |  |  |  | M007 |
| STD |  | OND | 6.30 | 87.7 |  |  |  | M017 |
| Test X02 | 2907.7Nm | JFM | 6.25 | 86.5 | 4mins | 800 lbs | 2 480 lbs | M017 |
| Test X02 |  | AMJ | 6.15 | 84.9 | 4mins | 1 200 lbs | 3 720 lbs | M008 |
| Test X02 |  | JAS | 6.14 | 85.0 | 4mins | 1 000 lbs | 3 100 lbs | M009 |
| Test X02 |  | OND | 6.27 | 87.1 | 3mins | 600 lbs | 1 860 lbs | M019 |
| Test X03 | 2903.3Nm | JFM | 6.27 | 87.1 |  |  |  | M020 |
| Test X03 |  | AMJ | 6.16 | 85.3 |  |  |  | M010 |
| Test X03 |  | JAS | 6.15 | 85.6 |  |  |  | M011 |
| Test X03 |  | OND | 6.29 | 87.8 |  |  |  | M023 |

- Depiction of routes

Test Route X03

Test Route X02

Standard Route

- Recommendation

3.1	The meeting is invited to note and review the content in this paper; and

  - The Russian Federation is invited to consider the proposal for a new optional routing from ORVIT to Japan.