Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4:   Provide Status on CPWG/8 Actions

Implementation of RNP 10 in the Anchorage Arctic Flight Information Region

(Action Item #03-30)

(Presented by the Federal Aviation Administration)

SUMMARY This paper requests endorsement of the implementation of 50 nautical miles (NM) lateral separation between aircraft authorized for Required Navigation Performance (RNP) 10 or RNP 4 operations in the Anchorage Arctic Flight Information Region (FIR).

- Introduction

  - As part of the discussions and updates from the Second Meeting of the Trans-Regional Airspace and Supporting ATM Systems Steering Group (TRASAS/2), the Fifth Meeting of the CPWG agreed to include TRASAS Conclusion 2/2 as part of its new CPWG Work Program. The CPWG identified a need to harmonize separation standards and improve efficiencies across the polar region.

- 1.2	The FAA, NAV CANADA and ISAVIA reported to the Seventh Meeting of the CPWG that they had formed a team to explore implementing 50 NM lateral separation between aircraft authorized for RNP 10 or RNP 4 operations in Anchorage Arctic FIR and Canadian Minimum Navigation Performance Specifications (CMNPS) airspace.

- 1.3	This working paper presents an update on the work conducted by the team, and requests endorsement of the implementation on 18 November 2010.

- Background / Discussion

  - The United States and Canada in coordination with the TRASAS are planning to implement a reduction of the lateral separation standard within the Control Areas (CTAs) of Anchorage Arctic FIR and Edmonton FIR/CTA. The lateral separation applied in the Anchorage Arctic FIR will be reduced from the existing 90 NM standard to 50 NM between aircraft authorized for RNP 10 or RNP 4 operations and will harmonize the application of 50 NM lateral separation across the Pacific CTA/FIRs, the Anchorage Arctic FIR and Edmonton FIR/CTA.

  - A Safety Risk Management (SRM) Panel composed of representatives from Anchorage Air Route Traffic Control Center, Edmonton Area Control Centre (ACC), NAV CANADA, FAA Western Service Center, FAA Flight Standards, and FAA Oceanic and Offshore Operations (AJE-32), and facilitated by CSSI Inc., was convened to ensure that hazards were identified and unacceptable risk was mitigated and accepted prior to any changes to the United States (US) National Airspace System (NAS).  Applying the principles of the SRM process, the Panel identified no high or medium risk hazards and only two low risk hazards.  It was determined by the SRM Panel that the reduction of lateral separation in the Anchorage Arctic FIR can be safely implemented.

  - A Safety Assessment conducted by the William J. Hughes Technical Center was completed in November 2009.  This safety assessment provides information that shows that RNP 10 will be a sufficient requirement for safe operations under the initial introduction of 50 NM lateral separation in the Arctic RNP 10 area. The 95% upper confidence estimate of lateral occupancy results were used to prepare the collision risk estimate, which should account for demand well into the future for this airspace. Under the assumptions outlined in the safety assessment and using the collision risk model parameter estimates, the estimate of lateral collision risk is 0.429 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation for the Arctic RNP 10 airspace when a lateral navigational performance of 7 NM/95% containment is used.

  - If it is assumed that aircraft population performance just meets RNP 10 performance (10 NM/95% containment), the resulting estimate of lateral collision risk is 0.528 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation.

  - Both of these values satisfy the ICAO-endorsed target level of safety value applicable to judging the safety of the lateral separation standard, 5 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation.  This assessment shows that given prevailing conditions and expected performance, the Arctic RNP 10 separation reduction will meet international guidelines for implementation.

  - The Safety Risk Management Document (SRMD) and supporting documents have been submitted and are awaiting approval.  NAV CANADA has completed their Hazard Identification Risk Assessment (HIRA) and submitted it to Transport Canada for approval.

  - The planned implementation date is 18 November 2010.

- Recommendation:

  - The meeting is invited to:

  - review the information contained in this working paper; and

  - endorse implementation of the project.