Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4: Provide Status on CPWG/8 Actions

TACTICAL RE-ROUTES PRIOR TO ENTERING RUSSIAN AIRSPACE

(Action Item CP06-12)

(Presented by United Airlines on behalf of International Air Transport Association)

SUMMARY This working paper presents information for the Group’s consideration in reference to Action item CP06-12 “Tactical Re-routes prior to entering Russian airspace.” A paper trial was conducted on April 7, 2010 between United Airlines, the Anchorage Center Traffic Management Unit, and the Russian Main Air Traffic Flow Management Center in Moscow. A power point presentation will be provided by the FAA Anchorage Center that further explains the actual test.

1.	Introduction

# At previous Cross Polar Working Group (CPWG) meetings, we have held discussions about the ability to tactically re-route prior to entering Russian airspace from the Anchorage Flight Information Region (FIR). The process can be somewhat complex due to various requirements by Russia, and the timeliness of transfer of control from Anchorage to the Russian Area Control Center (ACC) accepting the flight.

# At CPWG/6, action item CP06-12 was established. The status of this action item was updated at CPWG/8 with comments noting that the State ATM reported that they require Change Messages for tactical reroutes. It was agreed that Anchorage Air Route Traffic Control Center (ZAN) and State ATM would consult with airlines and consider possible methods to implement tactical re-routes, and a trial would be planned prior to CPWG/9 between United Airlines, Federal Air Transport Agency (FATA) and the Anchorage Traffic Management Unit (TMU).

- Discussion

# Tactical re-routes are conducted in many regions of the world with minimal complexity between the operator and the various air traffic service providers. Generally a re-route is either requested by the pilot when the filed routing and/or filed flight level are not available due to traffic. The Air Navigation Service Provider (ANSP) may also initiate a request to the operator, through the Airline Operations Center (AOC) or direct to the pilot, when a conflict of traffic will occur and a tactical re-route could eliminate that conflict.

# A ”simplistic” example would be a flight operating from Washington to Tokyo, within the Anchorage FIR, with a flight plan (FPL) to operate on North Pacific (NOPAC) Air Traffic Services (ATS) route R220. Due to traffic the flight may have to stay at an undesirable flight level, and therefore the pilot may request a tactical re-route to NOPAC ATS route R580. Generally there is coordination between the AOC and the flight to evaluate options and to amend the flight release. The actual re-route is then provided by the Anchorage Center controller and the FPL information is forwarded by ATS Interfacility Data Communication (AIDC) to Japan (in this example).

# There are several factors that make the same process more complex when a flight is approaching Russian airspace and the need for a tactical re-route exists:

# The transfer of control must be completed within 30 minutes of the Russian entry fix

# Russia requires a Change Message FPL to be sent, which is not consistent with global practices for operators

# Evaluation of the re-route is more essential for the operator due to onward routings within and or out of Russia to the west which, must be considered

# A paper test of a tactical re-route was proposed for April 7. 2010 for UAL803, a B777-200ER flight from Washington Dulles (KIAD) to Tokyo Narita (RJAA) airport.  The agreed procedures were:

# This would be a paper test only and no aircraft would be re-routed

# Approximately one hour prior to crossing ERNIK, United Airlines Flight Dispatch would simulate having a SATCOM discussion with UAL803 and a desire to make a tactical re-route to another Russian entry fix and ATS route

# United Dispatch would call the ZAN TMU and discuss route concerns and obtain their opinion of re-route options

# United Dispatch would send a SITA message to the MATFMC (MOWYWYA) with a “TEST ONLY” FPL of the new route, requesting approval for tactical re-route

# United Dispatch would make a direct telephone call to Duty Officer at MATFMC (7 495 601 0776) to confirm this proc3ess (SITA) had worked

# MATFMC would send a SITA response when approval was obtained (after checking with various ACCs) to United Dispatch

# United Dispatch would call ZAN TMU and confirm the approval from MATFMC and our desire to re-route to FRENK

# United Dispatch would advise UAL803 to accept the tactical reroute and the release would be amended

# A new FPL would be sent to PAZAZQ and MATFMC AFTN address and the appropriate Japan FIR addresses for UAL803

# The test was initiated at approximately 2300 UTC with the United Dispatcher contacting ZAN TMU with the “problem” flight UAL803. After some discussion, the re-route request to FRENK was agreed.

# 2.6	The Dispatcher sent a SITA message to MATFMC and followed this up with a direct phone call. The approval came back via SITA message 12 minutes after the initial “test re-route” message was sent by the dispatcher. The dispatcher called the ZAN TMU to confirm approval.

2.7	In summary, this process would have worked for an actual tactical re-route, although it is somewhat labor intensive and time consuming. It is believed that AIDC may in fact alleviate most of this process, and reduce the transfer of control time.

  - The process to tactically re-route flights into Russian airspace can be accomplished with some modification to the “test” we completed on April 7.

  - Our ultimate goal would be to simplify the procedure and eliminate most of the labor intensive process, however still pre-coordinating with the MATFMC are re-route approvals.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Working Paper and the power point presentation;

  - provide feedback and work towards the development of a workable process.