Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4:   Provide Status on CPWG/8 Actions

- REDUCTION OF RESTRICTIONS FOR AIRCRAFT FILING VIA PILUN OR LISKI

- (Action Item #CP08-05)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information regarding the reduction of Notice to Airmen (NOTAM) restrictions for aircraft filing into the Russian Trans East via either PILUN or LISKI.

- Introduction

  - The Anchorage Center (ZAN) airspace through which aircraft transit on their way to LISKI and PILUN is the border between reliable communication and surveillance, and limited communication with no surveillance.  In the Polar Region to the north of this area, ZAN has no VHF communications and spotty HF communications. This area has no radar surveillance.  Aircraft entering ZAN's airspace north of 74N exit ZAN's airspace at the same altitude so the need to issue control instructions is minimal.  To the south, in ZAN domestic airspace, VHF communication and radar surveillance at flight levels are the norm.  Control instructions are easily passed and relatively close separation can be safely maintained.  Aircraft transitioning to LISKI and PILUN must receive control instructions, specifically climbs and descents from feet to meters.  Failure to deliver these clearances results in operational deviations and possibly operational errors.  These essential clearances must be issued in an area in which passing these instructions is often cumbersome.

  - Creating a set of restrictions to ameliorate these concerns has proven to be a challenge.  Creating a northern limit at 141W for aircraft proceeding to LISKI or PILUN has met strong resistance from the airspace users.  Winds in the Canadian Arctic often dictate a more northerly 141W crossing point without which efficiencies are lost.  As of 1 April 2010 there were just a few NOTAMed restrictions in place for westbound traffic.  Aircraft filed over PILUN must cross 141W at or north of TAYTA and cross 157W at or north of 72N.  Aircraft filed over LISKI must cross 141W at or south of VOLOB and cross 157W at or south of 7030N.  NCA32 is closed.  Eastbound traffic using these fixes has the same 157W restriction as westbound traffic, but no 141W crossing restriction.

- Discussion

  - Some progress has been made to aid in achieving communications in this area.  ZAN put out a NOTAM making 141W reports mandatory for all aircraft crossing north of 72N, and added San Francisco ARINC frequencies as an option for HF communications.  Operators have emphasized to flight crews the need to make these reports.  ZAN and Edmonton Centre have amended their letter of agreement.  All westbound aircraft crossing 140W north of 70 30N are given a back up VHF frequency at 150W if contact on the primary frequency is unsuccessful.  The letter of agreement with Gander Radio has been modified to ensure aircraft forward a 141W estimate and report that position.  IFALPA’s London office has put out a notice to its member pilots reminding them of the need for timely 141W progresses.

  - Several events are underway that will help alleviate the problems of maintaining separation of LISKI and PILUN traffic, and reduce or eliminate the corresponding restrictions.  A safety risk management team has been compiling data to support implementing ZAN arctic RNP-10 airspace.  The change should be in place in late 2010 and will reduce the lateral separation minimum from 90 nautical miles (NM) to 50NM.  Magadan Area Control Centre (ACC) has offered to transition aircraft from feet to meters transitions in their airspace, thereby removing the necessity of control instructions being passed in ZAN airspace.  Negotiations are underway to hopefully have both of these improvements in place before the end of the year.

  - As these procedures are put in place, ZAN will conduct a thorough review of all of its NOTAMed restrictions.  For westbound traffic, ZAN hopes to reduce the 141W crossing restrictions, reduce or eliminate the 157W crossing restrictions, and reopen NCA32.  For eastbound traffic, ZAN plans to reduce the 157W crossing restriction.  Specific reductions will be delineated at the conclusion of negotiations with Magadan ACC, and upon the implementation of RNP-10 in the Arctic.

- Recommendation:

- The meeting is invited to note the information in this paper.