Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 6:	Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) issues

TRANSITION BETWEEN POLAR ROUTES WITHIN RUSSIA

(Presented by United Airlines on behalf of International Air Transport Association)

SUMMARY This working paper presents information for the Group’s consideration and is in reference to optimizing route capability within the polar region by establishing transitions between various polar routes.

1.	Introduction

# There are currently five polar routes within the Arctic region. Three of these routes run basically parallel to each other from the Anchorage/Magadan Flight Information Region (FIR) boundary to continental Russia.  This working paper proposes to add transitions between the various polar routes to enable optimization of winds and provide increased efficiency of operations.

# By adding transitions between polar routes, operators will gain significant flexibility in optimizing operations based on day to day winds, and more closely approach a User Preferred Route (UPR) capability within Russia.

# Operations between the North American Midwest/East Coast and Asia in both directions will benefit from these transitions.

- Discussion

# Operators are currently unable to plan UPRs over Russian airspace, and therefore the addition of many Air Traffic Service (ATS) routes within Russia will provide a “near UPR” environment for operations in this region.

  - Transitions joining the various polar routes will significantly improve flexibility within the polar region and provide efficiencies over current ATS routes.

# The following specific transitions shown in Appendix A are proposed for the consideration of the Federal Air Transportation Agency of Russia:

# 2.3.1    DEVID B480

- Transition route from DEVID to join G493 and G491 at GIKSI (N71 41.7 E128 54.0)

# 2.3.2    RAMEL G491

- Transition from RAMEL to TAKUN (G226)

- Transition from PETUL to RUTIN (G226)

- Transition from UNELI to HA (G226)

# 2.3.3    NIKIN G226

- Transition from NIKIN to UNELI (G491)

- Transition from TAKUN to TIGLA (G491)

- Transition from HA to TETKA (G491)

# 2.3.4    ORVIT G494

# Transition from ORVIT to TAKUN (G226)

# Transition from DILSA to RUTIN (G226)

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Working Paper;

  - provide suggestions and collaboration to consider the implementation, if possible, of the proposals within this Working paper.