| ## Request For Change form |  |  |  |  |  |  |  | RFC Nr: |  |  | 05-005 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. SUBJECT: |  |  |  |  |  |  |  |  |  |  |  |  |
| Request for specific chapter to highlight unique State (i.e. Military) aircraft requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. REASON FOR CHANGE: |  |  |  |  |  |  |  |  |  |  |  |  |
| State aircraft, i.e. military, have unique requirements insofar as using CPDLC. These requirements were never considered when the CPDLC message set was being developed, and so procedures have been developed utilizing a combination of existing CPDLC message elements as well as free text. There may be slight variations in procedures depending on the type of operations being conducted. Though this will be a start, this will promote harmonization in CPDLC procedures. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. DESCRIPTION OF PROPOSAL: [attach additional pages if necessary] |  |  |  |  |  |  |  |  |  |  |  |  |
| PROCEDURES FOR STATE AIRCRAFT 1 Introduction The data link and voice communication requirements for CNS/ATM are being defined by international, regional, and national civil aviation authorities and are based on use of commercial communication systems. There are currently no plans in the civil world to use MILSATCOM or other military communication systems for CNS/ATM because of the requirement for interoperability with civil aircraft and civil air traffic control facilities worldwide. In the Oceanic and Remote Regions, data link has seen increased use and will eventually replace voice as the primary means of communication. The military has unique requirements insofar as using CPDLC. These requirements were never considered when the CPDLC message set was being developed. Many air and maritime air forces have the capability to conduct air to air refueling (AAR) operations. Although detailed procedures are dependent on aircraft type, mode of employment and national requirements, there is sufficient commonality for standard procedures to be developed to enhance operational interoperability. Many of these air and maritime air forces are making the transition to aeronautical data links and the use of Controller Pilot Data Link Communications (CPDLC) and Automatic Dependent Surveillance (ADS). The procedures outlined below describe the procedures utilized by military aircraft in the attempt to promote harmonization in CPDLC procedures. These procedures have been developed utilizing a combination of existing CPDLC message elements as well as free text. To the maximum extent possible, data link capable aircraft should adhere to established message architecture and avoid extraneous free text. There may be slight variations in procedures depending on the type of operation being conducted 2 Aim The aim of this chapter is to provide a reference document covering procedures to be used in an aeronautical data link environment. This will: a) Provide guidance for air crews and air navigations service providers (ANSPs) in order to promote the effective AAR operations in an aeronautical data link environment. b) Lead to a better understanding of AAR procedures. c) Promote the development of harmonized mutual AAR procedures and terminology to be used in an aeronautical data link environment. 3 Common definitions, terms and phraseology AAR Area (AARA) A defined area encompassing both a racetrack shape AAR track and its protected airspace. AAR Bracket Designated segment of a route where AAR is planned. The bracket is defined by a refueling start point and stop point. AAR Exit Point The designated geographic point at which the refueling track terminates. AAR Time Planned elapsed time from ARCP to completion point. AAR Track A track designated for AAR. Abort Point A planned point along the receiver track at which the receiver must divert, if he is not in contact receiving fuel. Air Refueling Control Point (ARCP) The planned geographic point over which the receiver(s) arrive in the observation/pre-contact position with respect to the assigned tanker Air Refueling Control Time (ARCT) The planned time that the receiver and tanker will arrive over the ARCP. Air Refueling Initial Point (ARIP) The geographical point at which the receiver enters the refueling track (anchor), initiates radio contact with the tanker and begins manoeuvre to RV. Altitude Differential The difference between the receiver altitude and the tanker altitude. Altitude Reservation (ALTRV) An area of airspace reserved for AAR with the appropriate ATC authority. There are 2 types of ALTRV: moving and static. A moving ALTRV encompasses enroute activities and advances coincident with the mission progress. A static ALTRV consists of a defined geographic area, specific altitude(s) and time period(s). Anchor Point A defined reference point upon which an anchor refueling track is orientated. Anchor Refueling AAR performed as the tanker(s) maintain a prescribed pattern, which is anchored to a geographical point or fix. Approval Void Aircraft if Not Airborne (AVANA) The time period for which an ALTRV is valid. In most cases the AVANA is one hour after the last planned take-off, after which time the ALTRV is automatically cancelled. Base Altitude A reference altitude at which the lead aircraft of a tanker formation (or a single aircraft for individual AAR) will fly. End AAR A planned point or the actual position within the confines of the AAR track at which all AAR operations/requirements are complete. MARSA Military Assumes Responsibility for Separation of Aircraft - applies only to participating aircraft. Orbit Departure Time That time at which the tanker will depart the orbit point to affect the planned RV. Orbit Pattern The pattern flown by the tanker at the orbit point. Orbit Point A geographic point along the planned AAR track where the tanker will orbit. Rendezvous Procedure (RV) A procedure to join the receiver with the tanker. The purpose of a rendezvous (RV) procedure is to achieve close visual contact between the tanker and a receiver section or element. For the purpose of this Chapter, each RV procedure is written for one tanker. However, all procedures can be adopted when tankers are flying in Multi-Tanker - Echelon formation (usually in line astern) and in Cell formation. The RV is usually at the ARCP and at the ARCT. The type of RV utilized will be dictated by mission requirements and weather conditions. Rendezvous Initial Point (RVIP) A planned geographic point prior to the ARCP to which tankers and receivers time independently to effect an arrival at the RV control time. If the tanker/receiver is not already at its assigned RV FL/ altitude, it commences a climb/descent to that FL/altitude. This point may be a designated position established at the planning or briefing stage, or as directed by the tanker/GCI/AEW controlling the RV. RV Control Time (RVCT) A general term that applies to any control time to make good an RV between tanker/receiver at a specific point (ARCP, RVIP). RV Flight Level (FL)/Altitude The FL or altitude of the tanker during a RV procedure. Receivers are separated from the tanker by FL/altitude during the procedure; unless otherwise stated, the receiver is to be 1000 ft below the tanker. RV Point A designated point where tanker and receiver are planning to be joined in formation (ARCP, RVIP, etc). RV Track (Tanker Track) The track flown by the tanker during the RV procedure. Receiver(s) track to the RV Point is dependent on planned route and RV procedure. Airspace Reservations. Because of the large volume of airspace required, it is important to consult the relevant documents so that International Air Traffic procedures are strictly adhered to. In particular, some nations require AAR operations to be conducted in specific geographical areas. For flight safety considerations, it is normal to conduct AAR operations in reserved airspace. Such airspace can be divided into 2 broad categories: a) AARAs/Anchor Areas and AAR Tracks. AARAs/anchors areas and AAR tracks are areas of airspace established by the national authority for the conduct of routine AAR training and are normally activated by NOTAM. Bookings for this airspace are usually made through the designated national scheduling unit, who are also responsible for liaison with the appropriate ATC authority for activation of the airspace and issue of NOTAMs. If suitably positioned, AARAs/anchors areas or AAR tracks may be used for AAR deployments. Exercise/operational AARAs/anchors areas and AAR tracks are temporary areas established by NOTAM for the duration of the exercise or operation and may be either permanent or time restricted as dictated by the nature of the task. b) ALTRVs and Military Corridors. ALTRVs and military corridors are normally arranged with the appropriate national ATC authorities. For AAR purposes, moving ALTRVs are normally used to guarantee the required route and altitude(s) for an AAR supported deployment. Military corridors are activated by NOTAM and are essentially static ALTRVs. 4 Air Refueling 4.1 Air refueling is normally accomplished between 10,000 and 28,000 feet depending on receiver type, requiring both aircraft to descend for refueling. 4.2 Refueling tracks are numbered and depicted on charts in domestic airspace and a few depicted in oceanic airspace. Oceanic refueling may also be conducted on non-designated tracks with an Altitude Reservation (ALTRV). In both cases, the refueling procedure is part of the filed flight plan. The flight plan always includes time, requested altitude block, ARCP, ARIP, AREX and intermediate refueling track points. If the procedure is depicted, its designation (Arxxx) is sufficient to define the track. In the oceanic environment, a refueling pattern may be part of an existing ALTRV. 4.3 The type of RV utilized will be dictated by mission requirements and weather conditions. 5 Types of Rendezvous Procedures 5.1 A typical air refueling pattern is illustrated below. The light green track represents the tanker’s intended route to the ARCP (Air Refueling Control Point). The light blue track is the receiver’s intended route. Both aircraft file separate flight plans showing the specific aerial refueling locations. The dark blue track is the tanker’s orbit and rendezvous flight paths with the dark green track depicting the aerial refueling (AR) track. Three or more points can define the AR track. The Air Refueling Initial Point (ARIP) is the point where the receiver enters the AR track. The ARCP is the reference point for the holding pattern where the tanker awaits the receiver. The AR track is between the ARCP and the Air Refueling Exit Point (AREX). 5.2 RV Alpha (Anchor RV). This is a procedure directed by a radar control station, whether ground based, seaborne, or airborne (AEW). This procedure is a RV carried out under the control of a radar station on the ground, in the air or on-board ship. The RV Alpha is normally used to vector receivers to tankers operating on an AARA/anchor area but may be used as required in any situation. However, with the agreement of the tanker, the controller may give the tanker alterations of heading to effect a quicker join-up. The essential requirement for a RV Alpha is positive control by the radar station to bring the receiver to 1 nm astern and 1000 ft below the tanker. When visual with the tanker, the receiver calls ‘visual’ and is then cleared by the tanker to join (on the right unless directed otherwise by the tanker). The refueling anchor pattern is a left-hand racetrack with legs separated by 20 nm and a minimum leg of 50 nm. The location of the pattern is determined by the anchor point and the orientation of the inbound course. Single tankers or tanker formations/cells may be used in the anchor. Alternate Anchor Pattern. In the event that a radar control unit is not available to control an anchor RV, an alternative anchor pattern is to be flown. This pattern is shown below. 5.3 RV Bravo. This is a heading based procedure, which utilizes air to air equipment of both tanker and receiver. The tanker controls the procedure. The RV Bravo is a heading based procedure and is thus ideally suited for situations where accuracy of the navigation equipment of the tanker or receiver is in doubt or degraded. It has a further advantage in that it does not require a pre-briefed AAR track. As this procedure is heading rather than track based, it may not be suited to a busy ATC environment. 5.4 RV Charlie. This is a heading based procedure similar to the RV Bravo which allows receivers with an Airborne Intercept (AI) radar to control the procedure once positive AI radar contact is established. The RV Charlie is a heading based procedure and is thus ideally suited when accuracy of navigation equipment is in doubt or degraded. It does not require a pre-briefed AAR track. It requires the receiver to use an AI radar to complete the RV. As this procedure is heading rather than track based, it may not be suited to a busy ATC environment. 5.5 RV Echo. This procedure is intended for use in support of a combat air patrol (CAP). The RV Echo should be used in tactical situations where it is necessary to have available a tanker with which receivers can RV in a known area on an opportunity basis. 5.6 RV Foxtrot. This procedure is normally used when the tanker and receiver operate from the same base; accompanied/buddy climb/tailchase departures. The RV Foxtrot is normally used in VMC conditions when the tanker and receiver(s) are operating from the same airfield. Tanker and receiver take-offs occur within a few minutes of each other, which eliminates the fuel and time consuming racetracks of the other RV procedures. The collocated procedures have the added advantage that it is usually possible for the tanker to delay its take-off until assured of the receiver’s serviceability on start up. However, adverse climb out weather or ATC considerations may make these procedures impracticable. There are two methods of effecting a RV Foxtrot, the Accompanied Departure, Buddy Climb, and the Tailchase. 5.7 RV Golf (Enroute). This procedure facilitates join up on a common track to make good a scheduled time. The receivers may have departed either from the same or different bases. The RV Golf (En route) procedure is used when a join up is to be accomplished en route to an AAR track to join an ALTRV or an established military corridor. The tankers and receiver(s) navigate independently to arrive at the RVIP at a designated RV control time. It is advisable for the tanker to plan a 10 minute holding pattern prior to the RVIP to counter departure delays or receiver(s) arriving early. A common track length equivalent to 15 minutes flying time should be planned to allow for tanker descent to RV FL/altitude, visual acquisition and timing corrections. 15 minutes prior to the RV control time the tanker and receiver(s) are to confirm their altitude and timing. When established on the common track, the tanker crew is to use all available locating aids (EMCON state permitting) to gain visual contact between the tanker and the receiver. If EMCON 4 procedures are in force, the tasking instructions should include control times for both the RVIP and the ARCP 6 AAR CPDLC Messages 6.1 The procedures and CPDLC messages described below are based on the FANS-1/A message set. 6.2 Below is a listing of datalink exchanges that would occur between a FANS capable tanker and air traffic control. Pro-forma free text messages 671 and 67m have been created to support these operations. At approximately 10 minutes from the ARCP the pilots will make the following request: Message; Response; Message Intent. Pilot (Tanker); DL#25 REQUEST CLEARANCE. DL#67 TO DELAY FOR AIR REFUEL AT [position] UNTIL [time] DL#7 REQUEST BLOCK [altitude] TO [altitude]; The tanker is requesting a clearance to delay at the ARCP until the rendezvous with the receiver. [position] is the ARCP as filed in the tanker’s flight plan. [time] is the time the tanker expects to pass the ARCP and commence refueling along the refueling track. It is also the end of the delay time. The tanker is requesting the altitude block for air refueling.. Controller; UL#169 CLEARED TO DELAY AS REQUESTED If block is NOT immediately available UL#0 UNABLE. UL#166 DUE TO TRAFFIC; (optional) UL#9/10 EXPECT DESCENT AT [position/time]; or UL#7/8 EXPECT CLIMB AT [position/time]; and/or UL#130 REPORT PASSING [position]. If block is immediately available UL#31 CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#32 DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#30 MAINTAIN BLOCK [altitude] TO [altitude] (optional) UL#180 REPORT REACHING BLOCK [altitude] TO [altitude]; and/or UL#130 REPORT PASSING [position]; ROGER N/A (optional) ROGER WILCO (optional) ROGER; The tanker is cleared to delay at the ARCP as requested. AW note. This message will need to be improved. E.g. include area of operations etc. Also If the block clearance is not available, reject the request. An expectation could be issued or an instruction to report over the ARCP. If the block clearance is available, issue it. Additional instructions to report can be appended if required. When the Tanker crosses the ARCP to begin the delay: Message; Response; Message Intent. Pilot (Tanker); (optional) DL# 31 PASSING POSITION [position]; DL#31 notifies ATC that the tanker has crossed the ARCP and has entered the air refueling orbit for the delay. This message will only be sent if ATC has instructed the aircraft to report passing the ARCP.. Controller; If block was NOT previously available UL#31 CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#32 DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#30 MAINTAIN BLOCK [altitude] TO [altitude].; WILCO; Issue the block clearance for the tanker if it was not issued previously (traffic permitting of course). Receiver approaching the ARIP: Message; Response; Message Intent. Pilot (Tanker); DL#67 ACCEPT MARSA WITH [receiver callsign]; The tanker is accepting MARSA separation procedures with the receiver. Pilot (Receiver); DL#7 REQUEST BLOCK [altitude] TO [altitude]; The receiver is requesting an altitude change to conduct refueling. This request should be made prior to the receiver entering the ARIP – ARCP track. Controller (to Receiver); UL#31 CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#32 DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude]; or UL#30 MAINTAIN BLOCK [altitude] TO [altitude]; and UL#169 CLEARED FOR REFUELING; WILCO; The receiver is cleared to operate in the block required for refueling. Note. If no MARSA statement has been received, it must be confirmed prior to issuing a clearance that causes a loss of separation between the two aircraft. The received is cleared to refuel. Controller (to Tanker); UL#169 CLEARED FOR REFUELING; ROGER; The tanker is cleared to refuel. When the tanker is commencing the rendezvous with the receiver the tanker pilot sends the following: Message; Response. Pilot Tanker and Receiver; DL#11 AT [position] REQUEST CLIMB TO [altitude]; or DL#12 AT [position] REQUEST DESCENT TO [altitude]; [position] is the EXIT point. [altitude] is the requested level for each aircraft after refueling is complete. Approaching the end of refueling Message; Response; Message Intent. Pilot (Tanker); DL# 67 EXPECT END OF REFUEL AT [time]; The tanker pilot is providing notification that the end of refueling is imminent. Controller (to Tanker); UL#164 WHEN READY; and UL#19 MAINTAIN [altitude1]; and UL#129 REPORT LEVEL [altitude1]; WILCO. Controller (to Receiver); UL#164 WHEN READY; and UL#19 MAINTAIN [altitude2]; and UL#129 REPORT LEVEL [altitude2]; WILCO. Pilot (Tanker); DL#37 LEVEL [altitude1]; The tanker is maintaining the assigned level. Pilot (Receiver); DL#37 LEVEL [altitude2]; The received is maintaining the assigned level. Controller (to Tanker); UL#169 CANCEL MARSA; ROGER; MARSA procedures have been cancelled. 7 “Military Assumes Responsibility for Separation of Aircraft” (MARSA) 7.1 Prior to commencing aerial refueling or maneuvers with receiver aircraft, the tanker will notify ATC that the “Military Assumes Responsibility for Separation of Aircraft” (MARSA). The term “MARSA” is used in the call to ATC to notify ATC that the aircraft are accepting the responsibility for their actions within the AR Track and the tanker is the lead of the formation. ATC controls all other traffic to preclude conflicts between civil and military traffic involved in the AR while at the same time still controlling the tanker and receiver. The actual refueling commences at the ARCP and continues as the aircraft proceed down the refueling track. Normally, the refueling is completed prior to the aircraft reaching the AREX point. At AREX, both aircraft must receive ATC clearances to continue on their filed routing. MARSA Message; Response; Message Intent. Pilot; DL# 67 ACCEPT MARSA WITH [callsign(s) of other jets]; Stating acceptance of MARSA with other aircraft that will be in formation flight. Controller; UL#169 MARSA APPROVED; ROGER; MARSA has been approved. Jet or formation is joining an ALTRV Message; Response; Message Intent. Pilot; DL# 67 JOINING ALTRV [xxxx] AT [XXXXX]; [XXXXX] can be either a point (i.e. JOINING ALTRV CW413 AT HEMLO) or a time along the route of flight (i.e. JOINING CW413 AT 1530Z). Controller; UL#169 ROGER JOINING ALTRV; ROGER. MARSA Terminated Message; Response; Message Intent. Pilot; DL#67 MARSA TERMINATED WITH [xxxxx]; [xxxxx] callsign of aircraft leaving formation and continuing with ATC services.. Controller; UL#169 ROGER MARSA TERMINATED [WITH xxxxx if required]; ROGER. 8 FORMATION FLIGHTS. 8.1 Formation flying was born as a means of mutual support and protection in times of aerial warfare. It was quickly discovered that multiple aircraft working together achieve far greater results while also reducing risk to all formation members. Flying large numbers of aircraft in formation meant organization was necessary for safety and efficiency. With time and experience, standards and conventions were developed for formation flying. Some of these conventions vary depending on the country of origin, the organization, or the type of flying and is a routine element of day-to-day flight operations. A standard formation is usually one in which a proximity of no more than 1 mile laterally or longitudinally and within 100 feet vertically from the flight leader is maintained by each wingman. Non-standard formations are those operating under conditions other than standard formation dimensions that the flight leader has requested and air traffic control (ATC) has approved, or when operating within an authorized altitude reservation (ALTRV). 8.2 All data link equipped jets perform AFN logons at the correct time (typically 15-45 minutes prior to entering data link airspace. 8.3 Both lead and the wingman complete an AFN log-on at the normal time, but only lead makes CPDLC position reports (the same CPDLC position report sent out when single ship). 8.4 Use CPDLC standard messages for altitude requests, routing requests (if different from what was filed), and speed or ETA requests with ATC to effect any en-route changes. 8.5 In the event a formation wants to break-up the formation or depart an ALTRV the following data link procedures will be used. 8.5.1 Prior to formation break-up, wingmen desiring to break off of the formation will coordinate their departure a minimum of ten (10) minutes with appropriate requests. Formation Break-up or departure from ALTRV Message; Response; Message Intent. Pilot; DL# 67 REQUEST TO DEPART ALTRV [xxxx] AT [XXXXX]; Aircrew is providing notification to break-up from formation or depart from ALTRV. Controller; UL#169 ROGER REQUEST; ROGER. Pilot; DL#22 REQUEST DIRECT TO [position] or DL#24 REQUEST [route clearance]; Send DL#22 for direct to the desired point on the filed flight plan, or send DL#24 if not going to a point on the filed flight plan.. Controller; DL#76/77AT [time] PROCEED DIRECT TO [position] / AT [position] PROCEED DIRECT TO [position] or AT [position] CLEARED [route clearance]; WILCO; ATC responds with the proper UL based on the request.. Pilot/ATC; Send appropriate request for altitude desired. ATC responds with proper uplink. 9 ADS REPORTS. 9.1 All jets will leave ADS on if so equipped. As ADS contracts may be established by ATC. ATC will make the ADS contract with the lead aircraft as identified in the filed flight plan. |
| 4. REFERENCE(S): |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. PERSON INITIATING: |  |  | Allan Storm |  |  |  | DATE: |  | 1 Jun 2005 |  |  |  |
| ORGANISATION: |  |  | Air Force Flight Standards Agency, Civil/Military Aviation Issues |  |  |  |  |  |  |  |  |  |
| TEL/FAX/EMAIL: |  |  | +1-240-857-2146 |  |  |  |  |  |  |  |  |  |
| 6. CONSULTATION |  |  | RESPONSE DUE BY DATE: |  |  |  |  |  |  |  |  |  |
|  | Organisation |  | Name |  | Agree/Disagree |  |  |  |  | Date |  |  |
| 7. ACTION REQUIRED: |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. FOM EDITOR |  |  |  |  |  | DATE REC’D: |  |  |  |  |  |  |
| 9. FEEDBACK PASSED |  |  |  |  |  | DATE: |  |  |  |  |  |  |