THE TENTH MEETING OF THE

FANS INTEROPERABILITY TEAM

(FIT/10)

THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-12 July 2005)

CPDLC Weather Reporting

(Presented by the Federal Aviation Administration)

SUMMARY

This paper presents the results of a five week study of weather reporting by aircraft within the Oakland Flight Information Region (FIR) utilizing Controller Pilot Data Link Communications (CPDLC).

- Background

  - Data for this study was derived from system analysis recordings of the Oceanic Datalink System (ODL) over a 35 day period (16 May 2005 through 19 June 2005).

  - Two groups of message types were evaluated; aircraft movement requests with the message element dM65 (DUE TO WEATHER) appended, and messages containing pilot reported weather.

    - The message types in the first group included message elements, dM7 (REQUEST BLOCK [altitude] TO [altitude]), dM9 (REQUEST CLIMB TO [altitude]), and dM10 (REQUEST DESCENT TO [altitude]).

    - The message types included in the second group were dM48 (POSITION REPORT) and dM67 (freetext).

  - During the evaluation period less than 10% of the requests for aircraft movement due to weather conditions included a description of the weather.

| DUE WX Requests without WX Report | 601 |
| --- | --- |
| DUE WX Requests with Freetext WX Report | 72 |

  - Slightly more than 8/10ths of 1 percent of the CPDLC position reports (POS) received during the evaluation period contained weather information other than wind data.

| CPDLC Position Reports | 34102 |
| --- | --- |
| CPDLC PR with Weather Report | 284 |
| CPDLC PR with Turbulence Moderate or Greater | 39 |

- Discussion

  - The vast majority of requests for aircraft movement due to weather do not include a description of that weather.

  - Weather appears to be under reported in the datalink environment.

  - Freetext reports are unstructured and do not lend themselves to automated processing.

    - Inconsistent use of contractions - Light, L, LT, LGT, LGHT, LITE.

    - Missing or incomplete information on the area where the weather was encountered.

    - Use of Slang - "RIDE IS BUMPY", "BAD RIDE", "RUFF RIDE", "BUCKIN RIDE".

    - Humor - "SMOOOOOTH....SO FAR", "DARK.. VERY DARK... BUT SMOOTH....”

  - The one mops element that does make provisions for weather reporting (dM48 POSITION REPORT) will not be used in FIRs where ADS waypoint reporting is authorized.

- Recommendation

  - The meeting is invited to consider the contents of the working paper and the following recommendations.

  - Operators are encouraged to remind pilots of the need for weather reporting

  - The development of a purpose built downlink message element for weather reporting. This message should include fields for the reporting of aircraft position, altitude, wind, temperature, turbulence, and icing.

  - Consider canvassing other FANS groups to see if there is interest in forming a Task Force to recommend additions and/or deletions to the RTCA DO-258 CPDLC message set.