THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

THE TENTH MEETING OF THE FANS INTEROPERABILITY TEAM

(FIT/10)

(Tokyo, Japan, 11-15 July 2005)

The reports of compulsory reporting point using CPDLC

(Presented by Tokyo ACC)

Summary This paper is to confirm the items of compulsory reporting points using CPDLC.

- Background

  - Some aircraft reportedly downlink this kind of CPDLC waypoint messages containing only current flight level, without the flight level of the compulsory reporting points. The CPDLC waypoint report messages without the flight level over waypoint are not processed at Oceanic Air Traffic control Data Processing system (ODP3) in Tokyo Area Control Center (Tokyo ACC) as well as controller could not handle it as a sufficient waypoint reporting.

【example of CPDLC message lacking of flight level over waypoint 】

POSITION REPORT 2944.4N14313.8E 1349 FL311 SAGOP 1425 TEGOD 2100 -44 C 262 / 125 M.85 NOGAK 1347

  - When the above-mentioned CPDLC waypoint messages are downlinked, a controller at oceanic sector in Tokyo ACC requests the aircraft to send the correct message or the flight level over the compulsory reporting point.

- Procedures

  - According to ICAO Annex2, Section 3.6.3.1, position reports a controlled flight shall report to the appropriate air traffic services unit, as soon as possible, the time and level of passing each designated compulsory reporting point, together with any other required information.

  - According to ICAO Doc9694 Ⅳ-Chapter3 Appendix B. CPDLC message data glossary, Position report, the constituent parts of position report are as follows;

Position report. Uses the following data necessary to

provide an aircraft position report as follows:

－ Position current;

－ Time at position current;

－ Level;

－ Time ETA at fix next (optional);

－ Fix next plus one(optional);

－ Time ETA destination (optional);

－ Remaining fuel (optional);

－ Temperature (optional);

－ Winds (optional);

－ Turbulence (optional);

－ Icing (optional);

－ Humidity (optional);

－ Speed (optional);

－ Speed ground(optional);

－ Vertical change (optional);

－ Track angle (optional);

－ True heading (optional);

－ Distance to fix next (optional);

－ Reported waypoint position (optional);

－ Reported waypoint time (optional); and

－ Reported waypoint level (optional).

Reported waypoint level.  The level of the waypoint for which the report is being made.

2.3.1  The mandatory reporting items in the constituent parts of position report; “Position current, Time at position current, Level “ deems to be sufficient as current position reports when controller uplinks UL#132 “confirm position” in order to verify aircraft’s present position.

2.3.2  In accordance with ATC requirements as compulsory waypoint reporting, appropriate optional items in the above-described Position report should be selected. “Reported waypoint position, Reported waypoint time and Reported waypoint level” are required as the compulsory reporting items of waypoints reporting. Based on ATC regulation, ODP3 in Tokyo ACC is designed to process CPDLC waypoints messages contains these three items.

- Observations

  - CPDLC waypoint report messages lacking of the flight level over compulsory reporting points would increase the workload of not only controller but also flight crew due to exchanging further CPDLC messages to confirm flight level over waypoints.

  - In the case of aircraft changing their altitudes just over the compulsory reporting point, the flight level over compulsory reporting point may not be coincident with the current level in CPDLC waypoint report.

- Discussion

4.1 Under the ATC procedure, the current level in CPDLC messages will not be able to regard as the flight level over the compulsory reporting point.

- Recommendation

5.1　Operators are requested to review the regulations regarding position reporting and downlink CPDLC waypoints reporting contains the necessary items for the compulsory reporting point.