THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/22)

THE NINTH MEETING OF THE

FANS INTEROPERABILITY TEAM

(FIT/9)

(Los Angeles, USA, 24 - 28 January 2005)

Validation of Applicability of ADS report to Surveillance

(Presented by JCAB CRA)

Summary This paper prepares a discussion material for the applicability of ADS to surveillance, in place of CPDLC position report.

- Introduction

1.1       Tokyo ACC has planned to replace CPDLC Position Report (hereinafter referred to as CPDLC-PR) with ADS Waypoint Change Event Report (hereinafter referred to as ADS-WR) as the primary means of position reporting associated with aircraft equipped with ATS data link capability flying in the oceanic airspace within the Tokyo FIR.

1.2        As both of the CPDLC-PR and ADS-WR are thought to be produced almost as soon as FMS detects the aircraft passing a Waypoint, they are quite unlikely to show a considerable disparity in their contents and in their frequency of occurrence. There have been some comments, however, on the need to confirm no problem on the technical and operational aspects of operating ATC services based on ADS-WR without pilot interaction (or pilot interaction is not possible) and ceasing the use of position reporting with CPDLC-PR that is sent after a pilot checking.

1.3        The CRASA-J decided to investigate the difference between the CPDLC-PR and ADS-WR for use as the source of position information in the official operation.

- Surveyed item

2.1        To analyze the difference of frequency of occurrence between the CPDLC-PR and ADS-WR, and determine how the difference impacts on the ATC operation.

2.2        To investigate issues arising from operating ATC services based on ADS-WR and ceasing the use of position reporting with CPDLC-PR as well.

- Investigation method

  - Surveyed data

To perform the statistic analysis of the position information, all the CPDLC-PR and ADS-WR were collected from the recorded ATC system log for the Tokyo FIR oceanic airspace during April 1, 2004  0000-2359(JST).

3.2         Explanation of the CPDLC-PR and ADS-WR

The content of each PR is described as follows:

CPDLC DATA

IMI        : AT1

REG        : .JA001G

Time Stamp : 03:00.03

MIN        : 16

MRN        : -

MSG TYPE   : Single

Element No : 048  MSG : POSITION REPORT 3203.5N15956E 03:00 FL340 3616N155E 03:49 CALMA 06:08 -46C 263/94 M0.84 32N160E 02:59 FL340

Next waypoint ETA

Next waypoint

Current waypoint

Current waypoint ATO

ADS DATA

WayPoint Event Report

Basic ADS           3200N 15959E +34012ft HH:59:22

FOM  = 7

Pred.Route(Next  )  3615N 15459E +37996ft Basic ADS Time＋00:49:28

(Next+1)  3937N 15028E +37996ft

Current waypoint

Current waypoint ATO

Next waypoint

Next waypoint ETA information

3.3      Steps of the investigation

To investigate the difference between the collected CPDLC-PR and ADS-WR, the following steps were taken:

- To investigate the difference between the number of CPDLC-PR and ADS-WR,

- To compare CPDLC-PR and ADS-WR in view of the position information reported are identical,

- When the position information reported is different between the CPDLC-PR and ADS-WR, investigate the causes of the difference and classify them accordingly,

- To investigate the ratio of identical position information acquired by the ADS-WR to position information acquired by the CPDLC-PR.

4.         Investigation result

  - Comparison of the number of messages

The number of CPDLC-PR and ADS-WR reported is described in the Table-1. Note that time and a half ADS-WR were down linked in comparison with CPDLC-PR. Little difference exists between the ratios after deducting the number of re-sent messages from each of the CPDLC-PR and ADS-WR.

Table-1  The number of CPDLC-PR and ADS-WR

|  | CPDLC-PR | ADS-WR | Total |
| --- | --- | --- | --- |
| Total number of messages (Ratio between CPDLC-PR and ADS-WR) | 386 (39.1%) | 601 (60.9%) | 987 |
| Re-sent messages | 12 | 1 | 13 |
| Total number of messages after deducting the number of re-sent messages. (Ratio between CPDLC-PR and ADS-WR) | 374 (38.4%) | 600 (61.6%) | 974 |

4.2        Comparison of the reported waypoint information

Investigation was conducted about the 374messages of CPDLC-PR and 600 messages of ADS-WR in view of the waypoint information reported.

The Table-2 describes the number and ratio of CPDLC-PR/ADS-WR reporting identical waypoint information, CPDLC-PR/ADS-WR reporting un-identical waypoint information (i.e. the number of waypoint information reported solely by CPDLC-PR, or the number of waypoint information reported solely by ADS-WR) and incorrect content messages. According to the table, 25.9% of the CPDLC-PR did not have corresponding ADS-WR, and 53.7% of the ADS-WR did not have corresponding CPDLC-PR.

Table-2 Ratio of identical/un-identical Waypoint acquired between CPDLC-PR/ADS-WR

|  | CPDLC-PR | ADS-WR |
| --- | --- | --- |
| Identical Waypoint (Ratio) | 276 (73.8%) | 276 (46.0%) |
| Un-identical Waypoint (Ratio) | 97 (25.9%) | 322 (53.7%) |
| Incorrect messages (Ratio) | 1 (0.3%) | 2 (0.3%) |

4.3        The causes of reporting “un-identical” Waypoint information

4.3.1      Waypoint information reported solely by CPDLC-PR

The 97 messages of CPDLC-PR, which did not have corresponding ADS-WR, were found to be classified in the following categories described in Table-3.

Table-3 Waypoint information reported solely by CPDLC-PR

| First position report after establishing a CPDLC connection | 75 (77.3%) |
| --- | --- |
| CPDLC-PR from the aircraft to which an ADS contract unsuccessful | 15 (15.5%) |
| Other | 7 (7.2%) |

Table-3 shows 77.3% of the Waypoint information reported solely by CPDLC-PR were the first position report to be down linked after the establishment of a CPDLC connection, according to the procedure specified in the FANA-/A operations Manual. (For this timing there is no ADS-WR to be down linked.)

And the 15.5% of the CPDLC-PR shown on the table, were from the aircraft to which ADS contract were unsu8cessful.

    - Waypoint information reported solely by ADS-WR

Investigating the characteristics of the 322 messages of ADS-WR, which had no corresponding CPDL-PR, we learned nearly all of them were position reports of non-ATC waypoints. Information relating to these waypoints is not of interest to ATC procedure at present. (Though it is used in the ATC system to improve its accuracy to locate aircraft position.)

4.4        The ratio of acquired ADS-WRs to necessary CPDLC-PRs to be down linked

First position reports and re-sent messages, which were sent via CPDLC-PR, were not to be acquired via ADS-WR. Subtracting these 87 messages from the total number of CPDLC-PRs (386), we learn the number of CPDLC-PRs to be considered in this survey was 299, to which 276 ADS-WRs were acquired covering 92.3% of the position information to be conveyed via CPDLC-PR.

Table-4 Ratio of acquired ADS-WRs to necessary CPDLC-PRs.

| Number of CPDLC-PRs left from deducting the number of First position reports and resent messages | 299 |
| --- | --- |
| Ratio of acquired ADS-WRs to necessary CPDLC-PRs | 92.3% |

5.        Discussion

  - Difference in the frequency of occurrence between CPDLC-PRs and ADS-WRs

The ratio of acquired ADS-WRs was 92.3% to the necessary number of CPDLC-PRs. To the remainder i.e. 7.7% of the necessary CPDLC-PRs, we could not acquire corresponding ADS-WRs.

5% of the 7,7% were CPDLC-PRs sent from aircraft, to which ADS contracts were unsuccessful.

In order for ATSUs to cope with the situation when an unsuccessful ADS contract is detected, they should prepare such procedures as “depending controller’s judgment retry the ADS contract request or request to down link CPDLC-PRs.”

ATC system has a function to display a warning message on the ATC display, when it has not received a position report within a specified time limit from an aircraft passing over ATC waypoints. Triggered by the warning message, it is thought to be necessary for controllers to inquire of the pilot as to the ATC waypoint.

5.2        Things to be considered in ATC operations using ADS-WR as the primary means of surveillance

As far as we investigated ADS-WRs reported at ATC waypoints, 30% of the Next waypoint data were non-ATC waypoints. According to the current ATC procedure, controllers are required to check the Next ATC waypoint ETA  of the ADS-WRs down linked at ATC waypoints. Therefore, when CPDLC-PRs are dispensed with, by using ADS-WRs as the primary means of surveillance, 30% of the ADS-WRs need complementing in view of the Next ATC waypoint ETA data, by means of CPDLC triggered by controllers or ATC systems (automatically).

6.        Conclusion

This paper reports one of the necessary verification steps to be taken in evaluating the use of ADS in ATC operations.

Further study and verification activities in various aspects are required in order to extract technical/operational issues to be considered and devise and implement appropriate measures.