THE TWENTY-SECOND MEETING OF

THE INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/22)

THE NINETH MEETING OF

THE FANS INTEROPERABILITY TEAM

(FIT/9)

(Los Angeles, USA, 24 - 28 January 2005)

ADS Problems That Affect ATC Services With Distance Based Separation

(Presented by JCAB CRA)

Summary This paper provides the meeting with the information on the ADS related problems that are considered to affect the oceanic ATC operation with the distance based separation standard.

1.  Introduction

1.1	At FIT/8 meeting, the Co-chair asked if specific problems, which are considered to affect the application of distance-based separation minima using ADS/CPDLC, have been received or identified.  The meeting felt that there were no such problems at this stage; however, the JCAB CRA agreed to study this and report to the next meeting.

1.2	The application of distance based separation in the oceanic ATC services requires ADS for surveillance and CPDLC for the direct communication between pilots and controllers. In this point of view, it is the minimum requirement for surveillance that the appropriate ADS reports from aircraft are presented timely at the controllers’ workstation.

1.3	In the course of processing the Problem Reports sent from the end-users, especially Tokyo ACC, some ADS related problems were recognized to influence the oceanic ATC operation with the distance based separation standard.

2.  Requirements to use ADS for surveillance

  - The requirements to use ADS reports for the means of surveillance are as follows:

- Reliable aircraft position information available with Basic and Prediction data

- Timeliness of the aircraft positions updated and displayed on ATC screen

3.  Problem Reports affecting distance based ATC operation

3.1	The JCAB CRA reviewed the Problem Reports registered in the database to pick out the events that were estimated to impact upon the use of distance based separation standard.

3.1.1	The following events are caused by the incorrect prediction data in the ADS report.

[EVENT -1]

Despite of continuing deviation from the planned route, the flight path on ATC screen always proceeding back to a waypoint of the flight plan, and incorrect aircraft positions are updated on ATC screen. In this situation, the prediction data in ADS report depends on the flight planned waypoint, regardless the aircraft position and course actually flying.

[EVENT-2]

This event always occurs when an aircraft deviates from the planned route with a certain magnitude of miles exceeding the value set at the avionics. (21 NM for Boeing and 7 NM for Airbus aircraft)  In this case, though the actual flight path is forwarding to the next waypoint, the aircraft is displayed on ATC screen flying back to the waypoint that has already passed.

Note: When the aircraft sets ‘Off Set’ on FMC for deviation, this event does not occur.

[Possible impact on ATC operations]

The aircraft position and flight path are unreliable and should not be used.

[Countermeasures]

a. ATC operational procedures:

Controllers should be educated to recognize this situation and to disregard the aircraft position data during the aircraft deviating from the planned route.

b. ATC Systems:

A review of software upgrade on ATC systems is recommended.

    - Missed or delayed ADS report

The following events are caused by missing or delay of ADS report to arrive at ATC systems.

[EVENT-3]

The delivery delay or missing of messages could be occurred with the following situation.

Case-1: Missed ADS report caused by unscheduled network system stoppage

Case-2: Missed ADS report caused by unstable satellite communication

Case-3: Delay or missing of message caused by the particular onboard communication unit with which the aircraft is unable to communicate via satellite for hours

[Possible impact on ATC operations]

The aircraft which ADS report and/or CPDLC communication are not available will be determined to be an unqualified aircraft for applying distance based separation. Further, if a malfunction of ATS datalink network continues for a long period, the impact on ATC operations is notable and the alternative measures should be taken.

[Countermeasures]

For the individual aircraft with missing ADS/CPDLC, the contingency procedures for such aircraft should be applied, until such time the ATC established a conventional time separation instead.

For Case-3, the change to airborne system is expected.

[EVENT-4]

Capricious ADS reports cause missing, delayed delivery, incomplete contents or multiple forms of message, and, on this occasion, the controllers should take action to obtain the correct aircraft positions by other means.

Case-1: Unstable communication, when aircraft flies near the fringe of VHF coverage, causes

the missing of aircraft position gained by ADS reports.

Case-2: When the ADS message with incomplete contents is delivered to ATSU, it is rejected by ATC system.

Case-3: Delayed data processing in avionics causes ADS report containing unreasonable data.

Case-4: When an unreasonable multiple message with ‘ACK + ADS report + NAK’ in a report

is downlinked, ATC system rejects it.

[Possible impact on ATC operation]

The accuracy of aircraft position is downgraded or missed, until the correct report received by the ATC system.

[Countermeasures]

The ATC operational procedures responding to these events are required to develop, such as the procedure to uplink ADS Demand Request for obtaining the position.

4.  Action by the meeting

The meeting is requested to note the information in this paper.