| ## Request For Change form | RFC Nr: | 05-001 |
| --- | --- | --- |

To be used whenever requesting a change to any part of FOM.  This form may be photocopied as required.

| 1. SUBJECT: |  | Planned Airborne Re-Route Procedure – DARP (Datalink Aircraft) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. REASON FOR CHANGE: |  |  |  | Action Item from FIT/11 meeting Fiji (Feb 04) |  |  |  |  |  |
| This proposal is a result of airborne trials with NZZO/KZOA and Air New Zealand (B747) and simulated ground trials with NZZO and Air New Zealand (B747). The objective is to provide a generic process that both the Pilot and ATC will follow, without manual route modification by the Pilot or ATC. |  |  |  |  |  |  |  |  |  |
| 3. DESCRIPTION OF PROPOSAL: [attach additional pages if necessary] |  |  |  |  |  |  |  |  |  |
| PLANNED AIRBORNE RE-ROUTE PROCEDURE – DARP (Datalink Aircraft) A planned airborne re-route will occur periodically on long haul Pacific routes when a new forecast is issued after departure, indicating that significant time and/or fuel savings can be made. The following procedures apply when aircraft request a planned re-route while en-route. Note:(Currently restricted to routes transiting directly from/to Auckland / Oakland Oceanic Airspace.) - AOC will plan the re-route and uplink the route to the aircraft, commencing from the waypoint on the current route, ahead of the Aircraft and finishing at destination. - Flight crew will Downlink the unedited route request to the ATSU. - ATC (1) receives the downlink re-route request (message #24) “REQUEST Departure Airport:xxxx Destination Airport:xxxx (fix)(fix)(fix) etc” - The ground system will “auto populate” the uplink reply - ATC (1) will uplink route clearance (message #80) “CLEARED Departure Airport:xxxx Destination Airport:xxxx (fix)(fix)(fix) etc” Note :( When the new route is entered into the FMC as the Active Route, the aircraft will track direct to the first fix.) - ATC (1) will transmit the new route to ATC (2) via ABI messaging Or “UNABLE” due to traffic Where conflicting traffic prevents route clearance at the current flight level. The following operational requirements apply: - The requested route must commence from the waypoint on the current route ahead of the aircraft, and continue to destination. - The re-route request must be made at least 10min before that waypoint, to allow processing time. - The re-route request must not be made within 60min of the common FIR boundary to allow electronic route data transfer ATC(1) to ATC(2). [ABI messaging]. - The re-route request may be made, to the new Data Authority, immediately after crossing the common FIR boundary |  |  |  |  |  |  |  |  |  |
| 4. REFERENCE(S): |  |  |  |  |  |  |  |  |  |
| 5. PERSON INITIATING: |  |  | Graham Secker / Mark Shepherd |  |  |  | DATE: | 8 Dec 2004 |  |
| ORGANISATION: |  |  | Airways NZ / Air New Zealand |  |  |  |  |  |  |
| TEL/FAX/EMAIL: |  |  | seckerg@airways.co.nz / mark.shepherd@airnz.co.nz |  |  |  |  |  |  |
| 6. CONSULTATION |  |  | RESPONSE DUE BY DATE: |  |  |  |  |  |  |
|  | Organization |  | Name |  | Agree/Disagree |  |  |  | Date |
|  | FAA Qantas Tahiti Air Services Australia AFL SASL Air New Zealand Airways New Zealand |  | David Maynard Graham Rennie Jean-Pierre Carle Richard Stevens, Steve Kemp, Geoff Ross, Adam Watkin Jotame Ratabua Jovilisi Vatu Mark Shepherd Mark Goodall, Geoff DeBazin, Paul Radford |  |  |  |  |  |  |
| 7. ACTION REQUIRED: |  |  |  |  |  |  |  |  |  |
| 8. FOM EDITOR |  |  |  |  |  | DATE REC’D: |  |  |  |
| 9. FEEDBACK PASSED |  |  |  |  |  | DATE: |  |  |  |