:The Thirty-Fifth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/35)

The Twenty-Second Meeting of the IPACG FANS Interoperability Team (IPACG FIT/22)

(Hokkaido, Japan, 8 Nov 2011)

Report of the FIT

1.	Opening of the Meeting

1.1	The Twenty-second Meeting of the Informal Pacific Air Traffic Control Coordinating Group (IPACG) FANS Interoperability Team (FIT) was held in Hokkaido, Japan, 8 November 2011, in conjunction with the thirty-fifth meeting of IPACG, being hosted by JCAB.

1.2	Mr. Shoichi Kosugi, JCAB Co-chair, and Mr. David Burkholder, FAA Co-chair, welcomed the group and the participants introduced themselves.

1.3	The meeting considered 10 working papers (WP’s) and 6 information papers (IP’s).  All the papers and these minutes are available in full on the IPACG website.  (Note that these minutes report on the agenda items in agenda order, which is not necessarily the order in which they were discussed.)

2. 	Agenda Item 1: Review and approve agenda

2.1	WP-01	The proposed agenda was accepted as submitted.

3.	Agenda Item 2: Data Link Problem Report Review

3.1	WP-02	FAA and JCAB CRA Activities

3.1.1	IPACG FIT CRAs have continued to process Problem Reports (PRs) received from operators and service providers in the North and Central Pacific.  The IPACG FIT CRAs consist of two complementary organizations, each supporting their respective area.  The JCAB CRA processes reports originating in or relating to the occurrences in Fukuoka FIR.   Resources for the Supporting Agency of JCAB CRA are provided by the Air Traffic Control Association Japan (ATCA-J).  Mr. Yamasaki presented the CRASA-Japan data.  The FAA CRA processes reports originating in or relating to the occurrences in Oakland and Anchorage Oceanic FIRs.  Resources for the FAA CRA are provided by the Boeing Company.  Mr. Gordon Sandell reported the CRA activities for the FAA.

3.1.2	Since FIT/21, the FAA CRA has continued to process reports from stakeholders related to operations in the North Pacific and other regions (including SOPAC).  These have resulted in 7 new FIT Problem Reports relating to NOPAC operations.  Over the same time frame, the JCAB CRA has processed forty (40) PRs submitted from the Stakeholders regarding the events in Fukuoka FIR.  Among them, four (4) events are classified as “In progress,” eight (8) as “Waiting for information,” and twenty-five (25) as “Open.”  The FIT/22 meeting reviewed all PRs, discussed the nature and causes of issues, and closed several of the PRs.

3.2	IP-05	Data Link Monitoring Agency (DLMA) Problem Report (PR) Status for the North Atlantic (NAT) CNSG5

3.2.1	Mr. Gordon Sandell presented this paper as information for FIT/22 regarding the problem reports (PRs) that Boeing also tracks and monitors in the North Atlantic Communication, Navigation and Surveillance Group (NAT/CNSG).  PRs reported in one region are likely to be of interest to, or (especially in the case of airplane problems) common to other regions as well.  FAA CRA provided the report by the Data Link Monitoring Agency (DLMA), which is equivalent to the CRA.

4.	Agenda Item 3: ANSP GOLD Performance Analysis Review

4.1	IP-01	FANS 1/A Performance in Oakland and Anchorage FIRs (Per GOLD)

4.1.1	Ms. Julia Fuller provided observed performance measures as specified in the Global Operational Data Link Document (GOLD) from the operational data collected in Oakland and Anchorage Oceanic airspaces for Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance – Contract (ADS-C).  Information was also presented regarding Ground Earth Station (GES) and operator performance within Oakland and Anchorage FIRs.

4.2	IP-03	FANS 1/A over HFDL Performance

4.2.1	Ms. Julia Fuller provided observed performance of the High Frequency Data Link (HFDL) operations conducted within the Oakland FIR as part of the FANS 1/A over HFDL (FOH) evaluation project to evaluate the viability of FOH for some oceanic data link operations.  The paper outlined the HFDL performance by month for the Hawaiian Airlines (HAL) fleet of B763 participating in the evaluation.  The data shows that during the aggregate time period from July to September 2011, seven of the seventeen HAL-B763 airframes met the 95 percent ADS-C (RSP 400) criteria.

4.3	IP-04	FANS 1/A over I4 Classic Aero Performance

4.3.1	Mr. Tom Kraft provided observed performance of FANS 1/A over I4 Classic Aero (FOICA) data link operations by ground earth station (GES), by aircraft type and by operator/aircraft type combination within the Oakland, Anchorage and New York oceanic FIRs for the aggregate period from July to September 2011.  The purpose of this analysis was to present the most recent performance observed for FOICA and to aid in the evaluation of the global use of FOICA in data link operations.  The analysis focused on ADS-C data due to the limited data observed from the Controller Pilot Data Link Communication (CPDLC) system over I4 satellites.

4.4	WP-06	 GOLD Performance Analysis Tool (G-PAT) Status

4.4.1	Ms. Julia Fuller provided the current status of the GOLD – Performance Analysis Tool (G-PAT), which generates GOLD performance charts from input CPDLC and ADS-C data.  The G-PAT has a graphical user interface (GUI) which can be run on a MS Windows-based personal computer and is in beta test with a few selected users.  The current draft version of the G-PAT User Manual was also presented along with samples of how to input required data files.  Ms. Fuller indicated that Mr. Kosugi has a “beta test” version of tool, which is available for JCAB to participate in the evaluation.

5.	Agenda Item 4: GOLD Amendments

5.1	WP-03	GOLD Amendment Program

5.1.1	Mr. Tom Kraft provided an update on the GOLD Amendment Program.  Previously, the IPACG FIT had agreed to support follow-on work programs of the GOLD Working Group to ensure that Asia Pacific regional implementation was coordinated globally.  While the GOLD is maintained as a regional document by each of the participating regions, any region must first globally coordinate any amendments and obtain agreement with participating regions before it can be incorporated into the next edition of the GOLD.  In January 2011, the GOLD Working Group solicited input to establish a work program for follow-on work to amend the GOLD.  If anyone is participating in the GOLD Amendment Program, they must obtain access to the GOLD web site, as this web site will be used to post the latest draft versions of GOLD and the change proposal matrix, and to notify members of updates.

5.1.2	Mr. Kraft invited the FIT to recommend to IPACG to continue its participation to complete a GOLD, 2nd Edition, by first quarter 2013, and to identify and prepare proposals for specific GOLD amendments that are of interest to the IPACG and FIT and submit them to tom.kraft@faa.gov for consideration by the GOLD Working Group.  The FIT concurred with this proposal.

5.2	WP-04	CPDLC Route Clearance Messages

5.2.1	Ms. Julia Fuller discussed a proposal to remove the CPDLC transactions related to route clearances in the measurement of Actual Communication Performance (ACP) and to change the GOLD accordingly.  This paper provided an analysis to determine the impact these route clearance message elements have on ACP in various U.S. FIRs.  The percentage of CPDLC transactions used to measure ACP that is related to route clearances is approximately 1.2% in Oakland FIR (much higher in New York FIR).  It has been proposed that these route clearance messages do not reflect the conflict resolution capability of the controller and should thus be removed from all future ACP analysis.  Afterwards, the effect in and Pacific will be negligible due to the low 1.2% usage.  This change is also proposed for inclusion in the next edition of the GOLD.

5.2.2	The working paper requested that FIT comment on the proposal to remove the subject messages, and agree on the proposal to amend the GOLD, as appropriate.  Discussion ensued about whether this proposal was relaxing the criteria.  FAA confirmed that it was not relaxing the criteria and was only proposing the make the actual performance measurements more representative of the capability for the controller to intervene as defined by the collision risk model for the reduced separation minima.  JCAB asked for additional time to review the proposal and would provide comments at the next opportunity in the GOLD Working Group review process.

5.3	WP-05	CPDLC Message Element Usage

- 5.3.1	Ms. Julia Fuller provided information in support of the ICAO OPLINK Panel’s efforts to harmonize and optimize the CPDLC message set.  The paper provided s summary of the observed usage of the various elements in the existing CPDLC message set in the New York (KZNY), Oakland (KZAK) and Anchorage (PAZA) flight information regions (FIR).

- 5.3.2	The paper proposed that FIT comment on the proposal to collect statistics on CPDLC usage by message element, and agree to amend the GOLD to collect this type of information to support performance analysis and development of standards for next generation CPDLC implementations.  Discussion ensued about efforts underway and the need to have a common way of gathering and distributing this information.  JCAB already is compiling similar data and posting on its website.  Consensus was that this data was useful to collect.  FAA and JCAB agreed to work together and prepare a more detailed data collection proposal to be presented at the FIT/23.

6.	Agenda Item 5: Planning and Implementation

6.1	IP-02	FANS 1/A over Iridium Performance

6.1.1	Ms. Julia Fuller provided information on the observed performance of FANS 1/A over Iridium (FOI) data link operations conducted within the Oakland, Anchorage and New York flight information regions (FIRs) from April to September 2011.  This paper provided the more recent performance observed for FOI to aid in the evaluation of the viability of FOI for global use in data link operations.

6.2	WP-07	FANS 1/A over Iridium Status

6.2.1	Mr. Tom Kraft provided the FAA’s response to the Performance-Based Operations Aviation Rulemaking Committee (PARC) regarding the FANS 1/A over Iridium (FOI) and Performance-Based Concept Recommendations, dated 16 September 2010.  On 27 June 2011, the FAA responded to the PARC, indicating agreement with the recommendations in the report.  The paper reviewed the ICAO provisions and the FAA’s means of compliance for airworthiness and operational requirements that allows the use of FOI anywhere in the world unless otherwise restricted by airspace requirements.  The paper also highlighted the need to implement a performance-based framework for communications and surveillance, regardless of the type of media used for FANS 1/A operations.  The paper invited the IPACG FIT to take notice of these provisions and acknowledge FOI as a viable means for conducting air traffic service (ATS) communications, and that FOI aircraft are eligible for CPDLC and ADS-C operations in the Pacific Region.

6.3	WP-08	RCP and RSP Planning and Implementation

- 6.3.1	Mr. Tom Kraft presented this paper that stated that the Asia Pacific Region does not have a regional plan for the implementation of performance-based communication and surveillance.  With an increasing proliferation of different aircraft FANS 1/A systems, and given current ground system implementations, it is possible that separations may be misapplied because the qualification criteria for communication and surveillance are not formally applied within the Asia Pacific Region.  FIT/16 agreed with the concept of developing a template and performance specifications for global use in specifying criteria for communications and surveillance.

- 6.3.2	This paper recommended that the IPACG FIT include in its work program a planning and implementation initiative to effectively implement a performance based-framework in the North and Central Pacific regions.  The FIT agreed that this topic will be brought to the attention of IPACG/35 as a potential planning and implementation initiative and will be discussed further at FIT/23.

6.4	WP-10	Enhancement to AEIS Service

6.4.1	Mr. Gordon Sandell presented a paper outlining operator (Air New Zealand) problems with accessing the Japan AEIS (Volmet) service due to interoperability problems between the aircraft and the ground server.  The user interface on several operator aircraft allows only a 4-character airport identifier (with no spaces) to be entered.  The Japan AEIS system only provides a 3-character identifier.  The proposal is for the service provider for Japan’s AEIS service to respectfully consider the enhancement to accept both 3 and 4 character airport identifiers as a means of making this service available to additional operators.

6.4.2	JCAB acknowledged the issue and was aware of the need to expand the system to 4-characters.  JCAB is currently studying this and assessing the operator demand for this change, and then will work to secure appropriate funding to make this change.  JCAB welcomed operator comments on the demand for this service.  JAL and ANA stated that they were getting the data via their company pages (without CRC data protection).

6.5	IP-06	Route Loading Guide

6.5.1	Mr. Gordon Sandell presented this paper and noted that at IPACG/31 & FIT/18, Boeing provided a guide to assist ground system automation designers in developing their systems so that they can construct route uplinks that will load into the airplane’s flight management system correctly.  Since then, a number of minor revisions have been made and additional detail has been added to the document.  The meeting was asked to note the latest version of the document and make it available to ground system automation designers.

7.	Agenda Item 6: SATCOM Voice for ATS Communications

7.1	WP-09	SATCOM Voice Guidance Material

7.1.1	Mr. Tom Kraft provided an update on Inter-Regional Satellite Communications (SATCOM) Voice Task Force (IRSVTF) and the timeline for updating the guidance material.  The paper recommended that IPACG continue to participate in completing globally applicable guidance material for using SATCOM voice for air traffic services communication (SVGM) by staying active and providing Asia Pacific regional comments when prescribed in the accompanying presentation.  The FIT agreed with the recommendation.

8.	Agenda Item 7: ATS Inter-facility Data Communications (AIDC)

8.1	Verbal	Update on PAN-Regional ICD for AIDC

8.1.1	Mr. Tom Kraft reported that several items related to the status of the PAN-Regional ICD for AIDC in coordination between the North Atlantic and Asia-Pacific Regions.  First, it was noted that the NAT ICD has been updated very recently, and efforts will proceed to align the current draft PAN-Regional ICD with the most recent NAT ICD.  Secondly, the OPLINK Panel is monitoring the development of the PAN-Regional ICD for AIDC.  At their meeting in October, the OPLINK Panel agreed to review sections of ICAO Doc 9694 concerning AIDC, and propose that updated Doc 9694 material be incorporated into the PAN-Regional ICD for AIDC, accordingly.  Lastly, the ICAO EUR-NAT Regional Office in Paris is investigating the possibility of establishing a website on the ICAO Portal Secure web site for PAN-Regional ICD for AIDC to facilitate dissemination and gathering of comments on a global basis.  The FAA will continue to keep the FIT informed of the status of the PAN-Regional ICD for AIDC at future FIT meetings.

9.	Agenda Item 8: Any Other Business

9.1	No other business was brought before the FIT.

10.	Closing of the Meeting

10.1	The Co-chairs thanked the FIT meeting members for their participation, and on behalf of all participants, the Co-chairs expressed their appreciation for the efforts of the interpreters.

END