THE TWENTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/25)

(Tokyo, Japan, 24-27 October 2006)

Agenda Item 6:  Air Traffic Management (ATM) Issues

Application of 10-Minute Longitudinal Separation

Without the Mandatory Assignment of Mach Number Technique (MNT)

(Presented by the Federal Aviation Administration)

SUMMARY

On 27 January 2006, the attached proposal for amendment of the ICAO Regional Supplementary Procedures (Doc 7030/4) concerning the application of 10-minutes longitudinal separation without the mandatory assignment of Mach Number Technique (MNT) was approved by ICAO for use within the Oakland FIR.  This paper proposes to expand the application of 10-minute longitudinal separation without mandatory assignment of MNT to Anchorage and Fukuoka Flight Information Regions (FIR) and develop an implementation strategy for seamless integration.

1.		Background

  - Data collection and operational trials began simultaneously in 1998 for the application          of 10-minute longitudinal separation without mandatory assignment of MNT in the Oakland FIR.  This data was then compared to previous studies to determine if current practice would support reduced longitudinal separation.  A comparative safety analysis of the airspace where 10-minute longitudinal separation was applied was performed by the FAA and it was found that, due to improvements in aircraft performance and air traffic control practices, the risk of collision due to the loss of planned longitudinal separation was no greater than the risk found in the original study which supported 10 minutes with MNT.

  - The Seventeenth Meeting of the Review of the General Concept of Separation                  Panel (RGCSP) Working Group A (RGCSP/WG/A/17) agreed that sufficient provisions were in the PANS-RAC (subsequently revised to the PANS-ATM) to enable authorities to reduce separation       to 10 minutes without applying MNT and that the Regional Supplementary Procedures (Doc 7030) could be amended as appropriate

  - The United States proposed the Amendment of Regional Supplementary Procedures ICAO Doc 7030/4 to include the application of 10-minute longitudinal separation without mandatory assignment of MNT.  The amendment proposal was approved 27 January 2006.

2.	Discussion

  - An airspace analysis is currently being conducted within the Anchorage FIR to determine if current practice would support reduced longitudinal separation.  The gain-loss probability distribution for aircraft pairs throughout the Anchorage FIR is being investigated and the analysis centers on the description of aircraft pair-wise separation and their combined ability to maintain separation once it has been applied.

  - Provided the analysis results are favorable, it is anticipated that Anchorage will begin operational trials for the use of 10-minute longitudinal separation without the mandatory assignment of MNT after full transition to Ocean21 operations in 2007.

3.	Recommendation

  - In a continuing effort to harmonize global ATM, JCAB is invited to:

- Take note of the information in this paper; and

- Consider implementing 10-minute longitudinal separation without the mandatory assignment of MNT.

- END -