THE TWENTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/25)

(Tokyo, Japan, 24-27 October 2006)

Agenda Item 3:	 Report on relevant outcomes from other meetings

Automatic Dependent Surveillance In-Trail Climb/Descent Procedure in the Ocean

(Presented by the Federal Aviation Administration)

SUMMARY

The Federal Aviation Administration (FAA) and others are in the early stages of evaluating concepts for In-Trail Climb/Descent Procedures (ITP) in oceanic airspace using existing Automatic Dependent Surveillance-Contract (ADS-C) capabilities.  FAA is exploring options for ADS-C ITP; however, by using work accomplished by others, FAA may be able to expedite determination of whether ADS-C ITP is viable in the oceanic environment.

1.		Background

  - At an Action Plan 22: NAM/EUR Interoperability Trials (AP22) meeting in Glasgow, Scotland, during the week of 11-15 September 2006, representatives from the FAA, Eurocontrol     and other organizations met to discuss Automatic Dependent Surveillance-Contract (ADS-C) and ADS-Broadcast (ADS-B) In-Trail Climb/Descent Procedures (ITP).  FAA has presented information on an ITP concept for the ocean at previous oceanic work group meetings.

2.	Discussion

  - FAA is in the very early stages of pursuing ADS-C ITP in oceanic airspace.  Attachment A describes current FAA concepts for ADS-C ITP.

  - Although FAA supports ADS-C ITP, FAA may not be able to devote adequate resources toward quick development of ITP due to higher priority oceanic separation reduction efforts.  FAA has been advised that the United Kingdom (U.K.) is undertaking an analysis for possible reductions to time-based longitudinal separation between ADS-C equipped aircraft.  Additionally, during the recent and on-going Oceanic Tailored Arrival trial in Oakland Center (ZOA), NASA Ames noticed opportunities in oceanic airspace where an ADS-C ITP may have provided benefits.  Hopefully, data collected by the U.K. and NASA Ames may help further develop the ADS-C ITP.

  - Furthermore, we understand the Separation and Airspace Safety Panel (SASP) will be evaluating changes to the PANS ATM that relate to Global Navigation Satellite System (GNSS) references replacing current references to ground-based NAVAIDs.  By using work accomplished by others, FAA may be able to expedite determination of whether ADS-C ITP is viable in the oceanic environment.

3.	Recommendation

- 3.1	The meeting is invited to:

- Take note of the information presented; and

- Keep FAA apprised of other work that may be on-going concerning ADS-C enabled oceanic procedures.

–  END  –

ATTACHMENT A

ADS-C Oceanic In-Trail Climb/Descent Procedure (ITP)

Jointly Prepared by CAASD & FAA ATO-E

Introduction

- FAA in early stages of evaluating concept for Oceanic In-Trail Climb/Descent Procedure

  - Climb/descent to Requested Flight Level through one intermediate FL occupied by a blocking aircraft

  - Based on ADS-C reports & new ATC procedures

  - Builds on domestic DME rules in ICAO Doc. 4444, paragraph 5.4.2.3.2.3 & FAAO 7110.65, 6-4-2, c.1

  - Incorporate lessons learned from TCAS ITP, ADS-B ITP

- This briefing presents initial analysis

  - Need to refine & validate, identify issues

  - Work in progress

- Customer Benefits

  - More opportunities to reach optimal altitudes

  - Ability to climb to avoid turbulence

  - Potential to maximize revenue payload

  - Maximize on-board equipment use

Proposed Operational Concept

- ADS-C ITP is a procedure that can be requested by the pilot or controller-initiated

- Controller responsible for ensuring separation with all aircraft (assisted by Ocean21 conflict probe & tools)

- ADS-C ITP will have smaller longitudinal separation minima (xx nm) than standard rules (e.g., 30 NM, 50 NM) with aircraft at intermediate flight levels

- Pacific Trials First (currently using 30 or 50nm separation)

- Atlantic Trials Second (various standards in use)

Procedure may have near and long-term versions

- Goal is for early benefits without Ocean21 changes

  - Controller workload implications & some limitations

- Explore additional user benefits & reduced controller workload if Ocean21 is enhanced to automate procedure

ADS-C ITP Cases

FL350

FL360

FL370

xx nm minima

FL350

FL360

FL370

xx nm minima

Trailing Climb

Leading Climb

FL350

FL360

FL370

xx nm minima

FL350

FL360

FL370

xx nm minima

Trailing Descent

Leading Descent

Blocking aircraft

Requesting aircraft

(xx a distance less than standard longitudinal separation)

ADS-C ITP Concept Overview

| Preconditions |  |
| --- | --- |
| xx nm longitudinal separation minima while vertical separation is lost | Doc 4444, 5.4.2.3.2.3 |
| Direct controller pilot communication | Doc 4444, 5.4.2.3.1 |
| One aircraft maintains level (blocking aircraft) | Doc 4444, 5.4.2.3.2.3 (b) |
| Same track | Doc 4444, 5.4.2.3.2.3 |
| Simultaneous position reports (issue) | Doc 4444, 5.4.2.3.2.3 (c) |
| xx nm longitudinal separation minima while vertical separation is lost | Doc 4444, 5.4.2.3.2.3 |
| Direct controller pilot communication | Doc 4444, 5.4.2.3.1 |
| One aircraft maintains level (blocking aircraft) | Doc 4444, 5.4.2.3.2.3 (b) |
| Same track | Doc 4444, 5.4.2.3.2.3 |
| Simultaneous position reports (issue) | Doc 4444, 5.4.2.3.2.3 (c) |

ADS-C ITP Concept Timeline

Altitude change

request

Conflict probe reports altitude change clearance unavailable

Controller determines ADS-C ITP could be applied based on conflict probe

Send 0 sec. demand request to requesting &

blocking aircraft

Receive new ADS-C position reports

Determine  all ADS-C ITP criteria satisfied

Time interval_1

Uplink clearance response

(granted, unable)

Time interval_2

Aircraft reports reaching requested altitude

Time interval_3

Analysis Issues

- Additional analysis needed to determine performance criteria & procedural steps, such as

  - Definition of simultaneous reports

  - Maximum time from granting clearance to aircraft reestablishing vertical separation

  - Guidelines for maximum converging speed & initial separation distance (similar to TCAS ITP conditions)

  - Initial separation distance from Ocean21 displays

- Hazard analysis and safety assessment (collision risk) needed

Business Case

- Benefits Analysis

  - Estimate Oceanic Benefits

    - ZOA, ZAN & ZNY

- FAA Cost (ROM) for Implementation

- Complete Concept Analysis

  - Performance criteria

    - Ocean21 Performance

    - Communications system performance

  - Procedural steps

    - Definition of simultaneous reports

    - Max time from granting clearance to aircraft re-establishing separation

    - Guidelines for maximum convergence speed & initial separation distance

    - Initial separation distance

  - Controller Workload Preliminary Assessment

  - Preliminary Hazard Analysis & Safety Assessment

  - Pilot Situational Awareness requirements

  - Pilot Procedures

- Go / No Go Decision

Implementation Activities

- Resolve Datalink Performance Issues

- ATOP/ Ocean 21 NCPs

  - Not anticipated for initial implementation

- Separation Reduction Approval

  - Pacific

    - APANPIRG, ISPACG, IPACG, OWG, FIT

  - Atlantic

    - NAT SPG, IMG, ATMG, SARSIG, FIG

  - ICAO

    - SASP

  - FAA

    - OSRWG, AOV

- AT Implementation

  - Manuals

  - Procedures

  - Training

  - LMR

- Pilot Procedures / Situational Awareness

- Hazard Analysis & Safety Assessment

  - Collision Risk Model

ADS-C ITP Concept Summary

- Preliminary conclusion:  ADS-C ITP is feasible

- Resources should be allocated to begin next steps

- Next steps

  - Complete concept analysis, issues & initial procedure

  - Estimate controller workload effect on ADS-C ITP

  - Estimate oceanic benefits in ZOA, ZAN & ZNY

  - Safety assessment & hazard analysis

  - ICAO approval

  - Develop plan for ADS-C ITP implementation