THE TWENTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/25)

(Tokyo, Japan, 24-27 October 2006)

Agenda Item 6:	Air Traffic Management (ATM) Issues

Auckland/Oakland AIDC 2.0 Implementation

(Presented by the Federal Aviation Administration)

SUMMARY

This paper provides information regarding the implementation of ATS Inter-facility Data Communications (AIDC) 2.0 messaging between Auckland Area Control Centre (ACC) and Oakland Air Route Traffic Control Center (ZOA).

- Background

  - At the IPACG/23 meeting, held in July 2005, JCAB and FAA agreed to pursue the implementation of an ATS Inter-facility Data Communications (AIDC) 2.0 interface between Fukuoka ATMC and Oakland ARTCC.  The enhanced field 14 definitions employed in version 2.0 of the Interface Control Document (ICD) will permit data messaging to accomplish the coordination of mach, block altitude, and weather deviation/offset information. A similar effort between Auckland ACC and Oakland ARTCC was also undertaken.

- Discussion

  - On August 17, 2006 Auckland ACC and Oakland ARTCC implemented the enhanced field 14 portions of the AIDC 2.0 ICD.

  - Prior to the implementation of the enhanced interface the AIDC messages exchanged between the facilities was robust enough that coordination was accomplished via data messaging with no verbal communications required for routine flights.  The enhanced interface now in place allows for flights with speed restrictions, block altitudes, and weather deviation clearances to be coordinated using data communications exclusively.  Verbal communications between controllers for the purpose of coordinating flights is now required only in the most unusual circumstances.

  - After Oakland ARTCC implemented the new version of Ocean 21 software that supports use of the enhanced field 14 portions of the AIDC 2.0 ICD it was discovered that revised coordination with facilities supporting only AIDC 1.0 could cause problems in certain circumstances.  This problem occurs when an aircraft is originally coordinated with speed restrictions, block altitudes, or weather deviation clearances.  Since these conditions cannot be indicated in the AIDC 1.0 messages they must be verbally coordinated and entered into the aircraft flight profile by the Oakland controller.  After this is done the flight profile correctly includes all boundary crossing information.  If the sending facility subsequently sends a revised coordination message (CDN) this message will not include any information regarding the speed restrictions, block altitudes, or weather deviation since AIDC 1.0 does not allow for this type of information.  However, in the AIDC 2.0 environment the lack of this information in field 14 indicates that these conditions no longer apply (AIDC 2.0 paragraphs 4.4.1.2.2 and 4.5.2).  For example, if Oakland were to accept an initial transfer from an AIDC 1.0 facility for an aircraft on a weather deviation, the Oakland controller would manually enter the weather deviation information into the Ocean21 system.  If the AIDC 1.0 facility then sent a CDN message with a revised time the CDN message would not include the weather deviation and Ocean 21 would interpret this to mean that the aircraft be arriving at the boundary fix at a different time but would no longer be on a weather deviation.

  - Attachment 1 contains the test plan as conducted on August 17, 2006.

  - Attachment 2 contains system analysis data reductions for the final live test conducted on August 17, 2006.

- Recommendation

  - The meeting is invited to discuss the information presented in this paper and consider information that might be relevant to the ongoing effort to establish an AIDC 2.0 interface between Fukuoka ATMC and Oakland ARTCC.

- END -

Oakland ARTCC – Auckland ACC AIDC Test Plan

| Test Plan for AIDC Version 2 - Southbound KZAK-NZZO |  |
| --- | --- |
| Action | Description |
| KZAK: Inject AFP. | (AFP-ATOP1-IS -B744/H-SRDHIZW/C -KLAX-QUIGG/{{t-0,10}}F400 -M084F400 FICKY BELAN B577 TUT G457 SELKA -NZAA -REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT) |
| ABI Sent | KZAK: Verify ABI is sent when AFP is injected. KZAK: Verify receipt of LAM and correct Level/Estimate at PASSA sent. NZZO: Verify receipt of ABI |
| KZAK: Clear aircraft to climb maintain block F410F430 | KZAK: Verify ABI is sent with amended block level data F410F430F400A. Verify receipt of LAM from Auckland. NZZO: Verify receipt of ABI with F410F430F400A |
| Report aircraft in Block at F410 | KZAK: Verify amended ABI data sent F410F430. Verify receipt of LAM from Auckland. NZZO: Verify receipt of amended ABI data F410F430. |
| Clear aircraft on 5R weather deviation | KZAK: Verify amended ABI data sent F410F430/W5R. Verify receipt of LAM from Auckland. NZZO Verify receipt of ABI data with W5R. |
| Report aircraft back on route | KZAK: Verify amended ABI data sent F410F430. Verify receipt of LAM from Oakland. NZZO: Verify receipt of amended ABI data F410F430. |
| KZAK: Clear on weather deviation 30 either side. | KZAK: Verify amended ABI data sent with F410F430/W30E.Verify receipt of LAM from Auckland. NZZO: Verify receipt of ABI with weather deviation W30E. |
| KZAK: CPL is sent | KZAK: Verify CPL sent at correct VSP from boundary and that CPL contains F410F430/W30E. Verify receipt of LAM and ACP from Auckland. Verify sending of LAM on receipt of Auckland ACP. NZZO: Verify receipt of CPL F410F430/W30E, send ACP. |
| KZAK:Clear aircraft to maintain block F390F410 | KZAK: Send CDN to Auckland. Verify correct data F390F410F430B/W30E is sent. Verify LAM response and ACP. Verify LAM sent on receipt of ACP from Auckland.NZZO: Verify receipt of CDN with F390F410F430B and W30E. Send ACP. |
| Report aircraft in Block at F410 | KZAK: Send CDN to Auckland. Verify correct data F390F410/W30E is sent. Verify LAM response and ACP. Verify LAM sent on receipt of ACP from Auckland. NZZO: Verify receipt of CDN with F390F410 and W30E. Send ACP. |
| KZAK: Report aircraft back on route | KZAK: Send CDN to Auckland. Verify correct data F390F410 is sent. Verify LAM response and ACP from Auckland. Verify LAM sent on receipt of ACP from Auckland. NZZO: verify receipt of CDN F390F410 send ACP. |
| KZAK: Clear aircraft on offset 90L and speed GM085. | KZAK: Send CDN to Auckland. Verify correct data F390F410/GM085/090L sent. Verify receipt of LAM and ACP from Auckland. Verify LAM sent to Auckland ACP. NZZO: Verify receipt of F390F410/G085/090L. Send ACP. |
| Test Plan for AIDC Version 2 - Northbound NZZO-KZAK |  |
| Action | Description |
| NZZO: Copy AFP from scenario oakland_aidcv2_testing.s, and inject through MRCC. | (AFP-OCSIS1-IS -B744/H-SRDHIRZW/C -YSSY-11S170W/{{t-0,15}}F330 -M084F330 DCT WARTY 33S160E 3138S16300E 28S170E/M084F330 2500S17310E 15S176W 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W 24N130W LENNA FICKY -KLAX -REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT) |
| ABI Sent | NZZO: Verify ABI is sent when AFP is injected. NZZO: Verify receipt of LAM and correct Level/Estimate at 0500S16227W sent. KZAK: Verify receipt of ABI |
| NZZO: Clear aircraft to climb and maintain block F340F360 | NZZO: Verify ABI is sent with amended block level data F340F360F330A. Verify receipt of LAM from Oakland. KZAK: Verify receipt of ABI with F340F360F330A |
| Report aircraft in Block F340 | NZZO: Verify amended ABI data sent F340F360. Verify receipt of LAM from Oakland.KZAK: Verify receipt of amended ABI data F340F360. |
| Clear aircraft on 8L weather deviation | NZZO: Verify amended ABI data sent F340F360W8L. Verify receipt of LAM from Oakland. KZAK: Verify receipt of ABI data with W8L. |
| Report aircraft back on route | NZZO: Verify amended ABI data sent F340F360. Verify receipt of LAM from Oakland. KZAK: Verify receipt of amended ABI data F340F360. |
| NZZO: Clear on weather deviation 15 either side. | NZZO: Verify amended ABI data sent with W15E.Verify receipt of LAM from Oakland. KZAK: Verify receipt of ABI with weather deviation W15E. |
| NZZO: CPL is sent | NZZO: Verify CPL sent at correct VSP from boundary and that CPL contains F340F360W15E. Verify receipt of LAM and ACP from Oakland. Verify sending of LAM on receipt of Oakland ACP. KZAK: Verify receipt of CPL F340F360W15E, send ACP. |
| NZZO:Clear aircraft to maintain block F340F350 | NZZO: Send CDN to Oakland. Verify correct data F340F350 and W15E is sent. Verify LAM response and ACP. Verify LAM sent on receipt of ACP from Oakland. KZAK: Verify receipt of CDN with F340F350 and W15E. Send ACP. |
| NZZO: Apply speed control exact M085. | NZZO: Send CDN to Oakland. Verify correct data F340F360/EM085/W15E to Oakland.Verify LAM response and REJ from Oakland. KZAK: Verify receipt of CDN F340F360/EM085/W15E. Send REJ. |
| NZZO: Clear aircraft on offset 90L. | NZZO: Send CDN to Oakland. Verify correct data F340F360/EM085/O90L sent. Verify receipt of LAM and ACP from Oakland. Verify LAM sent to Oakland ACP. KZAK: Verify receipt of F340F360/EM085/O90L. Send ACP. |

-------------------------------------------------------------------------------------------------------------------------------

20060817 2124:57.125 TEXIN    fdpsb601  (AFP-ATOP1-IS

-B744/H-SDHIRZW/C

-KLAX-QUIGG/2114F400

-M084F400 FICKY BELAN B577 TUT G457 SELKA

-NZAA

-REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2125:02.769 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2214F400-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F400 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2125:41.743 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2217F330-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F330 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2125:44.881 Q_ACID   fdpsb601  QUEUE: ACID

VHF Clearance: [ATOP1-CLIMB TO AND MAINTAIN BLOCK F410 TO F430]

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2125:49.602 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2215F410F430F400A-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F410 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2126:30.697 FPPOS    fdpsb601  ATOP1    / 11    level= F410  level2= F410  tim_cur= 19700101 0000:00.000 pres_pos= E empty

ATOP1    / 11    source= FP_POSR_DEFAULT    timeest= 19700101 0000:00.000 subs_pos= E empty

-------------------------------------------------------------------------------------------------------------------------------

20060817 2126:35.119 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2215F410F430-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F410 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2126:56.892 Q_ACID   fdpsb601  QUEUE: ACID

VHF Clearance: [ATOP1-CLEARED TO DEVIATE UP TO R 005 NM]

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2126:58.201 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2220F340F360F330A-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2127:01.607 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2215F410F430/W5R-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F410 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2127:40.602 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2220F340F360-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2128:01.706 Q_SECTOR fdpsb601  QUEUE: SECTOR

ATOP1 :

FREE TEXT:  BACK ON ROUTE

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2128:06.603 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2215F410F430-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F410 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2128:26.078 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2220F340F360/W8L-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2129:06.648 Q_ACID   fdpsb601  QUEUE: ACID

VHF Clearance: [ATOP1-CLEARED TO DEVIATE UP TO L or R 030 NM]

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2129:11.604 TEXOUT   fdpsb601  (ABI-ATOP1/A0000-KLAX-PASSA/2215F410F430/W30E-NZAA

-8/IS

-9/B744/H

-10/SDHIRZW/C

-15/M084F410 QUIGG B577 TUT G457 SELKA

-18/REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2129:25.187 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2220F340F360-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2130:38.084 TEXIN    fdpsb601  (ABI-OCSIS1-YSSY-0500S16227W/2220F340F360/W15E-KLAX

-8/IS

-9/B744/H

-10/SRDHIRZW/C

-15/M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-18/REG/N179XX0 SEL/DEFG RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2136:35.734 TEXOUT   fdpsb601  (CPL-ATOP1/A0000-IS

-B744/H-SDHIRZW/C

-KLAX-PASSA/2215F410F430/W30E

-M084F410 QUIGG B577 TUT G457 SELKA

-NZAA

-REG/N180XX0 SEL/ABCD RMK/AIDCV2 TEST FLIGHT)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2136:46.556 TEXIN    fdpsb601  (ACP-ATOP1/A0000-KLAX-NZAA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2137:11.584 Q_ACID   fdpsb601  QUEUE: ACID

VHF Clearance: [ATOP1-DESCEND TO AND MAINTAIN BLOCK F390 TO F410]

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2137:39.767 TEXOUT   fdpsb601  (CDN-ATOP1/A0000-KLAX-NZAA

-14/PASSA/2214F390F410F430B/W30E

-15/M084F390 QUIGG B577 TUT G457 SELKA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2138:14.622 TEXIN    fdpsb601  (ACP-ATOP1/A0000-KLAX-NZAA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2138:42.007 FPPOS    fdpsb601  ATOP1    / 11    level= F410  level2= F410  tim_cur= 19700101 0000:00.000 pres_pos= E empty

ATOP1    / 11    source= FP_POSR_DEFAULT    timeest= 19700101 0000:00.000 subs_pos= E empty

-------------------------------------------------------------------------------------------------------------------------------

20060817 2138:59.519 TEXOUT   fdpsb601  (CDN-ATOP1/A0000-KLAX-NZAA

-14/PASSA/2214F390F410/W30E

-15/M084F390 QUIGG B577 TUT G457 SELKA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2139:54.702 TEXIN    fdpsb601  (CPL-OCSIS1-IS

-B744/H-SRDHIRZW/C

-YSSY-0500S16227W/2220F340F360/W15E

-M084F340 11S170W 0500S16227W 03S160W 05N151W 10N145W 15N139W

24N130W LENNA FICKY

-KLAX

-0)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2140:10.487 Q_SECTOR fdpsb601  QUEUE: SECTOR

ATOP1 :

FREE TEXT:  BACK ON ROUTE

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2140:32.513 TEXOUT   fdpsb601  (ACP-OCSIS1-YSSY-KLAX)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2140:43.855 TEXOUT   fdpsb601  (CDN-ATOP1/A0000-KLAX-NZAA

-14/PASSA/2214F390F410

-15/M084F390 QUIGG B577 TUT G457 SELKA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2141:27.572 TEXIN    fdpsb601  (ACP-ATOP1/A0000-KLAX-NZAA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2142:09.120 Q_ACID   fdpsb601  QUEUE: ACID

VHF Clearance: [ATOP1-OFFSET L 090 NM, Freetext : FLY A 30 DEGREE INTERCEPT ANGLE

TO JOIN THE OFFSET, MAINTAIN M085 OR GREATER]

EOM

-------------------------------------------------------------------------------------------------------------------------------

20060817 2142:27.428 TEXOUT   fdpsb601  (CDN-ATOP1/A0000-KLAX-NZAA

-14/PASSA/2217F390F410/GM085/O90L

-15/M085F390 QUIGG B577 TUT G457 SELKA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2143:36.926 TEXIN    fdpsb601  (ACP-ATOP1/A0000-KLAX-NZAA)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2146:23.902 TEXIN    fdpsb601  (CDN-OCSIS1-YSSY-KLAX

-14/0500S16227W/2220F340F350F360B/W15E)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2147:10.389 TEXOUT   fdpsb601  (ACP-OCSIS1-YSSY-KLAX)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2148:26.511 TEXIN    fdpsb601  (CDN-OCSIS1-YSSY-KLAX

-14/0500S16227W/2219F340F350F360B/EM085/W15E)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2149:18.085 TEXOUT   fdpsb601  (REJ-OCSIS1-YSSY-KLAX)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2152:03.874 TEXIN    fdpsb601  (CDN-OCSIS1-YSSY-KLAX

-14/0500S16433W/2210F340F350F360B/EM085/O90L

-15/M084F340 11S170W 0350S16324W 0152S16059W 0608N15159W 1109N14558W

2502N13112W 3139N12409W LENNA FICKY)

-------------------------------------------------------------------------------------------------------------------------------

20060817 2153:03.301 TEXOUT   fdpsb601  (ACP-OCSIS1-YSSY-KLAX)