THE TWENTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/25)

(Tokyo, Japan, 24-27 October 2006)

Agenda Item 6:  Air Traffic Management (ATM) Issues

Common Air Traffic Flow Management Terminology

(Presented by the Common ATFM Terminology Task Force)

SUMMARY

This working paper provides a summary of the work accomplished by the Common Air Traffic Flow Management (ATFM) Terminology Task Force, which was established during the 22nd Meeting of the IPACG (IPACG/22).

- Background

  - Centralized traffic management facilities are best able to communicate their national system’s ability to accept traffic from adjacent international air traffic service (ATS) providers.  As coordination and collaboration efforts intensify between the countries, effective communications are essential.  A key element in removing language barriers is establishing common terms and phrases.  Terminology and phraseology differences in air traffic flow management (ATFM) could be a potential source of confusion during communications among the Japan Civil Aviation Bureau (JCAB) Air Traffic Management Center (ATMC) and the Federal Aviation Administration (FAA) David J. Hurley Air Traffic Control System Command Center (ATCSCC).

  - IPACG/21 discussions resulted in a recommendation to develop common terms of reference for ATFM communications.  IPACG/22 supported the formation of a Task Force to address this issue.  The operation of the Task Force was outlined at IPACG/23.  ATCSCC and ATMC established a process to examine the ATFM common terminology and phraseology at IPACG/24.

  - The terminology will be an essential element in developing definitive, clear, and concise communication between international ATFM units.  Likewise, the phraseology will be a technical pattern of communication to exchange standardized and harmonized messages between international ATFM units.  This bilateral effort should be combined with an ICAO effort to standardize ATFM terms.

  - This working paper is largely based on the “Phraseology for the Exchange of ATFM Messages Handbook” dated February 2003, by the Multi-Agency Air Traffic Services Procedures Coordination Group (MAPCOG) ATFM Task Force, which is a joint effort between EUROCONTROL, NAV CANADA and the FAA.

- Discussion

  - ATFM is a service complementary to air traffic control.  The objective of ATFM is to ensure an optimum flow of air traffic to or through areas and aerodromes where traffic demand at times exceeds the available capacity of the ATC system.  This optimum flow will be achieved by maintaining, in continuous cooperation with related ATC units and airspace users, a balance between the traffic demand and the ability to accommodate that demand.

  - The operation of ATFM services, while respecting the intentions of the airspace users to the maximum extent possible, will ensure:

- Full exploitation of available ATC capacity

- Maximum flexibility in handling traffic flows

- Expeditious and orderly traffic flows

  - The operation of ATFM services includes the application of ATFM measures that are designed to achieve the optimum flow of traffic.  These measures include but are not limited to:

- Allocating and updating departure slots

- Allocating and updating arrival slots

- Re-routing of traffic

- Alternate flight profiles

- Time-in-trail assignments

- Miles-in-trail assignments

- Airborne holding

- Ground stops

  - ATFM is not considered an end in itself but is a tool to be applied when demand exceeds the existing capacity of the ATC system.  The prime effort is to be aimed at the ability of the ATC system to cope with the traffic demand rather than the ability of ATFM to adjust that demand.

  - The ATCSCC is the approving authority for ATFM measures within the United States National Airspace System (NAS).

  - ATFM is performed by the ATCSCC in 6 daily phases:

- Exploration

- Justification

- Implementation

- Validation

- Critique

- Review

  - The Exploration phase of ATFM consists of examining demand in relation to capacity.  This is done via periodic telephone conferences (TELCONs) and consideration of planned and anticipated events.  The Justification phase consists of determining the rationale for implementing an ATFM measure.  The Implementation phase consists of the notification and implementation of the ATFM measure.  The Validation phase consists of monitoring the implemented ATFM measure to validate the rationale for its implementation.  The Critique phase examines if the implemented ATFM measure achieved the desired outcome.  The Review phase documents the day's events and includes the retention of data.

  - The ATMC is the only organization designated by the JCAB for implementing ATFM measures within the Fukuoka Flight Information Region (FIR).  All the sectors of Japanese Area Control Centers (ACCs) and major airports are kept under surveillance.  Their capacity values are adjusted depending on weather, special use of airspace, and other affecting variables.  The forecasted excess demand is analyzed to make a go or no-go decision on ATFM measures.  The consultation with meteorological specialists and the talks on available ATC capacity with related air traffic control facilities are the elements to be considered.  The implemented ATFM measures are notified by NOTAM.  The effectiveness of the ongoing ATFM measures is monitored, and the measures would be fine-tuned if situation requires.

Terminology and Phraseology as Used in the Exchange of ATFM Messages

  - Terminology and phraseology for air traffic control are both standardized and documented to ensure that communications between air traffic controllers and pilots are brief, complete, accurate and understood.  Terminology and phraseology for the exchange of ATFM messages however, are neither standardized nor documented.

  - Historically, air traffic controllers were the initial operators of the ATFM system.  For example, a departure controller would contact a tower controller and say “stop departures.”  A tower controller would also have contacted an approach controller to say “hold arrivals.”  These two common ATC phrases are neither standardized nor documented.  Although there are elements of standardization for communications between air traffic controllers, there has always also been a plain language element of ATC communications.

  - The evolution of ATFM has brought about more robust and complex systems, but the use of plain language is still evident and is current practice.  Additionally, the transparency of current ATFM systems and the inclusion of non-ATC users also contribute to the use of plain language.

  - As ATFM becomes more global in nature, regional providers of ATFM service such as the ATMC and the ATCSCC may need to consider which plain language words and phrases are best suited for the exchange of ATFM messages.  This is especially important when linking adjacent ATFM systems or introducing ATFM in areas where it presently does not exist.

  - The task force has put together a foundation of ATFM message component and message types based on terminology and phraseology already endorsed by EUROCONTROL, NAV CANADA and the FAA.  This guideline should be reviewed and updated periodically once voice ATFM message exchanges between FAA and JCAB become more frequent.

  - The attached document contains terminology and phraseology for the exchange of ATFM messages, which has been developed based on the MAPCOG Phraseology for the Exchange of ATFM Messages Handbook.  The terminology and phraseology are not intended to be a requirement for ATFM communications, but may be used as a guideline for the exchange of ATFM messages.  It also contains those ATFM related abbreviations used by the ATCSCC and ATMC that are not defined in the ICAO Doc. 8400 (PANS-ABC).

- Recommendation

  - The meeting is invited to:

- Review and comment on the attached guideline for terminology and phraseology, which provides standard guidance for the exchange of ATFM messages;

- Adapt the attached guideline as an interim IPACG document which may be used as a basis for future implementation of ATFM communication between JCAB ATMC and FAA ATCSCC;

- Conclude that the work on Action Item IP/21-7 may be closed; and

- Agree that the ATFM Common Terminology Task Force, having completed their work, is dissolved.

Interim Guideline for ATFM Communication

Version 1.0

October 2006

FOREWORD

Centralized traffic management facilities are best able to communicate their national system’s ability to accept traffic from adjacent international air traffic service (ATS) providers.  As coordination and collaboration efforts intensify between the countries, effective communications are essential.  A key element in removing language barriers is establishing common terms and phrases.  Terminology and phraseology differences in air traffic flow management (ATFM) could be a potential source of confusion during communications among the Japan Civil Aviation Bureau (JCAB) Air Traffic Management Center (ATMC) and the Federal Aviation Administration (FAA) David J. Hurley Air Traffic Control System Command Center (ATCSCC).

IPACG/21 discussions resulted in a recommendation to develop the common terms of reference for ATFM communications.  IPACG/22 supported the formation of a Task Force to address this issue.  The operation of the Task Force was outlined at IPACG/23.  The ATCSCC and ATMC established a process to examine the ATFM common terminology and phraseology at IPACG/24.  This document was submitted by the Task Force at IPACG/25.  The bilateral effort herein should be combined with an ICAO effort to standardize ATFM terms in the future.

The terminology will be an essential element in developing definitive, clear, and concise communication between international ATFM units.  Likewise, the phraseology will be a technical pattern of communication to exchange standardized and harmonized messages between international ATFM units.  These terminology and phraseology are not intended to be a requirement for ATFM communications, but may be used as a guideline for the exchange of ATFM messages.

This guideline is largely based on the “Phraseology for the Exchange of ATFM Messages Handbook” dated February 2003, by the Multi-Agency Air Traffic Services Procedures Coordination Group (MAPCOG) ATFM Task Force, which is a joint effort between EUROCONTROL, NAV CANADA and the FAA.

TABLE OF CONTENTS

1.  General

2.  ATFM Message Components

3.  ATFM Message Types

4.  Abbreviations

Appendix:  Table of Abbreviations

1.	General

1.1	The primary goal of these guidelines is to develop terminology and phraseology for the exchange of ATFM messages between units providing ATFM services.  The terminology and phraseology contained herein are intended to both reflect the current use of plain language and provide a basis for standardization and harmonization.

1.2	Although there are various plain language words and phrases in use today by ATFM service providers, these words and phrases can be organized into a modular and structured method of delivery to ensure communication harmonization and reduce the incidence of misunderstanding between units providing ATFM service.

1.3	These guidelines include the concept of modular and structured ATFM messages and defines an ATFM message's components as who, what, where, when and why.  These five components are described as follows:

1). 	Who: The ATFM service unit being contacted followed by the ATFM service unit that is initiating the contact.

2). 	What: The ATFM objective to be achieved.

3).	Where: The location of the ATFM objective to be achieved.

4).	When: The time and/or duration of the ATFM objective to be achieved.

5).	Why: The reason for the ATFM objective.

1.4	There should be no module regarding “how” the ATFM restrictions should be achieved by the counterpart ATFM service provider. It is the counterpart’s responsibility how they fulfill the requested ATFM restrictions within their airspace. However, the center being asked for the ATFM restrictions may collaborate with the originated center on the type and method of ATFM measure application.

1.5	Below are the examples of possible ATFM messages:

- ATCSCC, this is ATMC…We need 100 miles interval regardless of altitude on R220, R580 and all the PACOTS tracks for traffic landing at Narita airport estimated FIR boundary from 0100 UTC until 0500 UTC due to severe weather.

- ATMC, this is ATCSCC…Information maybe developed into ATFM… Los Angeles has started flow control for all aircraft landing at Los Angeles airport due to earthquake. They are requesting ground stops for arrivals until further notice.

2.	ATFM Message Components

2.1	The use of a modular and structured ATFM message provides for consistent ATFM message design and delivery.  Each of the ATFM message's five components can contain plain language elements that when combined provide a complete ATFM message.  The harmonization achieved lies in the delivery of an ATFM message that has all of the required components in a structured format while making allowances for different plain language elements.  This is of particular benefit for ATFM service providers that use different ATFM terminology or for ATFM service providers that do not use English for their intra-ATFM coordination.

As the modular and structured ATFM message may contain several different elements of plain language, this section will examine each of the five components and detail some of the possible plain language words and phrases that are in use today.

2.2	WHO:  The who component identifies the ATFM service unit being contacted followed by the ATFM service unit that is initiating the contact.  Examples of the who component:

- ATMC, this is ATCSCC…

- ATCSCC, this is ATMC…

2.3	WHAT:  The what component identifies the ATFM objective to be achieved.  Objectives include but are not limited to:

I/WE NEED…

- (X) MILES/MINUTES INTERVAL AT THE SAME ALTITUDE…

- (X) MILES/MINUTES INTERVAL REGARDLESS OF ALTITUDE…

- A RATE OF (X) AIRCRAFT PER HOUR…

- (X) MILES-IN-TRAIL AT (specified altitude(s))…

- (X) MINUTES-IN-TRAIL AT (specified altitude(s))…

- TO BLOCK (specified altitude(s))

- TO LIMIT THE ACCEPTABLE ALTITUDE TO (specified altitude(s))

- TO SUSPEND THE FIR ENTRY…

2.4	WHERE:  The where component represents the location of the ATFM objective to be achieved.  It is often preceded by modifying clause, indicating what aircraft or traffic the restriction will apply to.  The modifying clause and the location combination are used to construct there where component.

Examples of the modifying clause:

- …FOR TURBOJET TRAFFIC…

- …FOR ALL AIRCRAFT…

- …FOR TRAFFIC GREATER THAN (X) KNOTS…

- …FOR HEAVY AIRCRAFT…

- …FOR TRAFFIC LANDING…

- …FOR AIRCRAFT DEPARTING…

- …FOR TRAFFIC OVERFLYING…

- …FOR AIRCRAFT PASSING…

Examples of there where clause:

- …OVER NIPPI…

- …NARITA AIRPORT…

- …ANCHORAGE APPROACH…

- …ON A337…

- …WESTBOUND ON PACOTS TRACK C…

- …EAST FLOW ON A590…

- …INBOUND ON G344…

- …ON PACOTS TRACK 2 LANDING AT SAN FRANCISCO AIRPORT…

- …ON PACOTS TRACK E BELOW FLIGHT LEVEL (X)…

- …ABOVE FLIGHT LEVEL (X)…

- …INBOUND TO TOKYO ACC…

- …INBOUND TO OCEANIC SECTOR 5…

- ... (compass direction) OF (a significant point/airway/location)…

2.5	WHEN:  The when component represents the time and/or duration of the ATFM objective to be achieved:

- …FROM 0300 UTC UNTIL 0600 UTC…

- …FROM NOW UNTIL 0600 UTC…

- …FROM 2300 UTC UNTIL FURTHER NOTICE…

- …UNTIL FURTHER NOTICE…

- …FOR THE NEXT (X) HOURS…

2.6	WHY:  The why component represents the reason for the ATFM objective:

DUE TO/FOR…

- RUNWAY CLOSURE.

- (SEVERE) WEATHER.

- COMMUNICATION SYSTEM OUTAGE.

- RADAR FAILURE.

- (significant event).

- (natural disturbance such as FIRE or VOLCANIC ASH).

- STATE ACTIVITY.

- MILITARY ACTIVITY.

- EQUIPMENT OUTAGE.

- EMERGENCY.

- ADJACENT ATFM MEASURES.

3.	ATFM Message Types

3.1	Information to be Shared Prior to Invoking the ATFM Restrictions:  The information-sharing should be facilitated not only during the actual flow control but also (and more importantly) well prior to invoking the ATFM restrictions when the possibility of flow control arises. The following phrases will make clear the distinction between the ATFM messages and the information provided for situation awareness:

- INFORMATION MAYBE DEVELOPED INTO ATFM.

- CAPACITY RELATED INFORMATION.

3.2	Examples of messages sent prior to invoking ATFM restrictions follow:

- ATCSCC, this is ATMC…Information maybe developed into ATFM… Narita airport has closed one of the runways and started snow removal.

- ATCSCC, this is ATMC…Capacity related information…Narita airport has entered the storm zone of the typhoon.

3.3	ATFM Initiative Message:  ATFM initiatives communicate air traffic restrictions from one nation to another.  They follow the five component structure described earlier:

1). 	Who: The ATFM service unit being contacted followed by the ATFM service unit that is initiating the contact.

2). 	What: The ATFM objective to be achieved.

3).	Where: The location of the ATFM objective to be achieved.

4).	When: The time and/or duration of the ATFM objective to be achieved.

5).	Why: The reason for the ATFM objective.

3.4	Examples of ATFM initiatives follow:

- ATMC, this is ATCSCC…I need a 30 minute interval at the same altitude for all aircraft landing at Chicago airport from 0800 UTC until further notice due to state activities.

- ATCSCC, this is ATMC…We need to block FL350 and below for aircraft overflying Japanese domestic airspace for the next 12 hours due to emergency.

3.5	Coordination of Aircraft Exempted from ATFM Initiatives:  The following phrases will be used for the coordination of aircraft which are deemed necessary to exempt from the ATFM restrictions:

- REQUEST EXEMPTION FROM ATFM.

- COORDINATION OF ATFM EXEMPTION.

3.6	The following types of aircraft may be exempted from the flow control restrictions:

- Aircraft in a state of emergency

- Aircraft engaged in search and rescue missions

- Aircraft operating for humanitarian reasons

- Aircraft carrying the head of state or distinguished visitors of state

- Aircraft carrying a patient who needs urgent treatment

3.7	Examples of messages requesting ATFM exemption follow:

- ATMC, this is ATCSCC…Request exemption from ATFM…UAL123 is carrying a patient who needs urgent treatment.

UAL123…Exemption is approved.

- ATCSCC, this is ATMC…Coordination of ATFM exemption… JA501A is operating for search and rescue missions.

3.8	Information for the Next Coordination:  If it is possible and appropriate, the expected time of next coordination will be forwarded with the ATFM messages:

- I WILL CALL YOU AT 0400 UTC FOR FURTHER COORDINATION.

- WE WILL CALL YOU AGAIN IN 30 MINUTES.

3.9	An example of a message with information for the next coordination follows:

- ATMC, this is ATCSCC…I need a 30 minute interval regardless of altitude for all aircraft on PACOTS track 8 from 1000 UTC until further notice due to military activity.  I will call you again in 60 minutes.

3.10	Amendment:  The amendment of an ATFM message should be structured as the initial message and include similar elements but with additional modifiers.  These modifiers may include:

- CHANGE

- AMEND

- REDUCE

- INCREASE

- DECREASE

3.11	Amendment messages should also identify which message is being amended, as several restrictions could be in place at one time.  Examples of ATFM amendment messages follow:

- ATCSCC, this is ATMC…We have changed the restriction on traffic flying PACOTS tracks C, E and F for Narita airport. We now need 20 minutes intervals at the same altitude on PACOTS tracks C, E and F for traffic landing at Narita airport from now until 0900 UTC.

- ATMC, this is ATCSCC…We have increased the inbound rate from 5 aircraft per hour to 10 aircraft per hour for traffic beyond Oakland FIR until further notice.

3.12	Cancellation:  The cancellation of an ATFM message should be structured as the initial message and include similar elements but contain a canceling word or phrase.  It is normally not necessary to state the why or reason for the cancellation.  A canceling word or phrase may include:

- CANCEL

- RESUME

- RESUME NORMAL

- RELEASE

3.13	Cancellation messages should also identify which message is being cancelled, as several restrictions could be in place at one time.  An example of an ATFM cancellation message follows:

- ATCSCC, this is ATMC…We have canceled the restriction on traffic beyond the Fukuoka FIR at this time. Resume normal traffic flow.

4.	Abbreviations

4.1        The abbreviations used by the ATCSCC and ATMC that are not defined in the ICAO Doc. 8400 (PANS-ABC), are shown in the appendix.  The shaded abbreviations are considered to be the common terms between the two centers.

4.2        The non-common abbreviations are deemed inappropriate for the inter-facility ATFM communication between ATCSCC and ATMC.

Table of Abbreviations

The abbreviations listed here are those used by ATCSCC and ATMC respectively that are not defined in the ICAO Doc. 8400 (PANS-ABC).  The shaded abbreviations are considered to be the common terms between the two centers.  The asterisk shows verbatim difference in the original collocation but the abbreviation still indicates the common object.

|  | ATCSCC | ATMC |
| --- | --- | --- |
| AAR | Airport Acceptance Rate |  |
| ACID | Aircraft Identification |  |
| ADL | Aggregate Demand List |  |
| ADR | Airport Departure Rate |  |
| ADZY | Advisory |  |
| AIM | Aeronautical Information Manual |  |
| ALTRV | Altitude Reservation | Altitude Reservation |
| ANP | Air Navigation Plan |  |
| AOA | Office of the Administrator |  |
| AOC | Airline Operations Center |  |
| AP | Air Patrol |  |
| APREQ | Approval Request | Approval Request |
| APVL | Approval | Approval |
| ARINC | Aeronautical Radio Incorporated |  |
| ARO | Airport Reservation Office |  |
| ARTCC | Air Route Traffic Control Center | Air Route Traffic Control Center |
| ARU | Airspace Reservation Unit (Canada) |  |
| ASM |  | Airspace Management |
| AT | Air Traffic |  |
| ATCSCC | Air Traffic Control System Command Center | Air Traffic Control System Command Center |
| ATMC | Air Traffic Management Center | Air Traffic Management Center |
| ATMetC |  | Air Traffic Meteorological Center |
| ATO | Air Traffic Operations Program |  |
| AUTODIN | Automatic Digital Network |  |
| CARF | Central Altitude Reservation Function |  |
| CCFP | Collaborative Collective Forecast Product |  |
| CCWSU | Command Center Weather Service Unit |  |
| CDM | Collaborative Decision Making | Collaborative Decision Making |
| CDR | Coded Departure Route(s) | Conditional Route |
| CDR | Continuous Data Recording |  |
| CDT | Controlled Departure Time |  |
| CFR | Code of Federal Regulations (formerly FAR) |  |
| CIWS | Corridor Integrated Weather System |  |
| COMSEC | Communications Security System |  |
| CR | Collaborative Routing |  |
| CT | Select Flights Ground Delay Program |  |
| CTA | Controlled Time of Arrival |  |
| CTAS-TMA | Center TRACON Automation System Traffic Management Advisor |  |
| CVRS | Computerized Voice Reservation System |  |
| CWA | Central Weather Advisory |  |
| CWSU | Center Weather Service Unit |  |
| DARC | Direct Access Radar Channel |  |
| DCCWU | ATCSCC Weather Unit |  |
| DOTS | Dynamic Ocean Track System | Dynamic Ocean Track System |
| DP | Departure Procedure |  |
| DSP | Departure Sequencing Program |  |
| EDCT | Expected Departure Clearance Time | Expected Departure Clearance Time |
| EFAS | Enroute Flight Advisory Service |  |
| EFTO | Encrypt For Transmission Only |  |
| EOF | Emergency Operations Facility |  |
| EOR | Emergency Operations Room |  |
| EPS | Engineered Performance Standards |  |
| ESCAT | Emergency Security Control of Air Traffic |  |
| ETE | Estimated Time Enroute | Estimated Time Enroute |
| ETMS | Enhanced Traffic Management System |  |
| EUCARF | European Central Altitude Reservation Facility |  |
| FA | General Ground Delay Program |  |
| FAA | Federal Aviation Administration | Federal Aviation Administration |
| FADT | Fuel Advisory Delay Time |  |
| FCA | Flow Constrained Area |  |
| FDMS |  | Flight Data Management System |
| FDPS |  | Flight Data Processing Section |
| FEA | Flow Evaluation Area |  |
| FP | Flight Plan |  |
| FPL | Full Performance Level |  |
| GA | General Aviation |  |
| GAAP | General Aviation Airport Program |  |
| GDP | Ground Delay Program |  |
| GS | Ground Stop |  |
| HARS | High Altitude Route System |  |
| HDTA | High Density Traffic Airport |  |
| IFCN | Interfacility Communication Network |  |
| IFPFP | Individual Flight Plan From this Point | Individual Flight Plan From this Point |
| IFSS | International Flight Service Station |  |
| INATS | Interruption of Air Traffic Service |  |
| JCAB | Japan Civil Aviation Bureau | Japan Civil Aviation Bureau |
| LAA | Local Airport Advisory |  |
| LADP | Local Airport Deicing Plan |  |
| LOA | Letter of Agreement | Letter of Agreement |
| MAP | Monitor Alert Parameter |  |
| MARSA | Military Assumes Responsibility for Separation of Aircraft | Military Assumes Responsibility for Separation of Aircraft |
| MEL | Minimum Equipment List |  |
| MINIT | Minutes in Trail |  |
| MIT | Miles in Trail |  |
| MOS | Military Operations Specialist |  |
| MTSAT | Multi-functional Transport Satellite | Multi-functional Transport Satellite |
| MVFR | Marginal Visual Flight Rules |  |
| NADIN | National Airspace Data Interchange Network |  |
| NAS | National Airspace System |  |
| NAVAID* | Navigational Aid | Navigation Aid |
| NFDC | National Flight Data Center |  |
| NMCC | National Maintenance Coordination Center |  |
| NOAA | National Oceanic and Atmospheric Administration |  |
| NOM | National Operations Manager |  |
| NOPAC | North Pacific | North Pacific |
| NOS | National Oceanographic Service |  |
| NRP | National Route Program |  |
| NTMO | National Traffic Management Officer |  |
| NWS | National Weather Service |  |
| OAG | Official Airline Guide |  |
| ODP |  | Oceanic Air Traffic Control Data Processing System |
| OPSNET | Operations Network |  |
| OTG |  | Oceanic Track Generator |
| OTR |  | Oceanic Transition Route |
| PACMARF* | Pacific Military Altitude Reservation Facility | Pacific Military Altitude Reservation Function |
| PACOTS | Pacific Organized Track System | Pacific Organized Track System |
| PMTC | Pacific Missile Test Center |  |
| PO | Plan of Operation |  |
| Pref Route | Preferential Route |  |
| PT | Planning Team |  |
| RA | Route Advisory |  |
| RAA | Remote Airport Advisory |  |
| ROT | Runway Occupancy Time |  |
| SAA | Special Activity Airspace |  |
| SOP | Standard Operating Procedure |  |
| STMP | Special Traffic Management Program |  |
| SUA | Special Use Airspace |  |
| SVRW | Severe Weather |  |
| SWAP | Severe Weather Avoidance Program |  |
| TEC | Tower-Enroute Control |  |
| TELCON | Telephone Conference |  |
| TFM | Traffic Flow Management |  |
| TIS | Traffic Information System |  |
| TMC | Traffic Management Coordinator | Traffic Management Coordinator |
| TMCIC | Traffic Management Coordinator in Charge |  |
| TMI | Traffic Management Initiative |  |
| TMU | Traffic Management Unit | Traffic Management Unit |
| TSTM | Thunderstorm |  |
| WSO | Weather Service Office |  |