THE TWENTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/25)

(Tokyo, Japan, 24-27 October 2006)

Agenda Item 6:  Air Traffic Management (ATM) Issues

Action Plan for Expansion of 30 NM Lateral / 30 NM Longitudinal Separation (30/30)

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides a status update on the planned expansion of 30 NM lateral / 30 NM longitudinal separation (30/30) in oceanic airspace delegated to the United States, including expansion of the 30/30 operational trials to additional airspace volumes within the Oakland Oceanic FIR.

1.		Background

  - At the IPACG/24 meeting held in January 2006, the Federal Aviation Administration (FAA) presented information regarding the success of the operational trial for the application of 30 NM lateral / 30 NM longitudinal separation (30/30) in Oakland Center Oceanic Control Sector 3 (OC3) for Reduced Navigational Performance (RNP) 4 approved aircraft.

  - Based on the outcome of the operational trial, FAA committed to expand the use and    application of 30/30 into at least two (2) additional airspace volumes within the Oakland Oceanic FIR by March 2007.  A further expansion schedule has not yet been determined.

2.	Discussion

  - Phase I of the plan expands the operational trial and application of 30/30 to the entire Oakland Oceanic FIR on a target of opportunity basis.  Two specific targets of opportunity are envisioned.  The most common is expected to be 30/30 eligible aircraft pairs that are longitudinally separated by more than 30 NM, but less than the smallest existing applicable standard.  The second involves 30/30 eligible aircraft that have deviated, offset or are flying a random route and will continue to be laterally separated from other 30/30 eligible aircraft by less than the current 50 NM standard, but not less than 30 NM.

  - The FAA established a 30/30 Scrutiny Group (SG) to evaluate the OC3 30/30 operational trial.  The SG examined data from the operational trial and data link communications within the entire Oakland Oceanic FIR.  The SG concluded that the March 2007 target date for expansion of 30/30 within the Oakland FIR is realistic and attainable provided certain issues are mitigated or resolved.  Three (3) issues are considered serious and, if not corrected, will force postponement of the planned March 2007 operational trial expansion.

    - Availability of Ground-Earth Stations (GES) is an issue.  The major concern is the frequency and duration of unplanned outages.  The Perth GES is scheduled to be upgraded in October; SG will review the November data to determine whether performance and availability has improved.  The communication service provider has announced upgrades for the Santa Paula GES; availability of Santa Paula will be monitored.

      - Mean Time Between Failure (MTBF), when combined with Mean Time To Repair (MTTR), will be monitored as an indication of availability.  A short MTBF will prevent expansion of the 30/30 operational trial until the reliability issues are resolved.

    - Overall message delivery times are an issue.  FOM performance targets are not being met, and the trend is that the proportion of messages greater than FOM targets is increasing.  It is anticipated that the upgrades mentioned in 2.2.1.1, above, will contribute to improvement in this area of concern.  Unless this trend is arrested, this will jeopardize expansion of the operational trial.

    - Lateral deviations, with or without a clearance, increase risk in the application of any reduced separation standard (e.g., weather deviations).  It is imperative that the controller be able to retain situational awareness during these maneuvers.  Data scrutinized to date may lead to an increased automatic update rate in the Ocean21 software.

  - The structure and timing of additional expansion phases will be determined by the results of     the Phase I expansion of the operational trial, including analysis of collected data, a collision risk assessment and a Safety Risk Management assessment.  Phase II foresees Oakland FIR-wide implementation of 30/30 to the maximum extent possible; FAA anticipates redesigning route structures to take advantage of the lateral component of the 30/30 standard as part of this effort.  Phases III and IV will expand application of 30/30 to the Anchorage Center (ZAN) and New York Center (ZNY) oceanic airspaces, respectively.

3.	Recommendation

3.1	The meeting is invited to discuss the information presented in this paper and continue to support the work of the 30/30 Task Force as 30/30 implementation continues and expands in other oceanic airspaces.

- END -