THE TWENTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/26)

(Anchorage, AK, 15 - 18 May 2007)

Agenda Item 6                        	Air Traffic Management (ATM) Issues

Consideration regarding KZOA OC5/6 revised routes

(Presented by Japan Civil Aviation Bureau)

Summary

At the Twenty-fifth meeting of the Informal Pacific ATC Coordinating Group  (IPACG/25), Oakland ARTCC proposed establishing revised routes in OC5/6 as the structure for RNP10 equipped aircraft. ATMC has given consideration to the operational issues or concerns in Fukuoka FIR.

- Background

1.1   The number of RNP10 equipped aircraft flying over the Pacific Ocean has been increasing year by year.

1.2   The route structures designed for the conventional lateral separation still exist, while NOPAC, PACOTS and some others are designed for RNP10 equipped aircraft.

1.3   ATMC agrees to the concept of the proposal which Oakland ARTCC explained about the revised routes in OC5/6.

1.4   ATMC has considered the proposal and brought some requirements on the operational issues or concerns in Fukuoka FIR.

- Discussion

2.1   When PACOTS Track H/I goes through Oakland sector OC2 and Track A goes through Oakland sector OC9 and then these tracks converge in the area of ATMC sector A03, ATMC currently initiates establishing a required separation between aircraft on Track H/I and aircraft on Track A.

2.1.1   ATMC is expecting that Oakland ARTCC will initiate establishing a required separation between aircraft on G339 and aircraft on A216 for southbound traffic. The reason is that G339 is controlled by sector A05 while A597 MONPI A216 is controlled by sector A04. (Refer to yellow arrows in the attached map)

2.1.2   ATMC is also expecting that Oakland ARTCC will initiate establishing a required separation between RNP10 equipped aircraft and Non-RNP10 aircraft on adjacent airways for southbound traffic. The necessity to change the stipulation about informing of Non-RNP10 aircraft in the LOA between Oakland ARTCC and ATMC might be examined. (Refer to blue arrows in the attached map)

2.2   BEEBO and RISBA should be modified by JCAB and FAA coincidently because the waypoints are described on the published documents such as AIP.

- Recommendation

  - The meeting is invited to note the information presented in this paper.

  - JCAB essentially intends to support the project.