Summary of Discussions

Twenty-Sixth Meeting of the

Informal Pacific Air Traffic Control Coordinating Group

(IPACG/26)

14-18 May 2007

Anchorage, Alaska USA

1.0 	Background

1.1 	The Twenty-Sixth Meeting of the Informal Pacific Air Traffic Control (ATC) Coordinating Group (IPACG/26) was held at the Anchorage New Federal Building in Anchorage, Alaska, from 14-18 May 2007.  The IPACG was established to provide a forum for air traffic service providers and airspace users to informally meet and explore solutions to near term ATC problems that limit capacity or efficiency within the Anchorage, Oakland, and Fukuoka Flight Information Regions (FIR).

2.0 	Welcome and Opening Remarks

  - The meeting was co-chaired by Mr. David Maynard, Manager, Oceanic and Offshore Operations, Federal Aviation Administration (FAA) and Mr. Satoshi Yamaguchi, Special Assistant to the Director, ATC Division, Japan Civil Aviation Bureau (JCAB). Mr. Maynard welcomed the participants to the meeting on behalf of the FAA.  Mr. Maynard noted that much progress had been made since the last IPACG meeting.  He encouraged active participation in discussions and was sure the participants would be pleased with the outcomes of the meeting.   Mr. Maynard introduced Mr. Kevin Ford, Manager Anchorage Center who welcomed the participants on behalf of the Western Service Area Director, Mr. Steve Osterdahl.

2.2	Mr. Yamaguchi, in his opening remarks, thanked the FAA team and Mr. Maynard for their hospitality, and stated that he was pleased to be back with the IPACG after a 4 year hiatus.

2.3	Mr. Maynard recognized Mr. Reed Sladen, Manager, Automation and Hiroshi Inoguchi, Special Assistant to the Director, ATS System Planning Division of the JCAB as co-chairs of the Thirteenth Meeting of the FANS Interoperability Team (FIT/13) meeting.

3.0	Submitted Papers

3.1	The following working and information papers were presented to IPACG/26 and were available on the IPACG website http://www.faa.gov/ats/ato/ipacg.htm.

| Paper Number | Agenda Item | Title | Presented by |
| --- | --- | --- | --- |
| IP/01 | 6 | Use of 30 NM Lateral / 30 NM Longitudinal Separation (30/30) | FAA |
| IP/02 | 6 | Advanced Communication for Air Traffic Flow Management (ATFM) Using Internet and Web Conferencing Technology | FAA |
| IP/03 | 6 | Report on the Business Case Development for ADS-C In-Trail Procedures | FAA |
| IP/04 | 6 | Airspace Effectiveness Review | FAA |
| IP/05 | 6 | Report on the Progress of DOTS+ Online Track Advisory | FAA |
| IP/06 | 6 | Report on FAA Central East Pacific Route Analysis | FAA |
| IP/07 | 6 | Plan for New Procedure of Establishing Hawaiian Tracks | JCAB |
| IP/08 | 6 | Brief Description of the Safety Assessment Process and Data Requirements to Support Operational-Trial Use of ADS-based Separation Standards In the Anchorage Flight Information Region | FAA |
| IP/09 | 6 | Status of the Operational Trials and Demonstrations for 10 Minute Longitudinal Separation Without Mandatory Mach Number Technique (MNT) in the Anchorage Flight Information Region (FIR) | FAA |
| WP/01 | 1 | Proposed Agenda and Timetable | Co-Chairs |
| WP/01a | 1 | Meeting Location and List of Information/Working Papers | Co-Chairs |
| WP/02 | 6 | Implementation of an Operational Trial Utilizing a Reduced Longitudinal Separation Minima in the Anchorage Oceanic Flight Information Region (FIR) | FAA |
| WP/03 | 6 | Russia Far East Trials | FAA |
| WP/04 | 6 | Proposed Changes to ATS Routes in the Oakland Flight Information Region (FIR) and Oceanic Control Area (OCA) Boundaries | FAA |
| WP/05 | 6 | Proposal for a Shared FAA/JCAB Oceanic Performance Measurement | FAA |
| WP/06 | 6 | Implementation of User Preferred Routes | FAA |
| WP/07 | 6 | The up-dated information of the 50 NM longitudinal separation based on the ADS within the Fukuoka Flight Information Region | JCAB |
| WP/08 | 6 | Study of a flexible routes system between Australia and Japan | JCAB |
| WP/09 | 6 | Implementation of RNAV5 and RNAV1 in Japan | JCAB |
| WP/10 | 6 | Consideration regarding KZOA OC5/6 revised routes | JCAB |
| WP/11 | 6 | Variations in True Airspeed Observed During the Operational Trial-Use of 30-nm Lateral and Longitudinal Separation Standards in Oakland Oceanic Airspace | FAA |

- Agenda Item 1:  Review and Approve Agenda

Proposed Agenda and Timetable

  - The following Agenda was proposed and adopted by the meeting:

Agenda Item 1		Review and approve agenda

Agenda Item 2   	Report on the outcome of the FANS Interoperability Team (FIT) Meeting

Agenda Item 3		Report on relevant outcomes from other meetings

Agenda Item 4	Report Review of relevant route requirements in the Asia/Pacific ATS Route Catalogue

Agenda Item 5  		Communications/Navigation/Surveillance (CNS) Issues

Agenda Item 6 		Air Traffic Management (ATM) Issues

Agenda Item 7		Develop necessary State contingency plans

Agenda Item 8	Review and Update of CNS/ATM Planning chart and Action Items

Agenda Item 9	Other Business

4.2	The Co-Chairs reviewed the proposed timetable and amended it as needed during progress of the meeting.  Mr. Maynard invited all participants to attend a reception on Wednesday evening sponsored by ARINC and SITA.

5.0	Agenda Item 2:  Report on the outcome of the Future Air Navigation System (FANS) Interoperability Team (FIT) Meeting

5.1	FIT Co-chairs presented a report of FIT/13, held on 14-15 May 2007, for review and comment.

5.2	This report is attached at Appendix B to this summary.

6.0 	Agenda Item 3: Report on relevant outcomes from other meetings

6.1	On Agenda Item 3, Mr. Maynard relayed regrets from the ICAO Asia Pacific Regional Office at not being able to participate in the meeting.

Report on the 21st. Meeting of the Informal South Pacific Air Traffic Services (ATS) Coordination Group (ISPACG/21)

6.2	Mr. Maynard provided a summary of the ISPACG 21 meeting.   He announced that since 13 March, FAA had expanded use of Automatic Dependent Surveillance (ADS) based separation throughout Oakland FIR though in a restricted fashion.   Oakland FIR would not allow aircraft to be at the same altitude in trail, only when climbing or descending.

6.2.1  	Even though the FAA made this restriction, the other ISPACG States chose not to restrict ADS-based flights in any way.  This determination was based on the lower percentage of Future Air Navigation System (FANS) equipped aircraft.

6.2.2	New Zealand presented a paper on variation in airspeed of aircraft.  The main point was that there was a distinct difference in interpretation by the air navigation service providers (ANSPs) and operators of Annex 2 of the Chicago Convention regarding change in airspeed.  Annex 2 states that "inadvertent speed changes" do not need to be reported to air traffic control (ATC).  Operators interpret Annex 2 to allow them to make changes in airspeed without having to notify ATC.  ATC, on the other hand believe operators must maintain airspeed or file a difference. The implications of these interpretations are significant, especially in the context of reduced separation.  The ISPACG has identified this topic as a top priority to address at its next meeting.  New Zealand planned to present a working paper on this issue at the Separation and Airspace Safety Panel (SASP) Working Group in Montreal during the week of 21 May.

Report on the Third Meeting of the Cross Polar Trans-East ATM Working Group (Cross Polar WG)

6.3 	The FAA provided a summary of the Cross Polar WG that was held in Washington DC, USA from 24-26 April 2007.

6.3.1	The FAA provided background on how the WG was formed.  Cooperative work on issues of Air Traffic Management and Communications (ATM), Navigation and Surveillance (CNS) as regard Cross Polar traffic, i.e. Flights between North America and Asia via Canadian, US and Russian controlled airspace began in earnest in 1993 with the first meeting of the Russian/American Coordinating Group for Air Traffic Control (RACGAT).  Over the years, RACGAT enjoyed many successes and was instrumental in the in the creation of the Cross Polar ATS route structure that now exists.  Unfortunately, and due to a variety of causes, including organizational restructuring of ANSPs, RACGAT's effectiveness as a medium for change diminished.  The last RACGAT meeting occurred in 2003.  In March 2006, NAV CANADA, FAA, the Federal Air Navigation Authority (FANA) and State Corporation for Russia, and several air carriers held a technical working group meeting, which became known as the Cross Polar WG.  A subsequent meeting was held in September 2006 in Montreal, Quebec, Canada.

6.3.2	The group established Terms of Reference (TOR), carefully crafted to accurately reflect the areas, i.e. traffic flows that the group would address.

6.3.3	Of particular interest to the IPACG is the group's work regarding the realignment and expansion of ATS route B932.  The route, currently established as a trial, provides an alternate routing for North Pacific (NOPAC) traffic north of ATS route R220.  Unfortunately, B932, as currently aligned, and with its current restrictions, such as unidirectional flight and lengthy in-trail separation minimums, provides limited utility to the user.  The group agreed to take on the work necessary to improve the route's utility by realigning it to de-conflict with R220 traffic, increase its operational hours and reduce the required in-trail separation.

6.4	IATA commented that transitioning from flight levels to metric altitudes had an adverse impact on RVSM and NOPAC operations.  The FAA and JCAB supported IATA’s comments.

7.0	Agenda Item 4:  Report Review of relevant route requirements in the Asia/Pacific ATS Route catalogue

7.1	No changes to the Asia/Pacific ATS Route Catalogue were submitted.

8.0	Agenda Item 5:  Communications/Navigation/Surveillance (CNS) Issues

8.1	No matters were discussed under this agenda item.

9.0	Air Traffic Management (ATM) Issues

The up-dated information of the 50 NM longitudinal separation based on the ADS within the Fukuoka Flight Information Region

9.1	JCAB provided updated status of operation for the 50 nautical miles longitudinal separation based on Automatic Dependent Surveillance, (ADS) within the Fukuoka FIR and future plans for that expansion.

9.2	On 12 April, 2007, Fukuoka Air Traffic Management Center (ATMC) expanded application of the 50NM longitudinal separation to aircraft flying at cruise within the Fukuoka FIR. JCAB had been continuously collecting the data and evaluating the status of the internal operation.

9.3	JCAB has been considering seamless operation of the 50NM longitudinal separation between Fukuoka FIR and Oakland FIR as well as between Fukuoka FIR and Anchorage FIR and plans to introduce these in a phased-manner.  JCAB will be ready to start the first step by 2008.  This first step should be limited to specific routes or airspace.  After that, JCAB can gradually expand the seamless operation to the other routes or airspace in this region based upon the evaluation of this limited application.  JCAB will provide an update to IPACG/27.

Use of 30 NM Lateral/30 NM Longitudinal Separation (30/30)

9.4	The FAA provided a status update on the use of 30/30 in the Oakland Flight Information Region (FIR).

9.5	Oakland Air Route Traffic Control Center (ZOA) introduced the 30/30 separation standard on a trial basis in Oceanic Control Sector 3 (OC3) in December 2005.

9.6     	As part of the trial, the FAA established a Scrutiny Group to evaluate the OC3 30/30 operational trial.  The Scrutiny Group was tasked to examine data from the operational trial and data link communications within the entire Oakland FIR.  The initial findings of the Scrutiny Group indicated that the initial trials were a success with one exception.  The exception was the reliability of the Perth Ground Earth Station (GES) which had suffered several outages over the course of the 30/30 trials.  Based on these findings, the FAA began work to expand the use of 30/30 to the rest of the Oakland FIR.

9.7   	During February 2007, the Perth GES again suffered from outages.  The FAA consulted directly with SITA on the causal factors for these outages and any planned remedies.  SITA advised the FAA that one of the causal factors was still undetermined and another identified but would not be corrected until the 2nd quarter of calendar year 2007.

9.8  	Based on this information and the knowledge that the Perth GES was still subject to outages due to known and unknown problems, the FAA made the determination to expand the 30/30 trials on the planned date of 13 March, 2007 but with a different operational concept.

  - In the interest of safety, the FAA decided to restrict the use of Automatic Dependant Surveillance, (ADS), based separation (30/30 and 50NM longitudinal).  ADS-based separation would only be authorized when climbing/descending through the altitude of another aircraft and/or when opposite direction traffic has passed each other and the appropriate distance had been achieved.

9.10  	In making this determination, the FAA looked at several scenarios, from terminating the 30/30 trials to disallowing the use of the Perth GES for data-link.  The restricted approach was seen to provide the best opportunity to continue to use ADS based separation while maintaining safety of flight within the Oakland FIR.

9.11	On 18 April, SITA successfully made the last planned calendar year 2007 upgrade to the Perth GES.  This action corrected all of the known problems with the Perth GES that caused the FAA to restrict the use of reduced separation standards that rely on data-link services (controller pilot data link communication, CPDLC, and ADS-C).

9.12	Based on this information, the FAA Scrutiny Group has recommended that the current restrictions be cancelled.  The FAA’s Oceanic Separation Reduction Working Group (OSRWG) has concurred with this recommendation.

9.13   ZOA has been given approval to remove the previously imposed restrictions.  Oakland ARTCC is now in the process of determining a date for the change to occur.

Brief Description of the Safety Assessment Process and Data Requirements to Support Operational-Trial Use of ADS-based Separation Standards in the Anchorage FIR

9.14	The FAA provided a brief description of the safety assessment process and data requirements for ADS-based separation standards in the Anchorage FIR and a preliminary summary of ADS operations observed to date in Anchorage oceanic airspace.

9.15	On 1 March 2007, the Ocean21 oceanic automation system was introduced into full-time operation at the Anchorage Air Route Traffic Control Center (ZAN) in oceanic sectors 10 and 11.  This event followed the introduction of the Ocean21 system at ZOA in October 2005.  The FAA initiated the use of the 50NM longitudinal separation standard in the Oakland FIR with the introduction of the Ocean21 system in October 2005.

9.16	After demonstration that necessary performance is adequate, the FAA intends to extend use of the 50NM longitudinal separation standard to portions of the Anchorage FIR.

9.17	The FAA presented a summary of the data sources that will be used to complete a safety assessment for the implementation of the ADS-based separation standards in Anchorage oceanic airspace.  The FAA also summarized the safety assessment methodology.

9.18	Preliminary estimates of ADS operations show that the average number of aircraft utilizing ADS in Anchorage oceanic airspace is 98 flights per day.  The average of number of ADS flights filing required navigation performance (RNP)-4 in their flight plans and using ADS is 13 flights per day.  There was extensive discussion by the meeting on the number of ADS flights for filing RNP-4.  United Airlines clarified that all their aircraft going through Anchorage FIR file RNP-10 in their flight plans.  Their Boeing 777 aircraft are RNP-4 capable but file as RNP-10 since there is additional cost without benefits filing as RNP-4.  Japan Airlines stated that JCAB is undergoing establishment of RNP-4 approval process which is expected in September.  Until that time, JAL will file RNP-10 for their aircraft which have RNP-4 capabilities.

9.19	The anticipated minimum data collection period needed to complete the safety assessment for the implementation of the 50NM longitudinal separation standard in Anchorage oceanic airspace is 3 months.  However, a sufficient number of observed ADS aircraft pairs separated by the longitudinal separation standard are needed to compute critical collision risk parameters.  Therefore, this data collection time period estimate is subject to change depending on a satisfactory number of observed longitudinally separated ADS aircraft pairs in data.

Variations in True Airspeed Observed During the Operational Trial-Use of 30NM Lateral and Longitudinal Separation Standards in Oakland Oceanic Airspace

9.20	The FAA provided results of preliminary analysis used to examine variations in aircraft true airspeed or Mach number evidenced in periodic ADS reports received at Oakland Oceanic Area Center (OAC).   The data used in this analysis are the aircraft filed flight plans, electronic coordination messages, and actual aircraft ADS reports collected from the Ocean21 system.  The data sample time period included 1 January through 31 March 2007.

9.21	Comparisons between the filed and actual airspeed were made in the analysis.  The sources of filed true airspeed or Mach number were the electronic coordination messages received by ZOA from adjacent ATS providers and the original filed flight plans.  When the true airspeed was provided, conversion to Mach number was completed using the Manual of the ICAO Standard Atmosphere. The analysis utilized the instantaneous Mach number provided in the ADS periodic reports as the actual airspeed.

9.22 	The proportions of ADS position reports with a difference between the filed speed and the ADS-reported Mach number greater than or equal to the five percent threshold were not uniform across all observed sample days, operators, city pairs and aircraft types.  Further investigation into the data will clarify the filed aircraft speeds against the available electronic HF and CPDLC messages contained within the Ocean21 data.

9.23 	ANSPs voiced their concerns over aircraft making speed changes without prior approval from ATC.  This is becoming more of a concern as reduced longitudinal standards are implemented.

9.24	IATA commented that airlines are flying by cost index, so there will be planned differences in speed on long-haul routes that should already be filed in Item 15 of the ICAO flight plan.  The question is then, does an already filed change of speed in the ICAO flight plan obligate the pilot to notify ATC, by referring to Annex 2, which states pilots are required to notify ATC of changes in airspeed of 5% or .01 Mach if speed “inadvertently deviates from its current flight plan”?   The implications of these interpretations, while maintaining a 30NM longitudinal minimum are significant.  Until this is resolved by a common understanding of the rules, IATA suggested the ATC assign a Mach number if the 30NM longitudinal rule is used for sustained application of longitudinal separation.  The FAA/JCAB responded that there are currently procedures contained in the PANS-ATM that address the situation.

9.25	Based on the discussions it was quite evident that there is a difference in how the operators, pilots and ANSPs interpret section 3.6.2.2 (b), of Annex 2, inadvertent deviations in true airspeed and the filing of a change of speed in Item 15 of the ICAO flight plan.  IPACG agreed that this needs to be clarified with ICAO to ensure a common understanding and application.  Also the current percentage allowed (plus or minus 5%) needs to be reviewed to ensure that it is valid for today’s operating environment.

Implementation of an Operational Trial Utilizing a Reduced Longitudinal Separation Minima in the Anchorage Oceanic FIR

9.26	The FAA provided details of an operational trial of a reduced longitudinal separation minima being conducted within the Anchorage FIR.

9.27	The current standard longitudinal separation minima available within the Anchorage Oceanic FIR is 15 minutes.  Successful implementation of a reduced standard, i.e. 10 minutes, has been achieved in the Oakland Oceanic FIR.  Based upon the experience obtained in the Oakland FIR, and upon the successful implementation of FAA’s Advanced Technologies and Oceanic Procedures (ATOP) automation system, i.e. Ocean21, at ZAN, the decision was taken to implement an operational trial of a reduced longitudinal separation minima, i.e. 10 minutes, within the Anchorage Oceanic FIR.

9.28	The trial began on 12 April and is expected to be completed by the end of the calendar year.  By that time sufficient data should have been collected to permit the completion of the required safety analysis.  The expectation is that the safety analysis will be successful and that the standard will be permanently adopted.

9.29	The FAA requested that JCAB consider the implementation of this standard in order to harmonize procedures.  JCAB agreed that they would consider this request and provide an update at IPACG/27.  JCAB requested that the FAA provide the methodology that they are utilizing to conduct the safety assessment associated with this trial.  FAA is in agreement.

Report on the Business Case Development for ADS-C In-Trail Procedures (ITP)

9.30	The FAA is continuing the background work associated with the development of an in-trail climb through and descent through procedure using ADS-C capabilities.  The FAA is currently focusing on the development of a business case for the procedure.  The safety case for this procedure is currently based on a 10NM distance between aircraft.

9.31	 The International Federation of Air Line Pilots Association (IFALPA) voiced a concern over who had the responsibility for the application of the procedure and separation assurance.  FAA clarified that the responsibility for the application of the procedure and separation assurance would be the responsibility of the controller not the pilot.  The ground systems will be responsible for the measurement of the distance between the aircraft.

9.31.1      JCAB mentioned that it should be reviewed in parallel with the business case if the assumption of 10NM as ITP separation would be allowed or possible in the current operational environment.

9.32	Pending successful completion of the ITP benefits case, the FAA will share the benefits findings with airlines, industry and global air navigation service providers for partnership in the planning of ADS-C ITP development. The FAA will provide an update on the progress of the business case development at IPACG/27.

Airspace Effectiveness Review

9.33	The FAA provided an update on ZOA’s review of the effectiveness, as follows:

9.33.1	The ADS based separation standards are being applied throughout the Oakland Control Area (CTA).

9.33.2	User Preferred Routes (UPRs) are being expanded, where feasible, within the Oakland CTA.

9.33.3	The ATS Route structures within the Oakland CTA have been reviewed and two areas have been identified for improvement.  Oakland is coordinating with adjacent ANSPs, to revise the ATS routes northwest of Guam and around Hawaii.  These changes will shorten the routes lengths by a combined 177 nautical miles.  This equates to an approximate savings of over 330,000 flying miles annually.

9.33.4	The FAA is beginning a review of the constraints on Pacific Organized Track System, (PACOTS), track generation within the airspace they are responsible to provide ATC services.  This review will provide improvements in the efficiency of PACOTS Tracks.

9.33.5	The Russia Far East, (RFE) trials have demonstrated large savings and an improvement in efficiency.

9.33.6	The FAA received approval from ICAO to implement 10 minutes longitudinal separation between turbojet aircraft in the Oakland FIR as a result of their 10 minute no Mach speed assignment trials.

9.33.7	The FAA is conducting a study of the Central East Pacific Route System (CEP), to look for the most efficient means of improving that airspace structure.

9.33.8	The FAA is looking into the feasibility of establishing an ADS-C ITP.

9.33.9	The airline operators suggested that the airspace review should also consider the continued effectiveness of the NOPAC route system as a fixed route verses flex route system. There are constraints associated with NOPAC routes that also should be reviewed.  They noted that recently that the Russian Federation has plans to implement several new routes in the Russian Trans East route system that will provide improved efficiency, reduce delays, and overall improve the operations.  It illustrates that “all” airspace should have a continual review.  This suggestion was noted by FAA and JCAB.

9.34	Another concern raised by the airline operators was the traffic flow between North America, Hawaii and Japan. They noted that the Wake Island Airport has not been available as an extended twin-engine operations (ETOPS) en-route alternate since Super Typhoon Ioke struck Wake in August 2006.  The U.S. Department of Defense (DOD) is planning to completely decommission the Wake Airport prior to the end of this year.  That would leave Midway Island Airport as the only available mid-Pacific en-route alternate airport. If any operational “hiccups” occur with Midway, it will be necessary for ETOPS aircraft to fly random routes to and from Japan, Hawaii and North America. The airline operators stated it was therefore essential that any current constraints in this traffic flow be identified and hopefully resolved in the future.

Proposed Changes to ATS Routes in the Oakland Flight Information Region (FIR) and Oceanic Control Area (OCA) Boundaries

9.35	The FAA provided details on proposed ATS route changes in the Oakland FIR and Oceanic Control boundaries.

9.36	The FAA stated that Ocean21, the air traffic control system utilized within Oakland FIR has been operational for just under two years.  The enhanced capabilities available through this system allow for application of reduced separation standards.  With this enhanced capability, and in support of the ICAO’s efforts to reduce operator costs and ecological impacts, the Federal Aviation Administration, (FAA), has been evaluating ATS routes and airspace structure in the vicinities of Guam and Hawaii.

9.37	The proposed ATS changes are realignment of ATS routes in the vicinity of Guam, realignment of ATS routes from Hawaii to South Pacific destinations, and realignment of CEP waypoints and A332 between the Continental United States.

9.38	The FAA stated its willingness to work with other ANSPs to develop an Action Plan to realign the proposed ATS routes and would consider further changes as appropriate.  The FAA assured that careful consideration would be given to ensure that route realignments will not create adverse effects.

9.39		Initial concerns raised by JCAB were resolved to the mutual satisfaction of JCAB and FAA.  In response, JCAB expressed agreement to the realignment of the ATS routes as proposed.  Tentative target date is 30 August 2007.

Implementation of RNAV5 and RNAV1 in Japan

9.40	JCAB provided an update on its implementation plans for implementing RNAV5/1 in Japan.  The RNAV Roadmap was updated in April 2007, and RNAV applications to be implemented in Japan will be fully consistent with the ICAO Performance Based Navigation (PBN) concept.  JCAB will start implementation of RNAV1 and RNAV 5, from 27 September 2007 and early 2008 respectively, in a phased manner.

9.41	JCAB also briefed on the requirement to indicate the approval status of RNAV/RNP in items 10 and 18 in the flight plan.

Consideration regarding KZOA OC5/6 revised routes

9.42	This paper was JCAB’s response to FAA WP/4, Proposed Changes to ATS Routes in the Oakland FIR and OCA Boundaries.  As previously noted, JCAB concurred with FAA proposal.

Report on FAA Central East Pacific Route Analysis

9.43	The FAA presented an update on the FAA CEP routes.  Based on the request of several airline partners, the FAA, with MITRE/CAASD, is conducting an airspace analysis on the fixed track network between Hawaii and the Continental US.  This track network is commonly referred to as the CEP.

9.44	Based on user feedback, the 2007 study will focus on three options for the airspace: Option 1:  Flex CEP tracks with same entry/exit fixes; Option 2:  UPRs above a specified altitude (e.g., FL390); Option 3:  Flex tracks with reduced restrictions.

9.45	These options will be analyzed in sequence beginning with option 1.  The analysis will consider potential fuel savings and potential disbenefits associated with traffic volume and complexity.   FAA stated the target date for completion of the analysis is fall 2007.

Implementation of User Preferred Routes

9.46	The FAA noted that the Oakland Air Route Traffic Control Center (ZOA) and the other South Pacific ANSPs have been supporting UPRs since December 2000.  UPRs have proven to be a very cost effective means for airspace operators to flight plan. The FAA is continuing to look for areas of expansion.

9.47	FAA requested that the group support beginning UPR Paper Trials between the South Pacific and Japan.  The data from the paper trials may allow the ANSP and Users to carefully evaluate the impacts of these UPRs.

9.48	JCAB has agreed to allow the use of UPRs between the city pairs, Noumea (New Caledonia), Auckland, (New Zealand) and Narita (Japan).  Details for the use of UPRs will be coordinated with the appropriate ANSPs and operators.  JCAB and FAA will provide an update on progress at IPACG/27.

Study of a flexible route system between Australia and Japan

9.49	At the IPACG/25 meeting, the FAA proposed that flights from South Pacific to Japan be included in the UPR trial.  In response to this proposal, JCAB reported that they had commenced a study of a flexible route system between Australia and Japan including evaluation of advantage/disadvantage of UPR.  JCAB will provide an update at IPACG/27.

Russia Far East Trials

9.50	The FAA provided an update on the RFE Trials in the Oakland, Vancouver, and Anchorage CTAs.

9.51	In March 2006, the FAA, in conjunction with NAV CANADA and the users, conducted a three week airborne trial for flights departing the San Francisco area and Los Angeles basin between 0500 and 0900 Coordinated Universal Times (UTC).  Cathay Pacific was the only participant in the airborne trial.  Thirty two aircraft flew the minimum time track to the RFE routes.  The results from the trial showed an average savings of 6.3 minutes flying time per flight.  One flight saved 21 minutes flying time and was able to carry an additional 4.8 tons of cargo.

  - The FAA encouraged users to participate in the RFE Airborne Trial when they resume.

Plan for new Procedure of establishing Hawaiian Tracks

9.53	JCAB reported its plan for a new procedure for establishing Hawaiian Tracks, in which   a new wind forecast that is updated at an interval closer to the time of the track generation is utilized.  JCAB commenced an assessment in February 2007 to determine if there is any value in changing the timing of the track generation.  This assessment would continue, for getting seasonal particulars, until the beginning of 2008.

Report on the Progress of DOTS+ Online Track Advisory

9.54	The FAA is continuing the development of web-enabled enhancements to the Dynamic Ocean Tracking System Plus (DOTS+) Track Advisory Program. This enhancement is known as On-Line Track Advisory (OLTA). The FAA completed the development of the operational and technical requirements for OLTA in early May.

9.55	The FAA plans to conduct a forum for airline dispatchers and operators to see demonstrations of the OLTA interface in early summer 2007.  The OLTA system development is currently progressing towards an operational implementation in early 2008.

Proposal for a Shared FAA/JCAB Oceanic Performance Measurement

9.56	The FAA proposed a shared FAA/JCAB Oceanic Performance Measurement focusing on the measurement of filed versus actual flown trajectories. The initial proposal is to measure vertical profiles for flights between US and Japan city pairs on the PACOTS.

9.57 	There was a sidebar discussion between JCAB and FAA which resulted in agreement to pursue oceanic performance measurement.  The initial focus will be that of filed altitude versus assigned for aircraft operating on the PACOTS.  Progress will be reported at IPACG/27.

Advanced Communication for Air Traffic Flow Management (ATFM) Using Internet and Web Conferencing Technology

9.58 	The FAA provided information on the implementation of an advanced communications capability for ATFM using Internet and web conferencing technology.

9.59	The FAA Air Traffic Control System Command Center (ATCSCC) and ATMC conducted a test of GoTo Meeting on 17 April 2007.  During the meeting, the basic GoTo Meeting capabilities such as handing the resource control capability to other meeting participants viewing organizer’s computer screens, as well as those of other participants, and interactive drawing overlay were demonstrated.

9.60	JCAB and FAA agreed that the test was successful, with minor network security problems. It was agreed to close this action item.

10.0	Agenda Item 7, Developed Necessary State Contingency Plans

10.1	There were no papers or discussions on this agenda item.

11.0	Agenda Item 8, Review and Update CNS/ATM Planning Chart and Action Items

11.1	JCAB and FAA presented updates to the CNS/ATM Planning Chart as documented in Appendix C to this summary.

12.0	Agenda Item 9, Other Business

Programmed Time of Landing (PTL) at Sydney

12.1	IATA presented the initiative of Airservices Australia PTL trials at Sydney Airport which often has a difficult time with early arrivals due to a 6:00 am curfew.  This trial is aimed at reducing the airborne holding duration for flights that are often required to hold prior to being allowed to land.  IATA noted the success of this trial and that fuel conservation and efficient aircraft operations are a high priority for airline operators and welcomed such activities by ANSPs.

    - IPACG is interested in getting additional information in regard to the PTL trials and will seek to obtain this for the next IPACG meeting.

13.0	Closing of the meeting

Next Meeting

13.1 	The next meeting will be held in Tokyo, Japan; 5-9 November 2007.  There will be a change in format for IPACG/27.  Day 1 will be for JCAB and FAA only.  Day 2 will be for the FIT meeting and days 3, 4 and 5 for IPACG/27.

13.2	For planning purposes the FAA proposed that IPACG/28 be held from 12-16 May 2008 in Henderson, Nevada.

14.0	Closing Remarks

14.1	IATA commended IPACG for its efforts in the areas of fuel savings and aircraft emission reduction and expressed their strong support.

/s/ Satoshi Yamaguchi				              /s/ David Maynard_______

Mr. Satoshi Yamaguchi					Mr. David Maynard

Co-chair for JCAB					Co-chair for FAA

Appendices:

Appendix A:    Meeting Attendees

Appendix B:    Report of FIT 13

Appendix C:    CNS/ATM Planning Chart

Appendix D:    Open Action Items