THE TWENTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/28)

(Las Vegas, USA, 12-16 May, 2008)

Agenda Item 5:  Air Traffic Management (ATM) Issues

Restructuring plan of the Kanto airspace surrounding the Tokyo and Narita international airport

(Presented by the Civil Aviation Bureau, Japan)

SUMMARY

This paper provides information of the restructuring plan of the Kanto airspace surrounding the Tokyo and Narita international airport at 2010.

- Background

  - It is expected that future demands on aviation in Japan (both international and domestic) will continue to increase, despite temporary stagnation due to events such as the Severe Acute Respiratory Syndrome epidemic. In particular, the demand on aviation operations between Japan and Asia is expected to vastly increase due to the economic expansion occurring in China and Southeast Asian country.

  - To cope this situation in Japan, The Civil Aviation Bureau of Japan (JCAB) is planning to open fourth runway of the Tokyo international (Haneda) airport, and to extend length of the runway 34R/16L of the Narita international airport. The fourth runway of the Haneda airport will open at autumn in 2010, and the capacity of the Haneda airport will be 407 thousand movements par year (1.4 times to current capacity). The runway 34R/16L of the Narita international airport will be extended to 2,500 meters early 2010, and the capacity of the Narita international airport will be 220 thousand movements or more par year (1.1 times to current capacity).

  - In order to handle increased traffic in the airspace over the Kanto district which includes Tokyo area safely and efficiently, JCAB has studied ideal traffic flows, airspace organization and ATC operational procedures for several years. At March 2008, JCAB finalize a draft of ATS routes, airspace configuration of terminal/ACC and ATC operational procedures.

- Overview of restructuring of the Kanto airspace

  - Terminal Approach Control Area

    - The Haneda terminal control area and the Narita terminal control area will be consolidated to a single terminal control area. This single terminal control airspace is smaller airspace than that combined current Tokyo and Narita airspace. The new terminal control facility is placed at Haneda.

    - As the terminal control airspace becomes small airspace and traffic would be increased, a sequencing function for arrival aircrafts to the Haneda and the Narita airport will be transferred to the Tokyo ACC.

    - The new terminal control area has two entry gates for each airport respectively, and each entry gate is a one-to-one correspondence to the runway (One gate one runway concept). The Tokyo ACC will provide sequencing service, and the new terminal control facility will provide metering service (establish appropriate in-trail separation for landing).

  - Tokyo Area Control Center

    - Six new sectors which are exclusively designed to control arrival/departure aircrafts of Haneda and Narita will be set up for the Tokyo ACC. These sectors will provide services both en-route ATC and sequencing function of current terminal ATC.

    - The sector A is for arrival aircrafts to the Narita airport from NOPAC and CENPAC. The sector C is for arrival aircrafts to the Haneda airport from western area of Japan. The sector D is for arrival aircrafts to the Narita airport from Asia. The sector E is for arrival aircrafts to the Haneda airport from the northern area of Japan, and for arrival aircrafts to the Narita airport from the Incheon, Beijing, Europe and Russia. The sector B is for arrival aircrafts to the Narita airport from the sector A and E.

  - Implementation schedule

    - JCAB is planning the project to implement step by step because it is a major change and impact to airspace configuration, routes and ATC operations. First step of the project is consolidation of the Haneda terminal control area and the Narita terminal control area. The consolidated terminal control facility is planned to operate from January 2010. At the same time, the sector A and F will be opened, and realign associated ATS routes.

    - Second step of the project is to open the sector C, D and E, and realign associated ATS routes. It is planned to operate from October 2010, it is the same time of the open of the fourth runway of the Haneda airport.

    - The last step of the project is to open the sector B, and realign associated ATS routes. To move to this step is required the approval of the local communities and other organization. Currently the target date of the last step is not determined.

- Action by the meeting

  - The meeting is invited to note the information provided in this paper.

- END -