THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Monitoring FANS-1/A Performance Against Oceanic SPR Standard

(Presented by United States of America)

Summary

This paper presents the results of an analysis of SATCOM data link performance in the New York oceanic airspace.  As measured against the surveillance and intervention capability requirements of Required Communications Performance (RCP) 240 most airline/airframe pairs fail to meet the continuity requirements for communication and surveillance. Additionally, the paper discusses the data collection and analysis methods used to develop these performance metrics.

# Introduction

## ICAO Annex 11 requires that data link performance is monitored to verify that an acceptable level of safety continues to be met. Data link performance requirements for the application of reduced separation standards, as defined in ICAO Doc4444, are contained in the RTCA DO-306/EUROCAE ED 122 Oceanic SPR standard.

## These requirements are specified in terms of Required Communications Performance (RCP) and surveillance. The analysis reported on in this paper used the RCP 240 values for evaluating the communications and controller intervention capability.  The evaluation of surveillance capability used the values specified for 30 NM lateral and distance-based longitudinal separation.

## Analysis of the data emphasizes the importance of end-to-end monitoring of performance by individual airline-aircraft types. Significant performance differences observed between aircraft types and between different airlines operating the same aircraft type indicate that monitoring should include not only the communication service provider (CSP) performance but also the airline flight deck procedures and aircraft equipment.

# Discussion

## This paper provides information derived from monitoring FANS-1/A data link performance in the New York Oceanic FIR during the period 1 May 2008 to 31 July 2008.

Communication Performance - CPDLC

## For RCP it was decided to monitor only those uplink messages that required a WILCO/UNABLE response. This decision was made because the critical communications requirement is provided by intervention messages when applying reduced separation standards. Incorporating other message types such as free text queries or information requests would skew the data because of the longer response times from the flight deck. Appendix A contains a listing of the uplink messages that are considered intervention messages when extracting data for analysis.

## While some analysis of messages from all media types was done the monitoring to date has concentrated on an analysis of round trip CPDLC intervention transactions i.e. the CPDLC intervention uplink and the crew response, under SATCOM. This analysis is based on that subset of message transactions where the MAS response is received from a satellite Ground Earth Station (GES), and the corresponding crew response downlink is also received through a satellite GES. It was decided to concentrate on SATCOM because we did not want to introduce issues associated with VHF/ SATCOM transitions.

## We used CPDLC uplink messages and their corresponding downlink responses from the aircraft in this assessment. Our analysis of CPDLC is based on the calculation of Actual Communication Performance (ACP) used to monitor RCP time allocations for communication transaction (TRN), Actual Communications Technical Performance (ACTP) used to monitor Required Communication Technical Performance (RCTP) time allocations, and Flight Deck Response (PORT) used to monitor the flight deck responder element of the transaction.

## To calculate ACP the difference between the times that the uplink message is originated at the Air Traffic Service Provider (ATSP) to the time that the corresponding response downlink is received at the ATSP is used. To calculate ACTP the difference between the downlink’s aircraft time stamp and the received time is added to half the round trip time determined by the difference between the uplink time when the message is sent from the ATSP and the receipt of the MAS response for the uplink at the ATSP ( (uplink transmission time – MAS receipt)/2 + downlink time). PORT latency is calculated by the difference between ACP and ACTP. Figure 1 illustrates these measurements.

Figure 1 – CPDLC Transaction Calculations

## As seen above the values for ACTP and PORT are only approximations.  The only way we can estimate the time taken for an uplink to reach the aircraft is to halve the time taken for the MAS response round trip. This assumption is flawed in a percentage of cases because we know it is possible for the MAS to be received at the ATSP some time after the operational response is received or the timestamp within the operational response is earlier than the MAS receipt time. Apparently, this will happen if the CSP does not hear the network ACK from the aircraft sent on uplink receipt and resends the uplink at a later time. The CSP receives the network ACK to this second uplink and sends the MAS to the ATSP. In the meantime the aircraft has already responded with the operational response. ATSP will see this issue reflected in their data with crew response times with negative or extremely small values. The time sequence diagram below in Figure 2 attempts to illustrate this.

Figure 2 – Issue with estimating uplink transit time as half MAS roundtrip

Surveillance performance – ADS C

## The Oceanic SPR also specifies surveillance performance requirements. The ADS downlink transit time is defined as the difference between the date/time derived from the Basic ADS Group timestamp and the time the message is received at the ATSP.

Categorizing the data and filtering

## The RCP and surveillance analysis was carried out for each airline and aircraft type operating in the New York FIR. Appendix B contains graphs of the observed performance within the New York Oceanic FIR. Operator-airframe combinations that had fewer than 20 RCP transactions or 100 ADS messages were dropped from the graphs. The parenthetical numbers in the chart legends indicate the number of transactions that comprise the curve.

## For this analysis it was decided that no filtering would be done for messaging that occurred during outages (e.g. GES, CSP network).  This decision was taken for two reasons.  Firstly, outage notification by the CSPs is not comprehensive. Secondly, what period of time should the filtering be carried on for; the notified period of the outage or the notified period of the outage plus some period of time after the event to allow for the inevitable RF storm or message deluge?

Observations and Results

## One area that has a significant impact on overall performance is the flight deck response times.  The charts for the A33x serve as a typical example of this impact.  Figure 3 below graphs the overall transaction (ACP) while Figure 4 and Figure 5 graph the technical (ACTP) and responder (PORT) sub-components of the communications performance, respectively.

## Figure 4 shows that 75% of the airlines operating the A33x meet the continuity requirement (99.9% at 180 seconds) for technical performance yet figure 3 shows that none of these operators meets the continuity requirement (99.9% at 210 seconds) for overall transaction performance (ACP). In fact one operator doesn’t even meet the 95% requirement.

## A contributing factor to this can be seen in Figure 5: flight deck response.  The best performance here is only 87% of the messages responded to within the 60 seconds allocated.  A performance improvement in this area may require nothing more than informing the aircrews of the response time allocation.  It should be noted that a “STANDBY” response from the crew does not end the transaction; only a “WILCO” or “UNABLE” end the transaction.

Figure 3 – ZNY – Operators w/A33x –ACP – May – July 2008

Figure 4 – ZNY – Operators w/A33x –ACTP – May – July 2008

Figure 5 – ZNY – Operators w/A33x –PORT – May – July 2008

## Nearly every operator-airframe combination fails to meet the continuity requirement (99.9% at 180 seconds) and there are a significant number of operator-airframe combinations that fail to meet the 95% requirement (150 seconds) for the surveillance component.  This seems peculiar when compared with the technical portion of communication performance, ACTP.  As discussed above ACTP is a combination of the estimated uplink time and the measured downlink time of the response.  However, at almost any point along the X-axis the performance of this round trip measurement is better than the performance of the one way ADS measurement.  Figure 6 graphs the actual delivery time of ADS position reports.

## A possible explanation for some of this performance degradation has to do with waypoint reporting.  Between the 27th and 29th of February 2008 Oakland ARTCC and Auckland ACC worked with ARINC and SITA to monitor downlink ADS performance on UAL B744 and ANZ B777 aircraft that operated across their common boundary.  During the analysis of the trace logs it was noted that after an aircraft sequenced a waypoint the first report that it downlinked was the ACARS position report to the Airline Operation Center.  Seven to ten seconds later it downlinked a report to the first ATSP that established an ADS Waypoint Change Event contract with the aircraft.  If a downstream ATSP also has an ADS Waypoint Change Event contract with the aircraft that report is transmitted seven to ten seconds later.

Figure 6 – ZNY - Operators w/A33x – ADS Position Report Delivery Performance – May – July 2008

Automation support for data collection

## Support software in Ocean 21 filters system data recordings to produce two files per day for data link performance analysis purposes.

## The first file contains data link message transit delay statistics consisting of transit delay and various other attributes for all uplink and downlink AFN, CPDLC, and ADS messages.  The output comma delimited text file is then imported into a relational database for analysis of the surveillance component of the RCP requirements.  Appendix C describes the layout of the transit delay text file.  Appendix C also shows a typical Ocean 21 data recording to illustrate the sources of the various data points for each transit delay analysis record.

## The second file contains various aspects of CPDLC transactions that modify the cleared profile of an aircraft.  The transactions included in the file are defined as those uplinked CPDLC messages where the MOPS Element ID in the first element sequence is contained in the Uplink Elements of Interest (see Appendix A). Data points extracted for each transaction allow analysis relative to aircraft type, aircraft operator, CSP, and media type. The output comma delimited text file is then imported into a relational database for analysis of the communications component of the RCP requirements.  Appendix D describes the layout of the CPDLC transaction text file.  Appendix D also shows a typical Ocean 21 data recording to illustrate the sources of the various data points for each transit delay analysis record.

Other considerations

## The data collection and analysis methods used for this paper were ported over from similar work being done at the Oakland and Anchorage ARTCCs.  The transit delay portion of the work done at those locations was itself a legacy analysis done to satisfy performance reporting requirements in the FANS 1/A Operations Manual.  While these methods met the needs of the present analysis some points to consider when developing a standardized data collection and reporting format are:

- Develop a single record that allows for both one way transactions and CPDLC intervention transactions (perhaps record types of AFN, COM, ADS, TRN).

- For ADS transactions add a data point that describes the type of message: Periodic, Waypoint Change, Other Event.  This could help with the identification of problems associated with a particular report type such as the one described above involving Waypoint Change reports.

- Add a data point to each transaction that describes the FIR ID (e.g. KZWY).  While this is obviously not necessary for analysis at each ACC it will be helpful when the region’s monitoring agency is analyzing data from a number of FIRs.

- Make the record extensible so that an ATSP could collect additional data points that they might find useful for their own analysis requirements but still allow for relatively easy future expansion of standardized data collection points.

- Develop a process for each ACC to report notified (or known) outage periods in their FIR.

## The region’s monitoring agency may find it useful to import the submitted data into a relational database to facilitate the analysis of the data.  In addition to providing analysis aids such as pivot tables and pivot charts the database can be used to eliminate duplicate transaction records.  The database used by the FAA includes the following tables with appropriate relationships defined:

- GS_Locids - contains ground station location ID and ground station type: VHF, SAT, or HF.

- CSP_ID - contains Communication Service Provider identifier (e.g. DDL) and CSP name (e.g. ARINC).

- MOPS - contains MOPS element number and MOPS text.

- Oper_ACType - contains information regarding satellite channel speed used by each operator-airframe combination.

- Operators - contains company name and ICAO identifier.

- Registration - contains aircraft registration number, aircraft type, and company name.

- Messages - contains data points for each transaction.

# Action

## The meeting is invited to:

- Note the performance metrics for data link operations within the New York Oceanic FIR and,

- Continue to collaborate with other regions via the GOLD Ad Hoc Working Group to develop a standardized reporting format for pertinent data points for each data link transaction, based on DO-306/ED-122.

Appendix A – Uplink Elements of Interest

Appendix B – RCP 240 Performance Monitoring (SATCOM Media) by FAA in KZWY FIR

Appendix C – Transit Delay File

Appendix D - CPDLC Transaction File

Appendix A – Uplink Elements of Interest

| ID | TEXT |
| --- | --- |
| 19 | MAINTAIN [altitude] |
| 20 | CLIMB TO AND MAINTAIN [altitude] |
| 21 | AT [time] CLIMB TO AND MAINTAIN [altitude] |
| 22 | AT [position] CLIMB TO AND MAINTAIN [altitude] |
| 23 | DESCEND TO AND MAINTAIN [altitude] |
| 24 | AT [time] DESCEND TO AND MAINTAIN [altitude] |
| 25 | AT [position] DESCEND TO AND MAINTAIN [altitude] |
| 26 | CLIMB TO REACH [altitude] BY [time] |
| 27 | CLIMB TO REACH [altitude] BY [position] |
| 28 | DESCEND TO REACH [altitude] BY [time] |
| 29 | DESCEND TO REACH [altitude] BY [position] |
| 30 | MAINTAIN BLOCK [altitude] TO [altitude] |
| 31 | CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude] |
| 32 | DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude] |
| 33 | CRUISE [altitude] |
| 34 | CRUISE CLIMB TO [altitude] |
| 35 | CRUISE CLIMB ABOVE [altitude] |
| 36 | EXPEDITE CLIMB TO [altitude] |
| 37 | EXPEDITE DESCENT TO [altitude] |
| 38 | IMMEDIATELY CLIMB TO [altitude] |
| 39 | IMMEDIATELY DESCEND TO [altitude] |
| 40 | IMMEDIATELY STOP CLIMB AT [altitude] |
| 41 | IMMEDIATELY STOP DESCENT AT [altitude] |
| 46 | CROSS [position] AT [altitude] |
| 47 | CROSS [position] AT OR ABOVE [altitude] |
| 48 | CROSS [position] AT OR BELOW [altitude] |
| 49 | CROSS [position] AT AND MAINTAIN [altitude] |
| 50 | CROSS [position] BETWEEN [altitude] AND [altitude] |
| 51 | CROSS [position] AT [time] |
| 52 | CROSS [position] AT OR BEFORE [time] |
| 53 | CROSS [position] AT OR AFTER [time] |
| 54 | CROSS [position] BETWEEN [time] AND [time] |
| 55 | CROSS [position] AT [speed] |
| 56 | CROSS [position] AT OR LESS THAN [speed] |
| 57 | CROSS [position] AT OR GREATER THAN [speed] |
| 58 | CROSS [position] AT [time] AT [altitude] |
| 59 | CROSS [position] AT OR BEFORE [time] AT [altitude] |
| 60 | CROSS [position] AT OR AFTER [time] AT [altitude] |
| 61 | CROSS [position] AT AND MAINTAIN [altitude] AT [speed] |
| 62 | AT [time] CROSS [position] AT AND MAINTAIN [altitude] |
| 63 | AT [time] CROSS [position] AT AND MAINTAIN [altitude] AT [speed] |
| 64 | OFFSET [distanceoffset] [direction] OF ROUTE |
| 65 | AT [position] OFFSET [distanceoffset] [direction] OF ROUTE |
| 66 | AT [time] OFFSET [distanceoffset] [direction] OF ROUTE |
| 67 | PROCEED BACK ON ROUTE |
| 68 | REJOIN ROUTE BY [position] |
| 69 | REJOIN ROUTE BY [time] |
| 72 | RESUME OWN NAVIGATION |
| 73 | [predepartureclearance] |
| 74 | PROCEED DIRECT TO [position] |
| 75 | WHEN ABLE PROCEED DIRECT TO [position] |
| 76 | AT [time] PROCEED DIRECT TO [position] |
| 77 | AT [position] PROCEED DIRECT TO [position] |
| 78 | AT [altitude] PROCEED DIRECT TO [position] |
| 79 | CLEARED TO [position] VIA [routeclearance] |
| 80 | CLEARED [routeclearance] |
| 81 | CLEARED [procedurename] |
| 82 | CLEARED TO DEVIATE UP TO [distanceoffset] [direction] OF ROUTE |
| 83 | AT [position] CLEARED [routeclearance] |
| 84 | AT [position] CLEARED [procedurename] |
| 91 | HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees] [direction] TURNS [legtype] |
| 92 | HOLD AT [position] AS PUBLISHED MAINTAIN [altitude] |
| 94 | TURN [direction] HEADING [degrees] |
| 95 | TURN [direction] GROUND TRACK [degrees] |
| 96 | FLY PRESENT HEADING |
| 97 | AT [position] FLY HEADING [degrees] |
| 98 | IMMEDIATELY TURN [direction] HEADING [degrees] |
| 106 | MAINTAIN [speed] |
| 107 | MAINTAIN PRESENT SPEED |
| 108 | MAINTAIN [speed] OR GREATER |
| 109 | MAINTAIN [speed] OR LESS |
| 110 | MAINTAIN [speed] TO [speed] |
| 111 | INCREASE SPEED TO [speed] |
| 112 | INCREASE SPEED TO [speed] OR GREATER |
| 113 | REDUCE SPEED TO [speed] |
| 114 | REDUCE SPEED TO [speed] OR LESS |
| 115 | DO NOT EXCEED [speed] |
| 116 | RESUME NORMAL SPEED |
| 176 | MAINTAIN OWN SEPARATION AND VMC |

Appendix B – RCP 240 Performance Monitoring (SATCOM Media) by FAA in KZWY FIR

Appendix B includes the following graphs of data collected at ZNY from May – July 2008, inclusive:

| Operators with | CPDLC |  |  | ADS |
| --- | --- | --- | --- | --- |
|  | CPDLC - ACP | CPDLC ACTP | CPDLC PORT | Position Report Delivery Time |
| A34x | Page B-2 | Page B-2 | Page B-3 | Page B-3 |
| B77x | Page B-4 | Page B-4 | Page B-5 | Page B-5 |
| Boeing Miscellaneous | Page B-6 | Page B-6 | Page B-7 | Page B-7 |
| Other Miscellaneous | Page B-8 | Page B-8 | Page B-9 | Page B-9 |

Appendix C – Transit Delay File

- Receive Date – date the downlink message is received from the aircraft or the downlink message assurance is received for the uplink message sent.  The format of this field is mm-dd-yyyy and the data comes from the date portion of the system data recording timestamp.

- Receive Time – time the downlink message is received from the aircraft or the downlink message assurance is received for the uplink message sent.  The format of this field is hh:mm:ss and comes from the time portion of the system data recording timestamp.

- Send Date – date portion of the system data recording timestamp for the uplink message sent from ATOP or the calculated date for the downlink message sent from the aircraft.  The format of this field is mm-dd-yyyy.

- Send Time – time portion of the system data recording timestamp for the uplink message sent from ATOP or the decoded timestamp from the downlink message sent from the aircraft.  The format of this field is hh:mm:ss.

- Delay – the difference in seconds between fields 3-4 and fields 1-2.

- Direction – “U” for uplink or “D” for downlink.

- Message Type – ADS, AFN, or COM.

- Aircraft ID – 7 characters

- Aircraft Registration – 7 characters

- Data Link Service Provider - 3 characters from the ACARS message header “DT” line.

- Ground Earth Station Antennae ID – 3 or 4 characters from the ACARS message header “DT” line.

- Message Assurance – “S” for successful, “F” for failed, and “T” for timeout or no message assurance received for the uplink message sent.

- Latitude - From the reported position information in the ADS Basic group.  The format is “+” for North or “-“ for South followed by a decimal number of degrees.

- Longitude - From the reported position information in the ADS Basic group. The format is “+” for East or “-“ for West followed by a decimal number of degrees

- Altitude - From the reported position information in the ADS Basic group. The format is ddddd optionally proceeded by a “-“ for a negative altitude.

- Next Latitude - From the ADS predicted routed group next position if available.

- Next Longitude - From the ADS predicted routed group next position if available.

- Message Length – in bytes of field data measured from 3rd line of the system data recording acars output record to the last line.

Fields 13 – 17 pertain to ADS downlink messages only.

1 & 3

2 & 4

8

9

-----------------------------------------------------------------------------------------------------------------------

20070213 1110:07.048 ACARSIN  fdpsb602    ACID= UAL840          tail_no= N187UA          link= #00   srln= 656   mas=

10

11

UAL840 : #0d

UAL840 : #01QU OAKODYA#0d

UAL840 : .QXSXMXS 131110#0d

UAL840 : #02FML#0d

UAL840 : FI UA840/AN N187UA#0d

UAL840 : DT DDL XXC 131110 F39A#0d

UAL840 : -  ADS.N187UA07F9D8145087C7EF09391D0DFC6FC460C8486C07F0FFFDD47D38C8AA800E0D80F2C0000F0EA99D0000101C937EE0377B#0d

UAL840 : #03 656

Above ADS message decoded:

13

14

15

4

ADS Downlink Message ADS - OAKODYA - .N187UA - CRC is valid

Basic,S8-39.4,W165-50.7,32496,9:50.2,TCAS,6,REDUNDANT

Route,S5-0.7,W162-59.3,34496,00:33:52,S0-0.7,W157-59.3,35496

EarthRef,Valid,38.0,485.5,0

AirRef,Valid,41.2,0.826,0

16

17

Meteor,28.5,Valid,109.0,-36.0

Results in antennae file entry:

02-13-2007,11:10:07,02-13-2007,11:09:50,17,D,ADS,UAL840,N187UA,DDL,XXC,,-8.65688323974609,-165.844287872314,32496,-5.01113891601562,-162.987499237061,189

Appendix D - CPDLC Transaction File

- Aircraft ID – 2 to 7 characters

- Aircraft Registration – 2 to 7 characters

- Send Date – date portion of the system data recording timestamp for the uplink message sent from Ocean 21.  The format of this field is mm-dd-yyyy.

- Send Time – time portion of the system data recording timestamp for the uplink message sent from Ocean 21.  The format of this field is hh:mm:ss.

- MOPS Element ID – 2 or 3 character element id of the first element sequence in the uplinked CPDLC clearance.

- Clearance Data Link Service Provider - 3 characters from the ACARS message header “DT” line in the MAS message.

- Clearance Ground Earth Station Antennae ID – 3 or 4 characters from the ACARS message header “DT” line of the MAS message.

- MAS Receive Date - date the downlink message assurance is received for the uplink message sent.  The format of this field is mm-dd-yyyy. The data comes from the date portion of the system data recording timestamp.

- MAS Receive Time - time the downlink message assurance is received for the uplink message sent.  The format of this field is hh:mm:ss. The data comes from the time portion of the system data recording timestamp.

- Response Data Link Service Provider - 3 characters from the ACARS message header “DT” line in the operational response message.

- Response Ground Earth Station Antennae ID – 3 or 4 characters from the ACARS message header “DT” line of the operational response message.

- PORT Send Date – date the operational response (WILCO or UNABLE) is sent.  The format of this field is mm-dd-yyyy.  The data must be calculated based on PORT Send Time, PORT Receive Time, and PORT Receive Date since the ATCmessageheader timestamp does not include date information.

- PORT Send Time – time the operational response (WILCO or UNABLE) is sent. The format of this field is hh:mm:ss.  The data comes from the ATCmessageheader timestamp in the decoded downlinked CPDLC operational response message. (See RTCA DO-258A section 4.6.3.3)

- PORT Receive Date – date the operational response (WILCO or UNABLE) is received.  The format of this field is mm-dd-yyyy.  The data comes from the date portion of the system data recording timestamp.

- PORT Receive Time – time the operational response (WILCO or UNABLE) is received. The format of this field is hh:mm:ss.  The data comes from the time portion of the system data recording timestamp.

- Operational Response – Nature of the operational response, “W” for wilco or “U” for unable.

- TRN – total transaction time in seconds.

- RCTP – Required Communication Technical Performance time in seconds (rounded up).

- PORT – Responder time in seconds.

- Transaction Completion Indicator – “S” for successful, “F” for failed.  The transaction is considered successful if TRN <= 1800 and items 1-19 can be determined.  If a transaction fails, all of the data that can be determined should be included in the record.

Sample transaction from system data recording

1. The aircraft makes a request.  This message is not part of the transaction but is included for clarity and to show the origination of message identification and message reference numbers that are pertinent to the transaction.

-------------------------------------------------------------------------------------------------------------------------------

20071119 0012:54.585 ACARSIN  fdpsa102    ACID= CAL008  tail_no= B18215  link= #00   srln= 64    mas= 65535   rcv_time= 20071119 0012:54.585  xmt_time= 20071119 0012:47.000  smi= FML

CAL008 : #0d

CAL008 : #01QU OAKODYA#0d

This decodes to the text below the dashed line.

CAL008 : .QXSXMXS 190012#0d

CAL008 : #02FML#0d

CAL008 : FI CI0008/AN B-18215#0d

CAL008 : DT QXT POR1 190012 F23A#0d

CAL008 : -  AT1B-182152A80CBC272BC6A2A#0d

CAL008 : #03 064

-------------------------------------------------------------------------------------------------------------------------------

ATC DL Downlink Message AT1 - OAKODYA - B-18215 - CRC is valid

21,,00:12:47

0(9)  : Request Climb To [alt]

alt(fl): 380

This is the Message ID Number which will be seen as the Message Reference Number in the uplinked clearance.

2. The climb clearance is sent.

Send Time

-------------------------------------------------------------------------------------------------------------------------------

Send Date

20071119 0015:12.131 ACARSOUT fdpsa102    ACID= CAL008  tail_no= B-18215  link= #00   srln= 691   mas= 751     rcv_time= 19700101 0000:00.000  xmt_time= 20071119 0015:12.000  smi= FMD

CAL008 : #0d

Message assurance sequence number and request for message delivery indication (the letter “A”).

CAL008 : #01QU FANS1XS#0d

CAL008 : .OAKODYA 190015#0d

CAL008 : #02FMD#0d

CAL008 : AN B-18215/MA 751A#0d

This decodes to the text below the dashed line.

CAL008 : -  /AA OAKODYA.AT1B-18215C22A2995E2072BC062C4#0d

CAL008 : #03 691

-------------------------------------------------------------------------------------------------------------------------------

ATC DL Uplink Message AT1 - OAKODYA - B-18215 - CRC is valid

4,21,

0(20) : Climb To And Maintain [alt]

Message Reference Number

alt(fl): 380

MOPS Element ID. See Appendix A for elements of interest in this study.

1(129): Report Level [alt]

alt(fl): 380

Message ID Number.  The aircraft will use this as the Message Reference Number in the Operational Response message.

Element Sequence Number. While a message can contain up to five elements only the first element (0) in an uplink message is of interest for this study.

3. The MAS for the uplinked clearance is received.

MAS Receive Date

MAS Receive Time

Message assurance sequence number and indicator (S or F). This sequence number can be used to relate the MAS to the uplinked CPDLC message.

Clearance Data link Service Provider

Clearance Ground Earth Station Antennae ID

-------------------------------------------------------------------------------------------------------------------------------

20071119 0015:17.185 ACARSIN  fdpsa102    ACID=   tail_no= B-18215  link= #00   srln= 85    mas= 751     rcv_time= 20071119 0015:17.185  xmt_time= 20071119 0015:00.000  smi= MAS

tail_no= : #0d

tail_no= : #01QU OAKODYA#0d

tail_no= : .FANS1XS 190015#0d

tail_no= : #02MAS#0d

tail_no= : AN B-18215/MA 751S#0d

tail_no= : DT QXT POR1 190015 S80A#0d

tail_no= : #03 085

-------------------------------------------------------------------------------------------------------------------------------

All of the text in this message is “in the clear” so no decoding is required.

4. The operational response for the uplinked clearance is received.

-------------------------------------------------------------------------------------------------------------------------------

PORT Receive Time

20071119 0015:33.342 ACARSIN  fdpsa102    ACID= CAL008  tail_no= B18215  link= #00   srln= 88    mas= 65535   rcv_time= 20071119 0015:33.342  xmt_time= 20071119 0015:26.000  smi= FML

CAL008 : #0d

PORT Receive Date

CAL008 : #01QU OAKODYA#0d

Response Data link Service Provider

CAL008 : .QXSXMXS 190015#0d

Response Ground Earth Station Antennae ID

CAL008 : #02FML#0d

CAL008 : FI CI0008/AN B-18215#0d

CAL008 : DT QXT POR1 190015 F24A#0d

This decodes to the text below the dashed line.

CAL008 : -  AT1B-182156B0803DA0089AF#0d

CAL008 : #03 088

-------------------------------------------------------------------------------------------------------------------------------

ATC DL Downlink Message AT1 - OAKODYA - B-18215 - CRC is valid

PORT Send Time

22,4,00:15:26

MOPS Element ID. This will be (0) Wilco, (1) Unable or, (2) Standby.  If “Wilco” or “Unable” is received they indicate a termination of the transaction.  If “Standby” is received it does not complete the transaction (i.e. the timer is still running).  A subsequent “Wilco” or “Unable” should be received with the same Message Reference Number terminating the transaction.

0(0)  : Wilco

Message Reference Number.  This references the Message ID Number of the uplinked clearance.

The above messages result in a rcp_mmddyy.txt file entry:

CAL008,B-18215,11-19-2007,00:15:12,20,QXT,POR1,11-19-2007,00:15:17,QXT,POR1,11-19-2007,00:15:26,11-19-2007,00:15:33,W,21,10,11,S

-END-