THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Proposed Changes to RTCA DO-258A/EUROCAE ED-100A, FANS 1/A INTEROP Standard

(Presented by United States of America)

Summary

This paper provides information on suggested changes to RTCA DO-258A/EUROCAE ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications (FANS 1/A INTEROP Standard).  These changes improve FANS 1/A operations related to logon correlation and the media selection criteria the communication service providers (CSPs) use to deliver messages.  The NAT FIG is invited to provide input on these improvements.

# Introduction

## Based on experience in FANS 1/A operations at Oakland Center, changes are being considered to improve RTCA DO-258A/EUROCAE ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications (FANS 1/A INTEROP Standard).

## This paper provides information on two changes and invites the NAT FIG to provide input on these changes.

- logon correlation

- media selection criteria

Note:  The proposed changes affect infrastructure and do not impact aircraft installations.

# Logon correlation

## In B767, B757, B737, and MD11 aircraft the ACARS MU/CMU (supplying the ACARS header registration used in messages) has a hard coded tail number. However, the pilot is required to manually enter the tail number in the FMC (supplying the aircraft registration used in the body of the message, and used by the ATSP to address their responses). This implementation can not protect against human input.

## Paragraph 3.4.2 requires that every uplink message from an ATS provider shall use the aircraft registration number which is contained within the FN_CON encapsulated by the CRC check. If an incorrect registration is entered into the FMC on a B767, B757, B737, or MD11 then the ATSP system response if compliant with DO258A will be addressed to this incorrect registration. In the example AFN log on below the ACARS header registration N257UP has been supplied by the ACARS MU and is correct. However, the registration contained in the body of the AFN logon message N267UP has been manually entered by the pilot and is incorrect.

QU OAKODYA
.QXSXMXS 220033
FML
FI UP77/AN N257UP
DT DDL HNL 220033 F02A
-  AFN/FMHUPS77,.N267UP,,003326/FPON21101W157271,1/FCOADS,01/FCOATC,017414
880

## The ATSP system response to this message will be addressed to the incorrect registration as illustrated below:

QU FANS1XS
.OAKODYA 220033
FMD
AN N267UP/MA 872A
- /A0 OAKODYA.AFN/FMHUPS77,.N267UP,,003329/FAK4,KZAKB919
866

## To ensure that the ATSP system response is directed to the correct aircraft and processed by the FMS it must be addressed using the registration numbers (ACARS header and message body) from the AFN Contact message as illustrated below:

QU FANS1XS
.OAKODYA 220033
FMD
AN N257UP/MA 872A
- /A0 OAKODYA.AFN/FMHUPS77,.N267UP,,003329/FAK4,KZAK8FDB
866

## Several regions have approved ground-ground messaging (e.g. ATS Interfacility Data Communications (AIDC) FANS Application Message (FAN)) to forward the aircraft and airborne ATS applications addressing information to the next ATS Provider.  Provision needs to be made for this to be an acceptable source of the aircraft registration number used in uplink messages.

## Proposed Change.  The following change to DO-258A/ED-100A is proposed:

3.4.2	Addressing of Uplink Messages

a) Every uplink message from an ATS Provider shall use the aircraft registration number which is contained within the portion of the AFN logon message (FN_CON) or ground-ground connection transfer message that is encapsulated by the Cyclical Redundancy Check (CRC).  The exception to this is an AFN Acknowledgement (FN_AK) message with an overall reason code of 4 which may be addressed using the aircraft registration number contained in the ACARS header (see note).

Note:	If an AFN Contact (FN_CON) message is received where the aircraft registration number in the ACARS header and the aircraft registration number in the body of the FN_CON message differ, the FN_AK (with reason code 4) should be addressed using the aircraft registration number that was contained in the ACARS header of FN_CON message to ensure the message is delivered to the correct aircraft. To ensure the message is processed by the FMS the body of FN_AK should use the registration number that was contained in the body of the FN_CON.

# Media selection criteria

## Paragraph 2.2.1 e) of RTCA DO-258A states "The requirement for a particular medium on a particular route will be specified by the appropriate authorities based on factors such as performance and coverage."  However, the section (3.4.3) proposed for change somewhat contradicts this by stating that “the CSP determines the routing of FANS-1/A messages to the aircraft according to the guidelines given in this document” and further limits the ability of the ATSP to determine the medium by stating that the AP, GL, and TP Text Elements (TEI) are not to be used in uplink messages.

## The CSP’s choice of which media to use for message delivery is based on contractual arrangements, Media Advisories (MA), and recently downlinked messages.  However, the CSP has no direct knowledge of the aircraft’s position and therefore cannot determine that the aircraft is at or beyond the limits of VHF coverage if a MA has not been received indicating the loss of VHF.  It has also been observed that some CSPs will continue to attempt delivery of a message via VHF even if a MA indicating loss of VHF is received during the retries at delivery.

## When aircraft are at the extreme limits of VHF communications (transitioning to SATCOM) it is not uncommon for uplink message delivery to be attempted several times (8 to 15) via VHF before attempting via SATCOM (if previously advised as established).  This can add significant delays to the message delivery time.  This situation is exacerbated when an aircraft’s routing takes it in and out of VHF coverage several times.

## ATSP Air Traffic Management (ATM) systems are able to determine an aircraft’s position with a great deal of accuracy based on received position reports and extrapolation of this information.  The ATM system can also be programmed with areas of marginal VHF coverage.  These two pieces of data could then be used by the ATM system to determine when an uplink message should be delivered via SATCOM to better ensure its speedy delivery to the aircraft.

## The ATSP would advise the CSP of this by including a TP TEI with the uplink message.  If the CSP has current media tracking information indicating that the aircraft has SATCOM established it would attempt to deliver the message via SAT.  If the CSP does not have current media tracking information indicating that the aircraft has SATCOM established or the delivery attempt via SAT has failed it would attempt delivery of the message using their standard routing logic.

## Proposed Change (Alternative 1).  The following change to DO-258A/ED-100A is proposed:

3.4.3	Message Routing

If an uplink message from an ATS Provider contains an AP, GL or, TP text element the CSP shall attempt to deliver the message as specified by the TEI. If the CSP's routing tables indicate that station or medium is not available for the aircraft, attempts to deliver as specified in the routing TEI fail, or no routing TEI is included, the CSP determines the routing of FANS-1/A messages to the aircraft according to the guidelines given in this document.

Note:	These text elements are used to direct the CSP to deliver a message via a specific air-ground facility i.e., VHF ground station, GES, or HF ground station.

## Proposed Change (Alternative 2).  The following change to DO-258A/ED-100A is proposed:

3.4.3	Message Routing

Although the AP, GL or TP text elements should not normally be used because they override the CSP’s own routing algorithms, there are specific cases where their use may be of benefit. Under such circumstances ANSPs may use these TEIs to instruct the CSP to attempt to deliver the message as specified by the TEI. If the CSP's routing tables indicate that station or medium is not available for the aircraft, attempts to deliver as specified in the routing TEI fail, or no routing TEI is included, the CSP determines the routing of FANS-1/A messages to the aircraft according to the guidelines given in this document.

Note:	These text elements are used to direct the CSP to deliver a message via a specific air-ground facility i.e., VHF ground station, GES, or HF ground station.

# Action

## The Group is invited to:

- note the information provided in this paper

- provide input on these proposed improvements.

-END-