THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Monitoring FANS1/A CPDLC RCP and ADS latency – the need for a common framework

(Prepared by Airways New Zealand; Presented by United States of America)

Summary

To successfully implement a global performance based system will require regional coordination to ensure that FANS1/A monitoring data can be easily aggregated in order to calculate overall performance indicators and assess system performance at a regional and global level. Some ANSP are now collecting data and monitoring FANS1/A CPDLC RCP and ADS latency using the performance requirements defined in DO306 Oceanic SPR. Airways New Zealand is interested in agreeing data collection requirements to enable automated data collection systems to be implemented without subsequent rework. This paper proposes that an ad-hoc group be established under the GOLD initiative to advance the concept of a common data collection specification for FANS1/A monitoring.

# Introduction

## Airways New Zealand has been monitoring FANS1/A performance to the DO306 Oceanic SPR Standard requirements for the past year. While developing this monitoring the methods employed have been quite labor intensive and we would like to commence work in the near term to automate this process. In an ideal world we would like to reach agreement on a common data set to avoid subsequent rework of the automation.

## The ICAO global plan calls for the implementation of a performance based system. Monitoring of FANS1/A data communications in terms of RCP and ADS latency is a part of this performance based system. Regions will need to agree on common monitoring requirements to enable RCP and ADS latency performance data to be aggregated in accordance with the guidelines provided in ICAO Doc 9883 Manual on Global Performance of the Air Navigation System.

## No framework currently exists for the aggregation of FANS1/A system performance data at other than a regional level. As an industry we should avoid a proliferation of regional solutions that may hinder aggregation of performance data at a global level.

# Discussion

## In our experience ANSP will develop the FANS1/A monitoring tools and internal reporting requirements which best suit their own environment. However, there will be regional requirements set for the periodic monitoring of FANS1/A CPDLC RCP and ADS surveillance latency using the DO306 Oceanic SPR Standards. To date these regional requirements for monitoring to the Oceanic SPR Standard do not exist.

## With our “low” traffic levels and by only monitoring on a monthly basis we have been able to get away with using simple script extraction from our recording files and processing using EXCEL spreadsheets. The FAA on the other hand with considerably more data to manage has modified existing relational database tools to give them the monitoring capability required. We support the FAA contention that a relational database will be required at regional level to successfully aggregate data from regional ANSP. However, we see a need to define both what is going to be monitored at a regional level, and also to define what data formats are used by individual ANSP when reporting to the regional central reporting agency. This definition would logically be contained in an interface control document.

## We are of the view that there may not be a need for the regional agency to have access to all available data. For example, in the SOPAC we have found that most problems and performance issues are identified through monitoring at the ANSP level, not at the regional level. It is at the ANSP level that detailed monitoring is required. Perhaps the regional agency just collates very high level performance data?

## We have yet to decide what is monitored at a regional level and the detail of how this monitoring is going to be achieved. The establishment of the GOLD ad-hoc group to progress the merging of the NAT Datalink Guidance Material with the Fans Operations Manual into a globally harmonized manual is a positive step towards enhanced global interoperability. We propose that the GOLD ad-hoc group be tasked with determining just what is monitored at a regional level, and the data collection (including data formats) that will be required to achieve this.

# Action

## The meeting is invited to:

- Collaborate with other regions via the GOLD Ad Hoc Working Group to develop a common interface control document specifying regional performance monitoring, reporting, and data transfer requirements for the FANS1/A datalink using the DO-306/ED-122 Oceanic SPR.

-END-