RCP 240 Specification

Coordination Draft 18-Oct-08

Table of contents
Contents		Page

Terms and acronyms	1

RCP 240	5

Airspace specific considerations	5

RCP parameter values	5

RCP monitoring and alerting criteria	5

RCP 240/D allocations	6

ATS Provider	6

CSP	10

Aircraft System	11

Aircraft Operator	14

# Terms and acronyms

The terms and acronyms are considered guidance material and are listed here only to coordinate on the RCP specifications themselves.  They are not considered part of the specification.

The terms applied to the RCP specification (1st part of the RCP specification) are taken directly from ICAO Doc 9869, First Edition, Manual on Required Communication Performance, dated 2008.

The RCP specification is derived mainly from a safety assessment.  However, in circumstances where it has been determined to be beneficial, an RCP specification may include criteria to support operational efficiency and orderly flow of air traffic.  Furthermore, it may have been determined to be beneficial to apply an RCP specification whose criteria are based purely on the operational utility of the service.  In either case the RCP specification shall indicate the distinction between safety and efficiency.

| RCP specification |  |
| --- | --- |
| Term | Description |
| Operational communication transaction | The process a human uses to initiate the transmission of an instruction, clearance, flight information, and/or request, and is completed when that human is confident that the transaction is complete. |
| RCP Expiration time (ET) | The maximum time for the completion of the operational communication transaction after which the initiator should revert to an alternative procedure. |
| RCP Nominal time (TT 95%) | The nominal time for the completion of the operational communication transaction at 95%. |
| RCP Continuity (C) | Probability that an operational communication transaction can be completed within the communication transaction time, ET or TT 95%. |
| RCP Availability (A) | Probability that an operational communication transaction can be initiated when needed. |
| RCP Integrity (I) | Acceptable rate of one or more undetected errors in a completed communication transaction. |

Designators are assigned to qualify different types of RCP allocations associated with an RCP specification.  The /D designator is used to qualify the RCP allocations associated with a data link system used to provide an intervention capability to the controller.

The definitions that follow are derived from the terms and acronyms for the RCP specification.  These terms clarify meaning and measurement points when the /D RCP allocations are used with the RCP 240.

The portion of any RCP performance parameter allocated to the human, i.e., Flight Crew/HMI or Controller/HMI, can be applied to machine functionality as substantiated by further safety and human factor assessments.

| /D transaction time |  |
| --- | --- |
| Term | Description |
| Monitored operational performance (TRN) | The portion of the transaction time (used for intervention) that does not include the times for message composition or recognition of the operational response. |
| Required Communication Technical Performance (RCTP) | The technical portion of the transaction time (used for intervention) that does not include the times for message composition, operational response, and recognition of the operational response. |
| Responder performance | The operational portion of the transaction time to prepare the operational response, and includes the recognition of the instruction, and message composition, e.g., flight crew/HMI for intervention transactions. |
| RCTPATSU for Intervention | The summed critical transit times for an ATC intervention message and a response message, allocated to the ATS unit system. |
| RCTPCSP for Intervention | The summed critical transit times for an ATC intervention message and a response message, allocated to the communication service provider system. |
| RCTPAIRCRAFT for Intervention | The summed critical transit times for an ATC intervention message and a response message, allocated to the aircraft system. |

| Continuity |  |
| --- | --- |
| Term | Description |
| C for TRN | The proportion of intervention messages and responses that can be delivered within the specified TRN for intervention. |
| C for RTCP | The proportion of intervention messages and responses that can be delivered within the specified RCTP for intervention. |
| C for RTCPATSU | The proportion of intervention messages and responses that can be delivered within the specified RCTPATSU for Intervention. |
| C for RTCPCSP | The proportion of intervention messages and responses that can be delivered within the specified RCTPCSP for Intervention. |
| C for RTCPAIRCRAFT | The proportion of intervention messages and responses that can be delivered within the specified RCTPAIRCRAFT for Intervention. |

| Availability |  |
| --- | --- |
| Term | Description |
| Communication service availability (ACSP) | Probability of available communication service on 24/7 operation. |
| Unplanned outage duration limit (min) | Time after the unplanned outage begins at which there is an operational impact. Measured from when an unplanned outage begins to when the ATS unit receives notification that the service has been restored. |
| Maximum number of unplanned outages | Measured for any 12-month period. Failures causing unplanned outages for multiple ATS units are only counted once. |
| Maximum accumulated unplanned outage time (min/yr) | Measured by accumulating only the duration times for unplanned outages greater than the unplanned outage duration limit during any 12-month period. |
| Unplanned outage notification delay (min) | Notification to the ATS unit of an unplanned outage. Measured from when the unplanned outage begins to when the ATS unit receives notification. |
| Aircraft system availability (AAIRCRAFT) | Aircraft equipage availability is the probability of available capability on an aircraft with an average flight of 6 hours. |

Ed Note:  More work may be needed on outage and restoration notifications.  During the last PARC CWG meeting, there was some discussion on Iridium service interruption announcements and warnings of future possible service degradation. The existing and necessary procedures may provide a starting point for developing guidance material related to outage and restoration notification to meet the “performance-based” criteria for RCP 240.  Note the criteria above is from when the outage begins to when the ANSP receives notification.  It is suggested also that if ANSPs are not notified of restoration, than that would seriously impact the overall yearly availability measurement against the 9 hour criteria for any 12-month period.

| RCP Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RCP type |  |  |  |  | # RCP 240 |  |
| ## Airspace specific considerations |  |  |  |  |  |  |
| Interoperability |  | Specify interoperability criteria, e.g., FANS 1/A |  |  |  |  |
| ATS Function |  | Specify ATS function(s), e.g., applicable separation standard |  |  |  |  |
| Application |  | Specify controller-pilot ATC communication intervention capability, e.g., CPDLC application per ICAO Doc 4444, and RTCA DO-306/EUROCAE ED-122, Annex A |  |  |  |  |
| ## RCP parameter values |  |  |  |  |  |  |
| Transaction time (sec) |  |  | Continuity (C) (probability) | Availability (A) (probability) |  | Integrity (I) (acceptable rate/flight hour) |
| ET = 240 |  |  | 0.999 | 0.999 0.9999 (efficiency) |  | 10-5 |
| TT 95% = 210 |  |  | 0.95 |  |  |  |
| ## RCP monitoring and alerting criteria |  |  |  |  |  |  |
| Ref: | Criteria |  |  |  |  |  |
| MA-1 | The system shall be capable of detecting failures and configuration changes that would cause the communication service to no longer meet the RCP type for the intended function. |  |  |  |  |  |
| MA-2 | When the communication service can no longer meet the RCP type for the intended function, the flight crew and/or the controller shall take appropriate action. |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Note 1: Rationale for the criteria provided in this specification can be found in ICAO Annex 11, ICAO Doc 4444, ICAO Doc 9689, and RTCA DO-306/ED-122. Note 2: The values for transaction times are to be applied to transactions that are representative of communication capability for the controller to intervene with a specific operator, aircraft type, and aircraft registration number or flight identification. Note 3: If changes are made to the system capacity limits, as specified by the airspace requirements, and the changes cause the system to perform below the RCP type, this would be considered a change in system configuration. Note 4: DO 306/ED 122 specifies an availability value based on safety assessment of the operational effects of the loss of the service. The availability value herein is more stringent, based on an additional need to maintain orderly and efficient operations. |  |  |  |  |  |  |

| ## RCP 240/D allocations |  |  |  |
| --- | --- | --- | --- |
| Application: CPDLC | System Component: | ### ATS Provider | 7 |

| #### RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Transaction Time Parameter | ET (sec), C = 99.9% | TT (sec), 95% | Compliance Means |
| Transaction Time Value | 240 | 210 | Analysis, CSP contract |
| RCP Time Allocations |  |  |  |
| Initiator | 30 | 30 | Analysis |
| TRN | 210 | 180 | Monitored, CSP contract |
| TRN Time Allocations |  |  |  |
| Responder | 60 | 60 | Monitored, estimated |
| RCTP | 150 | 120 | Monitored, estimated, CSP contract |
| RCTP Time Allocation |  |  |  |
| RCTPATSU | 15 | 10 | Pre-implementation demonstration |

#### RCP availability criteria

Note: See RCP 240/D allocations to CSP for RCP availability criteria allocated to the ATS Provider.

| #### RCP integrity criteria |  |  |
| --- | --- | --- |
| Integrity parameter | Integrity value | Compliance means |
| Integrity (I) | 10-5 | Analysis, safety requirements, development assurance level commensurate with integrity level, (compliance shown prior to operational implementation). |

| #### RCP monitoring and alerting criteria |  |  |
| --- | --- | --- |
| Ref: | Criteria | Compliance means |
| MA-1a | The ground system shall be capable of detecting ground system failures and configuration changes that would cause the communication service to no longer meet the requirements for the intended function. Note: If changes are made to the system capacity limits, as specified by the airspace requirements, and the changes cause the system to perform below the RCP type, this would be considered a change in system configuration. | System design, implementation |
| MA-1b | When the communication service no longer meets the requirements for the intended function, the ground system shall provide indication to the controller. | System design, implementation |
| MA-2 | When the controller receives an indication that the communication service no longer meets the requirements for the intended function (e.g., reduced longitudinal separation), the controller shall take action to resolve the situation, (e.g., apply an alternative form of separation). | System design, procedures, implementation |

| #### RCP related safety requirements |  |  |
| --- | --- | --- |
| Ref | Related RCP Parameter | Safety requirement |
| SR-1a | A | The ATSU shall display the indication provided by the aircraft system when a data link service request initiated by the ground system or the controller is rejected at the application layer. |
| SR-1b | A | The ATSU shall provide to the aircraft system an indication when it rejects a data link service request initiated by the flight crew at the application layer. |
| SR-2 | A, C | The ATSU shall indicate to the controller a detected loss of data link service. |
| SR-3 | A | Data link service shall be established in sufficient time to be available for operational use. |
| SR-4 | A, C | ATSU shall be notified of planned outage of data link service sufficiently ahead of time. |
| SR-5 | A, C | The ATSU shall indicate to the controller when a message can not be successfully transmitted. |
| SR-6 | C, I | The ATSU end system shall provide unambiguous and unique identification of the origin and destination with each message it transmits. |
| SR-7 | C, I | The ATSU shall indicate in each response to which messages it refers. |
| SR-8 | I | The ATSU shall send the route clearance information with the route clearance via data link. |
| SR-9 | C, I | The ATSU end system shall time stamp to within one second UTC each message when it is released for onward transmission. |
| SR-11 | C, I | Any processing performed by ATSU (data entry/ encoding/ transmitting/ decoding/ displaying) shall not affect the intent of the message. |
| SR-12 | C, I | The ATSU end system shall reject messages not addressed to itself. |
| SR-13 | C, I | The ATSU shall transmit messages to the designated aircraft system. |
| SR-14 | A, C, I | The ATSU system shall indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN). |
| SR-15 | C, I | When the ATSU receives a message whose time stamp exceeds ETTRN, the ATSU shall provide appropriate indication. |
| SR-16 | C, I | The ATSU shall prevent the release of clearance without controller action. |
| SR-17 | C, I | The ATSU shall prohibit operational processing by controller of corrupted messages. |
| SR-18 | C, I | The ATSU shall be able to determine the message initiator. |
| SR-19 | C, I | The ATSU shall prohibit to the controller operational processing of messages not addressed to the ATSU. |
| SR-20 | C, I | ATSU shall only establish and maintain data link services when the aircraft identifiers in data link initiation correlates with the ATSU’s corresponding aircraft identifiers in the current flight plan. |
| SR-21 | C, I | The aircraft identifiers used for data link initiation correlation by the ATSU shall be unique and unambiguous (e.g. the Aircraft Identification and either the Registration Marking or the Aircraft Address). |
| SR-23 | C, I | An ATSU system shall not permit data link services when there are non compatible version numbers. |
| SR-24 | C, I | The ATSU shall respond to messages in their entirety. |
| SR-25 | I | The ATSU end system shall be capable of detecting errors that would result in mis-delivery introduced by the communication service. |
| SR-26 | I | The ATSU end system shall be capable of detecting errors that would result in corruption introduced by the communication service. |

| RCP 240/D allocations |  |  |  |
| --- | --- | --- | --- |
| Application: CPDLC | System Component: | ### CSP | 11 |

| #### RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Transaction Time Parameter | ET (sec), C = 99.9% | TT (sec), 95% | Compliance means |
| RCTP Time Allocation |  |  |  |
| RCTPCSP | 120 | 100 | Contract terms |

| #### RCP availability criteria |  |  |  |
| --- | --- | --- | --- |
| Availability parameter | Efficiency | Safety | Compliance means |
| Communication service availability (ACSP) (probability) | 0.9999 | 0.999 | Contract terms |
| Unplanned outage duration limit (min) | 10 | 10 | Contract terms |
| Maximum number of unplanned outages | 4 | 48 | Contract terms |
| Maximum accumulated unplanned outage time (min/yr) | 52 | 520 | Contract terms |
| Unplanned outage notification delay (min) | 5 | 5 | Contract terms |
| Note: DO 306/ED 122 specifies a requirement to indicate loss of the service. Unplanned outage notification delay is an additional time value associated with the requirement to indicate the loss to the ATS provider per SR-4. |  |  |  |

| RCP 240/D allocations |  |  |  |
| --- | --- | --- | --- |
| Application: CPDLC | System Component: | ### Aircraft System | 12 |

| #### RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Transaction Time Parameter | ET (sec), C = 99.9% | TT (sec), 95% | Compliance Means |
| RCP Time Allocation |  |  |  |
| Initiator | 30 | 30 | Human-machine interface capability, Pre-implementation demonstration |
| TRN Time Allocation |  |  |  |
| Responder | 60 | 60 | Human-machine interface capability, Pre-implementation demonstration |
| RCTP Time Allocation |  |  |  |
| RCTPAIRCRAFT | 15 | 10 | Pre-implementation demonstration |

| #### RCP availability criteria |  |  |  |
| --- | --- | --- | --- |
| Availability parameter | Efficiency | Safety | Compliance means |
| AAIRCRAFT (probability) | N/A | 0.999 | Analysis, architecture, design, pre-implementation demonstration |

| #### RCP integrity criteria |  |  |
| --- | --- | --- |
| Integrity parameter | Integrity value | Compliance means |
| Integrity (I) | 10-5 | Analysis, safety requirements, development assurance level, e.g., Level C software, commensurate with integrity level, pre-implementation demonstration. |

| #### RCP monitoring and alerting criteria |  |  |
| --- | --- | --- |
| Ref: | Criteria | Compliance means |
| MA-1a | The aircraft system shall be capable of detecting aircraft system failures or loss of air/ground communication that would cause the aircraft communication capability to no longer meet the requirements for the intended function. | System design, implementation |
| MA-1b | When the aircraft communication capability no longer meets the requirements for the intended function, the aircraft system shall provide indication to the flight crew. | System design, implementation |

| #### RCP related safety requirements |  |  |
| --- | --- | --- |
| Ref | Related RCP Parameter | Safety requirement |
| SR-1a | A | The aircraft system shall provide to the ATSU an indication when it rejects a data link service request initiated by the ground system or the controller at the application layer. |
| SR-1b | A | The aircraft system shall display the indication provided by the ATSU when a data link service request initiated by the flight crew is rejected at the application layer. |
| SR-2 | A, C | The aircraft system shall indicate to the flight crew a detected loss of data link service. |
| SR-5 | A, C | The aircraft system shall indicate to the flight crew when a message can not be successfully transmitted. |
| SR-6 | C, I | The aircraft end system shall provide unambiguous and unique identification of the origin and destination with each message it transmits. |
| SR-7 | C, I | The aircraft system shall indicate in each response to which messages it refers. |
| SR-8 | I | The aircraft shall execute the route clearance per the route clearance received from the ATSU via data link. |
| SR-9 | C, I | The aircraft end system shall time stamp to within one second UTC each message when it is released for onward transmission. |
| SR-10 | C, I | The aircraft end system shall include in each ADS report the time at position to within one second of the UTC time the aircraft was actually at the position provided in the report. |
| SR-11 | C, I | Any processing performed by aircraft system (data entry/ encoding/ transmitting/ decoding/ displaying) shall not affect the intent of the message |
| SR-12 | C, I | The aircraft end system shall reject messages not addressed to itself. |
| SR-13 | C, I | The aircraft system shall transmit messages to the designated ATSU. |
| SR-15 | C, I | When the aircraft system receives a message whose time stamp exceeds ETTRN, the aircraft system shall provide appropriate indication. |
| SR-16 | C, I | The aircraft end system shall prevent the release of responses to clearances without flight crew action. |
| SR-17 | C, I | The aircraft system shall prohibit operational processing by flight crew of corrupted messages. |
| SR-18 | C, I | The aircraft system shall be able to determine the message initiator. |
| SR-19 | C, I | The aircraft system shall prohibit to the flight crew operational processing of messages not addressed to the aircraft. |
| SR-21 | C, I | The aircraft identifiers sent by the aircraft system and used for data link initiation correlation shall be unique and unambiguous (e.g. the Aircraft Identification and either the Registration Marking or the Aircraft Address). |
| SR-24 | C, I | The aircraft system shall respond to messages in their entirety or allow the flight crew to do it. |
| SR-25 | I | The aircraft end system shall be capable of detecting errors that would result in mis-delivery introduced by the communication service |
| SR-26 | I | The aircraft end system shall be capable of detecting errors that would result in corruption introduced by the communication service. |
| SR-27 | C, I | The aircraft and/or flight crew shall ensure the correct transfer into or out of the aircraft’s FMS of route data received/sent via data link that will be used to define the aircraft’s active flight plan. |

| RCP 240/D allocations |  |  |  |
| --- | --- | --- | --- |
| Application: CPDLC | System Component: | ### Aircraft Operator | 15 |

| #### RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Transaction Time Parameter | ET (sec), C = 99.9% | TT (sec), 95% | Compliance Means |
| RCP Time Allocations |  |  |  |
| Initiator | 30 | 30 | Procedures, flight crew training and qualification in accordance with safety requirements. |
| TRN Time Allocations |  |  |  |
| Responder | 60 | 60 | Procedures, flight crew training and qualification in accordance with safety requirements. |
| RCTP Time Allocation |  |  |  |
| RCTPAIRCRAFT | 15 | 10 | Aircraft type design approval, maintenance, properly configured user-modifiable software, e.g. ORT |
| RCTPCSP | 120 | 100 | CSP contract |

| #### RCP availability criteria |  |  |  |
| --- | --- | --- | --- |
| Availability parameter | Efficiency | Safety | Compliance means |
| AAIRCRAFT (probability) | N/A | 0.999 | Aircraft type design approval, maintenance, properly configured user-modifiable software, e.g. ORT |

| #### RCP integrity criteria |  |  |
| --- | --- | --- |
| Integrity parameter | Integrity value | Compliance means |
| Integrity (I) | 10-5 | Aircraft type design approval, establish procedures, training, and qualification to meet safety requirements |

| #### RCP monitoring and alerting criteria |  |  |
| --- | --- | --- |
| Ref: | Criteria | Compliance means |
| MA-2 | When the flight crew determines that the aircraft communication capability no longer meets the requirements for the intended function, the flight crew shall advise the ATC unit concerned. | Procedures, flight crew training and qualification |

| #### RCP related safety requirements |  |  |
| --- | --- | --- |
| Ref | Related RCP Parameter | Safety requirement |
| SR-22 | C, I | The flight crew shall perform the initiation data link procedure again with any change of the flight identifier. |
| SR-24 | C, I | The flight crew shall respond to a message in its entirety when not responded by the aircraft system. |
| SR-27 | C, I | The aircraft and/or flight crew shall ensure the correct transfer into or out of the aircraft’s FMS of route data received/sent via data link that will be used to define the aircraft’s active flight plan. |