THE TWENTY-NINTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/29)

(Fukuoka, Japan, 27-31 October 2008)

Agenda item 5:  Air Traffic Management (ATM) Issues

Proposal for the operational trial of User Preferred Routes (UPRs) between RJAA and YSSY/YBBN/YBCS

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This paper proposes starting the operational trial for the User Preferred Routes (UPRs) between RJAA and YSSY/YBBN/YBCS in 2009.

- Introduction

1.1	At the IPACG 26 meeting, Japan Airlines addressed their proposal of the group starting a study for the UPRs operation between Japan and Australia. JCAB presented the paper on flexible tracks between RJAA and YSSY/YBBN at the meeting. In this paper, they proposed to study for the introduction of flexible track system or UPRs to these city-pairs.

1.3 	JCAB and FAA discussed commencing the paper trial at the IPACG/27 Providers Meeting. They agreed to start a paper study to determine what the impact to ATC operations in the both FIRs. FAA undertook to coordinate a paper trial with the other ANSPs including the members of ISPACG.

1.4	At the ISPACG 22 meeting, JCAB and FAA discussed with the other ANSPs and airlines to reach an agreement on the details for the RJAA-YSSY/YBBN/YBCS UPR Paper Trial. The first paper trial had been conducted from April to May 2008.

1.5	Partial data from the Initial Paper Trial had been obtained at the IPACG/28 meeting.  The data and operational impacts were discussed.  It was determined that an additional paper trial would be required to determine the operational impacts during different seasonal conditions. The second paper trial began from October 6 2008 (last to November 24 2008) in cooperate with JAL and QFA.

2.	Discussion

2.1	The Guideline which has been applied to the paper trials is as follows. All of items should be reviewed prior to the beginning of the operational trial by ANSPs and airliners concerned.

UPR Generation Paper Trial Guidelines:

1.  The North and Southbound UPRs will remain within the Fukuoka, Oakland, Guam, Port Moresby and Brisbane CTAs.

2.  The UPR paper routes will start or end at the Waypoints along latitude 30 North and follow existing conventional ATS, departure and arrival routings north of 30N.

3.  The flying time between filed reporting points must not exceed 80 minutes flying time.

4.  Aircraft routes must include filed reporting points on the CTA boundary crossings in the FPL.

5.  Aircraft must file around NOTAMed and charted active military airspace.

6.  To segregate UPR traffic with domestic traffic operating along the eastern coast of Australia (e.g. Brisbane – Cairns), the following gates will be used for Japan/Australia UPRs:

“150NM Cairns” (i.e. BENKI, MEMIG, PUPEB, MUNEL), HM, KELPI, or SETER

Aircraft departing Australia are to plan ATS routes (or CS DCT BENKI) to the gate then UPR

Aircraft arriving in Australia may plan UPR to the gate then via ATS route (or BENKI DCT CS)

7.  When generating the data and routings for this UPR paper trial, volcanic activity or ash clouds will not be considered for this effort.

2.2	According to the review for the first paper trial, JCAB considers that no significant impact is found to conduct an operational trial on condition that some constraints are modified. Therefore, JCAB seeks the implementation of the operational trial at the earliest timing of 2009 so as to get the further experience for a study.

2.3	JCAB considers that the item of No. 2 which is included in the current Guidance should be modified to as below. Even if this modification will enforce the further limitation to the flexibility in UPRs operation, JCAB recognizes merits of getting the more controller’s experience and the practical data by the operational trials;

2.  The UPR operational  routes will start or end at the Waypoints along latitude 30 North and follow existing conventional ATS, departure and arrival routings north of 30N and remain within the east  airspace which is kept away from 25 NM of the center line of ATS route B586 within the Fukuoka FIR.

2.4	IPACG/29 and ISPACG/23 are the appropriate places to review the Guidance and to discuss how the operational trial would be conducted. If no significant impact on ATC operations is recognized through the paper trials, FAA and JCAB would consider proposing an operational trial to the next ISPACG meeting which is held on March 2009. JCAB considers the first operational trial could start by the mid 2009.

3.	Action by the meeting

  - The meeting is invited to;

- make the initial review for the second paper trial of UPRs between RJAA and YBBN/YSSY;

- review the current guidelines for the trial and;

- discuss the plan of implementation of the operational trial for UPRs between RJAA and YBBN/YSSY/YBCS.

- END -