| U.S. Department of Transportation Federal Aviation Administration DRAFT | Advisory Circular |
| --- | --- |

| ### Subject: Guidelines for Design Approval of Aircraft Data Link Communication Systems | ### Date: Initiated By: AIR-100 | ### AC No. 20-140A Change: |
| --- | --- | --- |

- What is the Purpose of this Advisory Circular (AC)?

- The guidance in this advisory circular (AC) provides one acceptable means of compliance, but not the only means, for type design approval of aircraft that has a data link system installed.  The acceptable means identifies safety and performance (SPR) standards and interoperability (INTEROP) standards for aircraft data link systems in different types of operating environments.

- Use the guidance in this AC to gain approval for type certificates (TC) and supplemental type certificates (STC) involving aircraft data link systems.  Also, it will facilitate standardization of these approvals among the aircraft certification offices (ACOs) in their assessment of aircraft data link systems for design approval.

- This AC was developed in consideration of the International Civil Aviation Organization (ICAO) Annexes and guidance material, RTCA Inc. documents (RCTA/DO) and European Organization for Civil Aviation Equipment (EUROCAE) related documents (ED).

- This AC is not mandatory and does not constitute a regulation.  It is issued to provide guidelines and to outline a means of showing that an aircraft data link system complies with airworthiness standards contained in Title 14 of the Code of Federal Regulations (14 CFR), chapter 1, subchapter C.

- This AC provides airworthiness requirements for the type design approval of aircraft with data link system installed.  AC 120-70B,Operational Authorization Process for use of Data Link Communication System, provides guidelines for the operational authorization needed to use the data link capability.

# Who Does this AC affect?

This AC provides guidelines for applicants seeking design approval of aircraft data link systems used for communication supporting air traffic services (ATS).

# Guidance on Standards Development and Compliance.

- RTCA/DO-264/ED-78A, Guidelines for Approval of the Provision and Use of Air Traffic Services supported by Data Communications, dated December 14, 2000, provides guidance on how to develop standards for an operational data link system and show that the data link implementation complies with the standards.  These guidelines take into consideration the different types of approvals for the aircraft data link system, the ground infrastructure, and the operational use of the data link capability. All standards specified in Tables 1 and 2 of this AC were developed using the RTCA/DO-264/ED-78A

- RTCA/ DO-264/ED-78A, Annex A, Compliance Matrix, provides a summary of the objectives that defines the evidence appropriate for each of the approval types, such as design approval per the guidance of this AC, or operational authorization per the guidance of AC 120-70B. This AC recognizes Annex A as a verification tool to achieve traceability to the means used to show compliance, and to ensure that all objectives relevant to the design approval of the aircraft data link system have been met. Complying with the objectives of this AC and using the standards specified in Tables 1 and 2 are considered to satisfy that criteria and RTCA/DO-264/ED-78A, especially sections 5 and 6, and the related Annex A Aircraft Type Design Approval column.

- For standards that are recognized in this AC, you will need to consider the intended uses of the aircraft data link system, choose the appropriate standards, and ensure your data link system complies with the requirements. As stated in RTCA/DO-264/ED-78A, you might need to show that the development and qualification of the aircraft data link system complies with the objectives in RTCA/DO-264/ED-78A, section 5 and section 6, as deemed relevant by Annex A.

- For any SPR and/or INTEROP standard that you would like to provide as a bench mark to a mean of compliance but not specified in this AC, you will need to provide these standards and show that they comply with the objectives provided in RTCA/DO-264/ED-78A, especially sections 4, 5, and 6.  Hence, per DO-264/ED-78A these standards are intended to define and validate the assumptions for the applicable operating environment and to determine the requirements for the aircraft data link system.

# Definitions for Different Types of Data Link Systems and Operations.

- Table-1 provides the criteria for different types of data link systems and operations based on standards.  For each type of data link system, a row is provided to assign a label to a particular aircraft capability, and correlate it to the applicable operating environment.  The applicable operating environment is characterized by the type of airspace, the capability of the ATS unit, and the use for which that aircraft capability is intended.  The aircraft label is used in the rotorcraft/airplane flight manual (R/AFM) to convey the functional and interoperability aspects of the aircraft data link system needed to support operations.

- Use Table-1 to identify the applicable operating environment(s) and intended uses for your specific aircraft data link system.  An aircraft data link system may support any combination of aircraft labels in the table.  For an aircraft data link system that combines aeronautical telecommunications network (ATN) B1 and future navigation system (FANS) 1/A capabilities that can be used on the same flight, you will need to develop an interoperability standard per paragraph 3.c.

Table 1, Criteria for Different Types of Data Link Systems and Operations

| Row # | Aircraft Data Link System | Operating Environment |  |  | Applicable Standards |
| --- | --- | --- | --- | --- | --- |
|  |  | Type of Airspace | ATS Unit System | Capability and Uses |  |
| 1 | ATN B1 | Continental (Domestic) | ATN B1 | Supplemental ATC communications: - CM application supports DLIC data link service - CPDLC application supports ACM, ACL, and AMC data link services Note 1: DCL, DSC, D-ATIS, and FLIPCY data link services are not supported. | - DO-290/ED-120, Chg 1 and Chg 2, Continental SPR Standard - DO-280B/ED-110B, ATN B1 INTEROP Standard |
| 2 | FANS 1/A+ | Continental (Domestic) | ATN B1-FANS 1/A | Same as Row 1, except: - Uses AFN application for DLIC data link service - For CPDLC application, UM 215, TURN [direction] [degrees] is not supported Note 2: FANS 1/A aircraft will require use of DM67 [free text] to mimic certain message elements per DO-290/ED-120 Chg 1 and 2. See DO-305/ED-154, paragraph 4.2.13.2. Note 3: In accordance with DO-290/ED-120, Chg 1 and Chg 2, FANS 1/A aircraft will require use of a message latency timer per DO-258A/ED-100A, paragraph 4.6.6.9 and is denoted by a “+” appended to the “FANS 1/A” label. Note 4: Only via VHF data link sub-network. | Same as for Row 1, plus: - DO-305/ED-154, FANS 1/A – ATN INTEROP Standard (Applies only to ATS Unit, except see Note 2) - DO-258A/ED-100A, FANS 1/A INTEROP Standard (Applies only to aircraft) |
| 3 | FANS 1/A+ or FANS 1/A | Oceanic and remote | FANS 1/A | Normal means of ATC communication uses AFN and CPDLC applications for DCPC Eligible for: - RCP 240 operations via VHF, Satcom Iridium, and Satcom Inmarsat sub-networks - RCP 400 operations via HF data link sub-network - No RCP operations Note 4: Aircraft capability that supports multiple RCP type operations needs to include appropriate indications and/or alerts to enable the flight crew to notify ATC when aircraft equipment failures result in the aircraft’s ability to no longer meet its criteria for any of the RCP types, per DO-306/ED-122, paragraph 5.2.6.a) and 5.2.6.b). Uses ADS-C application for automatic position reporting. | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier versions), FANS 1/A INTEROP Standard |
| 4 | FANS 1/A+ or FANS 1/A | Oceanic and remote | CADS | No CPDLC application Uses ADS-C application for automatic position reporting | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version), FANS 1/A INTEROP Standard (Applies only to aircraft). - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 5 | FMS WPR | Oceanic and remote | CFRS | No CPDLC application Uses FMS WPR application for automatic position reporting | - DO-306/ED-122, Oceanic SPR Standard - ARINC 702A, Advanced Flight Management Computer System (Applies only to aircraft). - CFRS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 6 | FANS 1/A ADS-C | Oceanic and remote | FANS 1/A or CADS | Same as row 4 | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version) FANS 1/A INTEROP Standard (If ATS unit is CADS, applies only to aircraft) - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit when ATS unit is CADS) |

ACRONYMS:

| ACL | ATC clearance |
| --- | --- |
| ACM | ATC communications management |
| ADS-C | Automatic dependent surveillance – contract (formerly ADS) |
| AFN | ATS facility notification |
| AMC | ATC microphone check |
| ATC | Air traffic control |
| ATN | Aeronautical telecommunication network |
| ATN B1 | ATN baseline 1 |
| ATS | Air traffic service |
| CADS | Centralized ADS-C system |
| CFRS | Centralized flight management computer waypoint reporting system |
| CM | Context management |
| CPDLC | Controller pilot data link communications |
| D-ATIS | Data link - automated terminal information service |
| DCL | Departure clearance |
| DCPC | Direct controller-pilot communications |
| DSC | Downstream clearance |
| FANS | Future air navigation system |
| FLIPCY | Flight Plan Consistency |
| FMS WPR | Flight management system waypoint position reporting |
| HF | High frequency |
| INTEROP | Interoperability |
| NAT FIG | North Atlantic FANS 1/A Implementation Group |
| RCP | Required communication performance |
| Satcom | Satellite communications |
| SPR | Safety and performance |
| VHF | Very high frequency |

- 5.	Means of Compliance for the Aircraft Data Link System.

- The means of compliance provided in this AC are described in terms of :

- Standards – that provide functional, safety, performance, and interoperability criteria for the aircraft data link system, and

- Objectives – that are satisfied by the processes used to develop and qualify the aircraft data link system.

- Table 2 refers to the appropriate criteria for aircraft data link systems for each of the data link definitions provided in Table 1.

- The criteria provided in each row of Table 2 are entirely applicable for the row.

- The design approval for the aircraft data link system is applicable when the assumptions are valid for the intended use.  Include in the R/AFM or other appropriate certification document any relevant information needed to the operational authorization.  See paragraph 8 in this AC.

- You may request changes from the provisions of this AC.  Any change to the data link definitions listed in Table 1 is considered a change to the provisions of this AC, including:

- Any standard not recognized in this AC

- Partial compliance to the criteria provided in the rows of Table 2

- Deviations from the standards recognized in this AC

- Deviations from the relevant objectives identified in Annex A of RTCA/DO-264/ED-78A.

Table 2.  Criteria for the Aircraft Data Link System

| Aircraft Label | Applicable Standard | Reference to criteria applicable to aircraft means of compliance |
| --- | --- | --- |
| ATN B1 | DO-290/ED-120, Chg 1 and Chg 2 | Annex A, Allocation of requirements (Denoted by “Aircraft System”) |
|  | DO-280B/ED-110B | Annex B, para. B.2.1, Airborne System CM P/OICS Annex B, para. B.4.1, Airborne System CPDLC P/OICS |
| FANS 1/A+ | DO-306/ED-122 | Annex B, Allocation of requirements (Denoted by “Aircraft System) |
|  | DO-258A/ED-100A | Section 6, Allocation of interoperability requirements (Denoted by “Air”) and relevant portions. |
|  | DO-290/ED-120, Chg 1 and Chg 2 | Annex A, Allocation of requirements (Denoted by “Aircraft System”) |
|  | DO-305/ED-154 | Paragraph 4.2.13.2, Downlink Message Elements |
| FANS 1/A | DO-306/ED-122 | Annex B, Allocation of requirements (Denoted by “Aircraft System”) |
|  | DO-258/ED-100 | Section 6, Allocation of interoperability requirements (Denoted by “Air”). Applicable restrictions: Oceanic and remote only |
| FANS 1/A ADS-C | DO-306/ED-122 | Annex B, Allocation of requirements (Denoted by “Aircraft System”), criteria only for the “surveillance element,” which are supported by the IER and PR data link services using AFN and ADS-C functionality. Applicable restrictions: Oceanic and remote only, not eligible for RCP operations |
|  | DO-258A/ED-100A | Section 6, Allocation of interoperability requirements (Denoted by “Air”) for AFN and ADS-C functionality only. |
| FMS WPR | DO-306/ED-122 | Annex B, Allocation of requirements (Denoted by “Aircraft System”), criteria only for the “surveillance element,” which are supported by the IER and PR data link services using FMS WPR functionality. Applicable restrictions: Oceanic and remote only, not eligible for RCP operations. |
|  | ARINC 702A | Paragraph 4.3.6, AOC Function, for automatic position reports. Note: Provided for definition of term FMS WPR and interoperability aspects of the AOC functionality. See DO-306/ED-122 for the safety assessment criteria with respect to use of this functionality. |

ADDITIONAL ACRONYMS for Table 2:

| AOC | Aeronautical operational control |
| --- | --- |
| IER | Information exchange and reporting |
| P/OICS | Protocol/operational implementation conformance statement |
| PR | Position reporting |

- 6.	Design Considerations for Aircraft Data Link Systems.

- Flight deck human-machine compatibility.

  - Consider other integrated systems when incorporating an aircraft data link system on the flight deck. Also take into account the existing flight deck philosophy for the aircraft type design and the airspace operating environment.

  - Make the human-machine interface compatible with the crew interface and flight deck design of the particular aircraft in which the data communications system and applications are installed.

  - If multiple ATS data link applications are available to the aircraft, base the crew interface and related crew procedures on a common and compatible flight deck philosophy.

- Flight deck human machine interface.

  - Flight Deck Annunciation. The aircraft data link system should have the following annunciation capabilities integrated into the flight deck:

(a)	Unless otherwise substantiated by the safety assessment, give an aural and visual indication for each uplink ATS message intended to be displayed to the flight crew.   Include messages not being displayed immediately because of lack of crew acknowledgement to an earlier ATS message.  Visual alerts alone should be used for non-ATS messages.

(b)	Alert the flight crew to system failures.

(c)	Make the status of the air/ground data communications system available to the flight crew, such as, loss of the data communications connection with communications management unit or its equivalent.

(d)	If message storage or printing capability is provided, include an indication when storage or printing is not possible.

(e)	Suppress the annunciation of the receipt of a message during critical flight phases (such as takeoff and landing) until after the critical flight phase.  Make the criteria that define critical flight phases consistent with the flight deck philosophy and the data link services supported.  For each type of service supported, define the flight phases where messages of particular data link services are received and flight phases when they are suppressed. Refer to AC 120-70B for more guidance on these issues.

  - Flight Deck Controls.  Aircraft data link systems must meet the following control capability criteria:

(a)	Provide a way for the flight crew to activate or deactivate each of the cockpit data communication applications.

(b)	Provide a way for the flight crew to identify, in real time, the identity of the ATS provider(s) connecting to the aircraft, and the applications involved with each connection.

(c)	Provide a way for the flight crew to acknowledge receipt of ATS messages.

(d)	Provide a way for the flight crew to list, select, and retrieve the most recent ATS messages received and sent by the flight crew. Have the status of each message, the time it was received or sent, accessible.

(e)	Provide a way for the flight crew to clear messages from the display.  However, protect this capability against inadvertent clearing.

(f)	Provide a way for the flight crew to create, store, retrieve, edit, delete, and send messages.

(g)	If a direct interface exists between the cockpit data communications application and other computer functions, (such as flight planning and navigation), provide a way for the flight crew to activate the computer function to use the data contained in the message.

  - Flight Deck Displays.  Flight deck displays must meet the following criteria:

(a)	Display all messages without being truncated.  Use a format that the flight crew can comprehend without the need for translation from English into another language, i.e use aviation phraseology.

(b)	Display the messages so the flight crew can read them without leaving their seats.

(c)	Display ATS messages until acknowledged, unless the flight crew selects another message, or in the case of a multi-function display, another display format or function is selected. In these cases include a visual reminder that indicates that pending messages are waiting for a response.

(d)	Display ATS messages so that messages are distinguishable from each other.  Display the status of each message, if acknowledged or not, together with the message.

(e)	When the data communication application is sharing a display with other aircraft functions, ensure appropriate priority for the information to be displayed.

(f)	If a message is greater than the available display area and only part of the message is displayed, provide a visual indication to the pilot to indicate the presence of the message remainder.

- Flight Deck Printer.  You can use a flight deck printer for storing data communications messages received or sent during the current flight.  Ensure it satisfies integrity and interface design criteria appropriate with the aircraft flight deck philosophy.

- Data Communication Recording.  For those aircraft required to install data communication recording equipment, AC 20-160, Onboard Recording of Controller-Pilot Data Link communication in Crash Survivable Memory, describes acceptable means of compliance.

- 7.	Ground and Flight Test Evaluation.

Test your aircraft data link system and application by end-to-end ground testing that verifies system interoperability and performance.  Test with either an appropriate ATS unit or with test equipment that is representative of an actual ATS unit.  You are responsible for submitting evidence that demonstrates compliance with the safety, performance and interoperability requirements. The ACO will determine the acceptability of the submitted evidence.

- 8.	Airplane/Rotorcraft Flight Manual (A/RFM) and A/RFM Supplement Wording.

Provide a description of all normal and submodes of data link system operation including what actions are expected by the flight crew for each case. You can reduce the material addressed by the A/RFM or A/RFM supplement if the information is included in other related references, such as a flight crew operating manual that is used by the operator as the basis for flight crew qualification.  See AC 120-70B, for guidance.

- Operating Limitations.

- Use operating limitations to control the use of data link applications that are intended for evaluation in operational service or if an operating limitation provides an alternative to satisfying the criteria contained in this AC.  For example, although the printer is optional, a limitation that requires a printer would be appropriate for implementations where the flight crew must rely solely on the printer to retrieve messages.

- Operating limitations are not necessary provided the data link system and applications have shown to operate in accordance with the criteria provided in Table 1.

- Operating Procedures.  Assure the operating procedures in the A/RFM or A/RFM supplement are consistent with and refer to the safety, performance and interoperability requirements that were used to demonstrate the system.  For example:

“The FAA has approved the aircraft data link system to the criteria contained in AC 20-140A. This approval was based on the standards for (insert appropriate aircraft labels from Table 1, e.g., FANS 1/A, ATN B1, etc.  Also, if the FAA approves variations to the criteria and those variations impact operational use of the data link system, include additional sentence(s) that either refer to the applicant’s document or provides text that describes the operational impact).  This design approval does not constitute operational authorization.   See AC 120-70B for operational authorization.”

Dave W. Hempe

Manager, Aircraft Engineering Division

Aircraft Certification Service

- Appendix 1. Related Documents.

- International Civil Aviation Organization (ICAO) documents:

  - Annex 2, Rules of the Air.

  - Annex 6, Operation of Aircraft.

  - Annex 10, Aeronautical Telecommunications.

  - Annex 11, Air Traffic Services.

  - Doc 4444, Air Traffic Management.

  - Doc 9869, Manual on Required Communication Performance (RCP).

- RTCA, Inc. Documents (RTCA/DO) and European Organization for Civil Aviation Equipment (EUROCAE) documents (ED):

  - RTCA/DO-160E, Environmental Conditions and Test Procedures for Airborne Equipment, dated December 9, 2004.

  - RTCA/DO-178B/ED-12B, Software Considerations in Airborne Systems and Equipment Certification, dated December 1, 1992.

  - RTCA/DO-258A/ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications, dated April 7, 2005.

  - RTCA/DO-264/ED-78A, Guidelines for Approval of the Provision and Use of Air Traffic Services Supported by Data Communications, dated December 14, 2000.

  - RTCA/DO-280B/ED-110B, Interoperability Requirements Standard for ATN Baseline1 (INTEROP ATN B1), dated June 26, 2007.

  - RTCA/DO-290 Changes 1 and 2/ED-120 Changes 1 and 2, Safety and Performance Requirements Standard for Air Traffic Data Link Services in Continental Airspace (Continental SPR Standard), dated April 29, 2004.

  - RTCA/DO-305/ED-154, Future Air Navigation System 1/A (FANS 1/A) – Aeronautical Telecommunications Network (ATN) Interoperability Standard, dated June 26, 2007.

  - RTCA/DO-306/ED-122, Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace (Oceanic SPR Standard), dated October 11, 2007.

  - ED-93, Minimum Aviation System Performance Standard for CNS/ATM Message Recording Systems, dated November 23, 1998.

- Federal Aviation Administration (FAA) documents:

  - AC 20-160, Onboard Recording of Controller-Pilot Data Link Communication in Crash Survivable Memory.

  - AC 120-70B, Operational Authorization Process for use of Data Link Communication System.