| NOTICE |  | U.S. DEPARTMENT OF TRANSPORTATION FEDERAL AVIATION ADMINISTRATION |  | N 8000.A056 _ |
| --- | --- | --- | --- | --- |
| National Policy |  |  | Effective Date: |  |
|  |  |  | Cancellation Date: |  |
| SUBJ: | OPSPEC/MSPEC/LOA A056: Data Link Communications |  |  |  |

1. Purpose of this Notice.  This notice provides revised guidance for Federal Aviation Administration (FAA) certificate-holding district offices (CHDO) and principal operations inspectors (POI) assigned to operators conducting airplane operations under Title 14 of the Code of Federal Regulations parts 91, 91 subpart K, 121, 125 (including the Letter of Deviation Authority (LODA A125) operators), and 135.  This notice revises operations specifications (OpSpecs), management specifications (MSpecs), and letters of authorization (LOA) to address changes to AC 120-70A.  This is a mandatory change to A056.

2. Audience.  The primary audience for this notice is FAA CHDOs and POIs assigned to operators conducting airplane operations under parts 91, 91 subpart K, 121, 125 (including the Letter of Deviation Authority (LODA A125) operators), and 135.  The secondary audience includes Flight Standards divisions and branches in the regions and in headquarters.

3. Where can I find this Notice?  Inspectors can find his notice and related templates in the Flight Standards Information Management System (FSIMS) at http://fsims.avs.faa.gov.  Operators may find this information on the Federal Aviation Administration (FAA) Web site at:  http://fsims.faa.gov.

4. Background.  The original Advisory Circular 120-70 was applicable to Aeronautical Telecommunications Network (ATN) only.  The revised AC 120-70A corrected the applicability to include all forms of Data Link operations.

5. Guidance.  The Flight Technologies and Procedures Division, AFS-400, in cooperation with the Air Transportation Division, AFS-200, the General Aviation and Commercial Aviation Division, AFS-800, and industry members of the Operations Specification Working Group (OSWG) developed this notice. This notice contains the following:

- The sample OpSpec A056 template in Appendix A that applies to part 121

- The sample OpSpec A056 template in Appendix B that applies to part 125

- The sample OpSpec A056 template in Appendix C that applies to part 135

- The sample MSpec MA056 template in Appendix D that applies to part 91 subpart K

- The sample LOA A056 template in Appendix E that applies to part 91

- The sample LOA A056 template in Appendix F that applies to part 125M

- The guidance for Inspectors that FAA will incorporate into Order 8900.1, Flight Standards Information Systems (FSIMS) Inspector’s Handbook, Volume 3, Chapter 18, Section 2.

6. Action.  Principal Operations Inspectors (POIs) should review the revised guidance for issuance of the paragraph contained in this notice.  POIs should provide this notice to the operators for whom they are responsible, alerting them to updated operating procedures as well as required pilot knowledge and training.  This authorization is mandatory, with a compliance date of 90 days from the date of this notice.

7. Disposition.  Direct questions or comments concerning this notice to the Flight Operations Branch, AFS-470, at 202-385-4623 / 4586.

James J. Ballough

Director, Flight Standards Service

Sample OpSpec Paragraph A056, Data Link Communications:  14 CFR Part 121

The certificate holder is authorized to conduct en route data link communications in accordance with the limitations and provisions of this operations specification.

a.	Authorized Aircraft and Equipment for data link.  The certificate holder is authorized to conduct data link communications using the following aircraft and FAA-certified data link communication systems:

Table 1

| Aircraft M/M/S | Data Link System M/M | Remarks |
| --- | --- | --- |
| TABL01 | TABL02 | TABL03 |

b.	The certificate holder is authorized to conduct en route operations using data link communications  in the areas listed in OpSpec B050 of these operations specifications where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions.

(1)	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

(2)	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

(3)	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

(4)	Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

(5)	Flight crews must have completed the certificate holder’s approved training program prior to conducting data link operations.  Flight crews must be familiar with and comply with:

(a)	All STC and AFM limitations

(b)	All provisions applicable to the use and operation of the installed data link system

(c)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and NOTAMS.

c.	The certificate holder’s continuous airworthiness maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication system.

TEXT99

Sample OpSpec Paragraph A056, Data Link Communications:  14 CFR Part 125

The certificate holder is authorized to conduct en route data link communications in accordance with the limitations and provisions of this operations specification.

a.	Authorized Aircraft and Equipment for data link.  The certificate holder is authorized to conduct data link communications using the following aircraft and FAA-certified data link communication systems:

Table 1

| Aircraft M/M/S | Data Link System M/M | Remarks |
| --- | --- | --- |
| TABL01 | TABL02 | TABL03 |

b.	The certificate holder is authorized to conduct en route operations using data link communications  in the areas listed in paragraph B050 of these operations specifications where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions.

(1)	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

(2)	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

(3)	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

(4)	Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

(5)	Flight crews must have completed the certificate holder’s training program prior to conducting data link operations.  Flight crews must be familiar with and comply with:

(a)	All STC and AFM limitations

(b)	All provisions applicable to the use and operation of the installed data link system

(c)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and NOTAMS.

c.	The certificate holder’s maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication system.

TEXT99

Sample OpSpec Paragraph A056, Data Link Communications:  14 CFR Part 135

The certificate holder is authorized to conduct en route data link communications in accordance with the limitations and provisions of this operations specification.

a.	Authorized Aircraft and Equipment for Data Link Communications.  The certificate holder is authorized to conduct data link communications using the following aircraft and FAA-certified data link communication systems:

Table 1

| Aircraft M/M/S | Data Link System M/M | Remarks |
| --- | --- | --- |

b.	The certificate holder is authorized to conduct en route data link operations in the areas listed in OpSpec B050 of these operations specifications where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions.

(1)	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

(2)	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

(3)	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

(4)	Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

(5)	Flight crews must have completed the certificate holder’s approved training program prior to conducting data link operations.  Flight crews must be familiar with and comply with:

(a)	All STC and AFM limitations

(b)	All provisions applicable to the use and operation of the installed data link system

(c)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and NOTAMS.

c.	The operator’s approved maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication system.

TEXT99

Sample MSpec Paragraph MA056, Data Link Communications:  14 CFR Part 91 Subpart K

The program manager is authorized to conduct en route data link communications in accordance with the limitations and provisions of this management specification.

a.	Authorized Aircraft and Equipment for data link.  The program manager is authorized to conduct data link communications using the following aircraft and FAA-certified data link communication systems:

Table 1

| Aircraft M/M/S | Data Link System M/M | Remarks |
| --- | --- | --- |
| TABL01 | TABL02 | TABL03 |

b.	The program manager is authorized to conduct en route operations using data link communications  in the areas listed in paragraph MB050 of these management specifications where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions.

(1)	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

(2)	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

(3)	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

(4)	Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

(5)	Flight crews must have completed the program manager’s approved training program prior to conducting data link operations.  Flight crews must be familiar with and comply with:

(a)	All STC and AFM limitations

(b)	All provisions applicable to the use and operation of the installed data link system

(c)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and NOTAMS.

c.	The program manager’s continuous airworthiness maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication system.

TEXT99

Sample LOA A056, Data Link Communications:  14 CFR Part 91

Letter of Authorization

Data Link Communications

1.	The Operator listed at the bottom of this document is authorized to conduct en route data link communications in accordance with the limitations and provisions of this Letter of Authorization (LOA).

2.	The Operator is authorized to conduct en route data link operations in airspace where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions:

a.	All aircraft conducting domestic data link operations must be equipped with an FAA-certified collision avoidance system that is on and operating.

b.	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

c.	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

3.	Authorized Aircraft and Equipment for Data Link Communications.  This LOA authorizes the Operator to operate the aircraft and FAA-certified data link communication systems listed in Table 1 below in the conduct of en route data link operations:

Table 1 - Aircraft and Data Link Equipment

| A/C Serial Number | Registration No. | Aircraft M/M/S | Data Link M/M/S | Remarks |
| --- | --- | --- | --- | --- |

4.	Flight Crew Training.  Flight crews must have completed an accepted training program, as described in this authorization, prior to conducting data link operations.  The Operator shall provide training conducted by TEXT02 for the flight crews using data link communications in accordance with the following:

a.	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

b.	Recurrent Training.  Recurrent training for digital communications should address any significant issues identified by operating experience, system changes, procedural changes, or unique characteristics.

c.	Flight crews must be familiar with and comply with the following:

(1)	All STC and AFM limitations and procedures.

(2)	All provisions applicable to the use and operation of the installed data link system.

(3)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and Notams.

(4)	When conducting instrument approach procedures, pilots are responsible to obtain and use the appropriate altimeter setting in accordance with 14 CFR Section 97.20.  Data link-issued altimeter settings are excluded for this purpose.

5.	Voice Communications Requirement.  Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF radios along the entire route of flight.

6.	Operator’s responsibilities regarding data link communications:

a.	Verify data link communications functionality for each environment to be used and when new or modified components or software are introduced.

b.	Assure follow-up and evaluation of exceptional data link events.

c.	Assure that appropriate data link communication software updates are incorporated when necessary, and that both air and ground system are able to identify and properly respond to the installed level of data link communication capability.

d.	Establish data link communications "return to service" policies to ensure proper data link communications functions when an aircraft is returned to service after a data link communication failure or maintenance action.

e.	 Ensure that the maintenance program includes the appropriate airworthiness requirements for the installed data link communication system.

f.	Provide adequate data link communication maintenance training in accordance with the appropriate regulation to ensure that their maintenance personnel or contract maintenance personnel at facilities not staffed by the Operator are able to properly implement digital communications-related maintenance programs.  This includes, but is not limited to, addressing installation, modification, correction of reported system discrepancies, use of test equipment, procedures, MEL relief, and "return-to-service" authorizations.

g.	The Operator’s continuous airworthiness maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication equipment.

7.	Operators must formulate necessary data link communications revisions to their MEL(s) for each particular fleet (e.g., B737, DC10).  MEL revisions must be consistent with the FAA’s MMEL established for each aircraft type.

8.	Responsible Person.  The Responsible Person for crew operations may be either an agent for service (who must be a U.S. citizen) or a person who is a U.S. citizen or holds a U.S. pilot certificate and accepts responsibility for complying with the stated regulations by signing this document.

a.	If the Responsible Person signing this LOA relinquishes responsibility, this LOA becomes invalid.

b.	Enter the name, email address, and telephone number in Table 2 of the Responsible Person signing this LOA:

Table 2 – Responsible Person

| Name | Email Address | Telephone |
| --- | --- | --- |

TEXT99

Sample LOA A056, Data Link Communications:  14 CFR Part 125M

Letter of Authorization

Data Link Communications

The operator is authorized to conduct en route data link communications in accordance with the limitations and provisions of this letter of authorization.

1.	Authorized Aircraft and Equipment for data link.  The operator is authorized to conduct data link communications using the following aircraft and FAA-certified data link communication systems:

Table 1

| Aircraft M/M/S | Data Link System M/M | Remarks |
| --- | --- | --- |
| TABL01 | TABL02 | TABL03 |

2.	The operator is authorized to conduct en route operations using data link communications  in the areas listed in letter of authorization B050 of these operations specifications where the air traffic service (ATS) infrastructure supports the use of Future Air Navigation System (FANS-1/A or equivalent) air traffic control data link equipment and/or Aeronautical Telecommunications Network (ATN)-compliant digital data link communications equipment, as applicable, in accordance with the following limitations and provisions.

a.	All aircraft conducting domestic operations using data link communications with air traffic services must be equipped with an approved collision avoidance system that is on and operating.

b.	Data link communications may be conducted only during the en route phase of flight, or departures and arrivals as applicable, to include domestic, foreign domestic, oceanic or remote airspace.

c.	Domestic data link operations must be conducted in an environment where air traffic surveillance is available (e.g., radar or ADS-B).

d.	Data link messages with air traffic services may be used as a supplement to voice communications.  Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

e.	Flight crews must have completed the operator’s training program prior to conducting data link operations.  Flight crews must be familiar with and comply with:

(1)	All STC and AFM limitations

(2)	All provisions applicable to the use and operation of the installed data link system

(3)	Regulations, policies and procedures applicable in individual countries and/or Flight Information Regions, as published in documents such as Aeronautical Information Publications and NOTAMS.

3.	The operator’s maintenance program must be revised to include the appropriate airworthiness requirements for the installed data link communication system.

TEXT99

VOLUME 3 GENERAL TECHNICAL ADMINISTRATION

CHAPTER 18  OPERATIONS SPECIFICATIONS

Section 3   Part A Operations Specifications—General

### OPSPEC/MSPEC/LOA A056, DATA LINK COMMUNICATIONS.

#### A.               Template A056 contains specific operational limitations and provisions for granting part 91, 121, 125, or 135 operators or part 91 subpart K program managers authorization to conduct data link communications with systems that are certified for air-ground air traffic services (ATS). All operators must be approved and authorized to use any data communication system and associated communications display unit(s) (CDU) that are used to communicate with ATS.

##### 1)                  Parts 91, 121, 125, and 135 operators and part 91 subpart K program managers conducting data link operations in oceanic and remote airspace may use data communication systems (i.e., Future Air Navigation System (FANS-1/A or equivalent). Data link operations in domestic airspace require very high frequency (VHF) radios called VDL Mode 2, certified for ATS.

##### 2)                  Data link may be used as a supplement to voice communications with ATS. Voice communications must be continually monitored since there is still a requirement for the aircraft to be equipped with operating VHF voice and, when required, HF voice radios along the entire route of flight.

##### 3)                  All data link operations in domestic airspace are limited to the en route phase of flight where radar or an equivalent surveillance system such as ADS-B is available for surveillance services.

##### 4)                  All aircraft used to conduct data link operations in domestic airspace must be equipped with an FAA-certified collision avoidance system that is on and operating. (Reference 14 CFR §§ 121.356, 125.224, and 129.18.)

##### 5)                  An exception to the requirement for data link communication systems is the FANS-1/A system in oceanic or remote airspace. The FANS-1/A communications system can only be approved for data link operations in oceanic and remote area airspace. FANS-1/A systems are not interoperable with the VHF-data link mode 2 (VDL-2) infrastructure for domestic data link communications.

#### B.               Parts 121 and 135 air carriers and part 91 subpart K program managers must have an approved data link training program as outlined in FAA AC 120-70A (as amended) , Operational Authorization Process for use of Data Link Communication System, current edition. All operators must provide data link communication training for their flight crews.

#### C.               For parts 91, 121, 125, and 135 operators  and part 91 subpart K program managers, the validation test process outlined in Volume 3, Chapter 29, Proving and Validation Tests, must be followed. Applicants who have no previous experience conducting data link must demonstrate competency to the FAA during a validation flight in the aircraft.

##### 1)                  A validation flight is not required in each make, model, and series of aircraft as long as the flight configuration and displays used are the same.

##### 2)                  During the validation flight test, the FAA evaluator must ensure that the operator’s procedures, knowledge, and use of the data link system are consistent with the operator’s data link training program and the recommendations outlined in AC 120-70A (as amended).

#### D.           For parts 91, 121, 125, and 135 operators and part  91 subpart K program managers, the POI will coordinate with the principal avionics and airworthiness inspectors for the certification and airworthiness approval review, the content of the OpSpec Authorization, the required communication performance, the aircraft flight manual, additional minimum equipment list requirements and relief, STC (if applicable) and other such areas necessary for the safe and effective use of data link. Identify the communications management unit (CMU)/VHF digital radio (VDR) make and model, etc. the equipment field of the OpSpec. Verify that software versions are noted in other appropriate maintenance manuals.

NOTE: POIs should be aware that there may be additional limitations and guidance for specific airplanes in Flight Standardization Board (FSB) reports.

E. CONTENT OF OPERATOR APPLICATION FOR OPERATIONAL AUTHORIZATION TO USE DATA LINK.

The operator’s application to obtain authorization to use Data Link must address and contain the following subjects including those identified in AC 120-70A (as amended).   The applicable AC 120-70A (as amended) paragraphs are referenced below.

1)  List of Source documents used:

a. For Generic Data Link  Operations (e.g., Aircraft/Avionics Manufacturer Documents)

b. For area of operations specific policy/procedures.  See 3 below

2)  Description of aircraft Data Link systems including certification documents and current configuration (e.g., current avionics load)

3)  General Information.

4)  Area of Operations/Routes where operator intends to use Data Link.

a. List of areas and/or routes where operator intends to conduct Data Link operations

b. List of Air Traffic Centers/Service Providers with which operator intends to communicate via Data Link

c. List policy/procedures source documents applicable to each area(s) of operations, such as:

- Operations Manuals for specific areas of operations (e.g., Fans Operations Manual (FOM) for operation in Asia Pacific FIR’s)

- State Aeronautical Information Publications (AIP)

- State NOTAMS

- FAA Chart Supplements (E.g., Pacific and Alaska Chart Supplement)

5.  Flight Crew Qualification Programs.

6.  Manuals and Other Publications.

7.  MMEL/MEL.

8.  Issues Unique To a Particular Operator.

9.  Maintenance Programs.

# F.   CONTENT OF FLIGHT-CREW QUALIFICATION PROGRAMS

1)  Academic Training Subjects.  Basic source document for Data Link procedures in oceanic areas is the FANs Operations Manual (FOM), part 5.  Policy and procedures applicable to specific FIR’s are in State AIPs and Notams.  AC 120-70A (as amended) calls for the following areas to be addressed:

- Acronyms   Source:  FANS Operations Manual (FOM) Part 2.

- General Concepts of Digital and Analog Communications.

- Expected Flight crew Response

- ATS Coordination

- Aircraft Digital or Analog Communication Equipment Components, Displays, Alerts.  SOURCES:   Aircraft manufacturer documents.

- Interface with other aircraft systems

- AFM Information MEL Provisions

- Data link Events Reports

- Data link Malfunction or Irregularity Reports

- Human Factors—Lessons learned

2).  Operational Use Training

- General requirement

- Simulators

- Computer Based Instruction (CBI)

- Policy on Initial Pilot Evaluation

- Recurrent Training and Evaluation

3)  Currency (Recent Experience)

4)  Line Checks and Route Checks (if applicable)

5)  LOFT (if applicable)

G.  OPERATIONAL AUTHORIZATION DOCUMENTS.

Approval to use Data Link in operations is granted through the issuance of paragraph A056.  The CMO should coordinate the approval with AFS-400.

H.  INSPECTOR RESOURCES

The following table describes the systems and their operating environment including the applicable criteria with references.

Table 3-7, Data Link Systems & Operating Environment

| Row # | Aircraft Data Link System | Operating Environment |  |  | Applicable Standards |
| --- | --- | --- | --- | --- | --- |
|  |  | Type of Airspace | ATS Unit System | Capability and Uses |  |
| 1 | ATN B1 | Domestic (Continental) | ATN B1 | Supplemental ATC communications: - CM application supports DLIC data link service - CPDLC application supports ACM, ACL, and AMC data link services Note 1: DCL, DSC, D-ATIS, and FLIPCY data link services are not supported. | - DO-290/ED-120, Chg 1 and Chg 2, Continental SPR Standard - DO-280B/ED-110B, ATN B1 INTEROP Standard |
| 2 | FANS 1/A+ | Domestic (Continental) | ATN B1-FANS 1/A | Same as Row 1, except: - Uses AFN application for DLIC data link service - For CPDLC application, UM 215, TURN [direction] [degrees] is not supported Note 2: FANS 1/A aircraft will require use of DM67 [free text] to mimic certain message elements per DO-290/ED-120 Chg 1 and 2. See DO-305/ED-154, paragraph 4.2.13.2. Note 3: In accordance with DO-290/ED-120, Chg 1 and Chg 2, FANS 1/A aircraft will require use of a message latency timer per DO-258A/ED-100A, paragraph 4.6.6.9 and is denoted by a “+” appended to the “FANS 1/A” label. Note 4: Only via VHF data link sub-network. | Same as for Row 1, plus: - DO-305/ED-154, FANS 1/A – ATN INTEROP Standard (Applies only to ATS Unit, except see Note 2) - DO-258A/ED-100A, FANS 1/A INTEROP Standard (Applies only to aircraft) |
| 3 | FANS 1/A+ or FANS 1/A | Oceanic and remote | FANS 1/A | Normal means of ATC communication uses AFN and CPDLC applications for DCPC Eligible for: - RCP 240 operations via VHF, Satcom Iridium, and Satcom Inmarsat sub-networks - RCP 400 operations via HF data link sub-network - No RCP operations Note 4: Aircraft capability that supports multiple RCP type operations needs to include appropriate indications and/or alerts to enable the flight crew to notify ATC when aircraft equipment failures result in the aircraft’s ability to no longer meet its criteria for any of the RCP types, per DO-306/ED-122, paragraph 5.2.6.a) and 5.2.6.b). Uses ADS-C application for automatic position reporting. | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier versions), FANS 1/A INTEROP Standard |
| 4 | FANS 1/A+ or FANS 1/A | Oceanic and remote | CADS | No CPDLC application Uses ADS-C application for automatic position reporting | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version), FANS 1/A INTEROP Standard (Applies only to aircraft). - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 5 | FMS WPR | Oceanic and remote | CFRS | Same as row 4 | - DO-306/ED-122, Oceanic SPR Standard - ARINC 702A, Advanced Flight Management Computer System (Applies only to aircraft). - CFRS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 6 | FANS 1/A ADS-C | Oceanic and remote | FANS 1/A or CADS | Same as row 4 | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version) FANS 1/A INTEROP Standard (If ATS unit is CADS, applies only to aircraft) - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit when ATS unit is CADS) |