THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

ATS Ground System SPR Compliance

(Presented by United States of America)

Summary

DO-306/ED-122, Oceanic SPR standard, allocates a number of safety requirements to the air traffic service provider (ATSP) ground systems. The FAA has completed an analysis of the Ocean 21 system to determine the level of compliance with the requirements.

# Introduction

## DO-306/ED-122 Oceanic SPR standard lists twenty seven safety requirements for the application of data link services in oceanic airspace. Twenty four requirements are applicable to ATSP ground systems.  The twenty four requirements that are allocated to the ground system are listed in Appendix A.

## The FAA has completed an analysis of the Ocean 21 system to determine the level of compliance with these requirements.  This paper provides information on results of the analysis.

# Discussion

## The FAA has determined that the Ocean 21 system is not compliant with two requirements and is partially compliant with one.

## The first safety requirement which the Ocean 21 system is not currently complaint with is SR-9 which relates to including a timestamp in every uplinked CPDLC message.  In the original CPDLC specification, RTCA DO-219, the timestamp for uplinked messages was optional.  An Ocean 21 software change proposal to include a timestamp with all uplinked CPDLC messages has been submitted.  The software version supporting this change is schedule for operational release in February 2009.

## The second safety requirement which the Ocean 21 system is not currently complaint with is SR-15 which relates to an appropriate indication to the controller when a clearance response is received that has a latency exceeding ETTRN. An Ocean 21 software change proposal to notify the controller when a clearance response is received after the expiry of ETTRN has been submitted. However, no date for implementation of this change has been set.

## Some discussion on implementation strategy is still underway.  A question exists as to whether to use the timestamp in the CPDLC header or the Ocean 21 message receipt time to determine if ETTRN has expired.  If the CPDLC timestamp is the appropriate value to use then the question arises whether the controller should be notified when a message is received after ETTRN expiry but the CPDLC timestamp is within the ETTRN value?  Another discussion point relates to what would be the appropriate timestamp to use for HF aircraft (to be measured against the RCP400 ETTRN value).

## The requirement that Ocean 21 is partially compliant with is SR-14 relating to an indication to the controller when a read back is not received within ETTRN. Ocean 21 has an adapted variable system parameter (RBK_WAIT_PRD) currently set at 300 seconds, which warns the controller if a clearance read back, delivered via CPDLC or HF, is not received within 300 seconds. A discussion is underway to decided as to whether to reduce this parameter value to 210 seconds (ETTRN for RCP240) and use it for both RCP240 and RCP400 as a temporary fix.  While this would bring the system into compliance it would also generate unnecessary controller notifications which carry their own risk.  An Ocean 21 software change proposal for a more permanent solution has been submitted.  The new logic would differentiate between RCP400 (ETTRN is 370 seconds) and RCP240 (ETTRN is 210 seconds) timeouts.  No date for implementation of this change has been set.

# Action

## The meeting is invited to:

- Note the safety requirements allocated to the ATSP ground system by the Oceanic SPR standard;

- Note the ongoing work by the FAA to comply with these requirements in the Ocean 21 ATSP ground system;

- Request participating states to report to the FIG on the compliance of their ATSP ground system(s) with the allocated safety requirements; and

- Track the status of any non-compliant systems.

Appendix A:  ATSP Ground System Safety Requirements (from RTCA DO-306 Table B-2)

| RQMT | Requirement Description | ATSP System |
| --- | --- | --- |
| SR-1 | An indication shall be provided to the initiator when a recipient rejects a data link service request at the application layer. Note: Rejection can also occur in a communication layer. In this case, a message could be discarded before it is processed by the application. | The ATSU shall display the indication provided by the aircraft system when a data link service request initiated by the ground system or the controller is rejected at the application layer. The ATSU shall provide to the aircraft system an indication when it rejects a data link service request initiated by the flight crew at the application layer. |
| SR-2 | A detected loss of data link service shall be indicated to the controller/flight crew | The ATSU shall indicate to the controller a detected loss of data link service. |
| SR-3 | Data link service shall be established in sufficient time to be available for operational use. | Data link service shall be established in sufficient time to be available for operational use. |
| SR-4 | ATSU shall be notified of planned outage of data link service sufficiently ahead of time | ATSU shall be notified of planned outage of data link service sufficiently ahead of time |
| SR-5 | There shall be an indication to the initiator when a message cannot be successfully transmitted. | The ATSU shall indicate to the controller when a message cannot be successfully transmitted. |
| SR-6 | The end system shall provide unambiguous and unique identification of the origin and destination with each message it transmits. | The ATSU end system shall provide unambiguous and unique identification of the origin and destination with each message it transmits. |
| SR-7 | A response shall indicate to which message it refers. | The ATSU shall indicate in each response to which message it refers. |
| SR-8 | The aircraft and the ATSU shall exchange via data link and process the same route information. | The ATSU shall send the route clearance information with the route clearance via data link. |
| SR-9 | The end system shall time stamp to within one second UTC for each CPDLC message when it is released for onward transmission. | The ATSU end system shall time stamp to within one second UTC each message when it is released for onward transmission. |
| SR-11 | Any processing (data entry/ encoding/ transmitting/ decoding/ displaying) shall not affect the intent of the message. | Any processing performed by ATSU (data entry/ encoding/ transmitting/ decoding/ displaying) shall not affect the intent of the message. |
| SR-12 | The end system shall reject messages not addressed to itself. | The ATSU end system shall reject messages not addressed to itself. |
| SR-13 | The initiator shall transmit messages to the designated end system. | The ATSU shall transmit messages to the designated aircraft system. |
| SR-14 | The ATSU system shall indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN) | The ATSU system shall indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN) |
| SR-15 | When the end system receives a message whose time stamp exceeds ETTRN the end system shall provide appropriate indication. Note 1: Appropriate indication may include the need for real time monitoring and alerting (such as via an uplink delay timer, see Table 4-4 and Table 4-8). This is determined based on further safety assessment and is a local matter. The safety assessment needs to consider message content, intended use, and environmental conditions that could potentially lead to an unacceptable risk of undetected late delivery of a message as determined by the continuity requirement. | When the ATSU receives a message whose time stamp exceeds ETTRN the ATSU shall provide appropriate indication. |
| SR-16 | The ATSU and aircraft end system shall prevent the release of clearance and operational responses without controller of flight crew action. | The ATSU shall prevent the release of clearance without controller action. |
| SR-17 | The recipient system shall prohibit operational processing of corrupted messages. | The ATSU shall prohibit operational processing by controller of corrupted messages. |
| SR-18 | The recipient shall be able to determine the message initiator. | The ATSU shall be able to determine the message initiator. |
| SR-19 | The recipient system shall prohibit operational processing of messages not addressed to the recipient. | The ATSU shall prohibit operational processing of messages not addressed to the ATSU. |
| SR-20 | ATSU shall only establish and maintain data link services when the aircraft identifiers in data link initiation correlate with the ATSU's corresponding aircraft identifiers in the current flight plan. | ATSU shall only establish and maintain data link services when the aircraft identifiers in data link initiation correlate with the ATSU's corresponding aircraft identifiers in the current flight plan. |
| SR-21 | The aircraft identifiers used for data link initiation correlation shall be unique and unambiguous (e.g. the Aircraft Identification and either the Registration Marking or the Aircraft Address). | The aircraft identifiers used for data link initiation correlation by the ATSU shall be unique and unambiguous (e.g. the Aircraft Identification and either the Registration Marking or the Aircraft Address). |
| SR-23 | The ATSU shall not permit data link services when there are no compatible version numbers. | An ATSU system shall not permit data link services when there are non compatible version numbers. |
| SR-24 | Messages shall be responded to in their entirety. | The ATSU shall respond to messages in their entirety. |
| SR-25 | The end system shall be capable of detecting errors that would result in mis-delivery introduced by the communication service. Note: A number of algorithms exist that have demonstrated error detection at acceptable levels of integrity, such as those based on cyclic redundancy check (CRC) or Fletcher's checksum. The specific algorithms found to be viable need to be defined in the Interoperability Standards appropriate for the technology used. | The ATSU end system shall be capable of detecting errors that would result in mis-delivery introduced by the communication service. |
| SR-26 | The end system shall be capable of detecting errors that would result in corruption introduced by the communication service. Note: A number of algorithms exist that have demonstrated error detection at acceptable levels of integrity, such as those based on cyclic redundancy check (CRC) or Fletcher's checksum. The specific algorithms found to be viable need to be defined in the Interoperability Standards appropriate for the technology used. | The end system shall be capable of detecting errors that would result in corruption introduced by the communication service. |

-END-