Controller Pilot Data Link
Communications (CPDLC)
REFERENCE GUIDE

Last Updated:  5 July 2008

Includes:

Operational Implementations
North Atlantic (NAT) Phase 4 CPDLC (NAT Guidance Material, version 17)
Asia-Pacific and other regions CPDLC (FANS 1/A Operations Manual, version 5)

Planned Implementations
Europe Link 2000+ (Implementing Rule for Data Link Services)

Standards
ICAO Doc 4444 (PANS/ATM), 14th and 15th editions
RTCA/EUROCAE
DO-306/ED-122, Oceanic SPR Standard
DO-305/ED-154, FANS 1/A – ATN INTEROP Standard
DO-290/ED-120, Chg 1 and Chg 2, Continental SPR Standard
DO-280B/ED-110B, ATN B1 INTEROP Standard
DO-258A/ED-100A, FANS 1/A INTEROP Standard

TABLE OF CONTENTS

1	Introduction	1

1.1	Purpose	1

1.2	Scope	1

1.3	Document organization	2

1.4	References	2

2	CPDLC application – message elements and standardized free text	4

2.1	CPDLC message elements and standardized free text messages	4

2.2	CPDLC communication transactions using free text messages	4

APPENDICES

Appendix A:	Conventions Used in this Standard

LIST OF TABLES

Table 2-1	CPDLC application – uplink message elements	5

Table 2-2	CPDLC application – downlink message elements	76

Table 2-3	CPDLC application – standardized uplink free text messages	106

Table 2-4	CPDLC application – standardized downlink free text messages	122

Table 2-5	CPDLC application – controller initiated standardized free text transactions	130

Table 2-6	CPDLC application – pilot initiated standardized free text transactions	134

# Introduction

This document provides the results of a review of CPDLC message sets that are defined in various standards, have been implemented, or are planned for implementation in various regions of the world.

Some of these documents have been published and are stable, others are continually being updated.  As this document is intended only for cross reference purposes, any discrepancies should be cross checked with the original source of the document listed in paragraph 1.4.

This document may be updated as the need arises.  If you would like to be on distribution of future updates or if you find any discrepancies, errors, or missing information, please advise Tom Kraft, Chief Scientific and Technical Advisor for Aeronautical Communications, Aviation Safety, Federal Aviation Administration (FAA), by email at tom.kraft@faa.gov.

## Purpose

This document is intended to be used as a cross-reference guide across a number of documents to support continuing efforts to converge use of the controller pilot data link communications (CPDLC) application throughout the world, with particular attention given to the message elements and free text capability.

## Scope

The scope of this document is intended to cover all standards and implementation programs that make specific reference to the CPDLC message elements or standardized use of free text.  It has considered the following work:

- ICAO Operational Data Link Panel (OPLINKP) – changes to the ICAO Doc 4444 (PANS/ATM), Appendix 5, CPDLC Message Set;

- Joint RTCA Special Committee (SC) 189 / EUROCAE Working Group (WG) 53 – development of

  - Safety and performance standards for data link services in oceanic and continental airspace, and

  - Interoperability standards for the FANS 1/A and ATN technologies;

- ICAO EUR/NAT Region Data Link Steering Group (DLSG) and ATN Accommodation Drafting Group (ADG) – development of data link and convergence strategy for the ATN and FANS 1/A technologies, specifically to support:

  - North Atlantic Region Phase 3 and 4 data link implementation, and

  - EUROCONTROL LINK 2000+ data link implementation;

- Informal South Pacific Air Traffic Services Coordinating Group (ISPACG), the Informal Pacific Air Traffic Control Coordinating Group (IPACG), and other activities in the Asia-Pacific Regions – continuing updates with FANS Operations Manual.

## Document organization

This document is organized as follows:

- Section 1 provides an introduction to this document.

- Section 2 provides a cross reference for the CPDLC application –message elements and standardized free text.

- Appendix A provides a description of the conventions used in this reference guide.  Refer to this appendix for annotations used and guidelines on how to read the tables.

## References

The references cited in this document are listed below:

- ICAO Doc 4444, 14th and 15th Editions, Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM), applicable date 27 November 2003.

- Asia Pacific FANS 1/A Operations Manual, Version 5, dated August 2007.

- North Atlantic Guidance Material for ATS Data Link Services in North Atlantic Airspace, version 17, dated June 2008.

- RTCA DO-306/EUROCAE ED-122, Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace (Oceanic SPR Standard), dated 11 October 2007.

- RTCA DO-305/EUROCAE ED-154, Future Air Navigation System 1/A – Aeronautical Telecommunication Network Interoperability Standard (FANS 1/A – ATN INTEROP Standard), dated 26 June 2007.

- RTCA DO-290/EUROCAE ED-120, Change 1 and Change 2, Safety and Performance Requirements Standard for Air Traffic Data Link Services in Continental Airspace (Continental SPR Standard), dated 26 June 2007.

- RTCA DO-280B/ED-110B, Interoperability Requirements Standard for ATN Baseline 1 (ATN B1 INTEROP Standard), dated 26 June 2007.

- RTCA DO-258A/ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications (FANS 1/A INTEROP Standard), dated 7 April 2005.

# CPDLC application – message elements and standardized free text

Two types of tables are provided in this document for the CPDLC application:

- CPDLC message elements and standardized free text messages; and

- CPDLC communication transactions using free text messages.

## CPDLC message elements and standardized free text messages

CPDLC uplink and downlink message elements are provided in the following tables.  (See paragraph A.2.1 for conventions used in these tables);

- Table 2-1 provides a cross-reference for the CPDLC application uplink message elements;

- Table 2-2 provides a cross-reference for the CPDLC application downlink message elements;

- Table 2-3 provides a cross-reference for the CPDLC application standardized uplink free text messages;

- Table 2-4 provides a cross-reference for the CPDLC application standardized downlink free text messages.

## CPDLC communication transactions using free text messages

CPDLC communication transactions for controller-initiated and pilot initiated transactions using CPDLC uplink and downlink [Free text] messages are provided in the following tables.  (See paragraph A.2.2 for conventions used in these tables);

- Table 2-5 provides a cross-reference for controller-initiated communication transactions, with pilot responses, using the CPDLC application standardized free text messages; and

- Table 2-6 provides a cross-reference for pilot-initiated communication transactions, with controller responses, using the CPDLC application standardized free text messages.

Table 2-1	CPDLC application – uplink message elements

| Ref # | Message Intent/Use | Message Element | Resp. | Ocean SPR | NAT | Asia-Pac | Cont. SPR | ATN B1 | FANS 1/A | FANS 1/A – ATN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Responses/ Acknowledgments (uplink) |  |  |  |  |  |  |  |  |  |
| UM 0 | Indicates that ATC cannot comply with the request. | UNABLE | N | All | N/A | N/A | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  |  |  | NE | All | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
| UM 1 | Indicates that ATC has received the message and will respond. | STANDBY | N | All | N/A | N/A | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  | Asia-Pac & NAT: The pilot is informed that the request is being assessed and there will be a short-term delay (within 10 minutes). The exchange is not closed and the request will be responded to when conditions allow. |  | NE | All | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
| UM 2 | Indicates that ATC has received the request but it has been deferred until later. | REQUEST DEFERRED | N | All | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: The pilot is informed that the request is being assessed and a long-term delay can be expected. The exchange is not closed and the request will be responded to when conditions allow. |  | NE | All | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 3 | Indicates that ATC has received and understood the message. | ROGER | N | All | N/A | N/A | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  |  |  | NE | All | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
| UM 4 | Yes. | AFFIRM | N | All | N/A | N/A | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  |  |  | NE | All | P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
| UM 5 | No | NEGATIVE | N | All | N/A | N/A | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  |  |  | NE | All | P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
| UM 235 | Notification of receipt of unlawful interference message. | ROGER 7500 | N | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A |  |
| UM 211 | Indicates that the ATC has received the request and has passed it to the next control authority. | REQUEST FORWARDED | N | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM211. | UM169x: REQUEST FORWARDED | R | N/A | N/A | N/A | Chg 2 ACL | N/A | [Free text] | Initial |
| UM 218 | Indicates to the pilot that the request has already been received on the ground. | REQUEST ALREADY RECEIVED | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 237 | Indicates that the request cannot be responded to by the current unit and that it should be requested from the next unit. | REQUEST AGAIN WITH NEXT UNIT | N | N/A | P4-(A) | N/A | Chg 1 All Chg 2 ACM | Rev B v1 ACM | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM237. | UM169ab: REQUEST AGAIN WITH NEXT UNIT | R | N/A | N/A | N/A | Chg 2 ACM | N/A | [Free text] | Initial |
|  | Vertical Clearances (uplink) |  |  |  |  |  |  |  |  |  |
| UM 6 | Notification that a level change instruction should be expected. | EXPECT [level] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | Initial ACL DSC DCL Chg 2 DCL | Rev A 112 Rev B v1 DCL | Initial | N/A |
| UM 7 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time. | EXPECT CLIMB AT [time] | R | IER | P4-(F) Not listed, currently being trialed by Reykjavik | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 8 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position. | EXPECT CLIMB AT [position] | R | IER | P4-(F) Not listed, currently being trialed by Reykjavik | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 9 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time. | EXPECT DESCENT AT [time] | R | IER | P4-(F) Not listed, currently being trialed by Reykjavik | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 10 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position. | EXPECT DESCENT AT [position] | R | IER | P4-(F) Not listed, currently being trialed by Reykjavik | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 11 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time. Asia-Pac: Due to different interpretations between the various ATS units this element should be avoided. | EXPECT CRUISE CLIMB AT [time] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
| UM 12 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position. Asia-Pac: Due to different interpretations between the various ATS units this element should be avoided. | EXPECT CRUISE CLIMB AT [position] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
| UM 13 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that an instruction should be expected for the aircraft to commence climb at the specified time to the specified level. | AT [time] EXPECT CLIMB TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 14 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that an instruction should be expected for the aircraft to commence climb at the specified position to the specified level. | AT [position] EXPECT CLIMB TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 15 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that an instruction should be expected for the aircraft to commence descent at the specified time to the specified level. | AT [time] EXPECT DESCENT TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 16 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that an instruction should be expected for the aircraft to commence descent at the specified position to the specified level. | AT [position] EXPECT DESCENT TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 17 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time to the specified level. Due to different interpretations between the various ATS units, this element should be avoided. | AT [time] EXPECT CRUISE CLIMB TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
| UM 18 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position to the specified level. Asia-Pac: Due to different interpretations between the various ATS units, this element should be avoided. | AT [position] EXPECT CRUISE CLIMB TO [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
| UM 19 | Instruction to maintain the specified level. | MAINTAIN [level] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 20 | Instruction that a climb to a specified level is to commence and once reached the specified level is to be maintained. | CLIMB TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | CLIMB TO AND MAINTAIN [altitude] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 21 | Instruction that at the specified time a climb to the specified level is to commence and once reached the specified level is to be maintained. | AT [time] CLIMB TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | N/A | N/A |
|  |  | AT [time] CLIMB TO AND MAINTAIN [altitude] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DSC | N/A | Initial | N/A |
| UM 22 | Instruction that at the specified position a climb to the specified level is to commence and once reached the specified level is to be maintained. | AT [position] CLIMB TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | N/A | N/A |
|  |  | AT [position] CLIMB TO AND MAINTAIN [altitude] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DSC | N/A | Initial | N/A |
| UM 185 | (Reserved) Instruction that after passing the specified position a climb to the specified level is to commence and once reached the specified level is to be maintained. PANS/ATM: 22-Nov-07 Change | N/A AFTER PASSING [position] CLIMB TO [level] | W/U | N/A Avoid Use | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
| UM 23 | Instruction that a descent to a specified level is to commence and once reached the specified level is to be maintained. | DESCEND TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial [Single and block level for ATN B1] |
|  |  | DESCEND TO AND MAINTAIN [altitude] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 24 | Instruction that at a specified time a descent to a specified level is to commence and once reached the specified level is to be maintained. | AT [time] DESCEND TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | N/A | N/A |
|  |  | AT [time] DESCEND TO AND MAINTAIN [altitude] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DSC | N/A | Initial | N/A |
| UM 25 | Instruction that at the specified position a descent to the specified level is to commence and once reached the specified level is to be maintained. | AT [position] DESCEND TO [level] | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | N/A | N/A |
|  |  | AT [position] DESCEND TO AND MAINTAIN [altitude] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DSC | N/A | Initial | N/A |
| UM 186 | (Reserved) Instruction that after passing the specified position a descent to the specified level is to commence and once reached the specified level is to be maintained. PANS/ATM: 22-Nov-07 Change | N/A AFTER PASSING [position] DESCEND TO [level] | W/U | N/A Avoid Use | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
| UM 26 | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified time. When this message element is not concatenated with another vertical clearance, the level specified is the assigned level which is to be maintained. PANS/ATM: 22-Nov-07 Change | CLIMB TO REACH [level] BY [time] | W/U | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 27 | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified position. When this message element is not concatenated with another vertical clearance, the level specified is the assigned level which is to be maintained. PANS/ATM: 22-Nov-07 Change | CLIMB TO REACH [level] BY [position] | W/U | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 28 | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified time. When this message element is not concatenated with another vertical clearance, the level specified is the assigned level which is to be maintained. PANS/ATM: 22-Nov-07 Change | DESCEND TO REACH [level] BY [time] | W/U | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 29 | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified position. When this message element is not concatenated with another vertical clearance, the level specified is the assigned level which is to be maintained. PANS/ATM: 22-Nov-07 Change | DESCEND TO REACH [level] BY [position] | W/U | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 192 | Instruction that a change of level is to continue, but at a rate such that the specified level is reached at or before the specified time. | REACH [level] BY [time] | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 209 | Instruction that a change of level is to continue, but at a rate such that the specified level is reached at or before the specified position. | REACH [level] BY [position] | W/U | N/A | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
| UM 30 | Instruction that a level within the defined vertical range specified is to be maintained. | MAINTAIN BLOCK [level] TO [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 31 | Instruction that a climb to a level within the vertical range defined is to commence. | CLIMB TO AND MAINTAIN BLOCK [level] TO [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 32 | Instruction that a descent to a level within the vertical range defined is to commence. | DESCEND TO AND MAINTAIN BLOCK [level] TO [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 34 | Instruction that a cruise climb to the specified level is to commence and continue until and, once reached the specified level is reached to be maintained. PANS/ATM: 22-Nov-07 Change Asia-Pac: Due to different interpretations between the various ATS units, this element should be avoided. | CRUISE CLIMB TO [level] | W/U | Avoid Use | P4-(F) Unsafe (Used in Reykjavik) | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
| UM 35 | Instruction to be used in conjunction with an associated level instruction indicating that a cruise climb can commence once above the specified level. PANS/ATM: 22-Nov-07 Change | WHEN ABOVE (level) COMMENCE CRUISE CLIMB ABOVE [level] PANS/ATM: 22-Nov-07 Change | W/U | Avoid Use | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Due to different interpretations between the various ATS units, this element should be avoided. | CRUISE CLIMB ABOVE [level] | W/U | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial |  |
| UM 219 | Instruction to stop the climb at the specified level and, once reached, this level is to be maintained. The specified level will be below the previously assigned level. PANS/ATM: 22-Nov-07 Change | STOP CLIMB AT [level] | W/U | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 220 | Instruction to stop the descent at the specified level and, once reached, this level is to be maintained. The specified level will be above the previously assigned level. PANS/ATM: 22-Nov-07 Change | STOP DESCENT AT [level] | W/U | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 36 | Instruction that the climb to the specified level should be made at the aircraft’s best rate. | EXPEDITE CLIMB TO [level] | W/U | CRD (Urgent) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 37 | Instruction that the descent to the specified level should be made at the aircraft’s best rate. | EXPEDITE DESCENT TO[level] | W/U | CRD (Urgent) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 38 | Urgent instruction to immediately climb to the specified level and, once reached, the specified level is to be maintained. PANS/ATM: 22-Nov-07 Change | IMMEDIATELY CLIMB TO [level] | W/U | CRD (Distress) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 39 | Urgent instruction to immediately descend to the specified level and, once reached, the specified level is to be maintained. PANS/ATM: 22-Nov-07 Change | IMMEDIATELY DESCEND TO [level] | W/U | CRD (Distress) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 40 | (Reserved) | (Not defined) | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: Urgent instruction to immediately stop a climb once the specified level is reached. | IMMEDIATELY STOP CLIMB AT [altitude] | W/U | Avoid Use | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 41 | (Reserved) | (Not defined) | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: Urgent instruction to immediately stop a climb once the specified level is reached. | IMMEDIATELY STOP DESCENT AT [altitude] | W/U | Avoid Use | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 171 | Instruction to climb at not less than the specified rate. | CLIMB AT [vertical rate] MINIMUM | W/U | CRD | P4+(A) Unsafe Listed in section 8 | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 172 | Instruction to climb at not above the specified rate. | CLIMB AT [vertical rate] MAXIMUM | W/U | CRD | P4+(A) Unsafe Listed in section 8 | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 173 | Instruction to descend at not less than the specified rate. | DESCEND AT [vertical rate] MINIMUM | W/U | CRD | P4+(A) Unsafe Listed in section 8 | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 174 | Instruction to descend at not above the specified rate. | DESCEND AT [vertical rate] MAXIMUM | W/U | CRD | P4+(A) Unsafe Listed in section 8 | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 33 | (Reserved) | (Not defined) | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Instruction that authorizes a pilot to conduct flight at any altitude from the minimum altitude up to and including the altitude specified in the clearance. further, it is approval for the pilot to proceed to and make an approach at the destination airport. Due to different interpretations between the various ATS units, this element should be avoided. | CRUISE [altitude] | W/U | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 Avoid Use | N/A | N/A | Initial | N/A |
|  | Crossing Constraints (uplink) |  |  |  |  |  |  |  |  |  |
| UM 42 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that a level change instruction should be expected which will require the specified position to be crossed at the specified level. | EXPECT TO CROSS [position] AT [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 43 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that a level change instruction should be expected which will require the specified position to be crossed at or above the specified level. | EXPECT TO CROSS [position] AT OR ABOVE [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 44 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that a level change instruction should be expected which will require the specified position to be crossed at or below the specified level. | EXPECT TO CROSS [position] AT OR BELOW [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 45 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Notification that a level change instruction should be expected which will require the specified position to be crossed at the specified level which is to be maintained subsequently. | EXPECT TO CROSS [position] AT AND MAINTAIN [level] | R | Avoid Use | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 46 | Instruction that the specified position is to be crossed at the specified level. This may require the aircraft to modify its climb or descent profile. | CROSS [position] AT [level] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 47 | Instruction that the specified position is to be crossed at or above the specified level. | CROSS [position] AT OR ABOVE [level] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 48 | Instruction that the specified position is to be crossed at or below the specified level. | CROSS [position] AT OR BELOW [level] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 49 | Instruction that the specified position is to be crossed at the specified level and that level is to be maintained when reached. | CROSS [position] AT AND MAINTAIN [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 50 | Instruction that the specified position is to be crossed at a level between the specified levels. | CROSS [position] BETWEEN [level] AND [level] | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 51 | Instruction that the specified position is to be crossed at the specified time. | CROSS [position] AT [time] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 52 | Instruction that the specified position is to be crossed at or before the specified time. | CROSS [position] AT OR BEFORE [time] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 53 | Instruction that the specified position is to be crossed at or after the specified time. | CROSS [position] AT OR AFTER [time] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 54 | Instruction that the specified position is to be crossed at a time between the specified times. | CROSS [position] BETWEEN [time] AND [time] | W/U | CRD | P4-(F) P4-(A) Unsafe | Ver 4 Ver 5 | Initial ACL DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL DSC | Initial | Initial |
| UM 55 | Instruction that the specified position is to be crossed at the specified speed and the specified speed is to be maintained until further advised. | CROSS [position] AT [speed] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 56 | Instruction that the specified position is to be crossed at a speed equal to or less than the specified speed and the specified speed or less is to be maintained until further advised. | CROSS [position] AT OR LESS THAN [speed] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 57 | Instruction that the specified position is to be crossed at a speed equal to or greater than the specified speed and the specified speed or greater is to be maintained until further advised. | CROSS [position] AT OR GREATER THAN [speed] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 58 | Instruction that the specified position is to be crossed at the specified time and the specified level. | CROSS [position] AT [time] AT [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 59 | Instruction that the specified position is to be crossed at or before the specified time and at the specified level. | CROSS [position] AT OR BEFORE [time] AT [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 60 | Instruction that the specified position is to be crossed at or after the specified time and at the specified level. | CROSS [position] AT OR AFTER [time] AT [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 61 | Instruction that the specified position is to be crossed at the specified level and speed, and the level and speed are to be maintained. | CROSS [position] AT AND MAINTAIN [level] AT [speed] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 62 | Instruction that at the specified time the specified position is to be crossed at the specified level and the level is to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [level] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 63 | Instruction that at the specified time the specified position is to be crossed at the specified level and speed, and the level and speed are to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [level] AT [speed] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
|  | Lateral Offsets (uplink) |  |  |  |  |  |  |  |  |  |
| UM 64 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction. | OFFSET [specified distance] [direction] OF ROUTE | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 65 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified position. | AT [position] OFFSET [specified distance] [direction] OF ROUTE | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | N/A | Initial | N/A |
| UM 66 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified time. | AT [time] OFFSET [specified distance] [direction] OF ROUTE | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 67 | Instruction that the cleared flight route is to be rejoined. | PROCEED BACK ON ROUTE | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 68 | Instruction that the cleared flight route is to be rejoined at or before the specified position. | REJOIN ROUTE BY [position] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 69 | Instruction that the cleared flight route is to be rejoined at or before the specified time. | REJOIN ROUTE BY [time] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 70 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified position. | EXPECT BACK ON ROUTE BY [position] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 71 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified time. | EXPECT BACK ON ROUTE BY [time] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 72 | Instruction to resume own navigation following a period of tracking or heading clearances. May be used in conjunction with an instruction on how or where to rejoin the cleared route. | RESUME OWN NAVIGATION | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
|  | Route Modifications (uplink) |  |  |  |  |  |  |  |  |  |
| UM 73 | Notification to the aircraft of the instructions Instruction to be followed from departure until the specified clearance limit. PANS/ATM: 22-Nov-07 Change | [departure clearance] | W/U | CRD | P4-(F) Not Used | Ver 4 Ver 5 Not Used | Initial DCL | Rev A 112 Rev B v1 DCL | Initial | N/A |
| UM 74 | Instruction to proceed directly from its present position to the specified position. | PROCEED DIRECT TO [position] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 75 | Instruction to proceed, when able, directly to the specified position. | WHEN ABLE PROCEED DIRECT TO [position] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 76 | Instruction to proceed, at the specified time, directly to the specified position. | AT [time] PROCEED DIRECT TO [position] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 77 | Instruction to proceed, at the specified position, directly to the next specified position. | AT [position] PROCEED DIRECT TO [position] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 78 | Instruction to proceed, upon reaching the specified level, directly to the specified position. | AT [level] PROCEED DIRECT TO [position] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 79 | Instruction to proceed to the specified position via the specified route. | CLEARED TO [position] VIA [route clearance] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL DCL | Initial Rev A 110, 111, 112 Rev B v1 ACL DCL | Initial | Initial |
| UM 80 | Instruction to proceed via the specified route. | CLEARED [route clearance] | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 81 | Instruction to proceed in accordance with the specified procedure. | CLEARED [procedure name] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 111, 112 Rev B Deleted, N/A | Initial | N/A |
| UM 236 | Instruction to leave controlled airspace. | LEAVE CONTROLLED AIRSPACE | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 82 | Approval to deviate up to the specified distance from the cleared route in the specified direction. | CLEARED TO DEVIATE UP TO [specified distance] [direction] OF ROUTE | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 83 | Instruction to proceed from the specified position via the specified route. | AT [position] CLEARED [route clearance] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 84 | Instruction to proceed from the specified position via the specified procedure. | AT [position] CLEARED [procedure name] | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
| UM 85 | Notification that a clearance to fly on the specified route may be issued. | EXPECT [route clearance] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | N/A | Initial | N/A |
| UM 86 | Notification that a clearance to fly on the specified route from the specified position may be issued. | AT [position] EXPECT [route clearance] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 87 | Notification that a clearance to fly directly to the specified position may be issued. | EXPECT DIRECT TO [position] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 88 | Notification that a clearance to fly directly from the first specified position to the next specified position may be issued. | AT [position] EXPECT DIRECT TO [position] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 89 | Notification that a clearance to fly directly to the specified position commencing at the specified time may be issued. | AT [time] EXPECT DIRECT TO [position] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 90 | Notification that a clearance to fly directly to the specified position commencing when the specified level is reached may be issued. | AT [level] EXPECT DIRECT TO [position] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 91 | Instruction to enter a holding pattern with the specified characteristics at the specified position and level. | HOLD AT [position] MAINTAIN [level] INBOUND TRACK [degrees] [direction] TURNS [leg type] | W/U | CRD | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees][direction] TURN LEG TIME [leg type] | W/U | N/A | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 92 | Instruction to enter a holding pattern with the published characteristics at the specified position and level. | HOLD AT [position] AS PUBLISHED MAINTAIN [level] | W/U | CRD | P4-(F) P4-(A) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 93 | Notification that an onwards clearance may be issued at the specified time. | EXPECT FURTHER CLEARANCE AT [time] | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 94 | Instruction to turn left or right as specified on to the specified heading. FANS 1/A – ATN: Direction as “left,” “right,” or “either side.” Use of “either side” discouraged. | TURN [direction] HEADING [degrees] | W/U | CRD | P4-(F) P4-(A) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 95 | Instruction to turn left or right as specified on to the specified track. | TURN [direction] GROUND TRACK [degrees] | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 215 | Instruction to turn a specified number of degrees left or right. | TURN [direction] [degrees] DEGREES PANS/ATM: 22-Nov-07 Change | W/U | N/A | P4-(A) | N/A | Initial ACL DSC Chg 1 Revised message element Chg2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial [Note: Use with FANS 1/A aircraft not feasible] |
| UM 190 | Instruction to fly on the specified heading. | FLY HEADING [degrees] | W/U | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial [ATN B1 UM190 converted to FANS 1/A UM94] |
| UM 96 | Instruction to continue to fly on the current heading. | CONTINUE PRESENT HEADING | W/U | CRD | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | FLY PRESENT HEADING | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 97 | Instruction to fly on the specified heading from the specified position. | AT [position] FLY HEADING [degrees] | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 221 | Instruction to stop turn at the specified heading prior to reaching the previously assigned heading. | STOP TURN HEADING [degrees] | W/U | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 98 | Instruction to turn immediately left or right as specified on to the specified heading. | IMMEDIATELY TURN [direction] HEADING [degrees] | W/U | CRD (Distress) | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 99 | Notification that a clearance may be issued for the aircraft to fly the specified procedure. | EXPECT [procedure name] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 112 Rev B v1 DSC | Initial | N/A |
|  | Speed Changes (uplink) |  |  |  |  |  |  |  |  |  |
| UM 100 | Notification that a speed instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 101 | Notification that a speed instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 102 | Notification that a speed instruction may be issued to be effective at the specified level. | AT [level] EXPECT [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 103 | Notification that a speed range instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] TO [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 104 | Notification that a speed range instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] TO [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 105 | Notification that a speed range instruction may be issued to be effective at the specified level. | AT [level] EXPECT [speed] TO [speed] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 106 | Instruction that the specified speed is to be maintained. | MAINTAIN [speed] | W/U | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 188 | Instruction that after passing the specified position the specified speed is to be maintained. | AFTER PASSING [position] MAINTAIN [speed] | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 107 | Instruction that the present speed is to be maintained. | MAINTAIN PRESENT SPEED | W/U | CRD | P4-(F) P4-(A) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 108 | Instruction that the specified speed or a greater speed is to be maintained. | MAINTAIN [speed] OR GREATER | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 109 | Instruction that the specified speed or a lesser speed is to be maintained. | MAINTAIN [speed] OR LESS | W/U | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 110 | Instruction that a speed within the specified range is to be maintained. | MAINTAIN [speed] TO [speed] | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 111 | Instruction that the present speed is to be increased to the specified speed and maintained until further advised. | INCREASE SPEED TO [speed] | W/U | CRD | P3 P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 112 | Instruction that the present speed is to be increased to the specified speed or greater, and maintained at or above the specified speed until further advised. | INCREASE SPEED TO [speed] OR GREATER | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 113 | Instruction that the present speed is to be reduced to the specified speed and maintained until further advised. | REDUCE SPEED TO [speed] | W/U | CRD | P3 P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 114 | Instruction that the present speed is to be reduced to the specified speed or less and maintained at or below the specified speed until further advised. | REDUCE SPEED TO [speed] OR LESS | W/U | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| UM 115 | Instruction that the specified speed is not to be exceeded. | DO NOT EXCEED [speed] | W/U | CRD | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 116 | Instruction that the aircraft’s normal speed be resumed. The previously issued speed restriction(s) are cancelled. Notification that the aircraft need no longer comply with the previously issued speed restriction. PANS/ATM: 22-Nov-07 Change | RESUME NORMAL SPEED | W/U | CRD | P4-(F) P4-(A) Unsafe | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 189 | Instruction that the present speed is to be changed to the specified speed. | ADJUST SPEED TO [speed] | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 222 | Notification that the aircraft may keep its preferred speed without restriction. | NO SPEED RESTRICTION | R | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM222. | UM169z: NO SPEED RESTRICTION | R with rejection procedure for flight crew | N/A | N/A | N/A | Chg 2 ACL | N/A | [Free text] | Initial |
| UM 223 | Instruction to reduce present speed to the minimum safe approach speed. | REDUCE TO MINIMUM APPROACH SPEED | W/U | N/A | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
|  | Contact/Monitor/Surveillance Requests (uplink) |  |  |  |  |  |  |  |  |  |
| UM 117 | Instruction that the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | CONTACT [unitname] [frequency] | W/U | TC | P4+(A) | Ver 4 Ver 5 | Initial ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | Initial | Initial |
| UM 118 | Instruction that at the specified position the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [position] CONTACT [unitname] [frequency] | W/U | TC | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 119 | Instruction that at the specified time the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [time] CONTACT [unitname] [frequency] | W/U | TC | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 238 | Notification that the secondary frequency is as specified. PANS/ATM: 22-Nov-07 Change | SECONDARY FREQUENCY [frequency] | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 120 | Instruction that the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | MONITOR [unitname] [frequency] | W/U | TC | P4+(A) | Ver 4 Ver 5 | Initial ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | Initial | Initial |
| UM 121 | Instruction that at the specified position the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [position] MONITOR [unitname] [frequency] | W/U | TC | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 122 | Instruction that at the specified time the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [time] MONITOR [unitname] [frequency] | W/U | TC | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 123 | Instruction that the specified code (SSR code) is to be selected. | SQUAWK [code] | W/U | IER | P4+(A) | Ver 4 Ver 5 | Initial ACM Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 124 | Instruction that the SSR transponder responses are to be disabled. | STOP SQUAWK | W/U | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 239 | Instruction that the ADS-B transmissions are to be terminated. PANS/ATM: 22-Nov-07 Change | STOP ADS-B TRANSMISSION | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 125 | Instruction that the SSR transponder responses should include level information. | SQUAWK MODE CHARLIE | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | SQUAWK ALTITUDE | W/U | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 240 | Instruction that the ADS-B transmissions should include level information. PANS/ATM: 22-Nov-07 Change | TRANSMIT ADS-B ALTITUDE | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 126 | Instruction that the SSR transponder responses should no longer include level information. | STOP SQUAWK MODE CHARLIE | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | STOP ALTITUDE SQUAWK | W/U | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 241 | Instruction that the ADS-B transmissions should no longer include level information. PANS/ATM: 22-Nov-07 Change | STOP ADS-B ALTITUDE TRANSMISSION | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 179 | Instruction that the ‘ident’ function on the SSR transponder is to be actuated. | SQUAWK IDENT | W/U | IER | P4+(A) | Ver 4 Ver 5 | Initial ACM Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| UM 242 | Instruction that the “ident” function of the ADS-B emitter is to be activated. PANS/ATM: 22-Nov-07 Change | TRANSMIT ADS-B IDENT | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Same intent as PANS/ATM UM242. | UM169ai: TRANSMIT ADS-B IDENT | R | N/A | N/A | Ver 5 | N/A | N/A | [Free text] | N/A |
| UM 243 | Instruction to report when the aircraft is clear of adverse meteorological conditions, and a clearance to regain cleared flight route can be accepted. PANS/ATM: 22-Nov-07 Change | REPORT CLEAR OF WEATHER | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Report/Confirmation Requests (uplink) |  |  |  |  |  |  |  |  |  |
| UM 127 | Instruction to report when the aircraft is back on the cleared route. | REPORT BACK ON ROUTE | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | R | IER | P4+(F) | Ver 4 Ver 5 | N/A |  | Initial | N/A |
| UM 128 | Instruction to report when the aircraft has left vacated the specified level that has either been maintained or passed through on climb or descent. PANS/ATM: 22-Nov-07 Change | REPORT LEAVING [level] | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: Either a level that has been maintained, or a level passed through on climb or descent. |  | R | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 129 | Instruction to report when the aircraft is in level flight at the specified level. | REPORT MAINTAINING [level] | W/U | IER | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
|  | Asia-Pac: Some States do not to use REPORT LEVEL [altitude] in order to avoid confusion because it does not comply with existing voice phraseology | REPORT LEVEL [altitude] | R | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 175 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: Instruction to report when the aircraft has reached the specified level Asia-Pac: To be interpreted as “Report reaching an assigned level.” NAT: This element is not be used to determine when an aircraft is in level flight at the specified level. | REPORT REACHING [level] | R | Avoid Use | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 200 | Instruction used in conjunction with a level clearance to report reaching maintaining the level assigned. PANS/ATM: 22-Nov-07 Change | REPORT REACHING MAINTAINING PANS/ATM: 22-Nov-07 Change | W/U | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 180 | Instruction to report when the aircraft is within the specified vertical range. | REPORT REACHING BLOCK [level] TO [level] | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 130 | Instruction to report when the aircraft has passed the specified position. | REPORT PASSING [position] | W/U | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 181 | Instruction to report the present distance to or from the specified position. | REPORT DISTANCE [to/from] [position] | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 184 | Instruction to report at the specified time the distance to or from the specified position. | AT TIME [time] REPORT DISTANCE [to/from] [position] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 228 | Instruction to report the estimated time of arrival at the specified position. | REPORT ETA [position] | Y DM104 | IER | N/A | N/A | N/A | Initial Rev A Deleted, N/A | N/A | N/A |
|  | Asia-Pac: Same intent as PANS/ATM UM228. | UM169d: REPORT ETA [position] Example: REQUEST ESTIMATE BILBO | R, then DM67n: [position] [time] | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| UM 131 | Instruction to report the amount of fuel remaining and the number of persons on board. | REPORT REMAINING FUEL AND PERSONS ON BOARD | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER (Urgent) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 132 | Instruction to report the present position. | REPORT POSITION | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CONFIRM POSITION | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 133 | Instruction to report the present level. | REPORT PRESENT LEVEL | Y DM32 | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | CONFIRM ALTITUDE | NE | IER | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 134 | Instruction to report the requested speed. | REPORT [speed type] [speed type] [speed type] SPEED | Y DM113 | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Instruction to report the present speed | CONFIRM SPEED | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Asia-Pac: The controller is requesting the pilot to report the present ground speed. | UM169b: REPORT GROUND SPEED | R, then DM67l: GS [speed] Example - GS 490 | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| UM 135 | Instruction to confirm and acknowledge the currently assigned level. PANS/ATM: 22-Nov-07 Change | CONFIRM ASSIGNED LEVEL | Y DM38 DM77 (TBC) | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | CONFIRM ASSIGNED ALTITUDE | NE | IER | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 136 | Instruction to confirm and acknowledge the currently assigned speed. PANS/ATM: 22-Nov-07 Change | CONFIRM ASSIGNED SPEED | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 137 | Instruction to confirm and acknowledge the currently assigned route. PANS/ATM: 22-Nov-07 Change | CONFIRM ASSIGNED ROUTE | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 138 | Instruction to confirm the previously reported time over the last reported waypoint. | CONFIRM TIME OVER REPORTED WAYPOINT | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 139 | Instruction to confirm the identity of the previously reported waypoint. | CONFIRM REPORTED WAYPOINT | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 140 | Instruction to confirm the identity of the next waypoint. | CONFIRM NEXT WAYPOINT | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 141 | Instruction to confirm the previously reported estimated time at the next waypoint. | CONFIRM NEXT WAYPOINT ETA | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 142 | Instruction to confirm the identity of the next but one waypoint. | CONFIRM ENSUING WAYPOINT | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 143 | The request was not understood. It should be clarified and resubmitted. | CONFIRM REQUEST | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 144 | Instruction to report the selected (SSR) code. | CONFIRM SQUAWK | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 145 | Instruction to report the present heading. | REPORT HEADING | Y | IER | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 146 | Instruction to report the present ground track. | REPORT GROUND TRACK | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 182 | Instruction to report the identification code of the last ATIS received. | CONFIRM ATIS CODE | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 147 | Instruction to make a position report. | REQUEST POSITION REPORT | Y | PR | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: To be used if the controller does not receive a scheduled position report. |  | NE | PR | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 216 | Instruction to file a flight plan. | REQUEST FLIGHT PLAN | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 217 | Instruction to report that the aircraft has landed. | REPORT ARRIVAL | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 229 | Instruction to report the preferred alternate aerodrome for landing. | REPORT ALTERNATE AERODROME | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 231 | Instruction to indicate the pilot’s preferred level. | STATE PREFERRED LEVEL | Y DM106 | IER | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | Asia-Pac: Same intent as PANS/ATM UM231. FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM231. | UM169c: STATE PREFERRED LEVEL | R, then DM67m: FL [altitude] Example: FL 350 | IER | P4-(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | [Free text] | Initial |
| UM 232 | Instruction to indicate the pilot’s preferred time and/or position to commence descent to the aerodrome of intended arrival. | STATE TOP OF DESCENT | Y DM109 | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM232 | UM169aa: STATE TOP OF DESCENT | R, then DM67v: TOD’ [time] | N/A | P4-(F) | N/A | Chg 2 ACL | N/A | [Free text] | Initial |
|  | Negotiation Requests (uplink) |  |  |  |  |  |  |  |  |  |
| UM 148 | Request for the earliest time or position at which the specified level can be accepted. PANS/ATM: 22-Nov-07 Change | WHEN CAN YOU ACCEPT [level] | Y DM81 DM82 | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  |  | NE Resp: DM67b DM67e | IER | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 149 | Instruction to report whether or not the specified level can be accepted at the specified position. | CAN YOU ACCEPT [level] AT [position] | A/N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 150 | Instruction to report whether or not the specified level can be accepted at the specified time. | CAN YOU ACCEPT [level] AT [time] | A/N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 151 | Instruction to report the earliest time or position when the specified speed can be accepted. PANS/ATM: 22-Nov-07 Change | WHEN CAN YOU ACCEPT [speed] | Y DM83 DM84 | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE Resp: DM67c DM67f | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 152 | Instruction to report the earliest time or position when the specified offset track can be accepted. PANS/ATM: 22-Nov-07 Change | WHEN CAN YOU ACCEPT [specified distance] [direction] OFFSET | Y DM85 DM86 | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Air Traffic Advisories (uplink) |  |  |  |  |  |  |  |  |  |
| UM 153 | ATS advisory that the altimeter setting should be the specified setting. | ALTIMETER [altimeter] | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 213 | ATS advisory that the specified altimeter setting relates to the specified facility. | [facility designation] ALTIMETER [altimeter] | R | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Same intent as PANS/ATM UM213, where xxxx is facility designation and yyyy is altimeter both sent as free text. | UM169y: [xxxx] ALTIMETER [yyyy] | R | N/A | P4-(F) | N/A | Chg 2 ACL | N/A | [Free text] | Initial |
| UM 154 | ATS advisory that the radar service is terminated. | RADAR SERVICE TERMINATED | R | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | RADAR SERVICES TERMINATED | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 244 | ATS advisory that the radar and/or ADS-B service is terminated. PANS/ATM: 22-Nov-07 Change | IDENTIFICATION TERMINATED | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Same intent as PANS/ATM UM244. | UM169aj: IDENTIFICATION TERMINATED | R | N/A | N/A | Ver 5 | N/A | N/A | [Free text] | N/A |
| UM 191 | ATS advisory that the aircraft is entering airspace in which no air traffic services are provided and all existing air traffic services are terminated. | ALL ATS TERMINATED | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 155 | ATS advisory that radar contact has been established at the specified position. | RADAR CONTACT [position] | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 156 | ATS advisory that radar contact has been lost. | RADAR CONTACT LOST | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 210 | ATS advisory that the aircraft has been identified on radar and/or ADS-B at the specified position. PANS/ATM: 22-Nov-07 Change | IDENTIFIED [position] | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 193 | Notification that radar and/or ADS-B identification has been lost. PANS/ATM: 22-Nov-07 Change | IDENTIFICATION LOST | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 157 | Instruction that a continuous transmission is detected on the specified frequency. Check the microphone button. | CHECK STUCK MICROPHONE [frequency] | N | IER (Urgent) | N/A | N/A | Initial AMC | Initial Rev A 110, 111, 112 Rev B v1 AMC | N/A | Initial |
|  |  |  | R | IER (Urgent) | P4+(A) | Ver 4 Ver 5 | Chg 2 AMC | N/A | Initial | Initial |
| UM 158 | ATS advisory that the ATIS information identified by the specified code is the current ATIS information. | ATIS [atis code] | R | IER | P4-(F) Unsafe | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 212 | ATS advisory that the specified ATIS information at the specified airport is current. | [facility designation] ATIS [atis code] CURRENT | R | N/A | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
| UM 214 | ATS advisory that indicates the RVR value for the specified runway. | RVR RUNWAY [runway] [rvr] | R | N/A | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
| UM 224 | ATS advisory that no delay is expected. | NO DELAY EXPECTED | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 225 | ATS advisory that the expected delay has not been determined. | DELAY NOT DETERMINED | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 226 | ATS advisory that the aircraft may expect to be cleared to commence its approach procedure at the specified time. | EXPECTED APPROACH TIME [time] | R | N/A | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
|  | System Management Messages (uplink) |  |  |  |  |  |  |  |  |  |
| UM 159 | A system generated message notifying that the ground system has detected an error. | ERROR [error information] | N | All (Urgent) | N/A | N/A | Initial All Chg 2 ACL ACM AMC | Initial Rev A 110, 111, 112 Rev B v1 ACL ACM AMC | N/A | Initial |
|  |  |  | NE | All (Urgent) | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 ACL ACM AMC | N/A | Initial | Initial |
| UM 160 | Notification to the avionics that the specified data authority is the next data authority. If no data authority is specified, this indicates that any previously specified next data authority is no longer valid. | NEXT DATA AUTHORITY [facility] | N | CE | N/A | N/A | Initial ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | N/A | Initial |
|  |  |  | NE | CE | P3 P4+(A) | Ver 4 Ver 5 | Chg 2 ACM | N/A | Initial | Initial |
| UM 161 | Notification to the avionics that the data link connection with the current data authority is being terminated. | END SERVICE | N | CT | N/A | N/A | N/A | Initial Rev A Deleted, N/A | N/A | N/A |
|  |  |  | NE | CT | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 162 | Notification that the ground system does not support this message. | SERVICE UNAVAILABLE MESSAGE NOT SUPPORTED BY THIS ATS UNIT PANS/ATM: 22-Nov-07 Change | N | All | N/A | N/A | Initial ACL DSC DCL Chg 2 All | Initial Rev A 110, 111, 112 Rev B v1 All | N/A | Initial |
|  |  | SERVICE UNAVAILABLE | NE | All | P4+(A) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial |
|  | NAT: standard free text message 003. The CPDLC downlink message was not part of the approved message set. Equivalent to UM162, PANS/ATM (22-Nov-07) Change. | UM169u: MESSAGE NOT SUPPORTED BY THIS UNIT Oceanic SPR: MESSAGE NOT SUPPORTED BY THIS FACILITY | R | IER | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
|  | NAT: standard free text message 007. Used by Reykjavik to indicate that part of a downlinked message was not part of the approved set. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | UM169ac: MESSAGE CONTAINS ELEMENT NOT SUPPORTED BY THIS UNIT. MESSAGE REJECTED | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
|  | NAT: standard free text message 008. Used by Reykjavik to indicate which element in a message is not part of the approved message set. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | UM169ad: DOWNLINK <abbreviated downlink message> NOT SUPPORTED BY THIS UNIT | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
|  | NAT: standard free text message 009. Used by Reykjavik to indicate that the un-supported message has been rejected. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | UM169ae: MESSAGE REJECTED | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 234 | Notification that the ground system does not have a flight plan for that aircraft. | FLIGHT PLAN NOT HELD | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 163 | Notification to the pilot of an ATSU identifier. | [facility designation] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | [icao facility designation] [tP4+Table] | NE | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 227 | Confirmation to the aircraft system that the ground system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACL ACM | Initial Rev A 110, 111, 112 Rev B v1 ACL ACM | N/A | Initial [Note: Use with FANS 1/A aircraft not feasible] |
| UM 233 | Notification to the pilot that messages sent requiring a logical acknowledgment will not be accepted by this ground system. | USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACL ACM [Optional] | Initial Rev A 110, 111, 112 Rev B v1 ACL ACM [Optional] | N/A | Initial [Note: Use with FANS 1/A aircraft not feasible] |
|  | Additional Messages (uplink) |  |  |  |  |  |  |  |  |  |
| UM 164 | The associated instruction may be complied with at any future time. | WHEN READY | N | CRD IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | CRD IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 230 | The associated instruction is to be complied with immediately. | IMMEDIATELY | N | N/A (Distress) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 165 | Used to link two messages, indicating the proper order of execution of clearances/ instructions. | THEN | N | CRD IER | N/A | N/A | Initial ACL DSC DCL Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  |  | NE | CRD IER | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| UM 166 | The associated instruction is issued due to traffic considerations. | DUE TO [traffic type] TRAFFIC | N | CRD IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | DUE TO TRAFFIC | NE | CRD IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 167 | The associated instruction is issued due to airspace restrictions. | DUE TO AIRSPACE RESTRICTION | N | CRD IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | NE | CRD IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 168 | The indicated communication should be ignored. Asia-Pac & NAT: The previously sent uplink CPDLC message shall be ignored. DISREGARD should not refer to a clearance or instruction. If DISREGARD is used, another element shall be added to clarify which message is to be disregarded. | DISREGARD | R | CRD (Urgent) IER (Urgent) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 176 | Notification Instruction that the pilot is responsible for maintaining separation from other traffic and is also responsible for maintaining visual meteorological conditions. PANS/ATM: 22-Nov-07 Change | MAINTAIN OWN SEPARATION AND VMC | W/U | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 177 | Used in conjunction with a clearance/instruction to indicate that the pilot may execute when prepared to do so. | AT PILOTS DISCRETION | N | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| UM 178 | (Reserved) | (not defined) | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: (Message intent not defined), Listed under Uplink – Route Modifications | TRACK DETAIL MESSAGE | W/U | Avoid Use | P4+(F) Unsafe Listed in section 8. | Ver 4 Ver 5 {Resp not defined) Not Used | N/A | N/A | Initial W/U | N/A |
|  | Free Text Normal-(uplink) |  |  |  |  |  |  |  |  |  |
| UM 169 | Normal urgency attribute, low alert attribute | [free text] | R | All | P3 P4+(F) | Ver 4 Ver 5 | Chg 2 All | N/A | Initial | Initial [FANS 1/A free text] |
|  | Free Text Distress (uplink) |  |  |  |  |  |  |  |  |  |
| UM 170 | Distress urgency attribute, high alert attribute | [free text] | R | CRD (Distress) IER (Distress) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Free Text – Other |  |  |  |  |  |  |  |  |  |
| UM 183 | Normal urgency attribute, medium alert attribute | [free text] | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACL ACM AMC DSC | Initial Rev A 110, 111, 112 Rev B v1 ACL ACM AMC DSC | N/A | Initial [ATN B1 free text] |
| UM 187 | low urgency, normal alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 194 | normal urgency, low alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 195 | low urgency, low alert | [free text] | R | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 196 | normal urgency, medium alert | [free text] | W/U | N/A | P4-(A) | N/A | Initial All Chg 2 ACL DCL (Not used) | Initial Rev A 110, 111, 112 Rev B v1 ACL DCL | N/A | Initial [Note: Use with FANS 1/A aircraft not feasible] |
| UM 197 | urgent urgency, medium alert | [free text] | W/U | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 198 | distress urgency, high alert | [free text] | W/U | N/A (Distress) | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 199 | normal urgency, low alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 201 | Not used, low urgency, low alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 202 | Not used, low urgency, low alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 203 | normal urgency, medium alert | [free text] | R | N/A | P4-(A) | N/A | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial [ATN B1 free text] |
| UM 204 | normal urgency, medium alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 205 | normal urgency, medium alert | [free text] | A/N | N/A | P4-(A) | N/A | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial [ATN B1 free text] |
| UM 206 | low urgency, normal alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 207 | low urgency, low alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 208 | low urgency, low alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2-2	CPDLC application – downlink message elements

| Ref # | Message Intent/Use | Message Element | Resp. | Ocean SPR | NAT | Asia-Pac FOM | Cont. SPR | ATN B1 | FANS 1/A | FANS 1/A – ATN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Responses/Acknowledgments (downlink) |  |  |  |  |  |  |  |  |  |
| DM 0 | The instruction is understood and will be complied with. | WILCO | N | All | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACM ACL | Initial Rev A 110, 111, 112 Rev B v1 ACM ACL | Initial | Initial |
| DM 1 | The instruction cannot be complied with. | UNABLE | N | All | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACM ACL | Initial Rev A 110, 111, 112 Rev B v1 ACM ACL | Initial | Initial |
| DM 2 | Wait for a reply. Asia-Pac & NAT: The controller is informed that the request is being assessed and there will be a short term delay (within 10 minutes). The exchange is not closed and the request will be responded to when conditions allow. | STANDBY | N | All | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 3 | Message received and understood. Asia-Pac & NAT: ROGER is the only correct response to an uplink free text message. Under no circumstances will ROGER be used instead of AFFIRM. | ROGER | N | All | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 4 | Yes. Asia-Pac & NAT: AFFIRM is an appropriate response to an uplinked negotiation request message (e.g. CAN YOU ACCEPT [altitude] at [time]). | AFFIRM | N | All | P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 5 | No. Asia-Pac & NAT: NEGATIVE is an appropriate response to an uplinked negotiation request message (e.g. CAN YOU ACCEPT [altitude] at [time]). | NEGATIVE | N | All | P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
|  | Vertical Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 6 | Request to fly at the specified level. | REQUEST [level] | Y UM0 UM1 UM19 UM20 UM23 UM26 UM27 UM28 UM29 UM46 UM47 UM48 UM159 + UM183 UM162 UM211 | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 7 | Request to fly at a level within the specified vertical range. | REQUEST BLOCK [level] TO [level] | Y | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| DM 8 | Request to cruise climb to the specified level. Asia-Pac & NAT: Due to different interpretations between the various ATS units, this element should be avoided. | REQUEST CRUISE CLIMB TO [level] | Y | CRD | P4-(F) Avoid Use Not listed in paragraph 6.7 | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 9 | Request to climb to the specified level. | REQUEST CLIMB TO [level] | Y UM0 UM1 UM19 UM20 UM23 UM26 UM27 UM28 UM29 UM46 UM47 UM48 UM159 + UM183 UM162 UM211 | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 10 | Request to descend to the specified level. | REQUEST DESCENT TO [level] | Y UM0 UM19 UM20 UM23 UM26 UM27 UM28 UM29 UM46 UM47 UM48 UM159 + UM183 UM162 UM211 | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 11 | Request that at the specified position a climb to the specified level be approved. | AT [position] REQUEST CLIMB TO [level] | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 12 | Request that at the specified position a descent to the specified level be approved. | AT [position] REQUEST DESCENT TO [level] | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 13 | Request that at the specified time a climb to the specified level be approved. | AT [time] REQUEST CLIMB TO [level] | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 14 | Request that at the specified time a descent to the specified level be approved. | AT [time] REQUEST DESCENT TO [level] | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 69 | Request that a descent be approved on a see-and-avoid basis. | REQUEST VMC DESCENT | Y | CRD | P4-(F) Not listed | N/A Not listed | N/A | N/A | Initial | N/A |
|  | Lateral Off-Set Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 15 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved. | REQUEST OFFSET [specified distance] [direction] OF ROUTE | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 16 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified position. | AT [position] REQUEST OFFSET [specified distance] [direction] OF ROUTE | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 17 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified time. | AT [time] REQUEST OFFSET [specified distance] [direction] OF ROUTE | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Speed Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 18 | Request to fly at the specified speed. | REQUEST [speed] | Y UM0 UM1 UM162 UM211 UM55 UM61 UM106 UM107 UM108 UM109 UM116 UM222 UM159 + UM183 | CRD | P3 P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 19 | Request to fly within the specified speed range. | REQUEST [speed] TO [speed] | Y | CRD | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Voice Contact Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 20 | Request for voice contact. | REQUEST VOICE CONTACT | Y | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 21 | Request for voice contact on the specified frequency. | REQUEST VOICE CONTACT [frequency] | Y | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Route Modification Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 22 | Request to track from the present position direct to the specified position. | REQUEST DIRECT TO [position] | Y UM0 UM162 UM211 UM74 UM96 UM190 UM159 + UM183 | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 23 | Request for the specified procedure clearance. | REQUEST [procedure name] | Y | CRD | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DSC | Rev A 111, 112 Rev B v1 DSC | Initial | N/A |
| DM 24 | Request for a route clearance. | REQUEST CLEARANCE [route clearance] | Y | CRD | N/A | N/A | Initial ACL DSC Chg 2 DSC | Rev A 111, 112 Rev B v1 DSC | N/A | N/A |
|  |  | REQUEST [route clearance] | Y | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DSC | N/A | Initial | N/A |
| DM 25 | Request for a clearance. | REQUEST [clearance type] CLEARANCE | Y | CRD | N/A | N/A | Initial ACL DSC DCL Chg 2 DCL DSC | Rev A 112 Rev B v1 DCL DSC | N/A | N/A |
|  | Asia-Pac & NAT: Either pre-departure or route | REQUEST CLEARANCE | Y | CRD | P4+(F) | Ver 4 Ver 5 | Chg 2 DCL DSC | N/A | Initial | N/A |
| DM 26 | Request for a weather deviation to the specified position via the specified route. | REQUEST WEATHER DEVIATION TO [position] VIA [route clearance] | Y | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 27 | Request for a weather deviation up to the specified distance off track in the specified direction. | REQUEST WEATHER DEVIATION UP TO [specified distance] [direction] OF ROUTE | Y UM0 UM162 UM211 UM64 UM74 UM82 UM96 UM190 UM159 + UM183 | CRD | P4+(A) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 70 | Request a clearance to adopt the specified heading. | REQUEST HEADING [degrees] | Y | CRD | P4-(F) Not listed | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| DM 71 | Request a clearance to adopt the specified ground track. | REQUEST GROUND TRACK [degrees] | Y | CRD | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | Reports (downlink) |  |  |  |  |  |  |  |  |  |
| DM 28 | Notification of leaving the specified level. | LEAVING [level] | N | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 29 | Notification of climbing to the specified level. | CLIMBING TO [level] | N | IER | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| DM 30 | Notification of descending to the specified level. | DESCENDING TO [level] | N | IER | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| DM 31 | Notification of passing the specified position. | PASSING [position] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 78 | Notification that at the specified time, the aircraft’s position was as specified. | AT [time] [distance] [to/from] [position] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 32 | Notification of the present level. | PRESENT LEVEL [level] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | PRESENT ALTITUDE [altitude] | N | IER | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| DM 33 | Notification of the present position. | PRESENT POSITION [position] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 34 | Notification of the present speed. | PRESENT SPEED [speed] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 113 | Notification of the requested speed. | [speed type] [speed type] [speed type] SPEED [speed] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac: Intent partial to PANS/ATM DM113. The pilot notifies the controller of present ground speed, in response to UM169b, REPORT GROUND SPEED | DM67l: GS [speed] | N | IER | N/A Not listed | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 35 | Notification of the present heading in degrees. | PRESENT HEADING [degrees] | N | IER | P4+(F) | Ver 4 Ver 5 | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | Initial | N/A |
| DM 36 | Notification of the present ground track in degrees. | PRESENT GROUND TRACK [degrees] | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 37 | Notification that the aircraft is maintaining the specified level. | MAINTAINING [level] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Initial Rev A 110, 111, 112 Rev B Deleted, N/A | N/A | N/A |
|  |  | LEVEL [altitude] | N | IER | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 72 | (Reserved) PANS/ATM: 22-Nov-07 Change | N/A | N | N/A | N/A | N/A | N/A | N/A | Initial | N/A |
|  | Asia-Pac & NAT: Notification that the aircraft has reached the specified level. | REACHING [level] | N | Avoid Use | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 76 | Notification that the aircraft has reached a level within the specified vertical range. | REACHING BLOCK [level] TO [level] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 38 | Read-back of the assigned level. | ASSIGNED LEVEL [level] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  |  | ASSIGNED ALTITUDE [altitude] | N | IER | P4+(A) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| DM 77 | Read-back of the assigned vertical range. | ASSIGNED BLOCK [level] TO [level] | N | IER | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | Initial | Initial |
| DM 39 | Read-back of the assigned speed. | ASSIGNED SPEED [speed] | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 40 | Read-back of the assigned route. | ASSIGNED ROUTE [route clearance] | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 41 | The aircraft has regained the cleared route. | BACK ON ROUTE | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 114 | Notification that the aircraft is clear of weather and is able to accept a clearance to regain cleared flight route. PANS/ATM: 22-Nov-07 Change | CLEAR OF WEATHER | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 42 | The next waypoint is the specified position. | NEXT WAYPOINT [position] | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 43 | The ETA at the next waypoint is as specified. | NEXT WAYPOINT ETA [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 44 | The next but one waypoint is the specified position. | ENSUING WAYPOINT [position] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 45 | Clarification of previously reported waypoint passage. | REPORTED WAYPOINT [position] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 46 | Clarification of time over previously reported waypoint. | REPORTED WAYPOINT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 47 | The specified (SSR) code has been selected. | SQUAWKING [code] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 48 | Position report. Asia-Pac & NAT: Reports the current position of the aircraft when the pilot presses the button to send this message. ATC expects position reports based on this downlink message. | POSITION REPORT [position report] | N | PR | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 79 | The code of the latest ATIS received is as specified. | ATIS [atis code] | N | IER | P4-(F) Not listed | Ver 4 Ver 5 | Initial ACL DSC Chg 2 DCL | Rev A 112 Rev B v1 DCL | Initial | N/A |
| DM 89 | The specified ATS unit is being monitored on the specified frequency. | MONITORING [unitname] [frequency] | N | IER (Urgent) | P4-(A) | N/A | Initial ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | N/A | Initial [ATN B1 only]] |
| DM 102 | Used to report that an aircraft has landed. | LANDING REPORT | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 104 | Notification of estimated time of arrival at the specified position. | ETA [position][time] | N | IER | N/A | N/A | N/A | Initial Rev A Deleted, N/A | N/A | N/A |
|  | Asia-Pac: Response to UM169d, REPORT ETA [position] | DM67n: [position][time] Example: BILBO 0413 | N | IER | N/A Not listed | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 105 | Notification of the alternative aerodrome for landing. | ALTERNATE AERODROME [airport] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 106 | Notification of the preferred level. | PREFERRED LEVEL [level] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL DCL | Initial Rev A 110, 111, 112 Rev B v1 ACL DCL | N/A | Initial |
|  | Asia-Pac: Response to free text UM169c, STATE PREFERRED LEVEL. FANS 1/A – ATN INTEROP: Response to UM231, STATE PREFERRED LEVEL. | DM67m: FL [altitude] | N | IER | P4+(F) Not listed | Ver 4 Ver 5 | Chg 2 ACL DCL | N/A | [Free text] | Initial |
| DM 109 | Notification of the preferred time to commence descent for approach. | TOP OF DESCENT [time] | N | N/A | P4-(A) | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A – ATN INTEROP: Response to UM232 or free text UM169aa, STATE TOP OF DESCENT. | DM67v: TOD [time] | N | N/A | N/A | N/A | Chg 2 ACL | N/A | [Free text] | Initial |
| DM 110 | Notification of the preferred position to commence descent for approach. | TOP OF DESCENT [position] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 111 | Notification of the preferred time and position to commence descent for approach. | TOP OF DESCENT [time] [position] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Negotiation Requests (downlink) |  |  |  |  |  |  |  |  |  |
| DM 49 | Request for the earliest time at which a clearance to the specified speed can be expected. | WHEN CAN WE EXPECT [speed] | Y | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 50 | Request for the earliest time at which a clearance to a speed within the specified range can be expected. | WHEN CAN WE EXPECT [speed] TO [speed] | Y | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 51 | Request for the earliest time at which a clearance to regain the planned route can be expected. | WHEN CAN WE EXPECT BACK ON ROUTE | Y | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 52 | Request for the earliest time at which a clearance to descend can be expected. | WHEN CAN WE EXPECT LOWER LEVEL | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | WHEN CAN WE EXPECT LOWER ALTITUDE | Y | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 53 | Request for the earliest time at which a clearance to climb can be expected. | WHEN CAN WE EXPECT HIGHER LEVEL | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | WHEN CAN WE EXPECT HIGHER ALTITUDE | Y | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 54 | Request for the earliest time at which a clearance to cruise climb to the specified level can be expected. | WHEN CAN WE EXPECT CRUISE CLIMB TO [level] | Y | IER | P4-(F) Not listed | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 87 | Request for the earliest time at which a clearance to climb to the specified level can be expected. | WHEN CAN WE EXPECT CLIMB TO [level] | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | DM67h: WHEN CAN WE EXPECT CLIMB TO [altitude] | Y | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 88 | Request for the earliest time at which a clearance to descend to the specified level can be expected. | WHEN CAN WE EXPECT DESCENT TO [level] | Y | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | DM67i: WHEN CAN WE EXPECT DESCENT TO [altitude] | Y | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
|  | Emergency Messages (downlink) |  |  |  |  |  |  |  |  |  |
| DM 55 | Urgency prefix. | PAN PAN PAN | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 56 | Distress prefix. | MAYDAY MAYDAY MAYDAY | Y | IER (Distress) | N/A | N/A | Initial ACL DSC Chg 2 Deleted, N/A | Rev A 112 Rev B Deleted, N/A | N/A | N/A |
|  |  |  | N | IER (Distress) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 112 | Indicates specifically that the aircraft is being subjected to unlawful interference. | SQUAWKING 7500 | N | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 57 | Notification of fuel remaining and number of persons on board. | [remaining fuel] OF FUEL REMAINING AND [persons on board] PERSONS ON BOARD | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | REPORT REMAINING FUEL AND SOULS ON BOARD | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 58 | Notification that the pilot wishes to cancel the emergency condition. | CANCEL EMERGENCY | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 59 | Notification that the aircraft is diverting to the specified position via the specified route due to an urgent need. | DIVERTING TO [position] VIA [route clearance] | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 60 | Notification that the aircraft is deviating the specified distance in the specified direction off the cleared route and maintaining a parallel track due to an urgent need. | OFFSETTING [specified distance] [direction] OF ROUTE | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 61 | Notification that the aircraft is descending to the specified level due to an urgent need. | DESCENDING TO [level] | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  | N | IER (Urgent) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 80 | Notification that the aircraft is deviating up to the deviating distance from the cleared route in the specified direction due to an urgent need. | DEVIATING UP TO [specified distance] [direction] OF ROUTE | Y | IER (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Asia-Pac & NAT: Listed under downlink reports. |  | N | IER (Urgent) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
|  | System Management Messages (downlink) |  |  |  |  |  |  |  |  |  |
| DM 62 | A system-generated message that the avionics has detected an error. | ERROR [error information] | N | All (Urgent) | P3 P4+(A) | Ver 4 Ver 5 | Initial All | Initial Rev A 110, 111, 112 Rev B v1 All | Initial | Initial |
| DM 63 | A system-generated denial to any CPDLC application message sent from a ground facility that is not the current data authority. | NOT CURRENT DATA AUTHORITY | N | IER | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | Initial | Initial |
| DM 99 | A system-generated message to inform a ground facility that it is now the current data authority. | CURRENT DATA AUTHORITY | N | N/A | P4-(A) | N/A | Initial ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | N/A | Initial |
| DM 64 | Notification to the ground system that the specified ATSU is the current data authority. FANS 1/A – ATN INTEROP: Used by FANS 1/A aircraft. | [facility designation] | N | CE | P3 P4+(F) | Ver 4 Ver 5 | Chg 2 ACM | N/A | Initial | Initial |
| DM 107 | A system-generated message sent to a ground system that tries to connect to an aircraft when a current data authority has not designated the ground system as the NDA. FANS 1/A – ATN INTEROP: Used by ATN B1 aircraft. | NOT AUTHORIZED NEXT DATA AUTHORITY | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACM | Initial Rev A 110, 111, 112 Rev B v1 ACM | N/A | Initial |
| DM 73 | A system-generated message indicating the software version number. FANS 1/A – ATN INTEROP: Used by FANS 1/A aircraft. | [version number] | N | CE | P3 P4+(F) | Ver 4 Ver 5 | Chg 2 ACM | N/A | Initial | Initial |
| DM 100 | Confirmation to the ground system that the aircraft system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACM ACL | Initial Rev A 110, 111, 112 Rev B v1 ACM ACL | N/A | Initial [ATN B1 only] |
|  | FANS 1/A – ATN INTEROP: Alternate means to obtain “Logical Acknowledgement” from FANS 1/A aircraft | Message Assurance (MA) | N/A | N/A | N/A | N/A | Chg 2 ACM ACL | N/A | Initial | Initial |
|  | Additional Messages (downlink) |  |  |  |  |  |  |  |  |  |
| DM 65 | Used to explain reasons for pilot’s message. | DUE TO WEATHER | N | CRD IER | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 66 | Used to explain reasons for pilot’s message. | DUE TO AIRCRAFT PERFORMANCE | N | CRD IER | P3 P4+(A) | Ver 4 Ver 5 | Initial All Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | Initial | Initial |
| DM 74 | States a desire by the pilot to provide his/her own separation and remain in VMC. | REQUEST TO MAINTAIN OWN SEPARATION AND VMC | Y | CRD | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | MAINTAIN OWN SEPARATION AND VMC | N | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 75 | Used in conjunction with another message to indicate that the pilot wishes to execute request when the pilot is prepared to do so. | AT PILOTS DISCRETION | N | CRD | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 101 | Allows the pilot to indicate a desire for termination of CPDLC application with the current data authority. | REQUEST END OF SERVICE | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 103 | Allows the pilot to indicate that he/she has cancelled IFR flight plan. | CANCELLING IFR | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 108 | Notification that de-icing action has been completed. | DE-ICING COMPLETE | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Free Text – Normal (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67 | Normal urgency, low alert | [free text] | N | IER | P3 P4+(F) | Ver 4 Ver 5 | Chg 1 ACM ACL DCL AMC Chg 2 All | Rev A 112 Rev B v1 DCL DSC | Initial | Initial [FANS 1/A free text] |
|  | Free Text - Distress (downlink) |  |  |  |  |  |  |  |  |  |
| DM 68 | Distress urgency, high alert | [free text] | Y | CRD (Distress) IER (Distress) | P3 P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial | N/A |
| DM 90 | normal urgency, medium alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 91 | normal urgency, low alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 92 | low urgency, low alert | [free text] | Y | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 93 | urgent urgency, high alert | [free text] | N | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 94 | distress urgency, high alert | [free text] | N | N/A (Distress) | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 95 | urgent urgency, medium alert | [free text] | N | N/A (Urgent) | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 96 | urgent urgency, low alert | [free text] | N | N/A (Urgent) | N/A | N/A | Initial All Chg 2 Deleted, N/A | N/A | N/A | N/A |
| DM 97 | low urgency, low alert | [free text] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 98 | normal urgency, normal alert | [free text] | N | N/A | P4-(A) | N/A | Initial All Chg 2 ACM ACL DCL AMC | Initial Rev A 110, 111, 112 Rev B v1 ACM ACL DCL AMC | N/A | Initial [ATN B1 free text] |
|  | Negotiation Responses (downlink) |  |  |  |  |  |  |  |  |  |
| DM 81 | We can accept the specified level at the specified time. | WE CAN ACCEPT [level] AT [time] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A: Preformatted free text. | DM67b: WE CAN ACCEPT [altitude] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | [Free text] | Initial |
| DM 115 | We can accept the specified level at the specified position. PANS/ATM: 22-Nov-07 Change | WE CAN ACCEPT [level] AT [position] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 82 | We cannot accept the specified level. | WE CANNOT ACCEPT [level] | N | IER | N/A | N/A | Initial ACL DSC Chg 2 ACL | Initial Rev A 110, 111, 112 Rev B v1 ACL | N/A | Initial |
|  | FANS 1/A: Preformatted free text. | DM67e: WE CANNOT ACCEPT [altitude] | N | IER | P4+(F) | Ver 4 Ver 5 | Chg 2 ACL | N/A | [Free text] | Initial |
| DM 83 | We can accept the specified speed at the specified time. | WE CAN ACCEPT [speed] AT [time] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | FANS 1/A: Preformatted free text. | DM67c: WE CAN ACCEPT [speed] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 116 | We can accept the specified speed at the specified position. PANS/ATM: 22-Nov-07 Change | WE CAN ACCEPT [speed] AT [position] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 84 | We cannot accept the specified speed. | WE CANNOT ACCEPT [speed] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | FANS 1/A: Preformatted free text. | DM67f: WE CANNOT ACCEPT [speed] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 85 | We can accept a parallel track offset the specified distance in the specified direction at the specified time. | WE CAN ACCEPT [specified distance] [direction] AT [time] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | FANS 1/A: Preformatted free text. | DM67d: WE CAN ACCEPT [direction] [distance offset] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 117 | We can accept a parallel track offset the specified distance in the specified direction at the specified position. PANS/ATM: 22-Nov-07 Change | WE CAN ACCEPT [specified distance] [direction] AT [position] | N | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| DM 86 | We cannot accept a parallel track offset the specified distance in the specified direction. | WE CANNOT ACCEPT [specified distance] [direction] | N | IER | N/A | N/A | N/A | N/A | N/A | N/A |
|  | FANS 1/A: Preformatted free text. | DM67g: WE CANNOT ACCEPT [direction] [distance offset] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |

Table 2-3	CPDLC application – standardized uplink free text messages

| Ref # | Message Intent/Use | Message Element | Resp. | Ocean SPR | NAT | Asia-Pac | Cont. SPR | ATN B1 | FANS 1/A | FANS 1/A – ATN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Free Text-Standardized Report/Confirmation Requests (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169b | Asia-Pac: The controller is requesting the pilot to report the present ground speed. Intent similar partially to PANS/ATM UM134 | REPORT GROUND SPEED | R, and then DM67l | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169c | Asia-Pac: The controller is requesting that the pilot advise the preferred flight level for the flight. Same intent as PANS/ATM UM231 | STATE PREFERRED LEVEL | R, and then DM67m | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | Initial [FANS 1/A only] |
| UM 169d | Asia-Pac: The controller is requesting an estimate for the specified waypoint. Same intent as PANS/ATM UM228. | REPORT ETA [position] Example – REPORT ETA BILBO | R, and then DM67n | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169e | Asia-Pac: The controller is requesting that the pilot notify when the specified traffic has been seen by visual contact and passed. The level specified in the traffic description is the level being maintained by the opposite direction aircraft. No equivalent to PANS/ATM. | REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time] The traffic description is to be inserted by the controller and shall include the aircraft identification (callsign), flight level and aircraft type. ETP = Estimated Time of Passing. Example of the traffic description: SIA228 B747 FL370 | R, and then DM67o DM67p | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169f | Asia-Pac: The controller is requesting from the pilot the time at which the aircraft will maintain the specified level. No equivalent to PANS/ATM. | WHEN WILL YOU MAINTAIN FL [altitude] | R, and then DM67r | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169g | Asia-Pac: The controller is requesting the distance from the specified position or waypoint at which the aircraft will maintain the specified level. The pilot shall include the direction from the waypoint as a cardinal point, e.g. N, NE, NW, S, SW, SE, E or W. No equivalent to PANS/ATM. | AT WHAT DISTANCE [position / waypoint] WILL YOU MAINTAIN FL [altitude] | R, and then DM67s | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169h | Asia-Pac: The controller is requesting that the pilot report the radial on which the aircraft is proceeding and the distance from the specified VOR. No equivalent to PANS/ATM. | REPORT RADIAL AND DISTANCE [to/from] [position] | R, and then, DM67t | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169i | Asia-Pac: The controller is requesting that the pilot makes voice contact / radio check call on the specified frequency. No equivalent to PANS/ATM. | REQUEST VOICE CONTACT [frequency] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free Text Instructions (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169j | Asia-Pac: The controller has detected that uplink messages exist that the pilot has not yet responded to. The pilot is required to check the ATC log page and to respond to unanswered uplink messages. No equivalent to PANS/ATM. | CHECK AND RESPOND TO OPEN CPDLC MESSAGES Asia-Pac: Formerly, “CHECK ATC LOG PAGE FOR OPEN MESSAGES” | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169w | FANS 1/A: Used by the aircraft to determine the time when messages received are delayed. No equivalent to PANS/ATM. | SET MAX UPLINK DELAY VALUE TO XXX SEC | R | CE | N/A | N/A | ACM? | N/A | Initial – N/A Rev A | N/A |
| UM 169ai | Asia-Pac: Instruction that the “ident” function of the ADS-B emitter is to be activated. Same intent as PANS/ATM UM242. | TRANSMIT ADS-B IDENT | R | N/A | N/A | Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free text Advisories (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169k | Asia-Pac: The controller is notifying the pilot that a selcal check will be made on the specified HF frequency. No equivalent to PANS/ATM. | EXPECT SELCAL CHECK HF [frequency] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169l | Asia-Pac & NAT: The controller is notifying the pilot that the CPDLC transfer process will not be completed at the FIR boundary and will be delayed until the specified time. If the CPDLC transfer is not completed by the specified time, the pilot shall manually disconnect and logon to the next center. NAT: standard free text message 006. No equivalent to PANS/ATM. | EXPECT CPDLC TRANSFER AT [time] | R | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169aj | Asia-Pac: ATS advisory that the radar and/or ADS-B service is terminated. Same intent as PANS/ATM UM244. | IDENTIFICATION TERMINATED | R | N/A | N/A | Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169m | Asia-Pac: The controller is notifying the pilot that CPDLC connection is not required by the next FIR (where the flight’s transition time of that FIR is short) and CPDLC connection will be transferred to the subsequent FIR. The [ATSU name] is the relevant four character ICAO code. No equivalent to PANS/ATM. | EXPECT NEXT CENTER [ATSU name]. CONTACT WITH [ATSU name] NOT REQUIRED | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169n | Asia-Pac: The controller is notifying the pilot of traffic significant to the flight. The description will include the aircraft type and any other relevant information to assist the pilot in sighting the traffic. The pilot may respond that the traffic has been sighted. No equivalent to PANS/ATM. | TRAFFIC IS [traffic description] | R and then, (optionally) DM67q | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169o | Asia-Pac: The controller is notifying the pilot of the secondary HF frequency for the area. No equivalent to PANS/ATM. | SECONDARY HF [frequency] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free Text Speed Messages (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169p | Asia-Pac: The controller is notifying the pilot that a speed instruction may be issued to be effective until the specified time. No equivalent to PANS/ATM. | EXPECT TO MAINTAIN [speed] UNTIL [time / position] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free Text Emergency Acknowledgment (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169q | Asia-Pac: The controller has acknowledged receipt of a MAYDAY downlink message. The controller shall attempt to make voice contact with the pilot. The pilot should only respond with ROGER if or when able to do so. If the aircraft is inbound to an airport within the FIR, a ROGER response is not required. NAT: standard free text message 005. The controller received DM56 MAYDAY MAYDAY MAYDAY. No equivalent to PANS/ATM. | ROGER MAYDAY | R | IER (Distress) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169r | Asia-Pac: The controller has acknowledged receipt of a PAN downlink message. The controller shall attempt to make voice contact with the pilot. The pilot should only respond with ROGER if or when able to do so. If the aircraft is inbound to an airport within the FIR, a ROGER response is not required. NAT: standard free text message 004. The controller received DM55 PAN PAN PAN. No equivalent to PANS/ATM. | ROGER PAN | R | IER (Distress) | P4+(F) | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| UM 169ak | Asia-Pac: When the ADS emergency mode is activated without a CPDLC emergency message or voice confirmation, and the demand contract report appears to indicate that the aircraft is maintaining normal operations (e.g. the aircraft is not in descent or involved in abrupt maneuvers), the aircraft may be subject to unlawful interference. To check for covert or inadvertent activation of the ADS emergency mode the free text uplink “Confirm ADS” shall be appended to a “Confirm Speed” data or voice request. If the aircraft continues with the ADS emergency mode activated ATC shall assume the aircraft is in emergency conditions and follow normal alerting procedures. No equivalent to PANS/ATM. | CONFIRM ADS | R | N/A | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free Text – NAT-specific Region (uplink) |  |  |  |  |  |  |  |  |  |
| UM 169s | NAT: standard free text message 001. The CPDLC downlink request was: 1) part of the approved message set; and 2) received by the controller. The aircraft will receive any further communication about the request via Gander aeradio. No equivalent to PANS/ATM. | REQUEST RECEIVED RESPONSE WILL BE VIA GANDER AERADIO | R | IER | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169t | NAT: standard free text message 002. The CPDLC downlink request was: 1) part of the approved message set; and 2) received by the controller. The aircraft will receive any further communication about the request via Shanwick aeradio. No equivalent to PANS/ATM. | REQUEST RECEIVED RESPONSE WILL BE VIA VOICE COMMUNICATION Oceanic SPR: REQUEST RECEIVED RESPONSE WILL BE VIA SHANWICK AERADIO | R | IER | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169u | NAT: standard free text message 003. The CPDLC downlink message was not part of the approved message set. Equivalent to UM162, PANS/ATM (22-Nov-07) Change. | MESSAGE NOT SUPPORTED BY THIS UNIT Oceanic SPR: MESSAGE NOT SUPPORTED BY THIS FACILITY | R | IER | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169v | NAT: standard free text message 007. No equivalent to ICAO Doc 4444. | UNABLE REQUESTED LEVEL | R | IER | N/A | N/A | N/A | N/A | N/A | N/A |
| UM 169ac | NAT: standard free text message 007. Used by Reykjavik to indicate that part of a downlinked message was not part of the approved set. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | MESSAGE CONTAINS ELEMENT NOT SUPPORTED BY THIS UNIT. MESSAGE REJECTED. | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169ad | NAT: standard free text message 008. Used by Reykjavik to indicate which element in a message is not part of the approved message set. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | DOWNLINK <abbreviated downlink message> NOT SUPPORTED BY THIS UNIT | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169ae | NAT: standard free text message 009. Used by Reykjavik to indicate that the un-supported message has been rejected. Adaptation of UM162, PANS/ATM (22-Nov-07) Change. | MESSAGE REJECTED | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169af | NAT: standard free text message 010. Used by Reykjavik to indicate that voice should be used to repeat the rejected message. No equivalent to PANS/ATM. | REPEAT VIA VOICE | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169ag | NAT: standard free text message 011. Used by Reykjavik to indicate the action to take in response to a rejected request for voice contact message. No equivalent to PANS/ATM. | TRY SATCOM VOICE OR RELAY THROUGH ANOTHER AIRCRAFT | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
| UM 169ah | NAT: standard free text message 012. Used by Santa Maria to indicate that the downlink message was not part of the approved message set. The message should be passed by voice. No equivalent to PANS/ATM. | MESSAGE NOT SUPPORTED BY THIS FACILITY, CONTACT RTF | R | N/A | P4+(F) | N/A | N/A | N/A | [free text] | N/A |
|  | Free Text – FANS 1/A use of ATN B1 data link services in Continental Airspace |  |  |  |  |  |  |  |  |  |
| UM 169x | Continental: Indicates that the ATC has received the request and has passed it to the next control authority. Same intent as PANS/ATM UM211. | REQUEST FORWARDED | R UM211 Response is N | N/A | N/A | N/A | N/A | N/A | [free text] | Initial [FANS 1/A only] |
| UM 169y | Continental: ATS advisory that the specified altimeter setting relates to the specified facility. Same intent as PANS/ATM UM213. | xxxx ALTIMETER yyyy Where xxxx is facility designation and yyyy is altimeter both sent as free text. | R | N/A | N/A | N/A | N/A | N/A | [free text] | Initial [FANS 1/A only] |
| UM 169z | Continental: Notification that the aircraft may keep its preferred speed without restriction. Same intent as PANS/ATM UM222. | NO SPEED RESTRICTION | R | N/A | N/A | N/A | N/A | N/A | [free text] | Initial [FANS 1/A only] |
| UM 169aa | Continental: Instruction to indicate the pilot’s preferred time and/or position to commence descent to the aerodrome of intended arrival. Same intent as PANS/ATM UM232. | STATE TOP OF DESCENT | R, then DM67v: TOD’ [time] | N/A | N/A | N/A | N/A | N/A | [free text] | Initial [FANS 1/A only] |
| UM 169ab | Continental: Indicates that the request cannot be responded to by the current unit and that it should be requested from the next unit. Same intent as PANS/ATM UM237. | REQUEST AGAIN WITH NEXT UNIT | R | N/A | N/A | N/A | N/A | N/A | [free text] | Initial [FANS 1/A only] |

Table 2-4	CPDLC application – standardized downlink free text messages

| Ref # | Message Intent/Use | Message Element | Resp. | Ocean SPR | NAT | Asia-Pac | Cont. SPR | ATN B1 | FANS 1/A | FANS 1/A – ATN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Free Text - Negotiation Responses (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67b | Asia-Pac: We can accept the specified level at the specified time. Intent equivalent to PANS/ATM DM81. | WE CAN ACCEPT [altitude] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67c | Asia-Pac: We can accept the specified speed at the specified time. Intent equivalent to PANS/ATM DM83. | WE CAN ACCEPT [speed] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67d | Asia-Pac: We can accept a parallel track offset the specified distance in the specified direction at the specified time. Intent equivalent to PANS/ATM DM85. | WE CAN ACCEPT [direction] [distance offset] AT [time] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67e | Asia-Pac: We cannot accept the specified level. Intent equivalent to PANS/ATM DM82. | WE CANNOT ACCEPT [altitude] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67f | Asia-Pac: We cannot accept the specified speed. Intent equivalent to PANS/ATM DM84. | WE CANNOT ACCEPT [speed] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67g | Asia-Pac: We cannot accept a parallel track offset the specified distance in the specified direction. Intent equivalent to PANS/ATM DM86. | WE CANNOT ACCEPT [direction] [distance offset] | N | IER | P4+(F) | Ver 4 Ver 5 | N/A | N/A | Initial - N/A Rev A | N/A |
| DM 67h | Asia-Pac: Request for the earliest time at which a clearance to climb to the specified level can be expected Intent equivalent to PANS/ATM DM87. | WHEN CAN WE EXPECT CLIMB TO [altitude] | N | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67i | Asia-Pac: Request for the earliest time at which a clearance to descend to the specified level can be expected. Intent equivalent to PANS/ATM DM88. | WHEN CAN WE EXPECT DESCENT TO [altitude] | N | IER | P4-(F) Avoid Use | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
|  | Free Text - Advisories (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67j | Asia-Pac: The pilot is offsetting due wake turbulence in accordance with RVSM procedures (offset will not exceed 2nm). The controller is not required to respond or issue a clearance. No equivalent in PANS/ATM. | WAKE DEV [direction] Direction L or R (left or right) as appropriate | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67k | Asia-Pac: The pilot is advising ATC of an update a waypoint ETA. No equivalent in PANS/ATM. | REVISED ETA [position] [time] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
|  | Free Text – Responses (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67l | Asia-Pac: The pilot responds to controller free text, UM169b REPORT GROUND SPEED. Intent partial to PANS/ATM DM113. The pilot notifies the controller of present ground speed. | GS [speed] Example - GS 490 | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67m | Asia-Pac: The pilot responds to controller free text, UM169c STATE PREFERRED LEVEL. Continental: Used for FANS 1/A aircraft. Same intent as PANS/ATM DM 106. | FL [altitude] Example - FL 350 | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67n | Asia-Pac: The pilot responds to controller free text, REPORT ETA [position], Same intent as PANS/ATM DM 104. | [position] [time] Example - BILBO 0413 | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67o | Asia-Pac: The pilot responds to controller free text, REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time]. No equivalent in PANS/ATM. | [traffic identification] SIGHTED AND PASSED Example - SIA228 SIGHTED AND PASSED | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67p | Asia-Pac: The pilot responds to controller free text, REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time]. No equivalent in PANS/ATM. | [traffic identification] NOT SIGHTED | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67q | Asia-Pac: The pilot optionally responds to controller free text, TRAFFIC IS [traffic description]. No equivalent in PANS/ATM. | TRAFFIC SIGHTED | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67r | Asia-Pac: The pilot responds to controller free text, WHEN WILL YOU MAINTAIN FL [altitude]. No equivalent in PANS/ATM. | FL [altitude] AT [time] Example - FL 350 AT 2317 | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67s | Asia-Pac: The pilot responds to controller free text, WHEN WILL YOU MAINTAIN FL [altitude]. No equivalent in PANS/ATM. | FL [altitude] AT [distance] NM [direction] [position / waypoint] | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67t | Asia-Pac: The pilot responds to controller free text, REPORT RADIAL AND DISTANCE [to/from] [position]. No equivalent in PANS/ATM. | [radial] R [distance] NM [to/from] [position] Example - 320 R 26 NM FROM MCY | N | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [Free text] | N/A |
| DM 67u | FANS 1/A: Used as response when a message is delayed. No equivalent in PANS/ATM. | UPLINK DELAYED IN NETWORK AND REJECTED - RESEND OR CONTACT BY VOICE | N | All | N/A | N/A | N/A | N/A | Initial – N/A Rev A | N/A |
| DM 67v | Continental & FANS 1/A – ATN: Response to UM232, STATE TOP OF DESCENT or free text UM169aa, STATE TOP OF DESCENT. Same intent as PANS/ATM DM 109. | TOD [time] | N | N/A | N/A | N/A | Initial ACL DSC | N/A | [free text] | Initial |
| DM 67ab | Asia-Pac: The pilot shall then check the status of the aircraft’s ADS Emergency Mode and if the emergency mode has been activated inadvertently, the pilot shall select ADS Emergency Mode to “OFF” and advise ATC by voice or the following CPDLC free text downlink. No equivalent in PANS/ATM. | ADS RESET | N | N/A | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
|  | Free Text – Reports (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67aa | The specified ATS unit is being monitored on the specified frequency. Urgent urgency attribute. Same intent as DM89. Preformatted message. | MONITORING [unitname] [frequency] | N | IER | N/A | N/A | Initial | N/A | [free text] | Initial |
|  | Free Text – Military (downlink) |  |  |  |  |  |  |  |  |  |
| DM 67w | Asia-Pac: The tanker is requesting a clearance to delay at the ARCP until the rendezvous with the receiver. [position] is the ARCP as filed in the tanker’s flight plan. [time] is the time the tanker expects to pass the ARCP and commence refueling along the refueling track. It is also the end of the delay time. DM67L in Asia-Pac. No equivalent in PANS/ATM. | TO DELAY FOR AIR REFUEL AT [position] UNTIL [time]; and | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| DM 67x | Asia-Pac: The tanker pilot is providing notification that the end of refueling is imminent. [xxxxx} may be either position or time. DM67n in Asia-Pac. No equivalent in PANS/ATM. | EXPECT END OF REFUEL AT [xxxxx] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| DM 67y | Asia-Pac: [XXXXX] can be either a point or a time DM67o in Asia-Pac. No equivalent in PANS/ATM. | JOINING ALTRV [xxxxx] AT [xxxxx] Example: JOINING ALTRV CW413 AT HEMLO or JOINING ALTRV CW413 AT 1530Z | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |
| DM 67z | Asia-Pac: The tanker is accepting MARSA procedures with the receiver [receiver callsign] is the flight planned callsign of the receiver. DM67p in Asia-Pac. No equivalent in PANS/ATM. | ACCEPT MARSA WITH [callsign(s) of other aircraft] | R | IER | N/A | Ver 4 Ver 5 | N/A | N/A | [free text] | N/A |

Table 2-5	CPDLC application – controller initiated standardized free text transactions

| CPDLC application standardized transactions using free text |  |  |
| --- | --- | --- |
| Transaction/message intent | Controller | Pilot |
| Uplink - Free Text Report/ Confirmation Requests |  |  |
| The controller is requesting that the pilot notify when the specified traffic has been seen by visual contact and passed. The level specified in the traffic description is the level being maintained by the opposite direction aircraft. | REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time] The traffic description is to be inserted by the controller and shall include the aircraft identification (callsign), flight level and aircraft type. ETP = Estimated Time of Passing. Example of the traffic description: SIA228 B747 FL370 | ROGER and then [traffic identification] SIGHTED AND PASSED Example - SIA228 SIGHTED AND PASSED or [traffic identification] NOT SIGHTED |
| The controller is requesting the pilot to report the present ground speed. | REPORT GROUND SPEED | ROGER And then, GS [speed] Example - GS 490 |
| The controller is requesting that the pilot advise the preferred flight level for the flight. Note: Same intent as PANS/ATM UM231 | STATE PREFERRED LEVEL | ROGER And then, FL [altitude] Example - FL 350 |
| The controller is requesting an estimate for the specified waypoint. Note: Same intent as PANS/ATM UM228. | REPORT ETA [position] Example - REQUEST ESTIMATE BILBO | ROGER Then, [position] [time] Example - BILBO 0413 |
| The controller is requesting from the pilot the time at which the aircraft will maintain the specified level. | WHEN WILL YOU MAINTAIN FL [altitude] | ROGER Then, FL [altitude] AT [time] Example - FL 350 AT 2317 |
| The controller is requesting the distance from the specified position or waypoint at which the aircraft will maintain the specified level. The pilot shall include the direction from the waypoint as a cardinal point, e.g. N, NE, NW, S, SW, SE, E or W. | AT WHAT DISTANCE [position / waypoint] WILL YOU MAINTAIN FL [altitude] | ROGER Then, FL [altitude] AT [distance] NM [direction] [position / waypoint] Example - FL 350 AT 26 NM W IPEMA |
| The controller is requesting that the pilot report the radial on which the aircraft is proceeding and the distance from the specified VOR. | REPORT RADIAL AND DISTANCE [to/from] [position] | ROGER Then, [radial] R [distance] NM [to/from] [position] Example - 320 R 26 NM FROM MCY |
| The controller is requesting that the pilot makes voice contact / radio check call on the specified frequency. | REQUEST VOICE CONTACT [frequency] | ROGER |
| Uplink - Free Text Instructions |  |  |
| The controller has detected that uplink messages exist that the pilot has not yet responded to. The pilot is required to check the ATC log page and to respond to unanswered uplink messages. | CHECK AND RESPOND TO OPEN CPDLC MESSAGES | ROGER |
| Instruction that the “ident” function of the ADS-B emitter is to be activated | TRANSMIT ADS-B IDENT | ROGER |
| The controller instructs the aircraft to set the time value to determine delayed messages. | SET MAX UPLINK DELAY VALUE TO XXX SEC | ROGER |
| Uplink – Free text Advisories |  |  |
| The controller is notifying the pilot that a selcal check will be made on the specified HF frequency. | EXPECT SELCAL CHECK HF [frequency] | ROGER |
| The controller is notifying the pilot that the CPDLC transfer process will not be completed at the FIR boundary and will be delayed until the specified time. If the CPDLC transfer is not completed by the specified time, the pilot shall manually disconnect and logon to the next center | EXPECT CPDLC TRANSFER AT [time] | ROGER |
| ATS advisory that the radar and/or ADS-B service is terminated. | IDENTIFICATION TERMINATED | ROGER |
| The controller is notifying the pilot that CPDLC connection is not required by the next FIR (where the flight's transition time of that FIR is short) and CPDLC connection will be transferred to the subsequent FIR. The [ATSU name] is the relevant four character ICAO code. | EXPECT NEXT CENTER [ATSU name]. CONTACT WITH [ATSU name] NOT REQUIRED | ROGER |
| The controller is notifying the pilot of traffic significant to the flight. The description will include the aircraft type and any other relevant information to assist the pilot in sighting the traffic. The pilot may respond that the traffic has been sighted. | TRAFFIC IS [traffic description] | ROGER Then, (optionally) TRAFFIC SIGHTED |
| The controller is notifying the pilot of the secondary HF frequency for the area. | SECONDARY HF [frequency] | ROGER |
| Uplink - Free Text Speed Messages |  |  |
| The controller is notifying the pilot that a speed instruction may be issued to be effective until the specified time. | EXPECT TO MAINTAIN [speed] UNTIL [time / position] | ROGER |
| Uplink - Free Text Emergency Acknowledgment |  |  |
| The controller has acknowledged receipt of a MAYDAY downlink message. The controller shall attempt to make voice contact with the pilot. The pilot should only respond with ROGER if or when able to do so. If the aircraft is inbound to an airport within the FIR, a ROGER response is not required. | ROGER MAYDAY | ROGER |
| The controller has acknowledged receipt of a PAN downlink message. The controller shall attempt to make voice contact with the pilot. The pilot should only respond with ROGER if or when able to do so. If the aircraft is inbound to an airport within the FIR, a ROGER response is not required. | ROGER PAN | ROGER |

Table 2-6	CPDLC application – pilot initiated standardized free text transactions

| CPDLC application standardized free text downlink messages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Transaction/message intent | Pilot |  | Controller |  |
| Downlink - Free Text Advisories |  |  |  |  |
| The pilot is offsetting due wake turbulence in accordance with RVSM procedures (offset will not exceed 2nm). The controller is not required to respond or issue a clearance. |  | WAKE DEV [direction] Direction L or R (left or right) as appropriate |  | R |
| The pilot is advising ATC of an update a waypoint ETA. |  | REVISED ETA [position] [time] |  | R |
| The aircraft advises ATC of receiving a delayed message. |  | UPLINK DELAYED IN NETWORK AND REJECTED - RESEND OR CONTACT BY VOICE |  | N |
| Downlink – Response to controller free text uplinks |  |  |  |  |
| The pilot responds to controller free text, REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time]. | [traffic identification] SIGHTED AND PASSED Example - SIA228 SIGHTED AND PASSED |  |  | N |
| The pilot responds to controller free text, REPORT SIGHTING AND PASSING OPPOSITE DIRECTION [traffic description] ETP [time]. | [traffic identification] NOT SIGHTED |  |  | N |
| The pilot optionally responds to controller free text, TRAFFIC IS [traffic description],. | TRAFFIC SIGHTED |  |  | N |
| The pilot optionally responds to controller free text, REPORT GROUND SPEED. | GS [speed] Example - GS 490 |  |  | N |
| The pilot responds to controller free text, STATE PREFERRED LEVEL. Note: Same intent as PANS/ATM DM 106. | FL [altitude] Example - FL 350 |  |  | N |
| The pilot responds to controller free text, REPORT ETA [position], Note: Same intent as PANS/ATM DM 106. | [position] [time] Example - BILBO 0413 |  |  | N |
| The pilot responds to controller free text, WHEN WILL YOU MAINTAIN FL [altitude]. Note: No equivalent in PANS/ATM. | FL [altitude] AT [time] Example - FL 350 AT 2317 |  |  | N |
| The pilot responds to controller free text, WHEN WILL YOU MAINTAIN FL [altitude]. Note: No equivalent in PANS/ATM. | FL [altitude] AT [distance] NM [direction] [position / waypoint] |  |  | N |
| The pilot responds to controller free text, REPORT RADIAL AND DISTANCE [to/from] [position]. Note: No equivalent in PANS/ATM. | [radial] R [distance] NM [to/from] [position] Example - 320 R 26 NM FROM MCY |  |  | N |
| Negotiation Responses (downlink) |  |  |  |  |
| We can accept the specified level at the specified time. Note: Intent same as PANS/ATM DM81. |  | WE CAN ACCEPT [altitude] AT [time] |  | N |
| We can accept the specified speed at the specified time. Note: Intent same as DM83. |  | WE CAN ACCEPT [speed] AT [time] |  | N |
| We can accept a parallel track offset the specified distance in the specified direction at the specified time. Note: Intent same as DM85. |  | WE CAN ACCEPT [direction] [distance offset] AT [time] |  | N |
| We cannot accept the specified level. Note: Intent same as PANS/ATM DM82. |  | WE CANNOT ACCEPT [altitude] |  | N |
| We cannot accept the specified speed. Note: Intent same as PANS/ATM DM84. |  | WE CANNOT ACCEPT [speed] |  | N |
| We cannot accept a parallel track offset the specified distance in the specified direction. Note: Intent same as PANS/ATM DM86. |  | WE CANNOT ACCEPT [direction] [distance offset] |  | N |
| Request for the earliest time at which a clearance to climb to the specified level can be expected. Note: Intent same as PANS/ATM DM87. |  | WHEN CAN WE EXPECT CLIMB TO [altitude] |  | Y |
| Request for the earliest time at which a clearance to descend to the specified level can be expected. Note: Intent same as PANS/ATM DM88. |  | WHEN CAN WE EXPECT DESCENT TO [altitude] |  | Y |

Appendix A:	Conventions Used in this Standard

This page intentionally left blank.

- CONVENTIONS USED IN THIS STANDARD

  - Introduction

This appendix provides a description of the conventions used in this standard.  These conventions provide cross reference of the information provided in this reference guide and to the source documents.

  - CPDLC application cross reference tables

There are two types of tables provided in this document for the CPDLC application:

- Message element tables – for CPDLC uplink and downlink message elements (See paragraph A.2.1); and

- Communication transaction tables – for controller-initiated and pilot initiated transactions using CPDLC uplink and downlink [Free text] messages (See paragraph A.2.2.

    - CPDLC application - message element tables

The CPDLC application message element tables provide information for uplink message elements, downlink message elements, and standardized free text.

The message element tables include the following columns:

| Ref # | Message Intent/ Use | Message Element | Resp. | Ocean SPR | NAT | Asia-Pac FOM | Cont. SPR | ATN B1 | FANS 1/A | FANS 1/A – ATN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

These columns and the notations used in each of the columns is described in Table AP- .  Also, the following guidelines apply when using the tables in this reference guide:

- Legend for response column.  “Closure” responses are implicit in the PANS/ATM when providing criteria for transfer of CPDLC.  FANS 1/A provides explicit “closure” responses.  Differences from the PANS/ATM are indicated.  Ambiguities in the FOM are clarified based on FANS 1/A interoperability criteria provided in RTCA DO-258/EUROCAE ED-100.

- Each of the remaining columns provides information from each of the references listed above.  For the initial release and Revision A of ED-110/DO-280, the differences are noted in the column for the INTEROP ATN B1.  Revision A introduced version codes:  110, 111, 112, to identify different versions of the ATN B1.  Each of the different versions has a different message set.  The table indicates which version contains which message element.  Revision A of DO-258/ED-100 has not been approved so the table does not include the results of this document.

- FANS 1/A aircraft do not have the capability to send UNABLE in response to an uplink message containing message elements with an “R” response attribute.  For these aircraft, the flight crew may use alternative means to UNABLE the message.  These alternative means will need to be taken into consideration to ensure proper technical and operational closure of the communication transaction.

- In cases where there is a choice for the message element or the response attribute, the first choice that appears in the row for that message element indicates the preferred choice.  The “NE” response attribute is used (by FANS 1/A) for message elements when the WILCO, UNABLE, AFFIRM, NEGATIVE, ROGER, and STANDBY responses are not enabled for flight crew selection.  However, when a message with an NE response attribute requires an operational response, only the correct operational response is presented to the flight crew.

- The following variables are considered operationally interchangeable in this document respecting range and resolution variations as defined in interoperability standards:

| ICAO Doc 4444 variable | Equivalent variable |
| --- | --- |
| [level] | [altitude] (See Item f)) |
| [specified distance][direction] | [direction][distance offset] |
| [departure clearance] | [predeparture clearance] |
| [unitname] | [icao unitname] |
| [code] | [beacon code] |
| [facility designation] | [icao facility designation] |
| [persons on board] | [souls on board] |

- ICAO Doc 4444 notes that message elements that contain the [level] variable can be specified as either a single level or a vertical range.  FANS 1/A only considers the [level] variable as a single level and uses message elements that are intended exclusively for specifying a vertical range, e.g., UM30, UM31, UM32, UM180, DM7, DM76, DM77, etc.  ATN uses the [level] variable to specify a vertical range and does not use the message elements intended exclusively for specifying a vertical range, except in cases where an ATN B1 ground system provides data link service to FANS 1/A aircraft.

Table AP- 	Conventions used in tables

| Notation | Description |
| --- | --- |
| All columns | General use of notation |
| N/A | Not applicable. The source documents do not refer to this item related to the message element. |
| Ref # | PANS/ATM, References: a) and b) |
| UM[x] | Uplink message reference number, where [x] is an integer value. |
| DM[x] | Downlink message reference number, where [x] is an integer value. |
| UM169[y] | Standardized uplink free text message reference number, where [y] is an alpha character for unique identification of the standardized free text. |
| DM67[y] | Standardized downlink free text message reference number, where [y] is an alpha character for unique identification of the standardized free text. |
| Message intent/use and Message elements columns | PANS/ATM, References: a) and b) Supplemental information, References: All |
| [Text] | Indicates the intended use of the message taken from ICAO Doc 4444, Appendix 5. |
| [Text] | Indicates text that was added by OPLINKP/1. [Text] indicates text that was deleted by OPLINKP/1. These changes are planned for incorporation into ICAO Doc 4444, effective 22 November 2007. |
| [Text] | Indicates text that was deleted by OPLINKP/1. These changes are planned for incorporation into ICAO Doc 4444, effective 22 November 2007. |
| [Text] | Supplemental information to indicate: a) ICAO changes; b) clarification of intended use in various regions of the world. The region or airspace to which the [Text] applies is indicated at the beginning of the text; and c) For standardized free text, to provide equivalent PANS/ATM message element reference, or to indicate no equivalent. |
| Response column | PANS/ATM, References: a), b), c), and i) Supplemental information, References: All |
|  | For uplink message |
| W/U | Response required: Yes PANS/ATM: WILCO, UNABLE, STANDBY, NOT CURRENT DATA AUTHORITY, NOT AUTHORIZED NEXT DATA AUTHORITY, LOGICAL ACKNOWLEDGMENT (only if required), ERROR FANS 1/A: WILCO, UNABLE, STANDBY, ERROR, NOT CURRENT DATA AUTHORITY |
| A/N | Response required: Yes PANS/ATM: AFFIRM, NEGATIVE, STANDBY, NOT CURRENT DATA AUTHORITY, NOT AUTHORIZED NEXT DATA AUTHORITY, LOGICAL ACKNOWLEDGMENT (only if required), ERROR FANS 1/A: AFFIRM, NEGATIVE, STANDBY, ERROR, NOT CURRENT DATA AUTHORITY |
| R | Response required: Yes PANS/ATM: ROGER, UNABLE, STANDBY, NOT CURRENT DATA AUTHORITY, NOT AUTHORIZED NEXT DATA AUTHORITY, LOGICAL ACKNOWLEDGMENT (only if required), ERROR FANS 1/A: ROGER, STANDBY, ERROR, NOT CURRENT DATA AUTHORITY |
| Y | Response required: Yes PANS/ATM: Any CPDLC downlink message, LOGICAL ACKNOWLEDGEMENT (only if required) |
| N | Response required: No, unless logical acknowledgement required. PANS/ATM: LOGICAL ACKNOWLEDGMENT (only if required), NOT CURRENT DATA AUTHORITY, NOT AUTHORIZED NEXT DATA AUTHORITY, ERROR FANS 1/A: Not used |
| NE | Response required: [Not defined] PANS/ATM: Not used. FANS 1/A: WILCO, UNABLE, AFFIRM, NEGATIVE, ROGER, STANDBY, not enabled Asia-Pac: Most messages with an NE attribute require an operational response. Only the correct operational response is presented to the pilot. The uplink message is considered to be closed on sending and does not require a response to close the dialogue. The WILCO, UNABLE, AFFIRM, NEGATIVE, ROGER, and STANDBY responses are not enabled for pilot selection. |
|  | For downlink messages |
| Y | Response required: Yes PANS/ATM: Any CPDLC uplink message, LOGICAL ACKNOWLEDGMENT (only if required) FANS 1/A: Response required. |
| N | Response required: No PANS/ATM: LOGICAL ACKNOWLEDGMENT (only if required), SERVICE UNAVAILABLE, FLIGHT PLAN NOT HELD, ERROR FANS 1/A: Response not required |
| Oceanic SPR column | Oceanic Safety and Performance Standard, Reference: g) |
| All | Used in all data link services listed for this column |
| CE | Used in to Connection Establishment |
| CT | Used in Connection Termination |
| TC | Used in Transfer of Communication |
| CRD | Used in Clearance Request and Delivery |
| IER | Used in Information Exchange and Reporting |
| PR | Used in Position Reporting |
| (Urgent) | (Urgent) used with CRD, IER, “All,” or N/A |
| (Distress) | (Distress) used with CRD, IER, “All,” or N/A |
| NAT column | North Atlantic Region, Reference: d), e), and f) |
| P3 | Planned for use in the NAT Phase 3 implementation |
| P4+(A) | Planned for use in the NAT Phase 4 implementation and within the scope of the ATN B1 message element set. |
| P4+(F) | Planned for use in NAT Phase 4 implementation and “out-of-scope” of the ATN B1 message element set, but in scope of the FANS 1/A message element set. |
| P4-(A) | Not planned for use in the NAT Phase 4 implementation and within the scope of the ATN B1 message element set. |
| P4-(F) | Not planned for use in the NAT Phase 4 implementation and within the scope of the FANS 1/A message element set. |
| Asia-Pac column | Asia-Pacific Region, Reference: c) |
| Ver 4 | Used in “Version 4” release of the document. |
| Ver 5 | Used in “Version 5” release of the document. |
| Cont. SPR column | Continental Safety and Performance Standard, Reference: h) |
| Initial | Used in the “Initial” release of the document. |
| Chg [x] | Added in “Change [x]” of the document, where [x] is an integer value that signifies the change number. |
| All | Used in all data link services listed for this column |
| ACM | Used in ATC Communications Management |
| ACL | Used in ATC Clearance |
| AMC | Used in ATC Microphone Check |
| DCL | Used in Departure Clearance |
| DSC | Used in Downstream Clearance |
| ATN B1 column | Aeronautical Telecommunication Network Baseline 1 Interoperability Standard (ATN B1 INTEROP Standard), Reference: j) |
| Initial | Used in the “Initial” release of the document. |
| Rev A | Used in “Revision A” of the document, for the message set versions listed. |
| 110 | Used in “Revision A” of the document, version 110 message set. |
| 111 | Used in “Revision A” of the document, version 111 message set. |
| 112 | Used in “Revision A” of the document, version 112 message set. |
| Rev B v1 | Used in “Revision B” release of the document, version 1 message set, for the data link services listed. |
| All | Used in all data link services listed for this column |
| ACM | Used in ATC Communications Management |
| ACL | Used in ATC Clearance |
| AMC | Used in ATC Microphone Check |
| DCL | Used in Departure Clearance |
| DSC | Used in Downstream Clearance |
| FANS 1/A column | Future Air Navigation System 1/A Interoperability Standard (FANS 1/A INTEROP Standard), Reference: i) |
| Initial | Used in the “Initial” release of the document. |
| Rev A | Added in “Rev A” release of the document |
| FANS 1/A – ATN column | Future Air Navigation System 1/A – Aeronautical Telecommunication Network Interoperability Standard (FANS 1/A – ATN INTEROP Standard), Reference: k) |
| Initial | Used in the “Initial” release of the document. |

    - Communication transaction tables

The CPDLC application communication transaction tables provide information for controller-initiated and pilot-initiated transactions using standardized free text.

The message element tables include a description of the intent of the transaction and messages.  For controller-initiated transactions, the controller is the initiator and the pilot is the recipient; for pilot-initiated transactions, the pilot is the initiator and the controller is the recipient.

| CPDLC application standardized transactions using free text |  |  |
| --- | --- | --- |
| Transaction/message intent | [Initiator] | [Recipient] |
| Description of intent of transaction and messages. Includes notes and references to PANS/ATM message elements, uses in particular regions, etc. | Standardized free text message for controller (when initiator) or pilot (when initiator). | Standardized free text message for pilot (when recipient) or controller (when recipient) |

  - Use of color

Red in the NAT, Asia-Pac, and Oceanic SPR columns indicate not supported, avoid use, or not used.  Green indicates the message element is supported, i.e., (it's in the regional documents),  Orange is indeterminate.  Yellow in the FANS 1/A column indicates which messages are in the FANS 1/A message set.  Blue in the Cont SPR, ATN B1, and FANS 1/A-ATN columns indicate the LINK 2000+ message set.