THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Operational authorizations and aircraft certification of data link systems

(Presented by United States of America)

Summary

This paper provides information on the status of the FAA’s guidance material for operational authorizations and aircraft certification of data link systems and relates it to issues with aircraft equipage requirements and flight crew training and qualification.

# Introduction

## In the North Atlantic Region, the FANS Implementation Group (NAT FIG) at their 17th meeting in Ayr, Scotland, 21-25 April 2008, discussed two issues related to aircraft data link equipage and operational use, particularly with the advent of general aviation use of ADS-C in NAT airspace:

- the need to ensure that aircraft will be properly equipped and that crews will be properly trained and qualified; and

- the need to clarify the meaning of FANS 1/A, and to establish appropriate designators to manage different aircraft capability and operational uses.

## The FAA is in the process of updating its guidance material for aircraft certification and operational authorization of data link systems.

## These updates are related to the whole of paragraph 3.1 in the NAT GM, and also paragraph 3.2 of the FANS 1/A Operations Manual (FOM), which addresses personnel licensing and training in accordance with ICAO Annex 1 – Personnel Licensing, and ICAO Annex 6 – Operation of Aircraft.

## The FAA guidance material may provide useful information for developing globally applicable guidance material to further address the related issues.

# Discussion

## FAA guidance material for aircraft certification and operational authorization of data link systems currently exists in the following advisory circulars:

- FAA Advisory Circular AC 120-70A, Operational Authorization Process for use of Data Link Communication System, dated December 29, 2005.

- FAA AC 20-140, Guidelines for Design Approval of Aircraft Data Communications Systems, dated August 16, 1999.

## These documents are currently under review and are being updated to take into account the experiences gained since their issuance.

## The scope of this guidance material is being defined according to Appendix A.  Appendix A is an excerpt from the two draft FAA ACs and provides criteria for different types of data link systems and operations.  For each type of data link system, a row is provided to assign a designator to a particular aircraft capability, and to correlate it to the intended operating environment.  The intended operating environment is characterized by the type of airspace, the capability of the ATS unit, and the intended use for that aircraft capability.  The FAA will refer to the designator for the aircraft data link system in the rotorcraft/airplane flight manual (R/AFM), as part of the type design approval for the aircraft, to convey the functional and interoperability aspects of the aircraft data link system needed to support operations, which are specified when obtaining an operational authorization to use the data link capability.

## The updates to AC 20-140 recognize published standards as an acceptable means to show that the aircraft data link system complies with applicable regulations.  If an applicant requests a change to the provisions of the AC, they will need to make a request to the FAA, which will assess the change to ensure that there is no significant global impact prior to granting an approval for the change.  Changes to the provisions of the AC are considered to be when the project involves:

- Any standard not recognized in the AC;

- Partial compliance to portions of a standard that are relevant to the aircraft;

- Deviations from the standards recognized in the AC; and

- Deviations from the relevant objectives identified in Annex A of RTCA/DO 264/ED 78A.

## Figure 1 provides an overview of the standards and guidelines that are recognized in the AC.

Figure 1:  Overview of data link standards

## The updates to AC 120-70A will provide guidance material for obtaining an operational authorization to use the aircraft data link systems approved in accordance with the provisions of AC 20-140A.  AC 120-70A will address flight crew training and qualification, maintenance, any additional criteria affecting the performance of system operation, and procedures for submitting problem reports and data.

## A mature draft of the AC 120-70A is not available at this time.  However, an internal FAA Notice N 8000.A056, which complements the AC, is mature and provides sample templates that will be used for issuing operational specifications to U.S. operators for the following:

- Operations specification (OpSpec) A056 template applicable to part 121

- OpSpec A056 template applicable to part 125

- OpSpec A056 template applicable to part 135

- Management specification (MSpec) MA056 template applicable to part 91 subpart K

- Letter of authorization (LOA) A056 template applicable to part 91

- LOA A056 template applicable to part 125M

- The guidance for Inspectors that FAA will incorporate into Order 8900.1, Flight Standards Information Systems (FSIMS) Inspector’s Handbook, Volume 3, Chapter 18, Section 2.

## AC 20-140A and AC 120-70B will include guidance material for qualification/monitoring, based on performance standards, using RCP, to support contracts for commercial communication services (from an aircraft operator’s perspective), to support operational authorizations, and aircraft certification.

## The guidance material being developed for AC 20-140A and AC 120-70B could be used to form the basis for developing global criteria to avoid region specific criteria being imposed on operators and aircraft/avionics manufacturers.

## Current drafts of FAA AC 20-140 and FAA N 8000.A056 are provided at Appendix B and Appendix C, respectively.

# Action

## The Group is invited to:

- note the information in this paper;

- consider this guidance material as a reference in the development of global and/or regional guidance material, such as provided by global operational data link document and NAT guidance material, for aircraft certification and operational authorization related to data link systems.

- consider global standardization of aircraft and ATS unit designators to facilitate the specification of airspace requirements related to data link and filing flight plans.

- contact Moin Abulhosn (moin.abulhosn@faa.gov) for more information or to provide comments on draft FAA AC 20-140A; and

- contact Bob Tegeder (robert.m.tegeder@faa.gov) for more information or to provide comments on draft FAA N 8000.A056 and the AC 120-70B development.

Appendix A – Excerpt from Draft FAA AC 20-140, Guidelines for Design Approval of Aircraft Data Link Communication Systems

Appendix B – Draft FAA AC 20-140A, Guidelines for Design Approval of Aircraft Data Link Communication Systems, September 9, 2008.

Appendix C – Draft FAA N 8000.A056, OPSPEC/MSPEC/LOA A056:  Data Link Communications, August 20, 2008.

Note:  Appendices are provided as separate files.

Appendix A – Excerpt from Draft FAA AC 20-140, Guidelines for Design Approval of Aircraft Data Link Communication Systems

Table 1:  Criteria for Different Types of Data Link Systems and Operations

| Row # | Aircraft Data Link System | Operating Environment |  |  | Applicable Standards |
| --- | --- | --- | --- | --- | --- |
|  |  | Type of Airspace | ATS Unit System | Capability and Uses |  |
| 1 | ATN B1 | Continental (Domestic) | ATN B1 | Supplemental ATC communications: - CM application supports DLIC data link service - CPDLC application supports ACM, ACL, and AMC data link services Note 1: DCL, DSC, D-ATIS, and FLIPCY data link services are not supported. | - DO-290/ED-120, Chg 1 and Chg 2, Continental SPR Standard - DO-280B/ED-110B, ATN B1 INTEROP Standard |
| 2 | FANS 1/A+ | Continental (Domestic) | ATN B1-FANS 1/A | Same as Row 1, except: - Uses AFN application for DLIC data link service - For CPDLC application, UM 215, TURN [direction] [degrees] is not supported Note 2: FANS 1/A aircraft will require use of DM67 [free text] to mimic certain message elements per DO-290/ED-120 Chg 1 and 2. See DO-305/ED-154, paragraph 4.2.13.2. Note 3: In accordance with DO-290/ED-120, Chg 1 and Chg 2, FANS 1/A aircraft will require use of a message latency timer per DO-258A/ED-100A, paragraph 4.6.6.9 and is denoted by a “+” appended to the “FANS 1/A” label. Note 4: Only via VHF data link sub-network. | Same as for Row 1, plus: - DO-305/ED-154, FANS 1/A – ATN INTEROP Standard (Applies only to ATS Unit, except see Note 2) - DO-258A/ED-100A, FANS 1/A INTEROP Standard (Applies only to aircraft) |
| 3 | FANS 1/A+ or FANS 1/A | Oceanic and remote | FANS 1/A | Normal means of ATC communication uses AFN and CPDLC applications for DCPC Eligible for: - RCP 240 operations via VHF, Satcom Iridium, and Satcom Inmarsat sub-networks - RCP 400 operations via HF data link sub-network - No RCP operations Note 4: Aircraft capability that supports multiple RCP type operations needs to include appropriate indications and/or alerts to enable the flight crew to notify ATC when aircraft equipment failures result in the aircraft’s ability to no longer meet its criteria for any of the RCP types, per DO-306/ED-122, paragraph 5.2.6.a) and 5.2.6.b). Uses ADS-C application for automatic position reporting. | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier versions), FANS 1/A INTEROP Standard |
| 4 | FANS 1/A+ or FANS 1/A | Oceanic and remote | CADS | No CPDLC application Uses ADS-C application for automatic position reporting | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version), FANS 1/A INTEROP Standard (Applies only to aircraft). - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 5 | FMS WPR | Oceanic and remote | CFRS | No CPDLC application Uses FMS WPR application for automatic position reporting | - DO-306/ED-122, Oceanic SPR Standard - ARINC 702A, Advanced Flight Management Computer System (Applies only to aircraft). - CFRS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit) |
| 6 | FANS 1/A ADS-C | Oceanic and remote | FANS 1/A or CADS | Same as row 4 | - DO-306/ED-122, Oceanic SPR Standard - DO-258A/ED-100A (or earlier version) FANS 1/A INTEROP Standard (If ATS unit is CADS, applies only to aircraft) - CADS Common Specification, Version 2.0, approved ICAO NAT FIG/10, Paris, March 29-April 2, 2004 (Applies only to ATS unit when ATS unit is CADS) |

-END-