THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Monitoring FANS1/A RCP and ADS latency – Airways New Zealand Update

(Prepared by Airways New Zealand, Presented by United States of America)

Summary

Airways New Zealand has been developing monitoring of FANS1/A system performance to the RTCA DO-306 requirements since September 2007. This paper provides an update on this development and also provides an overview of observed FANS1/A system performance in 2008 for the Auckland (NZZO) Oceanic Flight Information Region.

# Introduction

## RTCA DO-306/EUROCAE ED 122 Oceanic SPR standard provides ANSP’s with the requirements for the monitoring of FANS1/A CPDLC communication performance in terms of Required Communication Performance (RCP) and ADS surveillance latency.

## Airways New Zealand has been monitoring FANS1/A system performance using the CPDLC RCP and ADS surveillance latency requirements from DO306 since September 2007. This paper provides an update on this monitoring and an overview of observed system performance in 2008.

# Discussion

## Filtering of raw data before measuring performance. We have found that some filtering of the data is required in order for measured performance to better represent actual performance. Currently, we are filtering data as follows:

### Duplicated downlinks. A number of duplicated ADS downlinks are seen where the same message is repeated two and sometimes three times. These are filtered by checking for ADS downlinks with the same FMS timestamp and current latitude/longitude. Failing to remove these duplicated messages skews the observed results quite markedly. Figure 1 below illustrates the performance gain seen by removing duplicated ADS reports from an analysis of one airlines B777 fleet performance. While we are also filtering CPDLC downlinks for duplicates none have been observed to date.

Figure 1: Effect of duplicated ADS reports on latency measurement

### CSP Outages. Our analysis is done on a monthly basis. The raw data is assessed and where delayed messages ( e.g. ADS downlink > 90 secs ) are seen during notified periods of CSP outages they are removed. We have also detected an outage that was not notified. This was confirmed with the CSP and the affected messages removed from the data. An illustration of a typical monthly filter is illustrated below in Figure 2.

| Date | Outage | ADS Downlink ∆t |
| --- | --- | --- |
| 2008.05.02 | Not Notified | 467 |
| 2008.05.02 | Not Notified | 501 |
| 2008.05.02 | Not Notified | 1125 |
| 2008.05.02 | Not Notified | 358 |
| 2008.05.02 | Not Notified | 353 |
| 2008.05.30 | Notified | 203 |
| 2008.05.30 | Notified | 267 |
| 2008.05.30 | Notified | 706 |
| 2008.05.30 | Notified | 1274 |
| 2008.05.30 | Notified | 1169 |
| 2008.05.30 | Notified | 385 |
| 2008.05.30 | Notified | 396 |
| 2008.05.31 | Notified | 360 |

Figure 2: Delayed ADS reports due CSP outage filtered during May 2008

### B777 SATCOM/VHF RGS transition issue. An issue has been identified with the B777 fleet when transitioning from SITA VHF RGS into SATCOM. Boeing is aiming to have a fix available for this in January 2009. W are currently filtering delayed messages received in known areas of VHF/SATCOM transition from B777 aircraft when measuring B777 performance. The effect of this issue on B777 performance is illustrated in Figure 3 below.

Figure 3: B777 SATCOM/VHF transition issue

### Individual poor performing aircraft. We do observe degraded performance from an airline fleet on occasion that can be attributed to poor performance from an individual aircraft. When these aircraft are identified the airline is notified and the offending aircraft is filtered from the airline fleet performance indicators.

## CPDLC – Monitoring Required Communications Performance.

### In determining Actual Communications Performance (ACP) we are monitoring RCP, and Required Communication Technical performance (RCTP) as per RTCA DO306 Oceanic SPR Standard. We also monitor the pilots operational response time (PORT). Currently, we only monitor those CPDLC uplink messages that require a WILCO/UNABLE response. This approach is consistent with the explanatory note to para. 2.4.7 (n) in ICAO Doc 9882 that notes that where communication performance requirements are established, they shall be determined on the basis of the urgency of the communication and whether they are related to command and control ( i.e. intervention capability) communications.  Our early analysis showed that incorporating other non-intervention message types such as free text queries or information requests was skewing the data because of longer response times from the flight deck. Our monitoring to date has involved an analysis of round trip CPDLC intervention transactions i.e the CPDLC intervention uplink and the crew response, via a number of different communication paths. This analysis is based on that subset of message transactions where the MAS response is received from a RGS, and the corresponding crew response downlink is also received.  We are monitoring round trip performance via a number of different communication paths. These paths include: SATCOM Transmission Path Performance, measured by observing the results from all intervention uplinks where both the MAS and downlink response are received from a Satellite RGS; VHF Transmission Path Performance, measured by observing the results from all intervention uplinks where both the MAS and downlink response are received from a VHF RGS; Mixed VHF/SATCOM Transmission Path Performance, measured by observing the results from all intervention uplinks where one downlink response is received via Satellite RGS while the other is received via a VHF RGS; Overall System Performance, measured by observing results from all intervention uplinks regardless of the downlink path. Note that a limited number of CPDLC downlinks were received via HFDL from one airline through March 2008. However, because of a fleet change by this airline we no longer see HFDL used for CPDLC operational responses in NZZO.

### Observed CPDLC Performance. An indication of overall CPDLC system performance from all aircraft via all RGS is shown in the Figures 4 through 6 below. The data has been filtered for duplicates and outages but not for B777 SATCOM/VHF transition issues. Figure 4 below shows ACTP with 95% required in 120”, 99.9% in 150”, Figure 5 ACP with 95% required in 180” 99.9% in 210”, and Figure 6 PORT where DO306 allows for 95% within 60”.

Figure 4: CPDLC Actual Communications Technical Performance

Figure 5: CPDLC Actual Communications Performance

Figure 6: Pilot Operational Response Times

### Observed B744 CPDLC Performance by Airline Fleet.  An indication of fleet performance from B744 aircraft for one airline via SATCOM RGS is shown in the Figures 7 through 9 below. The data has been filtered for duplicates and outages. Figure 7 below shows B744 ACTP with 95% required in 120”, 99.9% in 150”, and Figure 8 B744 ACP with 95% required in 180” 99.9% in 210. Figure 9 shows A343 ACTP.

Figure 7: SATCOM B744 ACTP Airline AAA

Figure 8: SATCOM B744 ACP Airline AAA

Figure 9: SATCOM A343 ACTP Airline XXX

## ADS – Monitoring downlink latency.

### We are currently monitoring individual airline fleet performance via SATCOM. Figure 10 below displays typical B777 actual performance when VHF transition delays are filtered. The DO306 ADS latency requirements for the application of reduced distance based separations are 95% within 90” and 99.9% within 180”.

Figure 10: SATCOM B772 ADS Latency Airline DDD

## Observed system performance continues to be within the “ballpark” of DO306 requirements. Where observed performance has been seen to drop below the requirements then we are seeking to improve that performance. We see this as consistent with the performance based approach per the guidance in ICAO Doc 9883 Manual on Global Performance of the Air Navigation System. The monitoring of DO306 requirements and drilling down to an airline fleet and individual aircraft level has enabled us to identify a number of performance issues such as the B777 SATCOM/VHF transition problem.  The monitoring also shows possible issues with the current ARINC 618 SATCOM no-ack timers (airborne SAT7 timer is currently set at 180”) and Boeing is proposing a reduction in the timer value that would allow at least a first retry within the DO306 time allocations.

-END-