Coordination
Draft

Satellite Voice
Project Plan

30 September 2008

Prepared by:

Performance Based Operations
Aviation Rulemaking Committee

Communications Working Group
(PARC CWG)

Foreword

The Performance-Based Operations Aviation Rulemaking Committee (PARC) is an FAA-sponsored activity that operates according to the Administrator’s authority under 49 USC 106(p)(5).  The PARC comprises members from the FAA and the aviation community at large, provides recommendations to FAA’s Senior Management for action and implementation.  The PARC has been effective over the last decade in implementing RNAV/RNP.  In 2005, the PARC established the Communications Working Group (CWG) to address a number of issues related to the implementation of aeronautical communication systems.  These systems included, among others, the future air navigation system (FANS) 1/A, the aeronautical telecommunication network (ATN), and satellite voice communications.

The PARC CWG is committed to applying the performance-based concept, which aims to leverage existing capability and maximize benefits by:

- Enabling cost-effective alternatives, using different technologies and existing capabilities, that meet business needs in a more timely manner;

- Providing performance-based criteria to demonstrate aircraft equipment and capability without technological or implementation-specific constraints; and

- Enabling different levels of service in common airspace to a fleet of aircraft with varying capability and performance.

The PARC CWG develops recommendations that directly support matters that relate to the FAA’s regulatory criteria and guidance material for implementation of voice and data communications within the U.S. National Airspace System (NAS).  However, the PARC CWG recognizes that global harmonization is crucial to the success of any State or regional implementation initiative.  As such, the PARC CWG prepares Coordination Drafts for broad review and solicits input on such matters of interest to the aviation community.  If you would like more information on PARC CWG activities or would like to comment on this Coordination Draft, please contact either Arnold Oldach (aoldach@rockwellcollins.com) or Tom Kraft (tom.kraft@faa.gov).

Table of contents

Contents		Page

Background	1

Goal of this project plan	1

Prerequisites	2

Project Administration	2

Stakeholders	2

Success criteria	4

Description of project	4

Operations	4

Aircraft equipage	5

Qualification means	5

Air traffic service facility requirements	5

Satellite service requirements	5

Aircraft requirements	6

Project Plan Phase Definitions	6

Deliverables	6

Initial in-service operational trial	6

Project schedule	7

Cost of voice communications	7

Attachments

Attachment A – Criteria for Satellite Voice Project

Attachment B – Satellite Voice End-to-End Configurations

Attachment C – Satellite Voice Performance Evaluation Methods

Revision History

| Date | Description of changes |  |
| --- | --- | --- |
| 13-Aug-08 | Initial coordination draft of document |  |
| 2-Sep-08 | Incorporates discussions at PARC CWG/16 of the initial draft |  |
| 6-Sep-08 | Reformatted, added Foreword, editorial cleanup, added attachments A, B, & C |  |
| 30-Sep-08 | Incorporated comments from ARINC. It also includes suggested changes from Tom Kraft shown as “tracked changes and comments.” |  |

# Background

Oceanic and remote airspace flight communications have historically been conducted with high frequency (HF) radios due to the advantage of being able to transmit and receive air/ground communications for thousands of miles.  The HF communication centers are designated by air traffic service providers (ATSP) for air traffic control (ATC) services.  By comparison, very high frequency (VHF) radios used by controllers provide line-of-sight coverage of approximately 200 miles and do not follow the curvature of the earth.   Following the advent of satellites, new technologies were endorsed by ICAO in 1991 and have since updated communications, navigation, and surveillance (CNS) for air traffic management (ATM).

In 1995, the initial future air navigation system (FANS 1/A) provided an integrated CNS package.  In addition to required navigation performance (RNP) and global navigation satellite system (GNSS) capabilities, FANS 1/A includes controller pilot data link communications (CPDLC) and automatic dependent surveillance – contract (ADS-C) capabilities using satellite communications (SATCOM), VHF, and HF data links.  At the same time, aircraft became equipped with SATCOM voice capability.

In 1996, the FAA recognized technological advances in communications by a rule change to 14 CFR sections 1.1 and 121.351 that included use of a new term: long range communication system (LRCS). 14 CFR Section 1.1 defines LRCS as “A system that uses satellite relay, data link, high frequency, or other approved communication system which extends beyond line-of-sight.”  Examples of systems that meet this definition are HF voice, HF data link (HFDL), SATCOM voice and SATCOM data link.

The regulations, therefore, now address long range communication requirements in terms of LRCS.  With that as a basis, an aircraft on extended over water segments unable to utilize line-of-sight systems must have at least two operational LRCSs to honor regulatory communication requirements (unless specifically excepted under 14 CFR section 121.351(c)).  In addition, HF voice is the only LRCS currently acceptable for air traffic control communications in all areas.  Therefore, in areas requiring two operational LRCSs, at least one must be HF-voice and in areas requiring one LRCS, that system must be HF voice.

SATCOM voice and data have proven to be effective and reliable long range communication systems since their introduction into service.  Flight crews have used SATCOM voice (when voice communications are needed) instead of HF voice.  Some regulatory authorities have granted some operators dispatch relief for a limited time whereby the aircraft may be dispatched for a limited period (5 days) with only a single HF radio system and an operational SATCOM voice system.  In the interest of lowering operational costs and streamlining operational efficiency, aircraft operators are turning to SATCOM voice and data capability to meet long range communication system ( LRCS) requirements.  These capabilities include Inmarsat SATCOM voice using geosynchronous orbit satellites and Iridium SATCOM voice, which uses satellites in a low-earth orbit.

# Goal of this project

The goal of this project is to conclude on the recommendations to the FAA that would allow one HF radio system to be permanently replaced with a satellite voice system.  This project is specifically investigating Inmarsat and Iridium satellite voice communication capabilities to determine their viability as an FAA-approved long range communication system (LRCS).  If satellite voice can be FAA-approved as a LRCS, the Minimum Equipment List (MEL) could be changed to allow dispatch with one satellite voice communication system and only one HF radio system, when two LRCS are normally required.

# Prerequisites

Ed Note:  In work.  Will include prerequisites needed to participate in the trial.

Operators participating in the trial must meet the following criteria:

Only one SATCOM phone number per aircraft for ATC communications.

The aircraft satellite communication system shall be capable of displaying the call priority to the flight crew and the system will pre-empt a lower priority call.

Call security provided through a combined caller ID / call number checking procedure.

Caller ID displayed to individual receiving the call.

This does not exempt the service provider from having to meet the FAR requirements such as recording, message assurance and delivery time constraints.

Note:  When using SATCOM voice, maintain radio phraseology and discipline procedures.  Using the SATCOM phone like a regular phone can create misunderstandings and confusion.

In accordance with established PARC CWG procedures, participants must agree to provide information and data necessary to substantiate the goals of the project

The participating aircraft SATCOM voice communication system must be certified in accordance with AC 20-150, Satellite Voice Equipment as a Means for Air Traffic Services Communications, or equivalent.  The satellite data unit (SDU) meets ARINC 741.  The installations on the aircraft are integrated with the aircraft’s flight deck audio systems and the installation has been FAA approved.

# Project Administration

This project will be administered by the Performance-based Operations Aviation Rulemaking Committee's (PARC) Communications Working Group (CWG).  For further information on becoming an eligible participant in the project, contact the PARC CWG chairmen, Tom Kraft(tom.kraft@faa.gov), Arnold Oldach (aoldach@rockwellcollins.com).

Brad Cornell (Bradley.d.cornell@boeing.com) will be the coordinator for this project plan, update it, as necessary, and distribute the updates to the participants and stakeholders of this project.

The data collected and results of any analyses will be provided to the PARC CWG   The PARC CWG may rely on existing FANS Central Reporting Agency (CRA) as well as participating SATCOM radio facilities for analysis, problem resolution, and improvement to support the project.  Regardless, the PARC CWG will ensure processes are in place to secure individual confidentiality of any data or analysis.

The PARC CWG will also work together with the ISPACG sponsored SP6 HF radio working group focused on reducing HF congestion in the south pacific.

The PARC CWG will use the results from this project exclusively to substantiate the recommendations it formulates for the PARC to the FAA on the enabling criteria needed to approve the use of SATCOM as a long range communication system. Any other use that may come up during the course of this project will be negotiated by the parties involved prior to that use.

# Stakeholders

The participants in this trial are provided in Table 1.

Ed Note:  The participants in the table need to be confirmed, particularly the ANSPs listed and FAA participants.  Is GE part of this trial.  What about ICG and Inmarsat SDUs

Table 1 – Satellite Voice Project Participants

| Organization | Point of contact | email |
| --- | --- | --- |
| PARC CWG | Arnold Oldach Tom Kraft | aoldach@rockwellcollins.com tom.kraft@faa.gov |
| Aircraft Operators |  |  |
| Continental Airlines | Ron Renk Janet Weisner | ron.renk@coair.com Janet.Wiesner@coair.com |
| Continental Micronesia Airlines | Ralph Freeman Dion Johnson | ralph.freeman@coair.com dion.johnson@coair.com |
| Cargolux | Bernard Korsec | bernard.korsec@cargolux.com |
| UPS Global Network Systems | Tim Totten | ttotten@ups.com |
| Northwest | Jon Pendleton | jon.pendleton@nwa.com |
| Aircraft Manufacturers |  |  |
| Boeing Commercial Aircraft | Brad Cornell | bradley.d.cornell@boeing.com |
| Avionics Suppliers |  |  |
| ICG | Armin Jabs | ajabs@intcomgrp.com |
| Avionica | Tony Rios | arios@avionica.com |
| Air Navigation Service Providers |  |  |
| UK NATS | Dave Lacey | david.lacey@nats.co.uk |
| NAV Canada | Kevin Stevens (primary) Norm Dimock | stevens@navcanada.ca dimockn@navcanada.ca |
| Isavia (Iceland) | Leifur Hákonarson | leifur@isavia.is |
| Airways Corporation New Zealand | Paul Radford Alan London | paul.radford@airways.co.nz allan.london@airways.co.nz |
| FAA, Air Traffic Organization (ATO), Oceanic Operations | Karen Chiodini Dennis Addison (Oakland Center) Pete Hruz (New York Center) | karen.l.chiodini@faa.gov dennis.addison@faa.gov pete.hruz@faa.gov |
| Communication Service Providers |  |  |
| SITA | Eric Mehn (data) | eric.mehn@sita.aero |
| ARINC | Brian Pemberton (voice and data) Tammie Callahan (SFO) Jumper Leggio (NYC) | bpembert@arinc.com tcallaha@arinc.com gleggio@arinc.com |
| Satellite Companies |  |  |
| Inmarsat | Gary Colledge | gary_colledge@inmarsat.com |
| Iridium | Mike Meza | mike.meza@iridium.com |
| Operational Authorization and Aircraft Certification (FAA) |  |  |
| FAA, Flight Standards, Flight Technologies and Procedures Division | Robert M Tegeder (Primary) Madison Walton David Maloy Tom Parmer | robert.m.tegeder@faa.gov madison.walton@faa.gov david.maloy@faa.gov thomas.parmer@faa.gov |
| FAA, Flight Standards, Air Transportation Division | David Catey Robert M Davis (MEL Policy) | david.catey@faa.gov robert.m.davis@faa.gov |
| FAA, Aircraft Certification, Avionics Systems Branch | Dave Robinson | david.w.robinson@faa.gov |

# Success criteria

Although there are no formal performance requirements for HF communications, SATCOM voice communication performance will be evaluated using the RCP 400 specification per RTCA DO-306/EUROCAE ED-122, Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace, October 11, 2007.  See Attachment A.

# Description of project

## Operations

Initial STACOM long range communication service areas will be confined to airspace where enhanced SATCOM voice services are provided.  “Enhanced SATCOM services” means equivalent services to those provided for HF voice communications.  This means that each incoming SATCOM call would be answered by a designated SATCOM/radio operator.  SATCOM and HF calls would experience similar answer delays and have sufficient capacity to support expected demand. (Note: initially the demand for SATCOM voice services would be limited by the amount of commercial operators supporting the trial).  ARINC radio, Nav Canada, and UK NATS have all indicated a willingness to participate in the trial.

Standard voice radio phraseology used for HF and VHF will be maintained with SATCOM.  This is pre-established and should be the same as answering Selcal, always with the call sign.  Some minor changes in phraseology may be required as the operational trial progresses.  However there is no plan to have the flight crews say “SATCOM” or take any additional steps in support of long range communications over SATCOM.

## Aircraft equipage

This trial will be open to all SATCOM equipped operators that meet the requirements listed in the prerequisites section.  In general, SATCOM voice systems must be integrated into the aircraft’s communications systems and voice recorders.

# Qualification means

Data will be collected during the year long trial that will compare the quality of SATCOM voice and HF voice communication. The data will be compared against the requirements listed in RCP 400. If the SATCOM voice performance data meets the requirements listed in RCP 400 it is expected that SATCOM voice could be approved as a long range COMM system equivalent to an HF radio.

## Air traffic service facility requirements

Many ATC radio facilities provide the capability to initiate and receive SATCOM voice calls. However providing SATCOM voice radio services at a full operational level means more than simply accepting and placing a SATCOM voice call.  The necessary infrastructure will need to be developed and installed to  provide the capability to handle the expected SATCOM voice traffic demand.  For a fully operational service, the following criteria apply:

SATCOM/radio operators must have all the necessary equipment to provide SATCOM voice services equal to those provided by radio operators on HF.

- Incoming SATCOM voice calls must be automatically answered and routed to a SATCOM/radio operator with similar response times to those for HF calls.

- SATCOM voice service providers must have an auto dial capability to enable rapid call setup times.

- SATCOM/radio operators must have capabilities to exchange air traffic control information with ATC.

- Air traffic facilities must have specific SATCOM voice numbers published that are associated with current airspace sectorization (i.e. NAT, CAR, CEP, NP).

- Duplicating HF voice and VHF voice radio environments, SATCOM voice must be operated in a manner so that voice communication procedures are similar, regardless of the technology used.

- If a SATCOM/radio operator is communicating with a flight when another aircraft calls, the new caller must be able to hear the transmission already in progress and standby for a break in the traffic.

## Satellite service requirements

For a fully operational service, the following criteria apply to the satellite service: :

- a long term business plan dedicated to the provision of voice safety services

- The ground earth station must be capable of preventing unauthorized calls to aircraft.

- The satellite service must enable ATS access to the aircraft voice channel when required regardless if the flight crew are engaged in another call of a lower priority.  If passenger services are also provided on the same system used by the flight crew, passenger calls will be preempted if calls of a higher priority are placed to the flight deck.

- The SATCOM voice system must display Caller ID to the receiving party.

- The SATCOM service must use only one SATCOM phone number per aircraft for ATC communications.

Note: This SATCOM voice project will support both Iridium and INMARSAT communications.  INMARSAT systems typically support several levels of SATCOM voice communications.  Iridium systems are typically single channel or duel channel voice systems dedicated for flight deck communications.  Iridium based systems will require additional infrastructure in the Ground Earth Station to implement the necessary priority services needed for ATC communications.

## Aircraft requirements

AC 20-150 satellite voice equipment as a means for air traffic services communications

The satellite voice equipment must comply with RTCA, Inc.’s Document (RTCA/DO)-210D, Minimum Operational Performance Standards for Geosynchronous Orbit Aeronautical Mobile Satellite Services (AMSS), dated April 19, 2000.

## Project Plan Phase Definitions

- SATCOM meets the requirements outlined in the prerequisites

- End-to-end bench tests prior to operational trials

- In-service operational trials

## Deliverables

The deliverables are:

The necessary data to ensure compliance with of RCP 400 communication requirements will be collected. That data may include the following;

- Participating radio facilities will provide statistically sufficient data sample of SATCOM voice  connect times, voice quality, and percentage of lost, delayed or dropped calls.

  - For each SATCOM voice call , the data will include aircraft registration, time a voice call was initiated and the time a voice connection was established.

  - Participating operators will also collect data from flight crew on the intelligibility of SATCOM voice communications as compared to HF communications.

- The aircraft operators, controllers, communication service providers, or any participant should report any anomalies or potential problems using problem reporting procedures already in place for FANS 1/A data link operations as adapted for use in the Satellite Voice Project Plan.

It might also be possible to collect AOC dispatch call data in support of the trial.

## Initial in-service operational trial

Once PARC CWG agrees successful ground tests of the automatic answer and priority call override capabilities have been completed, operational trials will commence.  It is envisioned that the operational trial will be conducted over a one year period. During that time statistics will be collected as well as pilot and radio operator comment sheets to provide a comprehensive review of the system performance.  Similar data will be collected for HF performance during the same period.  At the conclusion of the one year trial it is anticipated that the data will show that SATCOM voice communications provide equal to or superior long range communication capability when compared to HF and therefore should be qualified as a SATCOM based Long Range Communication (SLRC).

# Project schedule

A schedule will be developed as more details from stakeholders are provided.  Key milestones prior to starting the trial will be upgrading necessary radio facilities.

# Cost of voice communications

The cost of SATCOM communications vs. the cost of HF communications is often a subject of some debate.  The goal of this project plan it to provide additional communication tools to both pilots and controllers certified to Long Range Communication requirements.  Operators and their internal standard operating procedures will dictate which communication method should be used.  During the trial and more than likely on into operational service the costs of each call will be owned by the originator

- – Criteria for Satellite Voice Project

  - DO-306/ED-122 performance requirements for RCP 400 relevant to voice communications

Note:  This attachment is currently under development.  It is intended to provide the necessary criteria for evaluating satellite voice equipment, capability and operations for routine ATC communications.  The criteria will be based on RCP 400 criteria, which was derived from operational requirements provided in the ICAO Doc 4444 for intervention and surveillance.

    - The criteria provided in this attachment are consistent with ICAO Doc 9869.  A satellite voice capability will be evaluated to provide:

- An alternative means of communication for 50 NM longitudinal, 30 NM longitudinal, and 30 NM lateral separation minima.  Per ICAO Doc 4444, these separation minima require a normal means of communication and an alternative means of communication.  While data link is required to provide the normal means of communication, a voice communication capability is still required when these separation minima are being applied.

- A normal means of communication used for intervention supporting lateral separations greater than or equal to 50 NM and existing time-based longitudinal separations, e.g., MNPS.  Per ICAO Doc 4444, these separation minima require only a normal means of communication.  While data link can be used as the normal means of communication, a voice communication capability is still required when these separation minima are being applied.

    - Table A- provides the intended uses for RCP 400.  RCP 400 is specified to support either an alternative (i.e., independent) communication means in reduced separation environments, or as a normal communication means when lateral separations are greater than or equal to 50 NM and existing time-based longitudinal separation, e.g., MNPS.

Note:  The scope of DO-306/ED-122 is for data communications.  Therefore, the allocations provided in the RCP 400 specification are only applicable only data communication technologies and would not be applicable to voice communication technologies.  However, the parameter values for the RCP types themselves should provide the basis for a performance-based specification for voice communication (other than HF voice).

    - Table A- provides the relevant RCP 400 time values when applied to a voice communication capability used to support intervention capabilities.

- The first time value provided in the ET column is the allocation for the communication transaction time per ICAO Doc 9869.  The communication transaction time is the maximum time for the completion of the operational communication transaction after which the initiator should revert to an alternative procedure.  The frequency at which the maximum time can be exceeded is specified by the continuity requirement.

- The second time value provided in the 95% column is the allocation for normal operations or the time at which 95% of all attempted transactions have been successfully completed.

Note:	As the criteria for RCP 400 applies to the intervention capability in reduced separation environments, the end-to-end monitoring need only consider voice communication transactions that are typical of an intervention capability, i.e., relatively simple clearances requiring an operational response.

Table A- – Applicability of RCP types (DO-306/ED-122, Table 5-10 (Partial))

| RCP type | Intended use | Transaction time (ET) (sec) | Continuity (C) (probability/ flight hour) | Availability (A) (probability/ flight hour) | Integrity (I) (acceptable rate/ flight hour) |
| --- | --- | --- | --- | --- | --- |
| RCP 400 | Alternative means of communication for application of 30 NM lateral separation and reduced distance-based longitudinal separation minima | 400 See paragraph 5.2.3 | 0.999 See paragraph 5.2.5 | 0.999 See paragraph 5.2.5 | 10-5 See paragraph 5.2.5 |
| RCP 400 | Normal means of communication for application of lateral separation greater than or equal to 50 NM and time-based longitudinal separation | 400 See paragraph 5.2.4 | 0.999 See paragraph 5.2.5 | 0.999 See paragraph 5.2.5 | 10-5 See paragraph 5.2.5 |
| Note: The values for parameters of RCP types have been adjusted from those provided in ICAO Doc 9869 based on the results of the operational safety and performance assessments provided in section 5. |  |  |  |  |  |

Table A- – 50 longitudinal and 30/30 - intervention (DO-306/ED-122, Table 5-6 (Partial))

| RCP type | RCP 400 |  |
| --- | --- | --- |
| Time Parameter | ET | 95% |
| Time Value | 400 | 350 |
| Note 1: Values shown in seconds. Note 2: Expiration time (ET) is at the continuity requirement, which is 99.9%. |  |  |

    - Table A- provides the time allocations for position report delivery times when applied to a surveillance capability supported by a voice communication capability.  The delivery times are specified to support surveillance capabilities for lateral separations greater than or equal to 50 NM and existing time-based longitudinal separation minima.  Per ICAO Doc 4444 and/or regional supplements, these separation minima only require position reports that are provided when the aircraft’s flight plan changes waypoint, although other types of position reports may be used.

    - Table A- provides the values assigned to the availability, continuity, and integrity parameters applicable to operational transactions associated with the communication capability used for intervention and position reporting.  These values are determined using DO-306/ED-122, Table 1-1 and the relevant safety objective(s) and qualitative probabilistic term(s) provided in DO-306/ED-122, Table 5-2.  The values for availability and continuity are defined as one minus the quantitative value assigned to the probabilistic term for the safety objectives associated with the parameter.

Note:  While these values are based on an operational safety assessment of data link services, the assessment for the RCP 400 specification was conducted in an operational context that would be applicable to either a voice or a data communication system.

  - Additional requirements

    - At the NAT SPG Technical Task Force meeting, it was further concluded that the RCP 400/D specification would include additional clarification to the 0.999 service availability requirement as follows:

- No more than 24 outages (affecting a significant portion of aircraft) greater than 20 minutes for any 12 month period;

- Failures causing outages for multiple OACs are not counted more than once; and

- No more than 9 hours of total downtime for any 12 month period.

    - This criteria should be consistently applied to an RCP 400/V specification.

    - Furthermore, it was concluded that while DO-306/ED-122 included requirements for monitoring and alerting, additional requirements were needed on the time it would take to report an outage to the OAC.  This time is referred to in the North Atlantic Region as the indication delay.  After an outage begins, indication delay is the time before the communications service provides ATC automation with a positive indication that there is an outage.  The indication delay is 10 minutes for RCP 400 operations.

Ed Note:  More work is needed on outage and restoration notifications.  During the last meeting, there was some discussion on Iridium service interruption announcements and warnings of future possible service degradation. The existing and necessary procedures may provide a starting point for developing guidance material related to outage and restoration notification to meet the “performance-based” criteria for RCP 400.  Note the criteria above are from when the outage begins to when the ANSP receives notification.  It is suggested also that if ANSPs are not notified of restoration, than that would seriously impact the overall yearly availability measurement against the 9 hour criteria for any 12-month period.

Table A- – >50 lateral and time-based longitudinal - surveillance
(DO-306/ED-122, Table 5-8 (Partial))

| Position report delivery times | Waypoint Position Report |  |
| --- | --- | --- |
| Time Parameter | ET | 95% |
| Time Value | 400 | 300 |
| Note 1: Values shown in seconds. Note 2: Expiration time (ET) is at the continuity requirement, which is 99.9%. |  |  |

Table A- – Continuity, integrity, and availability (DO-306/ED-122, Table 5-9)

| Parameter | Value See Note 1 and Note 2. | Source information |  |  |
| --- | --- | --- | --- | --- |
|  |  | Safety Objectives per Table 5-2 | Hazard/ qualitative term per Table 5-2 | Quantitative value per Table 1-1 |
| Availability of service provision for all aircraft | 0.999 [1-10-3] | SO-1 | Loss of voice communication capability (multiple aircraft, detected case) probable | 10-3 |
| Availability of an aircraft to use the service | 0.999 [1-10-3] | SO-2 | Loss of voice communication capability (single aircraft, detected case) probable | 10-3 |
| Continuity © | 0.999 [1-10-3] | SO-3 SO-4 SO-5 SO-6 | Unexpected interruption of the voice transaction (loss after initiation) probable | 10-3 |
| Integrity (I) | 10-5 | SO-7 SO-8 SO-9 SO-10 | Undetected corruption of the voice transaction remote | 10-5 |

- – Satellite Voice End-to-End Configurations

This Attachment is TBD.

  - Bench Tests

  - Operational Trials

- – Satellite Voice Performance Evaluation Methods

This Attachment is TBD.

  - Satellite Voice performance monitoring, analysis, and presentation