THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

PARC CWG Initiatives:  FANS 1/A over Iridium and Satellite Voice Communications

(Presented by United States of America)

Summary

This paper provides an update on the future air navigation system (FANS) 1/A over Iridium (FOI) and satellite voice communications work being done by the FAA-sponsored Performance-Based Operations Aviation Rulemaking Committee’s Communications Working Group (PARC CWG).

# Introduction

## At IPACG/28 FIT/15, the United States provided information (FIT/15 IP/05) on work being done by the FAA sponsored Performance-Based Operations Aviation Rulemaking Committee’s Communications Working Group (PARC CWG).  The work is related to two separate projects, the future air navigation system (FANS) 1/A over Iridium (FOI) project, and the satellite voice project (Inmarsat and Iridium).

## Since FIT/15, the PARC CWG held a meeting, 26-28 August 2008, in Tempe, Arizona, to review the progress on the FOI project and start work on a satellite voice project.

## These projects will include operational trials and, based on favorable results, the PARC CWG will develop recommendations for the FAA.  These recommendations will be in regard to the FAA policies and enabling criteria on the use of the Inmarsat and Iridium satellite networks for data and voice communication supporting air traffic management (ATM).

## The PARC CWG recognizes that global harmonization is crucial to the success of any State or regional implementation initiative.  As such, the PARC CWG prepares Coordination Drafts for broad review and solicits input on such matters of interest to the aviation community.

## This information paper provides an update on PARC CWG’s FOI and satellite voice projects and includes coordination drafts of the project plans, which can be found at Appendix A and Appendix B, respectively.

# FANS 1/A over Iridium (FOI)

## FOI Project evaluation criteria.  The PARC CWG agreed to use the draft RCP 240 specification, consistent with the North Atlantic and Asia-Pacific conclusions on communication performance requirements, as the basis for its evaluation of the Iridium satellite network supporting FANS 1/A operations.

## Iridium network availability.  Further improvements are needed to meet the RCP 240 service availability criteria of 0.9999 for operational efficiency.

## Iridium unplanned outage notification.  The timeliness in reporting unplanned outages has improved; however further work is needed.  The unplanned outage notification criterion indicates that air navigation service providers (ANSPs) are to be notified within 5 minutes of when the unplanned outage begins.  However, if the unplanned outage doesn’t last 10 minutes, do ANSPs still need to be notified?  The 10 minute criteria for outage duration is based on the time at which an ANSP would institute some procedural action, e.g., apply a different form of separation.  If the notification is provided more than 5 minutes from when the unplanned outage has already existed for 10 minutes or more, the notification at this time would be of diminished value to the ANSP.  It was also suggested that other types of notifications should be provided, such as weather conditions that are known to reduce the reliability of the Iridium system.

## Notification of restoration.  It would be helpful to receive notification of when the problem has been resolved.  More work is needed to clarify the criteria for providing notifications to ANSPs and the procedures that would include types of notifications.

## Message latency.  It was noted that perhaps further tuning of the ring alert may improve the actual latency of message delivery times.

## Data measurement method and common format for presenting data.  The communication service providers presented the proposal for data collection and presented data collection results.  The PARC CWG agreed that performance monitoring for the FOI project will be consistent with global monitoring requirements being development by the Ad-Hoc Working Group on the Global Operational Data Link Document (GOLD).  See IPACG/29 FIT/16 WP on GOLD.  The following is noted:

- Success percentages for data blocks may not represent true success.  Messages success rates provide more meaningful operational data.

- RCP 240 criteria for latency and continuity apply to all initiated transactions during the data collection period.  For example, the latency target value at 95% includes all initiated transactions, whether or not delivery was successful.

- The performance of the Iridium satellite network will continue to be improved and evaluated as part of regular data collection, review and performance tracking.  ARINC and SITA will coordinate to provide the CWG consistent presentation (e.g., format, labeling, and content) of the performance characteristics of the Iridium satellite network.

## Operational trial readiness status.  The PARC CWG recognizes the purpose of the project, and the differences between the prerequisites to start the trial and the recommended enabling criteria needed for full-up operations.  To start the trials, the Iridium satellite network message latency appears to be consistent with operational goals.  The message latency data collected between now and January 2009 will give the PARC CWG the confidence to recommend starting the trial.  The data indicates further improvements are needed on the availability, notification of outages, and notification of restoration.  The PARC CWG agreed that changes to improve the availability and number of outages of the Iridium satellite network can occur after the trial starts, and PARC CWG intends to address the issues as part of the recommended enabling criteria needed for full-up operations.

- Continental Micronesia (CMI) reported that all 9 of the CMI B737-800’s have been modified to add the necessary wiring for Iridium ACARS data link.  They are expecting to continue per schedule to have the CMI fleet equipped with FANS 1/A and FOI capable by January 2009.

- Cargolux (CLX) reported that all four 747 aircraft planned for the FOI project are currently operating with ACARS over Iridium (AOI).  The Boeing Service Bulletin to activate FANS 1 on these airplanes should be delivered to CLX in October 2008 for incorporation between October and December 2008.  The four airplanes should be FANS 1 compliant by January 2009.

- Regarding operational authorizations, FAA Flight Standards provided an overview of the updates to the FAA Notice, draft FAA AC 120-70B revision, and A-56 Ops Spec template revisions.  These materials are expected to be available by December 2008 to support operational authorizations for FOI trial.  See other IPACG/29 FIT/16 IP on operational authorizations and aircraft certification.

## FOI Project Plan.  PARC CWG updated the FOI Project Plan based on the inputs from PARC CWG/16.  Updates include mainly the incorporation of data collection, analysis and reporting methods, and schedule updates.  The FOI Project Plan can be found at Appendix A to this paper.

# Satcom Voice Project (Inmarsat and Iridium)

## Satellite voice objective.  This project is specifically investigating Inmarsat and Iridium satellite voice communication capabilities to determine their viability as an FAA-approved long range communication system (LRCS).  If satellite voice can be FAA-approved as a LRCS, the Minimum Equipment List (MEL) could be changed to allow dispatch with one satellite voice communication system and only one HF radio system, when two LRCS are normally required.  PARC CWG is looking for more than just a temporary relief; it is looking to permanently replace one HF radio system with a satellite voice system.

## Progress of work.  The project plan includes an operational trial that is based on using existing aircraft that are capable of satellite voice communications.  Areas of work are:

- Determine pre-requisite requirements for participating in the trial – based on FAA Advisory Circular AC 20-150 and ARINC 741.

- Develop recommendations for the FAA to establish the criteria for use of satellite voice communications in air traffic management operations on a routine basis, potentially leading to MEL relief on HF radios.  The success criteria will be based on criteria from previous satellite voice trials (e.g., North Atlantic) within the RCP framework (e.g., RCP 400/V specification).

- Establish requirements for data collection prior to and during the operational trials.

- Establish requirements for infrastructure (ground/satellite), aircraft satellite voice systems, procedures, including phraseology, and security – to meet the objectives of the project.

- Operators indicating their plans to participate are:

  - UPS – Tim Totten

  - Northwest – Jon Pendleton,

  - Continental Micronesia – Ralph Freeman,

  - Continental – Ron Renk.

  - Cargolux – Bernard Korsec

## Satellite voice project plan.  PARC CWG updated the Satellite Voice Project Plan based on the inputs from PARC CWG/16.  The Satellite Voice Project Plan can be found at Appendix B to this paper.

# Action

## The Group is invited to:

- note the information in this paper;

- note the next meeting (PARC CWG/17) is planned for February 3-4, 2009, hosted by ARINC, in Livermore, California.  The meeting will start on February 3 at 9:00 AM and adjourn at 5:00 pm on February 4 (2 full days); and

- contact either Arnold Oldach (aoldach@rockwellcollins.com) or Tom Kraft (tom.kraft@faa.gov) for more information on PARC CWG activities or to comment on the Coordination Drafts provided at Appendix A and Appendix B.

Appendix A – PARC CWG FANS 1/A over Iridium Project Plan – 9 September 2008

Appendix B – PARC CWG Satellite Voice Project Plan – 30 September 2008

Note:  Appendices are provided as separate files.

-END-