THE TWENTY-NINTH MEETING OF THE
INFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/29)

THE SIXTEENTH MEETING OF THE
FANS INTEROPERABILITY TEAM
(FIT/16)

(Fukuoka, Japan, October 27-31, 2008)

Performance Specification Template

(Presented by United States of America)

Summary

This paper proposes that a performance specification template be developed for specifying performance-based criteria for communication and surveillance capabilities supporting ATS functions.

# Introduction

## This paper suggests the need for a template to specify criteria for communication systems based on RCP and surveillance systems based on relevant performance characteristics.  For example, specifications, using a template, are needed for the following:

- criteria for RCP 240 when applied to data link systems (/D);

- criteria for RCP 400 when applied to data link systems (/D);

- criteria for RCP 400 when applied to “non-traditional,” (e.g., satellite) voice communication systems (/V);

- criteria for new RCP types, such as to support implementation of data link systems in continental (domestic) airspace, e.g., RCP 60/D or RCP 120/D; and

- criteria for surveillance, e.g., position reporting criteria for different separation standards.

## The intent is to provide a globally consistent way to convey performance-based criteria.

## The RCP 240/D and RCP 400/D specifications and surveillance criteria would be developed further in the GOLD using the template.  However, the template should be provided within a framework, and it may be appropriate to include in the GOLD provisions for RCP 400/V for satellite voice communications, work currently underway to develop air traffic data link services in continental airspace, and future implementation initiatives.  Guidelines for the framework and template can be found at Appendix C.

## In the longer term, performance specifications could be included as appendices to ICAO Doc 9869, Manual on Required Communication Performance (RCP).

# Discussion

Current activities

## North Atlantic (NAT) and Asia-Pacific (ASIA-PAC) regional implementation groups have developed Draft Performance Based Guidance Material for Communications material based on ICAO Doc 9869, Manual on Required Communication Performance (RCP) and RTCA DO-306/EUROCAE ED-122.  This material can be found at Appendix A and Appendix B, respectively.  While these draft materials are consistent in the criteria they specify, the presentation and format is different.

## The PARC CWG is evaluating Iridium as a viable network to support air traffic services.  It is using criteria consistent with the criteria provided at Appendix A and Appendix B to evaluate FANS 1/A over the Iridium network.  It has begun work to evaluate Satellite voice (Iridium or Inmarsat) communications for routine ATC communications, with a goal of approving this capability as a long range communication system (LRCS).  The PARC CWG recognizes the need for performance-based criteria to support a “non-traditional” voice capability without being specific to any particular technology or implementation (See IPACG/29 FIT/16 IP on PARC CWG Initiatives:  FANS 1/A over Iridium and Satellite Voice Communications).

## RTCA SC-214/EUROCAE WG-78 is developing standards for advanced data link services for air traffic services in continental (domestic) airspace.  These standards are considering a framework for specifying performance-based criteria to advance the continental data link services defined in RTCA DO-290/EUROCAE ED-122, Safety and Performance Requirements Standard for Air Traffic Data Link Services in Continental Airspace (Continental SPR Standard), first published in April 2004.  The framework was developed in accordance with the guidelines provided in DO-264/ED-78A, which was published in December 2000.  While DO-264/ED-78A is based on the ICAO concept of required communication performance (RCP), DO-290/EUROCAE ED-122 leaves part of the RCP type specification to the States.  Furthermore, the RCP concept and guidance on the implementation of RCP types has since become more mature and experience has been gained from practical applications in oceanic and remote regions.

## ICAO has made available a draft version of ICAO Doc 9869, Manual on Required Communication Performance (RCP) since September 2005.  It has recently published the First Edition, ICAO Doc 9869, dated 2008.  This Edition provides guidance for implementing performance-based communications based on RCP and provides criteria for RCP 240 and RCP 400, consistent with the criteria provided by Appendix A and Appendix B.

Review of the performance-based approach

## The performance-based approach characterizes function and performance based on operational criteria.  The operational criteria are determined from an assessment of the intended operations in all types of airspace, i.e., oceanic, en route, terminal, and airport domains, and are not based on any particular technology.

## For certain intended operations, operational criteria will need to be specified for communication and surveillance functions, taking into consideration global seamless operations.  The RCP types and other relevant operational criteria are intended to complement performance-based navigation specifications for the intended operations in any given airspace.

## It may be relevant to specify additional operational criteria to include functional capability or other performance characteristics, for example, the need for certain controller-pilot data link communications (CPDLC) messages, automatic dependent surveillance – contract (ADS C) reports, and voice communication phraseology, the need for an interactive capability of a voice communication or the air-ground integration capability of a data communication; the need for performance monitoring and alerts, indicating non-compliant performance and time to execute contingency procedures or to restore service.  Whilst RCP types provide performance characteristics, “performance-based” criteria include functional characteristics significant to the operation.

## The operational criteria should be used to determine the viability of a particular technology, qualify, certify, and approve various parts of an implementation and/or determine eligibility of an operator, and to manage performance levels needed to provide varying service levels to a fleet of aircraft with different capabilities in common airspace.

## Guidance material is needed in further detail for qualification and acceptance of data link and some voice communication services, such as Satcom voice, used for routine air traffic control communications.  The guidance material should support type design approval of the aircraft and the operational authorizations to use the communication services.  The material should include guidance on operational monitoring that may be necessary to ensure that the communication performance level continues to meet the operational criteria.

## It is noted that monitoring techniques in practice today cannot reliably predict actual operational performance trends and in some cases the actual technical performance measures are approaching or not meeting performance targets.  Other factors may need to be considered for qualification and acceptance, such as initial evaluations of system architecture, e.g., redundancy, dissimilarity, backups, configuration management, and operational and maintenance procedures.

## The performance-based approach will facilitate implementation of changes that provide new operational capabilities or employ systems using different technologies, and the change is predicated on communication and/or surveillance performance.  Some of these changes include:

- Reducing separation minimum or increasing airspace capacity that requires communication and/or surveillance capabilities and performances that are better than the communications equipment required for dispatch;

- Using available technologies for communication and/or surveillance, including those provided by commercial services, for use in air traffic management:

  - As the preferred, primary, normal, or replacement means for air traffic control (ATC) communications and/or surveillance, or

  - That may be suitable for use in some airspace, but not in others, e.g., ATN B1 aircraft may be suitable in core Europe, but not suitable in the NAT Region; or

  - When the technologies for communication and/or surveillance are mixed among the different parts of the system, e.g., the ground system is based on Future Air Navigation System 1/A (FANS 1/A) and the aircraft equipage is based on the Aeronautical Telecommunication Network (ATN).

Intended uses of performance-based criteria

## The template for performance-based specification should be designed to effectively meet the intended uses.  The intended uses of performance-based criteria are mainly to support:

- Safety oversight of air traffic service provisions and operations (See IPACG/29 FIT/16 WP on ATS Ground System SPR Compliance);

- Agreements/contractual arrangements that air navigation service providers and aircraft operators make with their respective communication service providers;

- Operational authorizations, flight crew training and qualification (See IPACG/29 FIT/16 IP on Guidance material for operational authorizations and aircraft certification of data link systems);

- Design approval of aircraft data link systems (See IPACG/29 FIT/16 IP on Guidance material for operational authorizations and aircraft certification of data link systems); and

- Operational-monitoring, analysis, and exchange of operational data among regions and states (See IPACG/29 FIT/16 WP on Monitoring FANS-1/A Performance Against Oceanic SPR Standard).

Considerations for performance specifications for communication and perfornance

## Limit RCP types and levels of surveillance performance.  While the RCP concept associates RCP types with ATS functions, it is envisaged that there would be a limited number of RCP types that would cover a wide range of performance for communication and surveillance capabilities.  The specifications for surveillance and performance should be developed so that they are not specifically coupled to any particular ATS function.  For example an RCP type that is applicable to a distance-based separation may also be applicable to a reduced time-based separation.

## Associate performance specifications to the ATS function.  The performance-based specifications for communication and surveillance together with performance-based navigation specifications, should allow for a practical balance of capability and performance of the CNS elements to attain a desired ATS function, e.g., reduced separation.  This is not to suggest that the performance-based specifications need to be closely coupled to an ATS function, in fact, it should not be, but at some the point, the specifications for CNS should be associated with an ATS function.

## Correlate voice and data performance.  In some cases, the ATS function may require a data link capability, but in cases where either voice or data can support the same ATS function, the performance criteria should be the same at the operational level, e.g., time to complete an intervention transaction and associated continuity, availability, and integrity.  However, the specification for data link performance will be different than the specification for voice communication since the allocations, which are part of the specification, will be different.

## Distinguish intervention capability from surveillance.  The criteria for communication should be separate from the criteria for surveillance.  Communication performance is characterized using RCP types based on a communication capability, e.g., controller intervention, needed to support an ATS function, e.g., reduced separation.  Surveillance performance is characterized based on a surveillance capability, e.g., position reporting, needed to support the ATS function, e.g., reduced separation.  The criteria for communication are derived differently than the criteria for surveillance.  The criteria may also be applied differently, e.g., surveillance criteria may be applied to an operation involving aircraft with only ADS-C or the flight management system waypoint position reporting (FMS WPR) applications.

## Parameters and units of measure.  While values for parameters of RCP type are specified in term of per flight hour, additional criteria should be provided to clarify the units of measure for the data collected:

- Time is measured in seconds and is associated with the continuity requirement (time at which an alternative procedure is executed, or expiration time); and 95% value (nominal).

- Continuity is a probability and whether measured on a per flight hour basis or by measuring the percentage of a population of transactions over a period of time in an airspace, the result will be the same.  It may be necessary, however, to measure continuity on a per aircraft type basis, or a per operator basis, or even on a tail number basis, to meet the intended uses of the data collected.

- Service availability can be measured against a specification that includes the following:

  - Duration limit of an unplanned outage and maximum number of outages exceeding the limit allowed over a specified period of time, e.g., No more than 4 outages (affecting a significant portion of aircraft) greater than 10 minutes for any 12 month period

  - Acceptable coverage criteria for an unplanned outage, e.g., Failures causing outages for multiple Oceanic ATS Centers (OACs) are not counted more than once;

  - Maximum allowable outage time over a specified period of time based on a 24/7 operation, e.g., No more than 50 minutes of total downtime for any 12 month period, which is approximately equivalent to a 0.9999 service availability requirement.  The operational service availability would be measured against this criterion by accumulating the total unplanned outage time determined from when the unplanned outage occurred to when the ATS facility received notification that the service has been restored.

- Aircraft system availability.  Criteria for availability of the aircraft data link equipage and capability should be specified to support design approval.

- Integrity is typically demonstrated by analysis, qualification, and approval of the system components through a variety of practices and techniques prior to operational provision and use, e.g., system architecture, monitoring and alerting, design considerations, software development assurance, etc.  Criteria should be provided in the form of procedural, functional, safety, and performance requirements and process controls.

- Other functional and performance metrics may need to be specified, for example, phraseology or message set, and monitoring and alerting, as discussed in paragraph 2.7.

## Points of measurement.  The specification should clearly define points at which practical measurements can be made or reasonably estimated, where necessary.

## Allocate according to institutional boundaries.  Allocations should be directed to the different system components consistent with approval mechanisms and/or along institutional boundaries.  For example, RCP allocations should sufficiently support operational authorizations, design approval of aircraft data link system, the communication service provision, and safety oversight of the ANSP’s air traffic service provision.  The allocations should be provided in a form that enables appropriate parties to show that the end-to-end system complies with operational criteria and to show that system components comply with relevant allocations:

- Operational monitoring.  For data link capability, RCP allocations should be established to support operational monitoring, such as the transaction times (TRN/ACP), technical performance (RCTP/ACTP), and operational response times (Responder/PORT).

- “Non-traditional” voice communication capability.  Allocations should be established based on operational criteria consistent with that criteria used for data link when the same ATS function is being considered.

- Technical performance.  Allocations should be established for aircraft data link systems, communication services, and automation within an ATS facility.

Note:  It is recognized that current allocations in the RCP 240/D and RCP 400/D specifications are based on an arithmetic sum of the system component time allocations at the continuity requirement and 95%.  Some discussions are currently underway to use statistical summation techniques to determine the allocations.  This paper does not address this issue as it is only intended to provide a template for specifying allocations once they have been determined and rationalized.

## RCP 240 Specification.  A coordination draft of the RCP 240 specification can be found at Appendix D.  This specification takes into account the considerations provided in paragraph 2.13 through paragraph 2.19, the material provided at Appendix A and Appendix B, and RTCA DO-306/ED-122.

# Action

## The Group is invited to:

- Note the information in this paper;

- Agree to the concept of developing a template and performance specifications for global use in specifying criteria that specifies the functional and performance characteristics of communication and surveillance capabilities supporting an ATS function; and

- Consider this paper as a starting point for the GOLD Ad Hoc Working Group to progress its development and coordination (See IPACG/29 FIT/16 WP on Global Operational Data Link Document (GOLD)).  Guidelines to progress this work can be found at Appendix C.

Appendices

Appendix A Summary of Discussions and Conclusions of the North Atlantic Systems Planning Group Technical Task Force on Data Link Applications Communications Requirements (NAT RCP Task Force), Appendix C, Paris, 20 to 22 February 2008

Appendix B ISPACG/22 FIT/15 WP/04, Appendix A, Updates required to the FOM in order to adopt Oceanic SPR standards, as agreed at Papeete, Tahiti, 11-12 March 2008

Appendix C Guidelines for a framework and template for performance specifications for communications and surveillance

Appendix D	RCP 240 Specification – Coordination Draft 18-Oct-08

Appendix A	Summary of Discussions and Conclusions of the North Atlantic Systems Planning Group Technical Task Force on Data Link Applications Communications Requirements (NAT RCP Task Force), Appendix C, Paris, 20 to 22 February 2008

APPENDIX C - Draft PERFORMANCE BASED GUIDANCE MATERIAL FOR COMMUNICATIONS

1	Application

Each State should ensure that the ANSPs provide communication services that meet the RCP type, and that contracted CSPs meet their RCP allocations for each oceanic area control centre (OACC) and the flights it serves. The risks represented by the requirements are regarded as being minimum for the specified ATS function to maintain operational efficiency and meets the safety needs.

2	Purpose

2.1	The requirements herein are intended to support operational and safety requirements as traffic grows, and as horizontal separation minima are reduced. Reduced longitudinal separations will require air traffic control to become more tactical, supported by enhanced intervention capability. The performance requirements will be subject to validation and change as safety risks are further analyzed, as monitoring is enhanced, and as operational experience is gained.

2.2	The requirements herein are intended to provide meaningful benchmarks for reference to safety oversight, initial qualification, and continued operational performance monitoring.

3	Relationship to standards documents

3.1	The requirements herein are based on the RTCA DO-306/EUROCAE ED-122 Safety and Performance Standard for Air TrafficData Link Services in Oceanic and Remote Airspace (Oceanic SPR Standard), which includes a supporting safety and performance assessments for existing separation standards provided in ICAO Doc 4444..

3.2	Whereas DO 306/ED 122 specifies an Availability value based on safety assessment of the operational effects of the loss of the service, the Availability requirement herein is more stringent, based on an additional need to maintain orderly and efficient operations.

3.3	Whereas the DO 306/ED 122 specifies a requirement to indicate loss of the service, an additional time value was associated with the requirement to indicate the loss to the ANSP.

4	Performance parameters and meanings

The following are RCP parameter definitions taken from DO 306/ED 122, which are consistent with the ICAO Doc 9869, Manual On Required Communications Performance, and augmented by derived meanings that pertain to the different parts of the end-to-end service.

4.1	Meanings for communication supporting intervention and surveillance

Intervention - operational communication transaction - The process a human uses to initiate the transmission of an instruction, clearance, flight information, and/or request, and is completed when that human is confident that the transaction is complete. (ICAO Doc 9869)

Surveillance – position report delivery

- Periodic report, from the start of the periodic interval. The start of the periodic interval occurs when the periodic report is sent by the aircraft/flight crew;

- Waypoint change event report, from the actual time the aircraft crosses the waypoint or is abeam the waypoint;

- Lateral deviation event report, from the time the aircraft system detects that the event has occurred

- Vertical deviation event report, from the time the aircraft system detects that the event has occurred

4.2	Communication transaction time - The maximum time for the completion of the operational communication transaction after which the initiator should revert to an alternative procedure.

Position report delivery time – The maximum time for the delivery of a position report from the aircraft to the ANSP.

Meanings for the end-to-end service -

- Monitored operational performance (TRN) - The portion of the operational communication transaction (used for intervention) that does not include message composition or recognition of the operational response.

- Required Communication Technical Performance (RCTP) – The technical portion of the operational communication transaction (used for intervention) that does not include message composition, operational response, and recognition of the operational response times.

Meanings for the aircraft –

- RCTPAIR for Intervention – The critical transit times for an ATC intervention message from the aircraft’s antenna to the flight crew’s indication of receipt of the message and from sending the message to the aircraft’s antenna.

- RCTPAIR for Surveillance – The critical transit time for a position report from the aircraft’s avionics to the antenna.

Meanings for communications service –

- RCTPCSP for Intervention – The summed critical transit times for an ATC intervention message and a response message, allocated to the CSPs.

- RCTPCSP for Surveillance – The critical transit time for a position report allocated to the CSPs.

4.3	Continuity - The probability that an operational communication transaction or position report delivery can be completed within the communication transaction time.

Meanings for the end-to-end service -

- The proportion of intervention messages and responses that can be delivered within the specified TRN for Intervention.

- The proportion of intervention messages and responses that can be delivered within the specified RCTP for Intervention.

Meanings for the aircraft -

- The proportion of intervention messages and responses that can be delivered within the specified RCTPAIR for Intervention.

- The proportion of position reports that can be delivered within the specified RCTPAIR for Surveillance.

Meanings for communications service –

- The proportion of intervention messages and responses that can be delivered within the specified RCTPCSP for Intervention.

- The proportion of position reports that can be delivered within the specified RCTPCSP for surveillance.

4.4	Availability – The probability that an operational communication transaction or position report delivery can be initiated when needed.

Meaning for communications service – Total outage proportion of communications service for any given observation period. An outage is an interval during which a communications service fault prevents the Continuity requirement from being met or service from being initiated, affecting multiple aircraft.

4.5	Integrity - The probability of one or more undetected errors in a completed communication transaction or position report delivery.  (Modified for clarity from ICAO Doc 9869).

5	RCP 400/D Specification

Per DO-306/ED-122, this specification is applicable to data communication.

The end-to-end service shall meet or better these performance parameter values

| RCP type | RCP 400 |  |
| --- | --- | --- |
| TRN | 370 | 320 |
| RCTP | 310 | 260 |
| Continuity | 0.999 | 0.95 |
| Integrity | 10-5 |  |

The aircraft shall meet or better these performance parameter values:

| RCP type | RCP 400 |  |
| --- | --- | --- |
| RCTPAIR for intervention ..(seconds antenna-HMI-antenna] | 15 | 10 |
| RCTPAIR for surveillance (seconds avionics-antenna] | 30 | 15 |
| Continuity | 0.999 | 0.95 |
| Aircraft equipage availability | 0.999 |  |

The communications service shall meet or better these performance parameter values:

| RCP type | RCP 400 |  |
| --- | --- | --- |
| RCTPCSP for intervention [seconds ATC-aircraft-ATC] | 280 | 240 |
| RCTPCSP for surveillance [seconds aircraft-ATC] | 340 | 270 |
| Continuity | 0.999 | 0.95 |
| Service Availability1 | 0.999 |  |
| Mean time between failures2 (MTBF) [days] | 15 |  |
| Maximum outage [minutes] | 30 |  |
| Outage indication delay 3 [minutes] | 10 |  |

Notes:

1 – Service Availability of 0.999 implies no more than 9 hours of total outage time in any 12-month period.

2 – A failure is any outage of more than 20 minutes affecting 5 or more aircraft within an OAC’s airspace.  Failures causing outages for multiple OACs are not counted more than once.

MTBF of 15 days implies no more than 24 failures in any 12-month period.

3 – After an outage begins, indication delay is the time before the communications service provides ATC automation with a positive indication that there is an outage.

6	RCP 240/D Specification

Per DO-306/ED-122, this specification is applicable to data communication.

The end-to-end service shall meet or better these performance parameter values:

| RCP type | RCP 240 |  |
| --- | --- | --- |
| TRN ..[seconds ATC HMI-aircraft-ATC HMI] | 210 | 180 |
| RCTP --[seconds ATC HMI-aircraft HMI & aircraft HMI-ATC HMI] | 150 | 120 |
| Continuity | 0.999 | 0.95 |
| Integrity | 10-5 |  |

The aircraft shall meet or better these performance parameter values:

| RCP type | RCP 240 |  |
| --- | --- | --- |
| RCTPAIR for intervention ..(seconds antenna-HMI-antenna] | 15 | 10 |
| RCTPAIR for surveillance (seconds avionics-antenna] | 5 | 3 |
| Continuity | 0.999 | 0.95 |
| Aircraft equipage availability | 0.999 |  |

The communications service shall meet or better these performance parameter values:

| RCP type | RCP 240 |  |
| --- | --- | --- |
| RCTPCSP for Intervention [seconds ATC-aircraft-ATC] | 120 | 100 |
| RCTPCSP for surveillance [seconds aircraft-ATC] | 170 | 84 |
| Continuity | 0.999 | 0.95 |
| Service Availability1 | 0.9999 |  |
| Mean time between failures2 (MTBF) [days] | 90 |  |
| Maximum outage [minutes] | 15 |  |
| Outage indication delay3 [minutes] | 5 |  |

Notes:

1 – Service Availability of 0.9999 implies no more than 50 minutes of total outage time in any 12-month period for orderly and efficient operations.  DO-306/ED-122 requires 0.999 for safety.

2 – A failure is any outage of more than 10 minutes affecting 5 or more aircraft within an OAC’s airspace.  Failures causing outages for multiple OACs are not counted more than once.  MTBF of 90 days implies no more than four failures in any 12-month period.

3 – After an outage begins, indication delay is the time before the communications service provides ATC automation with a positive indication that there is an outage.

7.	Monitoring and alerting

While aircraft, operators and air traffic service provision may be qualified for a specific RCP type operation, failures may occur which may cause degradation in the performance of the service to something below that which is required by the intended operation.  In such cases, real time monitoring and alerting may be necessary to provide indication to the flight crew and/or controller.  The monitoring and alerting criteria are yet to be defined.

8.	Applicability of RCP specifications

8.1	Applicability to data link communications

Per DO-306/ED-122RCP types, in conjunction with suitable navigation performance, are deemed to match the needs for safe, orderly and efficient operations as follows.

| RCP type | Satisfies requirements as |
| --- | --- |
| RCP 240 | Normal means of communication for application of 30 NM lateral separation and reduced longitudinal separation minima |
| RCP 400 | Alternative means of communication for application of 30 NM lateral separation and reduced longitudinal separation minima |
| RCP 400 | Normal means of communication for application of lateral separation greater than or equal to 50 NM and time-based longitudinal separation |

8.2	Applicability to voice communications.

End-to-end voice communications performance required for a given ATC application would be the same as for data link in the same application. However, one link in the end-to-end chain would be very different: Instead of the CSP domain for data link there would be the pilot and ground voice operator and their data displays and keyboard interfaces to their avionics and ATC automation. Regarding RCP 240 it would not be possible for a human-to-human-to-machine link to meet the CSP performance requirements. In fact the same is true for RCP 400, which in order to support safety improvements specifies integrity and availability well above the level of existing voice communications.

8.3	Fallback from data link to voice communications

In evaluating the suitability of voice as a fallback arrangement for a data link service outage it must be borne in mind that controllers might be unable to cope with the workload imposed by DCPC voice – data link and third-party voice share the characteristic of presenting data in a format lending itself to processing by FDPSs without any controller intervention.

From a practical point of view a suitable backup for data link communications arrangement may therefore be required to be either voice through a third-party (implying a need to retain sufficient voice communications operators to carry traffic during failures) or an independent backup data link system.

Appendix B	ISPACG/22 FIT/15 WP/04, Appendix A, Updates required to the FOM in order to adopt Oceanic SPR standards, as agreed at Papeete, Tahiti, 11-12 March 2008

Appendix A : Updates required to the FOM in order to adopt Oceanic SPR standards

Para 3.3, References

Add new reference for Oceanic SPR standard.

| Id | Name of the document | Reference | Date | Origin | Domain |
| --- | --- | --- | --- | --- | --- |
| 15 | Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace, (Oceanic SPR Standard) | RTCA DO-306/ EUROCAE ED-122 | October 11, 2007 | RTCA Eurocae | CPDLC ADS AFN |

Para 3.4, System Performance Criteria

Replace existing paragraph with new paragraph as follows:

RTCA DO-306/EUROCAE ED-122 Safety and Performance Standard for Air Traffic Data link Services in Oceanic and Remote Airspace (Oceanic SPR Standard) contains the safety and performance requirements for data link services that need to be met and verified.

Note:	The Oceanic SPR standard provides an availability requirement for safety of 0.999, however to enable operational efficiency in some environments the FANS1/A availability requirement is set at 0.9999. This 0.9999 availability requirement translates on a per ATSP basis to:

- No more than 4 outages (affecting a significant portion of aircraft) greater than 10 minutes for any 12 month period;

- Failures causing outages for multiple OACs are not counted more than once; and

- No more than 50 minutes of total downtime for any 12 month period.

The table below summarizes the Oceanic SPR Standard requirements.

| Performance Criteria | Definition | Values |
| --- | --- | --- |
| RCP 240/D | Normal means of communication for application of 30 NM lateral separation and reduced distance-based longitudinal separation minima | Communication Transaction time (ET) 240 (sec) Note: Communication Transaction time is defined as the maximum time for the completion of an operational transaction after which the initiator reverts to an alternative procedure. (ICAO Doc 8689) |
| RCP 400/D | Normal means of communication for application of lateral separation greater than or equal to 50 NM and time-based longitudinal separation. Alternative means of communication for application of 30 NM lateral separation and reduced distance-based longitudinal separation minima | Communication Transaction time (ET) 400 (sec) |
| Surveillance 50nm Longitudinal 30nm Longitudinal 30nm Lateral | Normal Surveillance: (position report delivery) Non-normal Surveillance: (Controller initiated position report request) | ET 180 (sec) ET 240 (sec) |
| Surveillance >50nm Lateral >=10mins time based | Normal Surveillance | ET 400 (sec) |
| Service Availability RCP 240 Surveillance 50 nm Longitudinal 30nm Longitudinal 30 nm Lateral | The probability that an operational communication transaction can be initiated when needed (ICAO Doc 8689) | 99.99% |
| Service Availability RCP 400 Surveillance >50 nm Lateral >=10mins time based | The probability that an operational communication transaction can be initiated when needed (ICAO Doc 8689) | 99.9% |
| Continuity | The probability that an operational communication transaction can be completed within the communication transaction time (ICAO Doc 9869) | 99.9% |
| Integrity | The probability of one or more undetected errors in a completed communication transaction. | 10-5/hour |

Appendix C	Guidelines for a framework and template for performance specifications for communications and surveillance

# Framework of performance specifications for communication and surveillance

## Consider expanding the scope of ICAO Doc 9869, Manual on RCP to address general application of performance specifications for communication to ATS function and operational capability.  The specifications would include details for the RCP types, and also other relevant factors.  Provide guidelines and a template suitable for developing a particular specification, e.g. RCP 240, specific to a data communication capability or a “non-traditional” voice communication capability.  Guidelines should be developed to address conventions used in framework for providing each particular specification, e.g., /D for data link, /V for voice.

## Appendices to the Manual on RCP

- RCP 240 Specification

  - RCP 240/D applicable to data link system, e.g., CPDLC over Satcom/VHF

  - …

- RCP 400 Specification

  - RCP 400/D applicable to data link system, e.g., CPDLC over HFDL

  - RCP 400/V applicable to non-traditional voice communication, e.g., Satcom voice

  - …

- RCP [tbd] Specification

  - RCP [tbd]/D applicable to data link system planned for continental (domestic) airspace

## Consideration should be give to progress surveillance performance, separate from RCP types, to consider ADS-B, ADS-C, Radar, etc.  Whether it be done in ICAO Doc 9869 or elsewhere, performance specifications for surveillance should consider how to characterize surveillance performance and varying levels of surveillance performance, for example:

- Required surveillance performance (type /180)

  - Type /180D1, e.g., applicable to FMS WPR over Satcom/VHF;

  - Type /180D2, E.g., ADS-C only over Satcom/VHF

  - …

- Required surveillance performance (type /400)

  - Type /400D1, e.g., applicable to FMS WPR over Satcom/VHF;

  - Type /400D2, e.g., ADS-C only over Satcom/VHF

  - Type /400V, e.g., Satcom voice

  - …

# Template for performance specifications

## The template for performance specifications for communication and surveillance should take into account:

- Considerations provided in paragraph 2.13 through paragraph 2.19 of this paper;

- Performance-based guidance material provided at Appendix A and Appendix B.

-END-