| ## Request For Change Form | RFC Nr: | 08-007 |
| --- | --- | --- |

To be used whenever requesting a change to any part of FOM.  This form may be photocopied as required.

| 1. SUBJECT: |  | Oceanic SPR standard |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. REASON FOR CHANGE: |  |  |  | Align FOM to DO-306 Oceanic Safety And Peformance Standard |  |  |  |  |  |
| 3. DESCRIPTION OF PROPOSAL: [attach additional pages if necessary] |  |  |  |  |  |  |  |  |  |
| Para 3.3 References Add new reference for Oceanic SPR standard. Id; Name of the document; Reference; Date; Origin; Domain. 15; Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace, (Oceanic SPR Standard); RTCA DO-306/ EUROCAE ED-122; October 11, 2007; RTCA/ Eurocae; CPDLC ADS AFN. Para 3.4 System Performance Criteria Replace existing paragraph with new paragraph as follows: RTCA DO-306/EUROCAE ED-122 Safety and Performance Standard for Air Traffic Datalink Services in Oceanic and Remote Airspace (Oceanic SPR Standard) contains the safety and performance requirements for datalink services that need to be met and verified. Note: The Oceanic SPR standard provides an availability requirement for safety of 0.999, however to enable operational efficiency in some environments the FANS1/A availability requirement is set at 0.9999. This 0.9999 availability requirement translates on a per ATSP basis to: - No more than 4 outages (affecting a significant portion of aircraft) greater than 10 minutes for any 12 month period; - Failures causing outages for multiple OACs are not counted more than once; and - No more than 50 minutes of total downtime for any 12 month period. - The table below summarizes the Oceanic SPR Standard requirements. Performance Criteria; Definition; Values. RCP 240/D; Normal means of communication for application of 30 NM lateral separation and reduced distance-based longitudinal separation minima; Communication Transaction time (ET) 240 (sec) Note: Communication Transaction time is defined as the maximum time for the completion of an operational transaction after which the initiator reverts to an alternative procedure. (ICAO Doc 8689). RCP400/D; Normal means of communication for application of lateral separation greater than or equal to 50 NM and time-based longitudinal separation. Alternative means of communication for application of 30 NM lateral separation and reduced distance-based longitudinal separation minima; Communication Transaction time (ET) 400 (sec). Surveillance 50nm Longitudinal 30nm Longitudinal 30nm Lateral; Normal Surveillance: (position report delivery) Non-normal Surveillance: (Controller initiated position report request); ET 180 (sec) ET 240 (sec). Surveillance >50nm Lateral >=10mins time based; Normal Surveillance; ET 400 (sec). Availability; The probability that an operational communication transaction can be initiated when needed (ICAO Doc 8689); 99.99%. Continuity; The probability that an operational communication transaction can be completed within the communication transaction time (ICAO Doc 9869); 99.9%. Integrity; The probability of one or more undetected errors in a completed communication transaction.; 10-5/hour. |
| 4. REFERENCE(S): |  |  | FOM 3.3 and 3.4 |  |  |  |  |  |  |
| 5. PERSON INITIATING: |  |  | Paul Radford |  |  |  | DATE: | 31.3.2008 |  |
| ORGANISATION: |  |  | Airways New Zealand |  |  |  |  |  |  |
| TEL/FAX/EMAIL: |  |  | paul.radford@airways.co.nz |  |  |  |  |  |  |
| 6. CONSULTATION |  |  | RESPONSE DUE BY DATE: |  |  |  |  |  |  |
|  | Organisation |  | Name |  | Agree/Disagree |  |  |  | Date |
|  | ISPACG/ISPACG FIT |  | Accepted by ISPACG/FIT15 and ISPACG 22 |  | Agree |  |  |  | 13.3.08 |
| 7. ACTION REQUIRED: |  |  |  |  |  |  |  |  |  |
| 8. FOM EDITOR |  |  |  |  |  | DATE REC’D: |  |  |  |
| 9. FEEDBACK PASSED |  |  |  |  |  | DATE: |  |  |  |