Ad Hoc Working Group on 
Global Operational Data Link Document (GOLD)

CPDLC Message Set Guidelines

Summary

This paper proposes guidelines for presenting the CPDLC message set and standardized/preformatted free text in the GOLD, and seeks comments and feedback on the guidelines and a way forward.  The guidelines were based on a comparative assessment of the CPDLC application against ICAO, industry, and regional documents.

This paper addresses comments 016, 017, and 029.

# Introduction

## Comments 016 and 017 suggest to review the CPLDC message set and preformatted/standardized free text in the FANS 1/A Operations Manual (FOM), paragraphs 5.9 and paragraph 5.10, against DO-306/ED-122, replace it with a reference to DO-306/ED-122, Annex A, and, as necessary, identify differences.

## In the NAT GM, paragraph 6.7, paragraph 7, and paragraph 8 maintain a similar CPDLC message set and preformatted/standardized free text.

## Comment 029 suggests to replace text in the FOM, paragraph 9, which addresses CPDLC implementation in continental airspace, with a reference to the appropriate standards for definition and requirements for continental CPDLC implementation.

## To address comments 16, 17, and 29, a comparative assessment was conducted and some guidelines have been formulated that are proposed to be used to formulate the text for the CPDLC message set and preformatted/standardized free text in the GOLD.

## This paper provides a summary of the comparative assessment results, proposes guidelines and considerations for the CPDLC message set and standardized/preformatted free text in the GOLD, and seeks comments and feedback on the guidelines and a way forward.

# Summary of comparative assessment results

## A comparative assessment of the CPDLC message set was conducted against the following documents related to air traffic data link services in oceanic and remote airspace:

- ICAO Doc 4444, 14th and 15th Editions, Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM), applicable date 27 November 2003.

- Asia Pacific FANS 1/A Operations Manual, Version 5, dated August 2007.

- North Atlantic Guidance Material for ATS Data Link Services in North Atlantic Airspace, version 17, dated June 2008.

- RTCA DO-306/EUROCAE ED-122, Safety and Performance Standard for Air Traffic Data Link Services in Oceanic and Remote Airspace (Oceanic SPR Standard), dated 11 October 2007.

- RTCA DO-258A/ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications (FANS 1/A INTEROP Standard), dated 7 April 2005.

## Additionally, the CPDLC comparative assessment included the air traffic data link services planned for continental airspace, as defined in the following documents:

- RTCA DO-290/EUROCAE ED-120, Change 1 and Change 2, Safety and Performance Requirements Standard for Air Traffic Data Link Services in Continental Airspace (Continental SPR Standard), dated 26 June 2007.

- RTCA DO-280B/ED-110B, Interoperability Requirements Standard for ATN Baseline 1 (ATN B1 INTEROP Standard), dated 26 June 2007.

- RTCA DO-305/EUROCAE ED-154, Future Air Navigation System 1/A – Aeronautical Telecommunication Network Interoperability Standard (FANS 1/A – ATN INTEROP Standard), dated 26 June 2007.

## The results of the comparative assessment can be found in the CPDLC Reference Guide, currently dated 5 July 2008, provided with this paper or it is available for download on the GOLD Ad Hoc WG website at:

https://ksn.faa.gov/km/avr/parc/parc/parcmem/communicWG/GOLD/default.aspx

## The following summarizes the results of the assessment:

- Revised message elements.  When the intent/use is the same, the PANS/ATM message elements are different than what is implemented, i.e., FANS 1/A message set.  These message elements can be found at Attachment A.

- Revised response attribute to message element.  The response attribute for some PANS/ATM message elements is different than the response attribute for message elements that are implemented, i.e., FANS 1/A message set.

  - Response attribute is N in PANS/ATM, NE in FANS 1/A:  Responses/ Acknowledgments (uplink):  UM0 through UM5; System Management Messages (uplink): UM159 through UM167.

  - Response attribute is W/U in PANS/ATM, R in FANS 1/A:  Report/Confirmation Requests (uplink): UM127-UM130, and UM180.

  - Response attribute is Y in PANS/ATM, NE in FANS 1/A:  Report/Confirmation Requests (uplink):  UM131 through UM147, and UM181; Negotiation Requests (uplink):  UM148, UM151, and UM152.

  - Response attribute is N in PANS/ATM, R in FANS 1/A:  UM157 {CHECK STUCK MICROPHONE [frequency]).

  - Response attribute is Y in PANS/ATM, N in FANS 1/A:  Emergency Messages (downlink):  DM55 through DM61, and DM80.

  - In some cases, when free text is used to mimic a PANS/ATM message element, the response attribute is different.  FANS 1/A generally uses UM169 which has a response attribute of R, and DM67, which has a response attribute of N.  Sometimes free text is used to mimic a PANS/ATM message element and the response attribute is different then the response attribute associated with the free text message element.

- Unsupported message elements.  Unsupported message elements are based on the FANS 1/A message set and use of free text to mimic PANS/ATM, edition 15, message elements.  These message elements can be found at Attachment B and take the following form:

  - Reserved message elements (R).  In edition 15 of PANS/ATM, some message elements were removed from edition 14.  These message elements are in the FANS 1/A message set, yet some of these message elements are still shown in either the NAT GM or the FOM as supported message elements.  DO-306/ED-122 shows which messages these are and indicates “Avoid Use.”  Also, some message elements were reserved prior to edition 15 and indicate being supported in the NAT GM and the FOM.

  - Unsafe (U, U(R), or D 17).  In the NAT GM, these messages are listed in paragraph 6.7.  Some message elements are listed as unsafe, but are used by Reykjavik (U(R)).  Other messages are listed in both paragraph 6.7 and paragraph 8 (D 17).  It is assumed that these are “unsupported” message elements.

  - Avoid Use (AU or NU).  In the NAT GM, these messages are annotated in paragraph 8 as “avoid use” (AU) or “not used” (NU).  In DO-306/ED-122, these messages are annotated in Annex A as “avoid use” and are correlated to message elements that are either reserved in PANS/ATM, edition 15, or annotated as “avoid use” in the FOM, version 4.  DO-306/ED-122 did not consider the NAT GM because at the time, phase IV CPDLC was still in the planning stages.

  - Not listed (NL,  NL(R), or S 17).  In the NAT GM, some messages elements are not listed (NL).  Some message elements are not listed but used on a trial basis by Reykjavik (NL(R).  Some message elements were struck through in version 17 (S 17).  It is assumed that these message elements in all cases are “unsupported” message elements.  In the FOM, DM69 REQUEST VMC DESCENT is the only message element in the FANS 1/A message set that is not listed.

Note:	In the NAT GM, paragraph 5.20 provides guidance for the treatment of unsupported message elements.  This guidance is inconsistent with paragraph 7, which includes a variety of free text messages used by Reykjavik, to respond to unsupported messages.

- Standardized/preformatted free text.  In the NAT GM, paragraph 7 appears to present only UM 167 preformatted/standardized free text messages.  For DM 67, preformatted free text messages are embedded in paragraph 8 together with the standard message elements.  The FOM lists some standardized (not necessarily pre-formatted) downlink messages mainly as responses to standardized/preformatted uplink messages or to mimic standardized CPDLC message elements defined in Doc 4444 (PANS/ATM).  Standardized/preformatted free text uplink and downlink messages from all sources can be found in the CPDLC Reference Guide, Table 2-3 and Table 2-4, respectively.

- Guidance material on use of free text.  An assessment of the guidance material provided in the DO-306/ED-122, Annex A, paragraph A.2.3, the NAT GM, paragraph 5.14, and the FOM, paragraphs 5.3 and 5.7 can be found at Attachment C.

- Guidance material on the use of emergency messages.  An assessment of the guidance material provided in the DO-306/ED-122, Annex A, paragraph A.2.2, the NAT GM, paragraph 5.19, and the FOM, paragraph 7.1 can be found at Attachment D.

# CPDLC Message Set Guidelines for the GOLD

## Message elements are based on ICAO Doc 4444 (PANS/ATM), edition 14, Appendix 5 for the columns Message ID, Intent/Use, Message Element, and Response Attribute columns.  Urgency and Alert Attribute columns are not tracked.  The same order and groupings are retained.

## Changes – resulting from edition 15 – to message elements for the Message ID, Intent/Use, Message Element, and Response Attribute columns are annotated.

## Changes (clarifications) – provided by the NAT GM or FOM – to the “intended use” column are annotated.

## The DO-258A/ED-100A (FANS 1/A INTEROP Standard) message is correlated to the ICAO Doc 4444 message set, edition 14 with edition 15 changes annotated.  Differences in the content of the message element and response attribute are annotated.

## Standardized/preformatted free text messages are defined.

- Preformatted free text messages are free text messages that are available to the controller/flight crew via automation.

- Standardized free text messages are free text messages that are defined in a Standard and procedurally controlled, i.e., manually entered by the flight crew/controller.

## DO-306/ED-122 provides a message set that meets the guidelines provided in paragraphs 3.1, 3.2, 3.3, 3.4, and 3.5, with minor exceptions owing to some minor errata and new free text messages that were added to the FOM since the time of publication.  The NAT GM was not considered since the CPDLC Phase IV message set was still under investigation at the time.

Note:	The scope of DO-306 was based on the FANS 1/A message set and free text that was used to mimicked standardized message elements since FANS 1/A implementation.  While the post-OPLINKP/1 (pre-edition 15) message set is included, without the specific message reference number (MRN) assigned, message elements that are not provided in FANS 1/A are annotated as N/A, unless the message element was mimicked using free text.

## Guidance on use of free text is provided based on DO-306/ED-122, and augmented as necessary (See Attachment C).

## Guidance on the use of emergency messages is provided based on DO-306/ED-122, and augmented as necessary (See Attachment D).

## Guidance on the treatment of unsupported message elements is provided taking into consideration the NAT GM, paragraph 5.20, and augmented as necessary.

## Messages that are not supported in any particular region are annotated as a regional supplement only as absolutely necessary.  Downlink messages that are not supported fall into two categories:

- Messages that exist in the implementation but are not supported procedurally or inhibited from use via automation.  Downlink messages of these type receive an uplink message, UM162 or standardized free text, indicating the message is not supported.

- Messages that exist in Doc 4444 and DO-306/ED-122, but are not implemented.

## Message elements, including use of free text that mimic PANS/ATM message elements, that are supported in the regions, but used in a different way, are annotated as a regional supplement only as absolutely necessary.

# Other considerations

## In the FOM, paragraph 9 addresses CPDLC implementation in continental airspace.  However, it needs to be updated based on the standards in paragraph 2.2 of this paper.

## Additionally, the ICAO EUR/NAT Region Data Link Steering Group (DLSG) and ATN Accommodation Drafting Group (ADG) developed a data link harmonization strategy for the ATN and FANS 1/A technologies, specifically to support:

- North Atlantic Region data link implementation based on FANS 1/A, and

- Eurocontrol LINK 2000+ data link implementation based on ATN;

## The Eurocontrol LINK 2000+ is based on the standards identified in paragraph 2.2 of this paper.  LINK 2000+ includes all the CPDLC message elements supporting only the ATC communications management (ACM), ATC clearance (ACL), and ATC microphone check (AMC) data link services.

## The DLSG/ADG concluded and reported to the NAT SPG meeting in June this year that an aircraft data link system built to the Link 2000+ standards is not viable for oceanic data link operations.

## The CPDLC comparative assessment included a correlation of these standards – only for the messages that are supported by LINK 2000+ – against the ICAO Doc 4444 message set.  This correlation is not included in DO-306/ED-122; however it is included in the CPDLC Reference Guide to annotate what CPDLC messages the aircraft would need to support for oceanic operations.

## In the GOLD, it may be desirable to address continental CPDLC implementation as provided by the CPDLC Reference Guide to determine the supplemental requirements needed for the LINK 2000+ aircraft to be viable in oceanic operations from a CPDLC perspective.

# Action

## The GOLD Ad Hoc Working Group is invited to:

- Note the information presented in this paper and the CPDLC Reference Guide, dated 5 July 2008.

- Agree to the guidelines provided in paragraph 3 of this paper for development of GOLD text related to CPDLC message set and its use.

- Provide the GOLD coordinator feedback with respect to the other considerations provided in paragraph 4 of this paper.

- Option 1 –Reference to DO-306/ED-122 (Comments 016, 017, and 029):

  - Consider replacing the FOM – paragraphs 5.9 and 5.10 and NAT GM – paragraphs 6.7, 7, and 8 with a reference to DO-306/ED-122;

  - Consider replacing the FOM, paragraph 9 with the references sited in paragraph 2.2 of this paper; and

  - Retain regional supplementary material that augments DO-306/ED-122, as necessary.

- Option 2 - Alternative to sub-paragraphs 5.1a):  Consider using a format similar to the CPDLC Reference Guide.  This format may facilitate future updates to the GOLD that would expand its scope to include operational implementations of data link services in continental airspace.  It will also facilitate future updates to the GOLD that would expand on the FANS 1/A message set.

- Find some volunteers to develop text for the GOLD that provides guidance on the following:

  - Use of free text, based on Attachment C of this paper (DO-306/ED-122, Annex A, paragraph A.2.3, NAT GM, paragraph 5.14, and FOM, paragraph 5.3);

  - Use of emergency messages, based on Attachment D of this paper (DO-306/ED-122, Annex A, paragraph A.2.2, NAT GM, paragraph 5.19, and FOM, paragraph 7.1); and

  - Treatment of unsupported message elements, based on NAT GM, paragraph 5.20.

Reference:  CPDLC Reference Guide

| MRN | Message Element (reserved in PANS/ATM) | NAT GM | FOM | FANS 1/A |
| --- | --- | --- | --- | --- |
| UM20 | CLIMB TO [level] or CLIMB TO AND MAINTAIN [altitude] | S | S | I |
| UM21 | AT [time] CLIMB TO [level] or AT [time] CLIMB TO AND MAINTAIN [altitude] | S | S | I |
| UM22 | AT [position] CLIMB TO [level] or AT [position] CLIMB TO AND MAINTAIN [altitude] | S | S | I |
| UM23 | DESCEND TO [level] or DESCEND TO AND MAINTAIN [altitude] | S | S | I |
| UM24 | AT [time] DESCEND TO [level] or AT [time] DESCEND TO AND MAINTAIN [altitude] | S | S | I |
| UM25 | AT [position] DESCEND TO [level] or AT [position] DESCEND TO AND MAINTAIN [altitude] | S | S | I |
| UM35 | WHEN ABOVE (level) COMMENCE CRUISE CLIMB ABOVE [level] or CRUISE CLIMB ABOVE [level] | U | AU | I |
| UM91 | HOLD AT [position] MAINTAIN [level] INBOUND TRACK [degrees] [direction] TURNS [leg type] or HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees][direction] TURN LEG TIME [leg type] | S | S | I |
| UM96 | CONTINUE PRESENT HEADING or FLY PRESENT HEADING | U | S | I |
| UM125 | SQUAWK MODE CHARLIE or SQUAWK ALTITUDE | S | S | I |
| UM126 | STOP SQUAWK MODE CHARLIE or STOP ALTITUDE SQUAWK | S | S | I |
| UM129 | REPORT MAINTAINING [level] or REPORT LEVEL [altitude] | S | S | I |
| UM132 | REPORT POSITION or CONFIRM POSITION | S | S | I |
| UM133 | REPORT PRESENT LEVEL or CONFIRM ALTITUDE | S | S | I |
| UM134 | REPORT [speed type] [speed type] [speed type] SPEED or CONFIRM SPEED or UM169b: REPORT GROUND SPEED (FOM) | S | S | I |
| UM135 | CONFIRM ASSIGNED LEVEL or CONFIRM ASSIGNED ALTITUDE | S | S | I |
| UM154 | RADAR SERVICE TERMINATED or RADAR SERVICES TERMINATED | S | S | I |
| UM162 | SERVICE UNAVAILABLE MESSAGE NOT SUPPORTED BY THIS ATS UNIT or SERVICE UNAVAILABLE or NAT-specific standardized free text: UM 169u: MESSAGE NOT SUPPORTED BY THIS UNIT or UM169ac: MESSAGE CONTAINS ELEMENT NOT SUPPORTED BY THIS UNIT. MESSAGE REJECTED or UM169ad: DOWNLINK <abbreviated downlink message> NOT SUPPORTED BY THIS UNIT or UM169ae: MESSAGE REJECTED | S | S | I |
| UM166 | DUE TO [traffic type] TRAFFIC or DUE TO TRAFFIC | S | S | I |
| DM24 | REQUEST CLEARANCE [route clearance] or REQUEST [route clearance] | S | S | I |
| DM25 | REQUEST [clearance type] CLEARANCE or REQUEST CLEARANCE | S | S | I |
| DM32 | PRESENT LEVEL [level] or PRESENT ALTITUDE [altitude] | S | S | I |
| DM37 | MAINTAINING [level] or LEVEL [altitude] | S | S | I |
| DM38 | ASSIGNED LEVEL [level] or ASSIGNED ALTITUDE [altitude] | S | S | I |
| DM52 | WHEN CAN WE EXPECT LOWER LEVEL or WHEN CAN WE EXPECT LOWER ALTITUDE | AU | S | I |
| DM53 | WHEN CAN WE EXPECT HIGHER LEVEL or WHEN CAN WE EXPECT HIGHER ALTITUDE | AU | S | I |
| DM57 | [remaining fuel] OF FUEL REMAINING AND [persons on board] PERSONS ON BOARD or REPORT REMAINING FUEL AND SOULS ON BOARD | S | S | I |
| DM74 | REQUEST TO MAINTAIN OWN SEPARATION AND VMC or MAINTAIN OWN SEPARATION AND VMC | S | S | I |

| MRN | Message Element (reserved in PANS/ATM) | Doc 4444 | DO-306 ED-122 | NAT GM | FOM | FANS 1/A |
| --- | --- | --- | --- | --- | --- | --- |
| UM6 | EXPECT [level] | S | IER | U | S | I |
| UM7 | EXPECT CLIMB AT [time] | S | IER | NL(R) | S | I |
| UM8 | EXPECT CLIMB AT [position] | S | IER | NL(R) | S | I |
| UM9 | EXPECT DESCENT AT [time] | S | IER | NL(R) | S | I |
| UM10 | EXPECT DESCENT AT [position] | S | IER | NL(R) | S | I |
| UM11 | EXPECT CRUISE CLIMB AT [time] | S | AU | U | AU | I |
| UM12 | EXPECT CRUISE CLIMB AT [position] | S | AU | U | AU | I |
| UM13 | AT [time] EXPECT CLIMB TO [level] | R | AU | U | S | I |
| UM14 | AT [position] EXPECT CLIMB TO [level] | R | AU | U | S | I |
| UM15 | AT [time] EXPECT DESCENT TO [level] | R | AU | U | S | I |
| UM16 | AT [position] EXPECT DESCENT TO [level] | R | AU | U | S | I |
| UM17 | AT [time] EXPECT CRUISE CLIMB TO [level] | R | AU | U | AU | I |
| UM18 | AT [position] EXPECT CRUISE CLIMB TO [level] | R | AU | U | AU | I |
| UM185 | AFTER PASSING [position] CLIMB TO [level] | R | AU | N/A | N/A | N/A |
| UM186 | AFTER PASSING [position] DESCEND TO [level] | R | AU | N/A | N/A | N/A |
| UM34 | CRUISE CLIMB TO [level] | S | AU | U(R) | AU | I |
| UM35 | WHEN ABOVE (level) COMMENCE CRUISE CLIMB ABOVE [level] or CRUISE CLIMB ABOVE [level] | S | AU | U | AU | I |
| UM40 | IMMEDIATELY STOP CLIMB AT [altitude] | R | AU | S | S | I |
| UM41 | IMMEDIATELY STOP DESCENT AT [altitude] | R | AU | S | S | I |
| UM171 | CLIMB AT [vertical rate] MINIMUM | S | CRD | D 17 | S | I |
| UM172 | CLIMB AT [vertical rate] MAXIMUM | S | CRD | D 17 | S | I |
| UM173 | DESCEND AT [vertical rate] MINIMUM | S | CRD | D 17 | S | I |
| UM174 | DESCEND AT [vertical rate] MAXIMUM | S | CRD | D 17 | S | I |
| UM33 | CRUISE [altitude] | R | AU | U | AU | I |
| UM42 | EXPECT TO CROSS [position] AT [level] | R | AU | U | S | I |
| UM43 | EXPECT TO CROSS [position] AT OR ABOVE [level] | R | AU | U | S | I |
| UM44 | EXPECT TO CROSS [position] AT OR BELOW [level] | R | AU | U | S | I |
| UM45 | EXPECT TO CROSS [position] AT AND MAINTAIN [level] | R | AU | U | S | I |
| UM50 | CROSS [position] BETWEEN [level] AND [level] | S | CRD | U | S | I |
| UM54 | CROSS [position] BETWEEN [time] AND [time] | S | CRD | U | S | I |
| UM70 | EXPECT BACK ON ROUTE BY [position] | S | IER | U | S | I |
| UM71 | EXPECT BACK ON ROUTE BY [time] | S | IER | U | S | I |
| UM73 | [departure clearance] | S | CRD | NU | S | I |
| UM85 | EXPECT [route clearance] | S | IER | U | S | I |
| UM86 | AT [position] EXPECT [route clearance] | S | IER | U | S | I |
| UM87 | EXPECT DIRECT TO [position] | S | IER | U | S | I |
| UM88 | AT [position] EXPECT DIRECT TO [position] | S | IER | U | S | I |
| UM89 | AT [time] EXPECT DIRECT TO [position] | S | IER | U | S | I |
| UM90 | AT [level] EXPECT DIRECT TO [position] | S | IER | U | S | I |
| UM92 | HOLD AT [position] AS PUBLISHED MAINTAIN [level] | S | CRD | U | S | I |
| UM94 | TURN [direction] HEADING [degrees] | S | CRD | U | S | I |
| UM95 | TURN [direction] GROUND TRACK [degrees] | S | CRD | U | S | I |
| UM96 | CONTINUE PRESENT HEADING | S | CRD | U | S | I |
| UM97 | AT [position] FLY HEADING [degrees] | S | CRD | U | S | I |
| UM98 | IMMEDIATELY TURN [direction] HEADING [degrees] | S | CRD | U | S | I |
| UM99 | EXPECT [procedure name] | S | IER | U | S | I |
| UM100 | AT [time] EXPECT [speed] | S | IER | U | S | I |
| UM101 | AT [position] EXPECT [speed] | S | IER | U | S | I |
| UM102 | AT [level] EXPECT [speed] | S | IER | U | S | I |
| UM103 | AT [time] EXPECT [speed] TO [speed] | S | IER | U | S | I |
| UM104 | AT [position] EXPECT [speed] TO [speed] | S | IER | U | S | I |
| UM105 | AT [level] EXPECT [speed] TO [speed] | S | IER | U | S | I |
| UM107 | MAINTAIN PRESENT SPEED | S | CRD | U | S | I |
| UM110 | MAINTAIN [speed] TO [speed] | S | CRD | U | S | I |
| UM115 | DO NOT EXCEED [speed] | S | CRD | U | S | I |
| UM116 | RESUME NORMAL SPEED | S | CRD | U | S | I |
| UM175 | REPORT REACHING [level] | R | AU | S | S | I |
| UM146 | REPORT GROUND TRACK | S | IER | U | S | I |
| UM182 | CONFIRM ATIS CODE | S | IER | U | S | I |
| UM231 | STATE PREFERRED LEVEL or UM169c: STATE PREFERRED LEVEL | S | IER | NL | FT | FT |
| UM158 | ATIS [atis code] | S | IER | U | S | I |
| UM178 | TRACK DETAIL MESSAGE | R | AU | D 17 | AU | I |
| DM8 | REQUEST CRUISE CLIMB TO [level] | S | CRD | AU | S | I |
| DM69 | REQUEST VMC DESCENT | S | CRD | NL | NL | I |
| DM19 | REQUEST [speed] TO [speed] | S | CRD | S 17 | S | I |
| DM70 | REQUEST HEADING [degrees] | S | CRD | S 17 | S | I |
| DM71 | REQUEST GROUND TRACK [degrees] | S | CRD | S 17 | S | I |
| DM113 | [speed type] [speed type] [speed type] SPEED [speed] or DM67l: GS [speed] | S | IER | NL | FT | FT |
| DM36 | PRESENT GROUND TRACK [degrees] | S | IER | S 17 | S | I |
| DM72 | REACHING [level] | R | AU | S | S | I |
| DM39 | ASSIGNED SPEED [speed] | S | IER | NL | S | I |
| DM40 | ASSIGNED ROUTE [route clearance] | S | IER | NL | S | I |
| DM41 | BACK ON ROUTE | S | IER | NL | S | I |
| DM42 | NEXT WAYPOINT [position] | S | IER | NL | S | I |
| DM79 | ATIS [atis code] | S | IER | S 17 | S | I |
| DM104 | ETA [position][time] or DM67n: [position][time] | S | IER | NL | FT | FT |
| DM106 | PREFERRED LEVEL [level] or DM67m: FL [altitude] | S | IER | NL | FT | FT |
| DM49 | WHEN CAN WE EXPECT [speed] | S | IER | AU | S | I |
| DM50 | WHEN CAN WE EXPECT [speed] TO [speed] | S | IER | S 17 | S | I |
| DM51 | WHEN CAN WE EXPECT BACK ON ROUTE | S | IER | S 17 | S | I |
| DM52 | WHEN CAN WE EXPECT LOWER LEVEL or WHEN CAN WE EXPECT LOWER ALTITUDE | S | IER | AU | S | I |
| DM53 | WHEN CAN WE EXPECT HIGHER LEVEL or WHEN CAN WE EXPECT HIGHER ALTITUDE | S | IER | AU | S | I |
| DM54 | WHEN CAN WE EXPECT CRUISE CLIMB TO [level] | S | IER | S 17 | S | I |
| DM87 | WHEN CAN WE EXPECT CLIMB TO [level] or DM67h: WHEN CAN WE EXPECT CLIMB TO [altitude] | S | IER | AU | S | I |
| DM88 | WHEN CAN WE EXPECT DESCENT TO [level] or DM67i: WHEN CAN WE EXPECT DESCENT TO [altitude] | S | IER | AU | S | I |

| DO-306/ED-122, Annex A, paragraph A.2.3 | NAT GM, paragraph 5.14 | FOM, paragraph 5.3 |
| --- | --- | --- |
| A.2.3 Use of free text The CPDLC message set supporting oceanic data link services includes free text message elements. While use of free text messages should be avoided, free text message elements may offer viable solutions to enhance capability, given local constraints and limitations of the data link system. For example, use of free text may enhance a data link service, where the data link system does not have the capability to operationally process specific ICAO Doc 4444 message elements needed to use that data link service. | 5.14 Free text Messages 5.14.1 The non-essential use of free text messages by ATC and flight crews is strongly discouraged. | 5.3 Use of Pre-Formatted and Free Text Messages Free text messages shall be used only when an appropriate pre-formatted message element does not exist. |
| In such instances, additional operational assessments will need to be conducted to determine safety and performance requirements associated with the use of free text. | 5.14.4 When a free text message is required, standard ATC phraseology and format should be used. 5.14.5 Non-essential words and phrases should be avoided. Abbreviations should only be included in free text messages when they form part of standard ICAO phraseology. | 5.3.1 Preferred use of pre-formatted messages (2nd paragraph) When a free text message is required, standard ATC phraseology and format shall be used. Non-essential words and phrases should be avoided. Abbreviations should only be included in free text messages when they form part of standard ICAO phraseology, e.g. ETA. |
| When the intent/use of the free text message element impacts operational seamlessness, global coordination will be necessary to define the operational contents, format, and use of free text as “pre-formatted free text messages” in appropriate international standards. | 5.14.6 ATSUs may develop lists of preformatted free text messages, defining their intent. Efforts have been made to develop a common list for use within NAT CPDLC airspace. This list can be found in section 7. | 5.3.2 Standardized free text messages While pre-formatted message elements are required to be used whenever possible, there are occasions where frequent use of free text allows the meaning and appropriate response to be standardized. The Standard Free text message set is shown in Section 5.10. |
| When free text is used to mimic ICAO Doc 4444 message elements, the following criteria applies: |  |  |
| a) The ATS unit shall apply any range, resolution, and units of measure restrictions prior to use of the message. |  |  |
| b) The ATS unit and aircraft system shall provide a human interface for message composition and recognition of free text messages consistent with operational, safety, and performance requirements associated with use of the free text message. |  | 5.3.3 Storing free text messages ATSUs capable of storing free text messages should select those message elements from the standard free text message set (see FANS-1/A CPDLC Standard Free Text Messages) appropriate to their particular environments. When the storage of free text messages is not possible, controllers shall use the same message formats when typing free text messages. |
| c) The ATS unit shall not use free text to mimic an ICAO Doc 4444 message element with a W/U response attribute. | 5.14.2 Uplink free text messages require that the flight crew send DM3 ROGER prior to responding to the content of the message. For that reason, among others, ATC clearances should not be issued using free text. | 5.3.1 Preferred use of pre-formatted messages (1st paragraph) Free text messages shall be used only when an appropriate pre-formatted message element does not exist. In particular, the creation of a clearance request and the issuing of a clearance shall be performed by the use of pre-formatted message elements only. The use of pre-formatted message elements allows on board data processing such as the automatic insertion of the clearance information into the FMC. It also allows the controller to respond more quickly when the ATS system has the capability to automatically link a pre-formatted request to a pre-formatted response. Additionally, this process minimizes the risk of input errors. |
| d) The ATS unit shall provide for operational closure of the dialogue that uses a free text message to mimic a message element with a Y response attribute. Note: The message is considered technically closed when the ground system receives from the aircraft the appropriate response to the uplink free text message. | 5.14.3 Except for messages containing DM68 (see 8.1 - emergency downlink message elements) downlink free text messages do not require a closure response. Responses to non-distress free text messages should not include MRNs (see 6.4.1 for information relating to ATS automation and MRNs). | 5.7 Message Closure 5.7.1 General … A normal downlink free text message (based on downlink message element DM#67) does not require a response from the controller to close the CPDLC exchange. However, a downlink free text message based on downlink message element DM#68 (Distress attribute) does require a response and the message will remain open until a referenced response is received. Any uplink message containing only free text requires a ROGER response. The message will remain open until a referenced response containing ROGER is received. |

| DO-306/ED-122, Annex A, paragraph A.2.2 | NAT GM, paragraph 5.19 | FOM, paragraph 7.1 |
| --- | --- | --- |
| A.2.2 Processing CPDLC emergency messages ICAO Doc 4444, paragraph 14.3.5, includes provisions for use of CPDLC in emergency situations. Furthermore, effective November 22, 2007, ICAO Annex 10, Volume II, includes provisions that allow the flight crew to use “all available means” to communicate an urgency or distress situation. It is expected that, in an emergency, the flight crew will immediately revert to voice communications. However, the flight crew may use CPDLC if the capability is on the aircraft. Table A-2 includes the CPDLC downlink emergency message elements. | 5.19 Emergency Messages 5.19.1 It is expected that, in an emergency, flight crews will immediately revert to voice communications. This does not preclude crews from using CPDLC for emergency communications if unable to establish voice contact. | 7.1 Emergency procedures Although an emergency CPDLC message such as MAYDAY or PAN does not require a closure response, the controller must acknowledge receipt of the message and attempt to determine the nature of the emergency and ascertain any assistance required. |
| When processing emergency CPDLC messages, the following criteria apply: a) When an emergency message has been received from the aircraft, the ground system should display the message to the controller. |  |  |
| b) When the controller receives an emergency message, other than one containing message element DM112, the controller should respond by sending a UM3 ROGER to the aircraft. | 5.19.4 Upon receipt of an emergency downlink message, the controller shall indicate to the aircraft that the message was received by: a) responding with preformatted freetext message 004: ROGER PAN if the message contains DM55 PAN PAN PAN; b) responding with preformatted freetext message 005: ROGER MAYDAY if the message contains DM56 MAYDAY MAYDAY MAYDAY; or c) responding with UM3 ROGER if the message contains DM57, DM58, DM59, DM60 or DM61. | 7.1.4 CPDLC acknowledgment A CPDLC acknowledgment shall be in the form of a free text message using the words ROGER MAYDAY or ROGER PAN. This uplink free text message requires a response from the pilot to close the CPDLC exchange. Depending on the nature of the emergency, the free text message may or may not be acknowledged by the pilot. 7.1.4.1 Voice contact When an emergency is acknowledged by CPDLC, controllers may also attempt to make voice contact with the aircraft. |
| Note 1: Procedures for confirming the status and intentions of an aircraft and acting on the emergency message are a local matter. | 5.19.2 Any downlink message that contains an emergency message element (see 8.1 for the list of emergency message elements) should be treated as an emergency message. 5.19.3 In the event that a controller receives an emergency downlink message he/she should take immediate action to confirm the status and intentions of the aircraft via voice. … 5.19.6 Once an emergency downlink message is received, controllers will consider the aircraft to be in an emergency state until confirmed otherwise via voice contact with the flight crew. | 7.1.1 Response to an emergency message When a CPDLC or ADS emergency message is received the controlling authority, in order to better assess the nature of the emergency shall respond as in Section 7.1 above, and may also choose to: a) Increase the PERIODIC contract reporting rate to 5 minutes, or b) Send an ON DEMAND contract request. (Note. This is not required if the periodic reporting rate has been increased – an ADS report will have already been triggered by the avionics when the new periodic contract is received). Note. Increasing the ADS-C reporting rate also reduces the period between cancellation of the ADS emergency and receipt of the ADS Cancel Emergency downlink. 7.1.2 Confirmation of emergency activation When the ADS emergency mode is activated without a CPDLC emergency message or voice confirmation, and the demand contract report appears to indicate that the aircraft is maintaining normal operations (e.g. the aircraft is not in descent or involved in abrupt maneuvers), the aircraft may be subject to unlawful interference. To check for covert or inadvertent activation of the ADS emergency mode the free text uplink “Confirm ADS” shall be appended to a “Confirm Speed” data or voice request: Controller Confirm Speed Confirm ADS The pilot shall then check the status of the aircraft’s ADS Emergency Mode and if the emergency mode has been activated inadvertently, the pilot shall select ADS Emergency Mode to “OFF” and advise ATC by voice or the following CPDLC free text downlink. Pilot ADS reset If the aircraft continues with the ADS emergency mode activated ATC shall assume the aircraft is in emergency conditions and follow normal alerting procedures. 7.1.3 Acknowledgement of an emergency message When an ADS emergency accompanied by a CPDLC emergency message is received, the controller shall immediately acknowledge receipt of the emergency with the pilot by the most appropriate means (voice or CPDLC). 7.1.5 Retaining the active connection If CPDLC is the best (or only) communications medium available between the aircraft and any ATSU, the ATSU with the active connection should maintain that connection until better assistance can be provided by another means. In this case, transfer of the connection should not occur to another unit, and any automatic transfer capability should be disabled, if possible, in order to improve the chances of the CPDLC connection being retained. 7.1.5.1 Communications responsibility It is recognized that if a transfer of the CPDLC connection does not occur, then the responsibility for maintaining communications with the aircraft is retained by the current ATSU. 7.1.5.2 Executive control responsibility In accordance with established procedures, the responsibility for the control of the flight rests with the ATSU within whose airspace the aircraft is operating. If the pilot takes action contrary to a clearance that has already been coordinated with another sector or ATSU and further coordination is not possible in the time available, then this action would be performed under the pilot’s emergency authority. 7.1.6 Normal emergency procedures After receipt of the emergency message is acknowledged, normal emergency response procedures shall be followed. 7.1.7 Coordination in the case of emergency When the ADS emergency mode is observed by an ATSU that is not in control of the aircraft, that ATSU shall coordinate with the controlling authority to ensure that the emergency report has been received. Adjacent ATSUs shall not increase the reporting rate of the periodic contract. |
| Note 2: Procedures associated with receipt of message element DM112 are a local matter and will likely differ dependent upon each individual situation. | [message currently not implemented.] | [message currently not implemented.] |
| Note 3: The controller should not rely on any additional information that may be concatenated with DM55 or DM56, e.g., altitude. | 5.19.7 Controllers should be aware that altitude information included with DM55 or DM56 may not be reliable. In some cases, this information is included automatically and may not accurately reflect the current altitude or attitude of the aircraft nor the intentions of the flight crew. |  |
| Note 4: DM80 is part of the FANS 1/A message set, however, it is not processed as an emergency message element. | [Implicit in message set provided in Paragraph 8.] | [Implicit in message set provided in paragraph 5.9.] |
| c) When an emergency message is inadvertently sent, the flight crew should send DM58 CANCEL EMERGENCY as soon as practicable. After sending DM58, the flight crew should confirm their status and intentions via voice. | 5.19.5 If an emergency downlink message is inadvertently sent, the flight crew should send DM58 CANCEL EMERGENCY as soon as practicable. After sending DM58, the flight crew should confirm their status and intentions via voice. |  |