THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May, 2009)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Continuous Descent Arrivals (CDA)

(Presented by the Civil Aviation Bureau, Japan)

SUMMARY

This paper provides information on the use of Continuous Descent Arrivals (CDA) which has started on 7th May at Kansai International Airport.

- Introduction

  - Japan Civil Aviation Bureau (JCAB) began their initial Continuous Descent Arrivals (CDA) trial to Kansai International Airport (RJBB) from 7th May 2009. The CDA is a descent profile clearance which is cleared via Radio Communication to participating aircraft, currently only Japanese air carriers. The aircraft can then use its Flight Management System (FMS) to fly the most efficient descent to the arrival airport.

- Discussion

  - The CDA provides the following savings;

    - Reduced fuel burn and carbon dioxide (CO2) emissions by having the aircraft fly an optimized descent profile.

    - Reduced noise pollution since the aircraft is flying a power off descent.

    - Reduced engine wear

  - The initial CDA trial will run for a two or three month with only Japanese air carriers. The trial is limited between 23:00JST and 07:00JST when there is very little airport demand.

  - A complete CDA is projected to save each flight between 500 pounds (lbs) (B763) and 1,000 pounds (B744) of fuel over the standard arrival profile into RJBB. Due to airport demand, it is not always possible for an aircraft to fly the full CDA. When the arrival demand for the airport increase, aircraft flying the CDA may need to be vectored for arrival spacing, causing the CDA to be terminated. There are, however, still savings to be gained for the aircraft flying partial CDA.

  - When developing a CDA, several considerations must be included. The goal is to create a profile that allows the aircraft to fly an engine idle descent to the runway. If  the CDA profile contains restrictions that require the aircraft to level off, the aircraft must add power to maintain altitude and the overall fuel savings is reduced. Ideally, the descent profile, when viewed from the side, is a wedge of airspace that keeps the aircraft on a continuous descent. Different factors affect the optimum altitudes at which the aircraft cross waypoint (fix) on the CDA profile. These factors include such items as initial altitude, aircraft descent speed, aircraft type, and winds.

  - When building the RJBB CDA, the first factors that were considered were minimum en-route altitude (MEA) of these exiting routes. Presently altitude restrictions are only at or above several fixes to much MEA. There are no speed restrictions, but it will be needed for increase full CDA.

  - The following procedures are applied for RJBB CDA trial;

- Participating aircraft: FMS equipped aircraft which is operated by Japanese airlines with ETA between 2300JST and 0700JST.

- Airspace where operational trial for CDA is operated: Kansai approach control area and ACC’s control area.

- Route applicable to operational trial for CDA: Those arrival route for RJBB via KARIN, KAZRA or EVERT and join STAR’s.

- Conditions on operational trial for CDA: Radar must be in operative at relevant ACCs and Kansai Radar Approach Control Area.

- Request and Clearance for CDA: CDA routes named as packages are used when pilot request CDA and when ATC clears CDA. There are altitude restrictions on CDA routes. ATC may add altitude restrictions when necessary due to traffic situations. CDA may not be cleared due to traffic situations.

- Timing for requesting CDA: Pilot should request CDA not later than 10 minutes before reaching Top of Descend (TOD) with position of TOD and estimated over time of KARIN, KAZRA or EVERT.

- Report of beginning descend: Once CDA is cleared by ATC, Pilot should report ATC of beginning of descend.

- Suspension or cancellation of CDA: ATC may suspend or cancel CDA due to traffic situation even after CDA is cleared. Alternate instructions will be issued when CDA is suspended or canceled.

- Further plan

  - JCAB will continue operational trial for CDA at RJBB only Japanese airlines two (2) or three (3) months for evaluations. After evaluations, CDA will be applied to all aircraft with ETA between 2300JST and 0700JST.

  - JCAB will continue study and discuss with airline operators to extend time period at RJBB and possibility to establish for other airports.

- Action by the meeting

  - The meeting is invited to note the information in this paper