THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May 2009)

Agenda Item 5:  	ATM Issue

RNAV5 PREFERNTIAL OPERATION

(Presented by JCAB)

SUMMARY

This paper provides outline of the RNAV5 Preferential Operation (Sky Highway) which will be in force from October 2010 in domestic airspace of the Fukuoka FIR.

- Introduction

  - Around year 2000, JCAB made the airways realignment program utilizing Japanese RNAV standards which are establishing double tracks between major city pairs and/or establishing airways for one-way using in order to cope future air traffic demand. September 2007, JCAB established the RNAV1/5 standards in accordance with ICAO PBN manual, replaced Japanese RNAV routes to RNAV5 routes. JCAB is establishing new RNAV5 routes for shortening flight routes and alternating use of conventional VOR routes after that, and 89 RNAV5 routes are operating now.

  - In other hand, conventional VOR routes are still existing and available for flight planning of non-RNAV5 approved aircraft. Currently aircraft operators may file flight plan whichever RNAV5 route and VOR route.

  - This circumstances that RNAV5 routes and VOR routes are established in the same airspace lead to increase workload of air traffic controllers. Air traffic controller always have to care of separation between aircrafts on RNAV5 route established closely VOR route. Because lateral separation between RNAV5 route and VOR route are not considered when establishing RNAV5 route. And aircraft on RNAV5 route has much possibility to overtake aircraft on VOR route.

  - Increasing air traffic controller’s workload means decreasing ATC capacity. In order to make the most of capacity enhancement using RNAV5, JCAB plan to introduce RNAV5 preferential operation named the Sky Highway Project.

  - The Sky Highway is to separate flights on the VOR routes and RNAV routes operationally at FL290. The Sky Highway will be in force at October 2010 at when fourth runway of the Tokyo International Airport (Haneda) will open. An Aeronautical Information Circular regarding the Sky Highway will be published for all parties concerned.

- Operational Procedure

  - The Sky Highway is not RNAV5 mandate, and not exclusive RNAV5 operations at or above FL290 in the Fukuoka FIR. The Sky Highway is one of ATC operational procedures which is to avoid increasing air traffic controller’s workload with securing safety.

  - Before departure, air traffic controller will give an ATC clearance of cruising altitude between FL290 and FL410 for only RNAV5 approved aircrafts. For aircraft coming from adjacent FIR or oceanic airspace, air traffic controller will instruct altitude change to at or below FL280 if the aircraft is not RNAV5 approved.

  - Following aircraft/cases are exception of the Sky Highway. Air traffic controller will approve altitude between FL290 and FL410 for non RNAV5 approved depend on the traffic situation.

    - The state aircraft will flight plan at altitude between FL290 and FL410 because of its mission; or

    - In case of turbulence or weather deviation at or below FL280/at or above FL430; or

    - Air traffic controller instruct to enter altitude between FL290 and FL410 in order to resolve conflict; or

    - While midnight time period when low traffic is expected, and ATC facilities approved.

- Flight Plan

- 3.1.      Aircraft operators are requested to flight plan depend on the aircraft whether the RNAV5 approved or not.

- 3.1.1.	If the aircraft is RNAV5 approved and plan to operate at between FL290 and FL410, the operator shall flight plan along RNAV5 routes; and

- 3.1.2.	If the aircraft is not RNAV5 approved, the operator shall flight plan at or below FL280/at or above FL430.

- 3.2.	Air traffic controller will assign cruising altitude as to notations on the flight plan. Aircraft operators are requested to flight plan correctly as follows.

- 3.2.1.	If the aircraft is RNAV5 approved;

a.	Annotate “Z” in Item 10 with additional information in Item 18; and

- b.	Annotate “NAV/RNAV5” in Item 18 (Other Information).

- Action by the meeting

  - The meeting is invited to:

- Note the information in this paper, and;

- Support the Sky Highway as one of the program to ensure airspace capacity and efficiency, and;

- Request IATA to circulate this information to all member airlines.

- END -