THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May, 2009)

Agenda Item 5:  Air Traffic Management (ATM) Issues

Simulation analysis of UPRs of TRACK 1

(Presented by JCAB)

SUMMARY

UPRs over North Pacific Area are desired. The tendency of UPRs of TRACK 1 is simulated. These UPRs usually intersect to adjacent route in winter.

- Introduction

  - At IPACG/29, IATA proposed the paper trial between Asia and North America for flights within the Fukuoka, Anchorage and Oakland FIRs. FAA and JCAB agreed to continue discussions with IATA on UPR trials in the NOPAC region. JCAB has been conducting a modelling study of the possibility of UPRs and flex tracks in the NOPAC region of Fukuoka FIR.

  - Over the north pacific area, NOPAC routes and the current Pacific Organized Track System (PACOTS) keep 50 nautical miles (NM) lateral separation each other. UPRs may not keep lateral separation with other PACOTS or NOPAC routes. So it is necessary to analyze the tendency of UPR intersections with other routes and potential conflict.

  - Electronic Navigation Research Institute (ENRI) has been studying safe and efficient oceanic route system using Dynamic Aircraft Route Planning System (DARPS) simulator. This paper presents the simulation results to tendency of UPR for TRACK 1.

- Simulation Methods

  - Minimum fuel tracks as optimum flight routes and flight profile are computed for a day by DARPS simulator. The weather condition depends on the GRIB data of Japan Metrological Agency. PACOTS are also computed in the simulator. The comparing the intersection for TRACK2, TRACK3 or NOPAC is shown in this paper.

  - The simulator also computes the optimum flight profile for PACOTS.The numbers of conflicts are estimated by simulator by simple detection.

- Simulation Results

  - The variety of UPR depends on variety of the jet stream. When the jet stream is strong, as in winter, UPRs of TRACK 1 causes the intersections with TRACK 2 and TRACK 3 shown as Figure 1. Blue lines show TRK2 and TRK3. Red lines show UPRs of TRK1.Grean arrows show jet stream. Most east stream is strong. Middle stream is strong, but the location is south for optimum route.

Figure 1 UPRs at 17 Feb. 2007

  - Even in the winter when the jet stream is strong, UPRs are near the great circle routes as shown as Figure 2. It is because that the core of the jet stream is too south for efficiency.

Figure 2 UPRs at 27 Feb. 2007

  - For 28 days at February 2007, rate of intersected days is shown in Figure 3. TRK1 is baseline (100%). TRK2 indicates 85.7%. It means that some UPR indicate to TRK 2 at 85.7% day in February. In February, there is no day that UPRs intersect free adjacent routes.

Figure 3 Rate of day in Feb. UPRs intersected with adjacent routes.

  - East of 170E, the rate of intersected days is shown in Figure 4. TRK2 indicates 53.6%. UPRs tend to intersect with TRK2 even east of 170E in February. The intersection occurs usually in winter.

Figure 4 Rate of day in Feb. UPRs intersected with adjacent routes east of 170E.

  - When the jet stream is weak, as in summer, the UPRs are close to the great circle routes as shown as Figure 5. Especially, when the core of jet stream is far north, the UPRs intersect to NOPAC routes.

Figure 5 UPRs at 26 Aug. 2007

  - When the TRACK 1 is established over the core of the jet stream, UPRs are close to original TRACK 1 as shown as Figure 6. And these tend to separate with TRACK 2. However, these cases are not so often.

Figure 6 UPRs at 26 Oct. 2007

  - UPR from Narita airport is different with other UPR from the airports west from Narita.

  - Potential conflict is detected in the DARPS simulator for two days. This detection is based on flight profile data without departure delays. When UPRs are intersect free east of 160W with TRACK 2 and east of 170W with TRACK 3 (17 Feb. 2007, see Figure 1), the number of conflict in original PACOTS are 49. However the number of conflict in UPRs, TRACK 2 and TRACK 3 are 78. UPRs increase the conflict, especially with TRACK 2, so it means that some flight are forced lower flight for safety.

  - When UPRs keep separations with TRACK 2 (26 Oct. 2007, see Figure 6), the number of conflict in original PACOTS are 26 and the number of conflict in UPRs, TRACK 2 and TRACK 3 are 33. UPR increase the conflict but the altitude which some flight are changed is small.

- Recommendations

4.1	The meeting is invited to note and to discuss the simulation results as presented in this paper.

- END -