THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May, 2009)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

The status of considering introduction of 10 Minute Longitudinal Separation without Mandatory Mach Number Technique (MNT) in the Fukuoka Flight Information Region (FIR)

(Presented by the Civil Aviation Bureau, JAPAN)

SUMMARY

This paper provides the status of considering introduction of 10-minute longitudinal separation without mandatory Mach Number Technique (MNT) in the Fukuoka FIR after the result of the safety assessment.

- Introduction

  - JCAB announced the result of the safety assessment prior to implementation of 10-minute longitudinal separation minimum without mandatory Mach Number Technique (MNT) in the Fukuoka FIR oceanic airspace at IPACG/29.

  - As the result of the safety assessment, the estimated collision risk is below the Target Level of Safety (TLS) if the distance position reports is 350NM or less for NOPAC routes and 370NM or less for other routes.

  - JCAB reviewed this result and concluded that for the implementation of 10-minute longitudinal separation minimum without mandatory MNT, modifications of the route structure, upgrade of the ODP System, or limitations of applying 10-minute longitudinal separation standard with mandatory MNT are required in the portion of routes within the Fukuoka FIR.

  - Despite the conclusion above, JCAB plans to introduce the 10-minute longitudinal separation minimum without mandatory MNT in the Fukuoka FIR in a phase manner.

JCAB plans to apply the separation for aircraft flying within the Fukuoka FIR in;

a. Stage-1:  case of ADS capability failing during the 30 or 50 NM longitudinal separation based on the ADS applied.

b. Stage-2:  the portion of airspace/routes under the current route structures and the ODP system.

- Discussion

  - JCAB studied a concrete introduction method of the trial operation to introduce the 10-minute longitudinal separation minimum without mandatory MNT in the Fukuoka FIR.

  - The current ATS routes in Fukuoka FIR that can be introduced the 10-minute longitudinal separation minimum without mandatory MNT are in limited routes to connect to the Oakland FIR. Other ATS routes, NOPAC, PACOTS and UPRs do not meet the requirement of intervals of the compulsory reporting points required by the safety assessment.

  - JCAB consider that the modification of reporting points from non-compulsory to compulsory and the ODP system are required, to expand the 10-minute longitudinal separation minimum without mandatory MNT to whole airspace in Fukuoka FIR.

  - JCAB propose that firstly as the trial operation, it will apply the 10-minute longitudinal separation minimum without mandatory MNT within Fukuoka FIR in case of ADS capability failing during the 30 or 50 NM longitudinal separation based on the ADS applied or occurring contingency situation on summer 2009.

  - The expansion of further application is assumed to be going to continue examining in the future.

- Action by the meeting

  - The meeting is invited to note the information presented in this paper.