THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May 2009)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Response to the improvement of operations of Air Traffic Services Route B932

(Presented by the Civil Aviation Bureau, Japan)

SUMMARY

This paper presents the Japan Civil Aviation Bureau’s (JCAB’s) response to the improvement of operations of ATS route B932 through the Fukuoka, Yuzuho-Sakhalinsk and Petropavlovsk-Kamchatsky Flight Information Regions (FIRs).

- Introduction

  - ATS Route B932 was established as a demonstration route in mid 2003. Unfortunately however, the route received little utilization.  This was thought to have been mainly due to the route’s limited hours of availability, i.e. only between 2030UTC and 0830UTC, and its unidirectional orientation, that is westbound flight only.

  - This time frame is decided according to the operation time of Memanbetsu VOR/DME (TBE), where locates at the end of B932. In the middle of 2007, JCAB concluded to extend the operation time of TBE to twenty-four hours. As a result, the B932 became available to 24 hours use from 28 August 2008.

  - The Federal Air Navigation Authority of Russia (FANA) represented its perception that it is necessary to move the feet/meter flight level conversion areas to Sapporo ACC, which was located in Yuzhno-Sakhalinsk ACC, for the bidirectional operation of B932, because of lack adequate radar coverage. Also, Yuzhno-Sakhalinsk requested Russia’s State Air Traffic Management Corporation (State ATM) to approve installation of additional VHF relays to expand the coverage area for reduction of longitudinal separation.

  - In July 2008, the FANA and JCAB completed negotiations resulting in the realignment of B932  to permit bidirectional use and to reduce the longitudinal separation to 15-minute, because JCAB accepted to move the feet/meter flight level conversion areas to Fukuoka FIR with all ATS routes between Yuzhno-Sakhalinsk FIR and Fukuoka FIR. These changes of operation were to be effective on 20 November 2008.

  - During the Third Meeting of the Cross Polar Working Group (CPWG/6), held in Hong Kong, from 3 to 5 November 2008, FAA recognized that final success of the B932 realignment would require FAA to complete corresponding and necessary changes within the Anchorage FIR.

- Discussion

  - The FANA and JCAB agreed to assume the improvement of B932 was effective on 20 November, however the effective date was delayed by the simultaneous period with renewal of the Air Traffic Control System at Sapporo ACC, and preliminary work of Yuzhno-Sakhalinsk ACC.

  - As the result of continuously coordination with the FANA and JCAB, the operational improvement of the B932 agreed with beginning since 12 February 2009, and new LOA was established between Yuzhno-Sakhalinsk ACC and Sapporo ACC at the same date.

  - Because the Anchorage ARTCC was not ready to open for eastbound traffic on the B932 from Petropavlovsk-Kamchatsky FIR, the unidirectional operation of the B932 had been continued for the time being.

- Action by the meeting

  - The meeting is invited to;

    - Note the information presented.

    - Continue support to improve operational efficiency and enhance capacity.