THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

THE SEVENTEENTH MEETING OF THE FANS INTEROPERABILITY TEAM

(FIT/17)

(Seattle,Washington, 19 May, 2009)

Proposal of the downlink report forms to relay the turbulence information by CPDLC

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This paper proposes about the downlink report forms in FREETEXT to relay the turbulence information by CPDLC quickly.

1      Background

1.1    At IPACG/29, the JCAB attempted to propose the downlink report forms to relay the turbulence information by CPDLC. After the meeting, the FAA and the JCAB reviewed it, and the JCAB decided to propose the downlink report forms in FIT/17.

2     Discussion

2.1    An objective of relaying the turbulence information by CPDLC is a quickly automatic transmission to the aircraft with down link messages from the aircraft are analyzed by the system. Therefore, it has been reviewed about the standard report forms from the aircraft.

2.1.1   The following downlink information (inconsecutive) becomes necessary for the system to analyze the turbulence information.

- The term which means a turbulence (TURB, TURBULENCE, and so on)

- The term which shows the intensity of the turbulence (MOD, MODERATE, MDT, and so on)

2.1.2   Examples of downlink messages which the system analysis possible

- MOD TURB

- MODERATE TURB ON FL350

2.1.3   Examples of downlink messages which the system analysis impossible (controllers need confirmation of it)

Multiple representations of observation point

- MOD TURB FL350 STARTED 100 MILE WEST OF N42E160

- MOD/SVR TURB FL320 LAST 10 MILES ANY REPORT OTHER FL

- MDT TURB FOR LAST 10 MIN

Intensity of the turbulence is unknown

- TURB

2.2    The proposal report forms (It is based on PANS-ATM Doc4444, RTCA/DO-258A)

  - TURB MOD  (level)

(level) (position)

(level) (position) TO (position)

(level) TO (level)

(level) TO (level) (position)

(level) TO (level) (position) TO (position)

  - TURB SEV   (level)

(level) (position)

(level) (position) TO (position)

(level) TO (level)

(level) TO (level) (position)

(level) TO (level) (position) TO (position)

*In some cases, (position) may become as follows.

[distance] NM [direction] (position)

*(position)

[Fix Name]

[Navaid]

[Airport]

[Latitude Longitude]

[Place Bearing Distance]

2.3    The advantages by the proposal

- The quick and exact dissemination becomes possible to the target aircraft

- The workload of the controllers are reduced

- Entry work in FREETEXT for the flight crew is simplified

2.4     About the uplink information

2.4.1   The following contents are included in the turbulence information to provide the aircraft.

- Intensity of the turbulence

- Observation point

- Observation altitude

- Reported time

- Reported aircraft type

2.4.2   If the other information is added in the turbulence information of the downlink message, an air traffic controller may sometimes confirm the message contents first. And then the air traffic controller relays that information to the target aircraft.

2.4.3   The target aircrafts are automatically selected by the system.

3      Action by the meeting

3.1    The meeting is invited to discuss these proposals.