THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

THE SEVENTEENTH MEETING OF THE FANS INTEROPERABILITY TEAM

(FIT/17)

(Seattle, USA, 18-22 May 2009)

ATS Ground System SPR Compliance in the ATM-ODP system

(Presented by Civil Aviation Bureau, Japan)

Summary

The JCAB has completed an analysis of the ATM-ODP system to determine the level of compliance with the requirements.

# 1.     Introduction

## The JCAB has completed an analysis of the ATM-ODP system to determine the level of compliance with these requirements.  This paper provides information on results of the analysis.

# 2.     Discussion

## 2.1    The JCAB has determined that the ATM-ODP system is not compliant with three requirements and is partially compliant with one.

## 2.2    The first safety requirement which the ATM-ODP system is not complaint with is SR-2 which relates to indicate to the controller a detected loss of data link service. The ATM-ODP system does not indicate to the controller in the timing of the loss of data link service. The controller will grasp the loss of data link service by non-delivery of the message.

## 2.3    The second safety requirement which the ATM-ODP system is not complaint with is SR-14 which relates to indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN). The ATM-ODP system does not inspect whether or not the downlink message to the uplink message is within the required time (ETTRN).

## 2.4    The third safety requirement which the ATM-ODP system is not complaint with is SR-15 which relates to an appropriate indication to the controller when a clearance response is received that has a latency exceeding ETTRN. The ATM-ODP system does not inspect whether or not the time stamp is within the required time (ETTRN).

## 2.5    The requirement that the ATM-ODP system is partially compliant with is SR-8 relating to send the route clearance information with the route clearance via data link. The ATM-ODP system is not adaptable to set the request route to UL#83 by the quotation entry. The uplink is possible with the manual input by the controller, but the ATM-ODP system includes an anxious factor in the point, SAME.

- 3.     Recommendations

3.1    The meeting is invited to note the information provided in this paper.

ATSP Ground System Safety Requirements (from RTCA DO-306 Table B-2)

| RQMT | Requirement Description | ATSP System |
| --- | --- | --- |
| SR-2 | A detected loss of data link service shall be indicated to the controller/flight crew | The ATSU shall indicate to the controller a detected loss of data link service. |
| SR-8 | The aircraft and the ATSU shall exchange via data link and process the same route information. | The ATSU shall send the route clearance information with the route clearance via data link. |
| SR-14 | The ATSU system shall indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN) | The ATSU system shall indicate to the controller when a required response for a message sent by the ATSU is not received within the required time (ETTRN) |
| SR-15 | When the end system receives a message whose time stamp exceeds ETTRN the end system shall provide appropriate indication. Note 1: Appropriate indication may include the need for real time monitoring and alerting (such as via an uplink delay timer, see Table 4-4 and Table 4-8). This is determined based on further safety assessment and is a local matter. The safety assessment needs to consider message content, intended use, and environmental conditions that could potentially lead to an unacceptable risk of undetected late delivery of a message as determined by the continuity requirement. | When the ATSU receives a message whose time stamp exceeds ETTRN the ATSU shall provide appropriate indication. |