THE THIRTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/30)

(Seattle, Washington, USA, 18-22 May 2009)

Agenda Item 5:  	Air Traffic Management (ATM) issues

The updated report the implementation of the operational trial of the 30NM Longitudinal Separation Standard based on the ADS within the Fukuoka FIR

(Presented by the Civil Aviation Bureau, Japan)

SUMMARY

This paper provides the updated information on the operational trial of the 30NM longitudinal separation standard based on the ADS within the Fukuoka FIR, which has started since August 28 2008 for Step1 and December 1 2008 for Step2.

- Introduction

  - JCAB considered the earliest way so as to introduce the 30 NM longitudinal separation in the Fukuoka FIR since 2007. JCAB addressed to the IPCAG their plan to reduce the 50NM longitudinal separation to 30NM from the summer of 2008. It will be forwarded in the phased manner which is limitedly applied to the climb/descent phase or specific routes.

  - Prior to the introduction of the 30 NM separation, JCAB Regional Monitoring Agency (JCAB RMA) demonstrated the safety assessment in accordance with the provision of the ICAO PANS-ATM (Doc. 4444). The RMA concluded that if the reduction to the 30 NM separation would be applied in the whole airspace in the Fukuoka FIR, the application of 10 minutes periodic interval for the ADS downlink report is required so as to satisfy the Target Level of Safety (TLS) though ICAO PANS-ATM provisions indicate that the maximum interval of ADS periodic reports for RNP4 is 14 minutes. The reason of this modification is required is that the amount of traffic and the density of the North Pacific routes system (NOPAC) is too heavy.

  - From the result of this evaluation, JCAB determined to propose taking the 10 minutes periodic interval for the ADS report from aircraft equipped with the Required Navigation Performance 4 (RNP4) capability in the Fukuoka FIR.

  - JCAB planned three-steps for implementing the ADS 30NM longitudinal separation standard in the Fukuoka FIR as follows;

Step-1: Trial operation to the Climb/Descent in the Fukuoka FIR

Step-2: Trial operation to the Cruse phase in the Fukuoka FIR

Step-3: The regular operation of 30/30 in the Fukuoka FIR

Step-4: To apply it to the crossing-FIR boundary traffic between the neighbouring FIR

  - The Fukuoka Air Traffic Management Center (Fukuoka ATMC) introduced the Step1 on 28 August 2008, the Step2 on 1 December 2008. JCAB consider that some issues are existed to progress to the next step, which has intermittent problems with satellite communications due to the number of outage, network load of satellite data link system, and low percentage of flights for application of the ADS 30NM longitudinal separation.

  - This minimum is applied only between aircrafts with RNP4 approval.  Between RNP4 approved aircraft and non-RNP4 approved aircraft, and between non-RNP4 approved aircrafts, 50NM or 10 minutes with MNT or 15 minutes longitudinal separation shall be applied.

  - This separation reduction will provide aircrafts with more opportunity to fly at optimum altitude or closer to the optimum altitude than before as well as increase airspace capacity and ATC flexibility to respond to operators requests.

- Discussion

- Status of data link communications

  - Fukuoka ATMC and JCAB also gathered the data regarding the status of Data link communications since August 28 2008.

  - The Graphs of A, B and C show status of Data link messages and the ADS capability in the Fukuoka FIR from January 2007 to March 2009.

    - Total numbers of the ADS downlink messages and periodic report slightly decrease from September 2008.

    - The average of Date link message per logoned aircraft is almost no variation from September 2008.

    - The total numbers of RNP4 capable aircraft have still maintained low (17.7 percents on March 2009).

Graph-A

Graph-B

Graph-C

Data link Success Rate

  - Figures of A and B show the data of the 95 and 99 percentile for uplink and downlink messages respectively between October 2008 and March 2009. Figure C shows “Uplink Success Rate” in the same period.

  - There is almost no variation between October 2007 and September 2008. These data indicates no adverse trend compared with those before the ADS30 applied.

Figure-A

Down link 95 and 99 Percentile

Figure-B

Up link 95 and 99 Percentile

Figure-C

CPDLC Up link Message Success Rate

|  | Oct. 2008 | Nov. 2008 | Dec. 2008 | Jan. 2009 | Feb. 2009 | Mar. 2009 |
| --- | --- | --- | --- | --- | --- | --- |
| Total number of uplink message | 31,913 | 26,210 | 26,536 | 26,067 | 21,743 | 27,076 |
| Total number of unsuccessful uplink message | 154 | 189 | 161 | 255 | 120 | 148 |
| (%) | 0.48% | 0.72% | 0.61% | 0.98% | 0.55% | 0.55% |
| Total number of successful uplink message | 31,759 | 26,021 | 26,375 | 25,812 | 21,623 | 26,928 |
| (%) | 99.52% | 99.28% | 99.39% | 99.02% | 99.45% | 99.45% |

Conclusion

  - JCAB concluded the tendency shown in these data does not indicate any impact to the Data link communications in the Fukuoka FIR after the 30 NM separation standard based on the ADS separation standard.

  - JCAB will continue to review the status of Data link communications within the Fukuoka FIR.

- Action by the meeting

  - The meeting is requested to note the information in this paper.