THE THIRTY-FIRST MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/31)

THE EIGHTEENTH MEETING OF THE FANS INTEROPERABILITY TEAM

(FIT/18)

(Tokyo, JAPAN, 5-9 October 2009)

PARC CWG initiative – FANS 1/A over Iridium

(Presented by the United States of America)

Summary

This paper provides an update on the FANS 1/A over Iridium (FOI) project being done by the FAA-sponsored Performance-Based Operations Aviation Rulemaking Committee’s Communications Working Group (PARC CWG).

# Introduction

## At IPACG/30, the FAA and JCAB held a side-bar meeting to discuss the Continental/Air Micronesia (COA/CMI) FANS 1/A over Iridium (FOI) trial to support the PARC CWG’s investigation of Iridium as a viable sub-network for FANS 1/A operations.

## This paper provides a summary of a successful flight test of the FOI capability, responses to FAA actions taken at the side-bar meeting, Friday, 22 May 2009, and an update on the PARC CWG project.

# Discussion

## Continental Micronesia (CMI) operates 9 Boeing 737-800 aircraft based in Guam and fly daily extended over water operations outside of VHF coverage (see   Figure 1) in airspace currently supporting FANS 1/A services.

Figure 1.  CMI routes supporting FOI project

## Continental Micronesia (CMI) is in the process of outfitting these aircraft with Avionica satLINK, an ARINC 741 compliant voice and data capable Iridium-based satellite system.  These aircraft use Collins CMU-900 and GE Aviation MCDU and will be equipped with a FANS 1/A package.  CMI uses SITA as their communication service provider.  The aircraft registration for the CMI fleet and FOI status is shown in Table 1.

Table 1 – CMI FOI fleet configuration

| A/C Registration | FOI Status |
| --- | --- |
| N25201 | Planned |
| N13227 | Planned |
| N14228 | Planned |
| N26232 | Planned |
| N14235 | FOI Ready |
| N35236 | Planned |
| N14240 | Planned |
| N27246 | Planned |
| N14249 | Planned |

## On 5 September, CMI, FAA and JCAB conducted a successful test of N14235 on CMI Flight 975, departing Guam (GUM) to Nagoya (NGO).  Table 2 provides a log of the test.  All times and dates are UTC.

Table 2 – FOI Flight Test Log, 5-Sep-09, CMI Flt 975 (N14235)

| UTC | Description |
| --- | --- |
| 0627 | 05 Sept out 0627 off 0635 ETA RICHH 0712 |
| 0700 | LOGON sent KZAK |
| 0702 | Accepted |
| 0703 | ATC comm established RJJJ ACT CTR RJJJ |
| 0704 | ATC select off ATC comm terminated |
| 0705 | LOGON sent again KZAK |
| 0706 | accepted ATC comm established ACT CTR KZAK NXT CTR RJJJ |
| 0712 | established voice contact with ARINC |
| 0734 | sent free text message |
| 0736 | received free text message |
| 0736 | accept sent |
| 0742 | received message to contact RJJJ on HF 11384 |
| 0742 | sent accepted WILCO |
| 0745 | ATC select off ATC comm terminated |
| 0818 | received verbal request via HF to LOGON to RJJJ |
| 0818 | LOGON sent RJJJ |
| 0819 | accepted |
| 0820 | ATC comm established |
| 0850 | free text sent |
| 0851 | free text received |
|  | System continued to work until VHF voice contact was established |
|  | ATC comm terminated |

## The FAA conducted analysis for this flight based on the guidelines for post-implementation monitoring and the Required Communication Performance (RCP) 240 specification provided in the Global Operational Data Link Document (GOLD).  This first test provided one CPDLC transaction with a WILCO/UNABLE (W/U) response attribute and five ADS-C position reports.  Table 3 provides the results of the actual performance measurements for these data points.

Table 3 – Actual performance results

| Metric | Result |
| --- | --- |
| CPDLC actual communication performance (ACP) (1) | 41 |
| CPDLC actual communication technical performance (ACTP) (1) | 15.5 |
| CPDLC pilot operational response time (PORT) (1) | 25.5 |
| ADS delivery time (1) | 18 |
| ADS delivery time (2) | 4 |
| ADS delivery time (3) | 6 |
| ADS delivery time (4) | 11 |
| ADS delivery time (5) | 18 |

## Table 4 provides FAA responses to actions taken at the side-bar during IPACG/30.

Table 4 – Status of action items from side-bar

| Who | Action Item | Status |
| --- | --- | --- |
| FAA | Find out if CMI can make CPDLC position reports as well as periodic ADS reports and advise JCAB. This would be for the Fukuoka FIR only. | If required, CMI can make CPDLC position reports in addition to ADS reports while in the Fukuoka FIR. However, CMI would like to keep procedures the same as KZAK, if possible. |
| FAA | Find out if CMI can file “I” at the end of the call sign for aircraft participating in the trial. | CMI can include an "I" code on the end of flight number on the ICAO flight plan (indicating Iridium equipped) as previously requested from the JCAB if necessary. CMI would prefer to not include the "I" on the call sign or on the CMI dispatch release and flight plan due to potential conflicts with CMI’s internal computer systems. |
| FAA | Confirm that none of the CMI aircraft in the trial are Inmarsat equipped. | None of these aircraft are equipped with Inmarsat satellite system. All are planned to be outfitted with satellite system as described in paragraph 2.2. |

## At side-bar, we had discussed plans to start the trial on August 1, 2009.  The project had experienced some delay, and CMI is planning a revised target date to start the trial by the end of October.  CMI will coordinate further with JCAB and FAA as we approach this date.

- CMI are on track to have about 7 aircraft equipped by the end of October 2009 and the complete fleet equipped by July 2010; and

- CMI is proceeding with crew training in anticipation of having Ops Specs approval by end of October.

## We also note that Cargolux (CLX) participates in the PARC CWG FOI project.  CLX operates 14 Boeing 747-400 aircraft in over 55 countries and services over 60 flight destinations.  CLX is in the process of outfitting these aircraft with ICG ICS-220A, an ARINC 741 compliant voice and data capable Iridium-based satellite system.  These aircraft use ACARS MU Rockwell Collins model DLM-900 and Honeywell MCDU and will be equipped with FANS 1/A package.  CLX uses SITA as their communication service provider.

- CLX was granted FANS operational approval on September 9, 2009 by their authorities. CLX is beginning the application process for FANS 1/A for the different parts of the world;

- CLX Crew Training are preparing the crews and documentation is being updated to include FANS 1/A; and

- CLX plans to have SATCOM installations on their complete fleet and Flight Operations ready by end of October.

# Recommendations

## The meeting is requested to note this information.

## For more information on the PARC CWG FOI project, please contact Mr. Arnold Oldach (aoldach@rockwellcollins.com) or Mr. Tom Kraft (tom.kraft@faa.gov), the co-chairmen of the PARC CWG.