THE THIRTY-FIRST MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/31)

THE EIGHTEENTH MEETING OF THE FANS INTEROPERABILITY TEAM

(FIT/18)

(Tokyo, JAPAN, 5-9 October 2009)

PARC CWG initiative - satellite voice for ATC communications

(Presented by the United States of America)

Summary

This paper provides an update on the satellite voice communications work being done by the FAA-sponsored Performance-Based Operations Aviation Rulemaking Committee’s Communications Working Group (PARC CWG).

# Introduction

## The Performance-Based Operations Aviation Rulemaking Committee (PARC) is an FAA-sponsored activity that operates according to the Administrator’s authority under 49 USC 106(p)(5).  The PARC comprises members from the FAA and the aviation community at large and provides recommendations to FAA’s Senior Management for action and implementation.  The PARC has been effective over the last decade in implementing area navigation and required navigation performance (RNAV/RNP).

## In 2005, the PARC established the Communications Working Group (CWG) to address a number of issues related to the implementation of aeronautical communication systems.  These systems included, among others, the future air navigation system (FANS) 1/A, the aeronautical telecommunication network (ATN), and satellite voice communications.

## The PARC CWG is committed to applying the performance-based concept, which aims to leverage existing capability and maximize benefits by:

- Enabling cost-effective alternatives, using different technologies and existing capabilities, that meet business needs in a more timely manner;

- Providing performance-based criteria to demonstrate aircraft equipment and capability without technological or implementation-specific constraints; and

- Enabling different levels of service in common airspace to a fleet of aircraft with varying capability and performance.

## The PARC CWG develops recommendations that directly support matters that relate to the FAA’s regulatory criteria and guidance material for implementation of voice and data communications within the U.S. National Airspace System (NAS).

## The PARC CWG recognizes that global harmonization is crucial to the success of any State or regional implementation initiative.  As such, the PARC CWG prepares Coordination Drafts of material for broad review and solicits input on such matters of interest to the aviation community.

## This paper discusses a PARC CWG initiative on the use of satellite voice for ATC communications.

# Discussion

Background

## Oceanic and remote airspace flight communications have historically been conducted with high frequency (HF) radios due to the advantage of being able to transmit and receive air/ground communications for thousands of miles.  The HF communication centers are designated by air traffic service providers (ATSP) for air traffic control (ATC) services.  By comparison, controllers use very high frequency (VHF) radios for line-of-sight coverage of approximately 200 miles and the radio transmission does not follow the curvature of the earth.   Following the advent of satellites, new technologies were endorsed by ICAO in 1991 and have since updated communications, navigation, and surveillance (CNS) for air traffic management (ATM).

## In 1995, the initial future air navigation system (FANS 1/A) provided an integrated CNS package.  In addition to required navigation performance (RNP) and global navigation satellite system (GNSS) capabilities, FANS 1/A includes controller pilot data link communications (CPDLC) and automatic dependent surveillance – contract (ADS C) capabilities using satellite communications (SATCOM), VHF, and HF data links.  At the same time, aircraft became equipped with SATCOM voice capability.

## In 1996, the FAA recognized technological advances in communications by a rule change to Title 14 Code of Federal Regulations (CFR) sections 1.1 and 121.351 that included use of a new term - long range communication system (LRCS). 14 CFR section 1.1 defines LRCS as “A system that uses satellite relay, data link, high frequency, or other approved communication system which extends beyond line-of-sight.”  Examples of systems that meet this definition are HF voice, HF data link (HFDL), SATCOM voice and SATCOM data link.

## The regulations, therefore, now address long range communication requirements in terms of LRCS.  With that as a basis, an aircraft on extended over water segments unable to utilize line-of-sight systems must have at least two operational LRCSs to honor regulatory communication requirements (unless specifically excepted under 14 CFR section 121.351(c)).  In addition, HF voice is the only LRCS currently acceptable for air traffic control communications in all areas.  Therefore, in areas requiring two operational LRCSs, at least one must be HF-voice and in areas requiring one LRCS, that system must be HF voice.

## SATCOM voice and data have proven to be effective and reliable long range communication systems since their introduction into service.  Flight crews have used SATCOM voice (when voice communications are needed) instead of HF voice.  Some regulatory authorities have granted some operators dispatch relief for a limited time whereby the aircraft may be dispatched for a limited period (5 days) with only a single HF radio system and an operational SATCOM voice system.  In the interest of lowering operational costs and streamlining operational efficiency, aircraft operators are turning to SATCOM voice and data capability to meet long range communication system ( LRCS) requirements.  These capabilities include Inmarsat SATCOM voice using geosynchronous orbit satellites and Iridium SATCOM voice, which uses satellites in a low-earth orbit.

## In March 2008, the PARC CWG initiated a project to investigate uses of satellite voice for ATC communications.

Satellite voice project goal

## The goal of this project is to conclude on the recommendations to the FAA that would allow one HF radio system to be permanently replaced with a satellite voice system.  This project is specifically investigating Inmarsat and Iridium satellite voice communication capabilities via a radio operator to determine their viability as an FAA-approved long range communication system (LRCS).  If satellite voice can be FAA-approved as a LRCS, the Minimum Equipment List (MEL) could be changed to allow dispatch with one satellite voice communication system and only one HF radio system, when two LRCS are normally required.

Note.—  This project will not consider the use of SATCOM voice for ATC communications directly to a controller.

Pre-requisites for the project

## The pre-requisites for the project include:

- In accordance with established PARC CWG procedures, participants must agree to provide information and data necessary to substantiate the recommendations described in the goal of this project.

- Each contributor, ATSP and aircraft operator has volunteered to participate in the trials, and must agree to bear their own costs.

- An operational implementation or trial for the use of SATCOM voice for ATC communications through a radio operator must be in place.  The data will be obtained from operational implementations and/or trials of the use of SATCOM voice for air traffic control (ATC) communications.  The PARC CWG is relying on other implementation initiatives under the auspices of planning and implementation regional groups (PIRGs) or States when data can be provided to the PARC CWG.

- Although the underlying technology lends itself to a conversational mode of communications, such use can create misunderstanding and confusion.  Therefore, when using SATCOM voice, normal RTF conventions must be followed in accordance with standard ICAO phraseology, as defined in Annex 10, Volume II, Chapter 5 and Doc. 8400.

Participating operators

## Currently, Continental, Continental Micronesia, Cargolux, and Delta collectively intend to provide 103 aircraft to participate in the project.

## 57 aircraft are equipped with Iridium and 46 aircraft are equipped with Inmarsat.  The installations have been approved by the State of Registry or State of the Operator in accordance with FAA AC 20-150 (or equivalent) and the satellite radio meets ARINC 741.

## Additionally, aircraft operators participating in this project must meet the following criteria:

- Distribute advisory information within the flight operations department to ensure that all personnel concerned are aware of SATCOM concepts and procedures associated with the use of SATCOM for routine ATC communications (i.e. phreasiology).

- Assess operational requirements, establish policy and procedures, and incorporate them in appropriate company documents, including:

  - Procedures to ensure that each participating aircraft is registered with their communication service provider;

  - Flight crew responsibilities for establishing and maintaining HF/VHF voice communications (SELCAL) with the appropriate OCA/FIR’s;

  - Procedures on use of SATCOM voice in accordance with ATC communication procedures, including its use in relationship to other means of communication on the specific aircraft, e.g., CPDLC/ADS-C, FMS WPR, Oceanic clearance, etc.;

  - Contact information for the Radio Stations;

  - Procedures when SATCOM voice fails;

  - Procedures for reporting problems associated with SATCOM voice to the PARC CWG.

Note.—  Participation in this project does not relieve the operator of the responsibility to ensure that the aircraft is fully equipped for oceanic communications as prescribed for the airspace operated in or the State of registry for the aircraft or operator.

Participating radio facilities

## Currently, ARINC radio facilities are participating in the project.  Radio facilities participating in this project must meet the following criteria:

- Establish procedures for operators to register SATCOM voice capabilities, phone number, etc.

- Establish procedures to distribute to ANSPs and radio operators the relevant information needed to establish two-way communications with the aircraft.

- Establish procedures to notify users when service conditions change, i.e., service outages, degraded performance, restoration of service.

Note.—  Participation in this project does not relieve the communication service provider of the responsibility to fulfill its contractual requirements with ANSPs and operators or regulatory requirements as a provider of aeronautical communication services involving the radio facility as prescribed by ICAO and its member States.

## Success criteria

## SATCOM voice communication performance will be evaluated using the RCP 400 specification “top sheet” per Appendix B of the GOLD, which is presented in SOCM WP/5, and is shown in Figure 2-1.  The PARC CWG will develop as part of this project the recommendations for the RCP allocations needed to support application of the RCP 400 specification “top sheet” to satellite voice communications.  Figure 2-2 provides a suggested allocation for a controller-initiated third-party satellite voice communication transaction.  The RCP allocations will be validated and adjustments may be necessary during the course of its evaluation.

| RCP Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RCP type |  |  |  |  | RCP 400 |  |
| Airspace specific considerations |  |  |  |  |  |  |
| Interoperability |  | Specify interoperability criteria, e.g., FANS 1/A |  |  |  |  |
| ATS Function |  | Specify ATS function(s), e.g., applicable separation standard |  |  |  |  |
| Application |  | Specify controller-pilot ATC communication intervention capability, e.g., CPDLC application per ICAO Doc 4444, and RTCA DO-306/EUROCAE ED-122, Annex A |  |  |  |  |
| RCP parameter values |  |  |  |  |  |  |
| Transaction time (sec) |  |  | Continuity (C) (probability) | Availability (A) (probability) |  | Integrity (I) (acceptable rate/flight hour) |
| ET = 400 |  |  | 0.999 | 0.999 |  | 10-5 |
| TT 95% = 350 |  |  | 0.95 |  |  |  |
| RCP monitoring and alerting criteria |  |  |  |  |  |  |
| Ref: | Criteria |  |  |  |  |  |
| MA-1 | The system shall be capable of detecting failures and configuration changes that would cause the communication service to no longer meet the RCP type for the intended function. |  |  |  |  |  |
| MA-2 | When the communication service can no longer meet the RCP type for the intended function, the flight crew and/or the controller shall take appropriate action. |  |  |  |  |  |

Figure 2-1.	RCP 400 specification “top sheet”

Figure 2-2.	Typical voice communication transaction – controller initiated
third party satellite voice communication allocation

## In addition to the RCP 400 specification, PARC CWG is proposing additional criteria:

- All parties will use standard ICAO phraseology;

- 75% of all calls will be answered in 10 seconds or less;

- 94% of all calls will be answered in 60 seconds or less;

- ATC clearances will be delivered in 3 minutes or less;

- ATC requests and advisories will be delivered in 5 minutes or less;

- Weather related messages will be delivered in 10 minutes or less; and

- Company messages will be delivered in 10 minutes or less.

Note.—  While radio facilities strive to meet the above time criteria the actual “turn around” time from aircraft request to receipt of a clearance may be longer due to ATC processing delays which are out of the control of the radio facility.

Data collection and evaluation

## The PARC CWG is coordinating data collection and analysis methods with participating stakeholders.  The following are being considered in determining the data that will be collected:

- Identify any security, operational and technical issues through normal problem reporting means;

- Determine connection times compared to HF Radio for ground and airborne initiated calls;

- Determine delivery times of clearances, requests, advisories, and company messages and compare with similar transactions using HF voice;

- Determine impact on existing radio operator and flight crew procedures;

- Determine clarity and strength of satellite voice communications; and

- Determine reliability, availability and integrity of satellite voice services.

## The PARC CWG estimates that it will collect data from approximately 10,000 transactions using satellite voice communication services within a year to support its investigations.

## Based on favorable results from its analysis, PARC CWG would provide recommendations to the FAA on criteria to use of satellite voice communications as an FAA-approved LRCS, potentially enabling one satellite voice communication system to meet MEL where two LRCSs are required.

# Recommendations

## The meeting is requested to note this information.

## For more information on PARC CWG activities, please contact either Mr. Arnold Oldach (aoldach@rockwellcollins.com) or Mr. Tom Kraft (tom.kraft@faa.gov), the co-chairmen of the PARC CWG.