THE NINETEENTH MEETING OF THE IPACG FANS INTEROPERABILITY TEAM

(IPACG FIT/19)

(Honolulu, Hawaii, USA, 11 May 2010)

FANS 1/A over Iridium Project Update

(Presented by Federal Aviation Administration)

SUMMARY

This paper provides an update on the FANS 1/A over Iridium (FOI) project being done by the FAA-sponsored Performance-Based Operations Aviation Rulemaking Committee’s Communications Working Group (PARC CWG).

- Introduction

  - Since IPACG/31, on 12 December, Continental Micronesia (CMI) began FANS 1/A over Iridium (FOI) operations.  Currently, CMI is operating 7 FOI B737-800 aircraft (See Table 1 for aircraft registration numbers) and two FOI 737-700 aircraft, N13720 and N27734, were added the first week of May.

  - In addition, on 25 March, Cargolux (CLX), in Luxemburg, began FOI operations with 13 Boeing 747 400s to over 60 flight destinations worldwide.  We do not expect any of these aircraft to operate in the Fukuoka flight information region (FIR).

  - This paper provides preliminary results of the performance evaluations of these flights in Oakland and New York airspace.

- Discussion

  - The Performance Based Operations Aviation Rulemaking Committee Communications Working Group (PARC CWG) held its 20th meeting on 6-7 April, 2010, in Phoenix, Arizona.

  - At that meeting, the PARC CWG reviewed preliminary performance data collected from Oakland and New York centers.  The ADS-C downlink latency performance for March is presented in Figure 1 and Figure 2, and for April in Figure 3.  Actual communication technical performance (ACTP) for all FOI CPDLC transactions with W/U response is provided in Figure 4.

  - PARC CWG requested the Central Reporting Support Agency (CRASA) to facilitate an FOI Investigation Team, comprising representatives for all the components of the end-to-end system, to look at possible reasons the actual performance of the CMI fleet is not meeting ADS-C type 180 specification, provided in the global operational data link document (GOLD).

  - Possible suspect is that the Iridium SATCOM system is telling the ACARS management unit (MU) that it is in a NO-COMM condition.  Also a possible contributor is that these aircraft are equipped with high frequency data link (HFDL), which may be interfering with performance measurements.  As indicated in Table 1, although we did not expect the HF radios to be in data mode, we are still receiving some ADS-C reports on HFDL.  The investigation team is looking further into these suspects.

Table 1 – Counts of ADS-C reports by media – March data

  - In addition, a problem has been identified with the ring alert processing by the short burst data (SBD) server.  A change to correct the ring alert processing has been incorporated into a Release 5.2 update to the SBD server.  Release 5.2 was implemented on 28 April.  We expect to see a performance improvement in the actual performance data.

  - CMI is also reporting problems received by their flight crews to the CRASA, some of which are not related to the Iridium project.  These problems are being investigated and corrected.

  - The PARC CWG is expecting to conclude its evaluations on the viability of Iridium for FANS 1/A operations after it gathers a statistically sufficient sample of consistent data from a stable configuration indicating acceptable performance.  PARC CWG believes this will occur before September.

- Conclusion

  - The meeting is invited to note the information provided.

Figure 1.	ADS-C downlink latency – by operator - March data (ZOA and ZNY)

Figure 2.	ADS-C downlink latency - by aircraft registration – March data (ZOA)

Figure 3.	ADS-C downlink latency – by Month (IGW1)

Figure 3.	ACTP – by Month (IGW1)