THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, 10-14 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

FAA’s ADS-B ITP and ADS-C CDP Trials Update

(Presented by the Federal Aviation Administration)

SUMMARY

The purpose of this information paper is to present the U.S. Federal Aviation Administration (FAA) plan for conducting an Automatic Dependent Surveillance – Broadcast (ADS-B) In-Trail Procedures (ITP) Operational Trial and an Automatic Dependent Surveillance – Contract (ADS-C) Climb/Descend Procedure (CDP) Operational Trial in the Oakland Oceanic Flight Information Region (FIR).

- Introduction

  - The FAA is continuing the work associated with the development of a climb-through and descend-through procedure using ADS-C capabilities.  ADS-C analysis has determined that this oceanic procedure supports the FAA goals for fuel efficiency, emission reductions, and air traffic capacity.

    - This procedure is based on in-trail Distance Measuring Equipment (DME) rules in ICAO Doc 4444, paragraph 5.4.2.3.2.  Aircraft pair distance verification is performed by ground flight tracking systems, such as the FAA Ocean21 system, using near simultaneous ADS-C demand contract reports.  As with the existing DME procedure, responsibility for separation assurance remains with air traffic control (ATC).

  - The FAA Surveillance and Broadcast Services (SBS) Program Office is developing a number of airborne ADS-B applications that are expected to provide benefits to operators who chose to equip their aircraft with appropriate avionics, including “ADS-B In” (i.e., the ability to receive, process, and display ADS-B data from surrounding aircraft).  In addition to providing benefits to operators who equip, these applications will help accelerate the understanding and acceptance of airborne ADS-B and provide an increased user-base of equipped aircraft that will support future applications.  One such airborne ADS-B application being developed is ADS-B In-Trail Procedures (ITP).

    - The FAA has entered into an agreement with United Airlines for the purpose of performing an operational trial of ADS-B ITP in the South Pacific, on revenue flights.  This agreement includes the development and certification of onboard systems that provide the ADS-B ITP criteria and display that information to the pilot.

- Discussion

  - ADS-C CDP Operational Trial

    - The ADS-C procedure is designed to improve FAA service to properly equipped aircraft by allowing an oceanic air traffic controller to have another option for granting an altitude change request when other standard separations, such as ADS-C distance-based 30 NM longitudinal separation minima, do not allow for a climb or descent through the altitude of a blocking aircraft.  It is strictly an air traffic control tool that is controller-initiated after a pilot request, to be applied between Required Navigation Performance 4 (RNP4) manoeuvring and blocking aircraft pairs.

    - The objective of ADS-C CDP activities in fiscal year (FY 2009) was to begin development of operational trials over Pacific airspace.  These activities included the refinement of procedures developed in FY 2008, the development of necessary safety case documents and obtaining ICAO endorsement.

    - During FY 2010, collision risk modelling will be completed, and a demonstration plan and procedure will be finalized.  The operational trial will be a manual execution of the procedure using Ocean21 displays but without changes to the Ocean21 software.  During the operational trial, Safety Management System monitoring will be performed, and benefits metrics will be collected.  Although not determined, it is expected that the operational trial will be limited to specific sectors such as Oakland Air Route Traffic Control Center (ARTCC) Oceanic Sectors 1 and 2.

      - Based on Oakland ARTCC traffic, with a 35% Future Air Navigation System (FANS) equipage rate, benefits analysis projected total combined fuel savings up to 44,000 kilograms (kg ) or 139,040 kg of carbon dioxide (CO2)  per day.

      - A FANS equipage rate nearing 100% increased the potential benefits to nearly 88,000 kg of fuel or 278,080 kg CO2 per day.

    - The proposed requirements for an operational trial of ADS-C CDP are listed in Attachment A to this paper.

  - ADS-B ITP Operational Trial

    - Aircraft operating in oceanic airspace are, at times, held at non-optimal flight levels due to conflicting traffic either at the desired flight level or at flight levels between the existing flight level and the optimal flight level. The use of flight level change procedures, enabled by ADS-B, can supplement oceanic standards creating greater operational efficiency.

    - ADS-B ITP will enable flight level changes for aircraft equipped with ADS-B receivers and on-board automation.  For ADS-B ITP, the maneuvering (trailing or leading) aircraft obtains the flight identification (ID), altitude, position and ground speed transmitted by proximate ADS-B equipped non-maneuvering (leading or trailing) aircraft.  Based on the ADS-B data from the non-maneuvering or reference aircraft, a pilot can make an ITP altitude change request to ATC.  The controller, who maintains separation responsibility at all times, would verify that the ITP and reference aircraft were same track and that the maximum closing Mach differential was not exceeded.

    - If the controller determines that standard separation minima will be met with all aircraft other than the ITP reference aircraft, a clearance for the climb or descent may be issued. The maneuvering aircraft may then vertically transition through the altitude of the non-maneuvering aircraft.  To perform these procedures, the aircraft desiring to climb or descend must be equipped with an ADS-B transceiver and an appropriate onboard decision support system, both of which would have to be certified for this application. Those aircraft operators choosing to equip in this manner would be able to take advantage of this procedure when operating in proximity to aircraft equipped with a suitable ADS-B transmitter.

    - While there has been a significant amount of developmental work for ADS-B ITP, the next step is to conduct an operational trial, or evaluation, of ADS-B ITP.  The objective of FY 2010 activities will include, but is not limited to, development and certification of onboard systems that provide the ADS-B ITP criteria and display that information to the pilot, performing required safety management system (SMS) processes and analyses, and obtaining FAA aircraft certification and flight standards approvals.

    - During FY 2011 a demonstration plan and procedure will be finalized for an operational trial of a manual execution of ADS-B ITP using Ocean21 displays but without changes to the Ocean21 software.  During the operational trial, SMS monitoring will be performed and benefits metrics will be collected.

      - The trial will take place on equipped United Airlines 747-400’s operating between the US West Coast and Australia and on return flights.  The flights will be regularly scheduled passenger revenue flights, not test flights.

      - There will be designated data collection activity for both United Airlines and Oakland ARTCC.  The data collected will be used to enhance the understanding of the economic and operational impact.  Any significant adverse operational issues that are discovered (such as communication or workload) will result in an immediate suspension of all operational evaluation activity.

      - Initial expectations are that between one and two ITP procedures will be performed per week of operations.

    - A concept overview of the proposed requirements for an operational trial of ADS-B ITP is presented in Attachment B to this paper.

- Recommendations

  - The meeting is requested to note the information presented in this paper.

Proposed requirements for an operational trial of ADS-C CDP are as follows:

- The maneuvering aircraft is flying level prior to executing ADS-C CDP

- The maneuvering aircraft is approved for Reduced Vertical Separation Minimum (RVSM)

- The maneuvering  aircraft has active FANS-1/A ADS-C and Controller-Pilot Data Link Communication (CPDLC) connections

- The blocking aircraft is flying level

- The blocking aircraft is approved for RVSM

- The blocking aircraft has active ADS-C and CPDLC connections

- The blocking aircraft has an assigned altitude that is 1,000 feet above or below maneuvering aircraft assigned flight level

- Blocking and maneuvering aircraft are on same track, same direction

- Blocking and maneuvering aircraft are eligible for distance-based separation

- Neither the maneuvering nor blocking aircraft are on a deviation from course or are requesting a deviation from course

- Neither the maneuvering nor blocking aircraft are out of conformance

- The maneuvering aircraft will execute an altitude change of 2000 feet

- For a manual operational trial, maneuvering and blocking aircraft must be qualified for RNP-4

For a manual operational trial, the following separation criteria shall be used:

- The controller must set RNP-4 distance-based separation flags for the maneuvering and blocking aircraft.

- ADS-C CDP can be used if the Conflict Probe results for the requested altitude change show a conflict, and either of the following two distance and speed checks is passed.

- When the maneuvering aircraft is probed for an altitude change, the blocking and maneuvering aircraft display an ACTUAL conflict (NOW indicated in red, to the right of “LOS” in the Conflict Report Window), and all of the following checks are satisfied:

  - The longitudinal separation distance at the conflict start time is greater than or equal to parameter (16) nm

  - The leading aircraft groundspeed is greater than or equal to the trailing aircraft groundspeed, based on the displayed groundspeeds after ADS DEMAND reports are received

  - The leading aircraft reported Mach number is greater than or equal to the trailing aircraft reported Mach number

- When the maneuvering aircraft is probed for an altitude change, the blocking and maneuvering aircraft have an ACTUAL or IMMINENT conflict (Red box will be displayed to the right of “LOS” in the Conflict Report Window), and all of the following checks are satisfied:

  - The longitudinal separation distance at the conflict start time is greater than or equal to parameter (26) NM

  - The trailing aircraft groundspeed is greater than the leading aircraft groundspeed by at most parameter (10) knots, based on the displayed groundspeeds after ADS DEMAND reports are received

  - The trailing aircraft reported Mach number is greater than the leading aircraft reported Mach number by at most parameter (.02)

- The uplink clearance shall be a CPDLC message (Appendix A of ATOP Ops Manual [5]–UL Messages 26 and 28)

  - CLIMB TO REACH (alt) by (time)

  - DESCEND TO REACH (alt) by (time)

    - (time) is a parameter (15) number of minutes after the uplink time of the ADS Demand message for the maneuvering aircraft

ADS-B ITP CONCEPT OVERVIEW

Procedure supports FAA goals for fuel efficiency, emissions reductions, capacity, and safety.

Other Aircraft

ITP Aircraft

Reference Aircraft

FL360

FL340

FL350

Current

Longitudinal

Separation Minimum

Other Aircraft

Other Aircraft

Current

Longitudinal

Separation Minimum

ITP

Criteria

ITP Behind-Climb

ITP Behind-Climb: The ITP aircraft is behind a reference aircraft which is at a higher intervening flight level (FL350). Standard ATC procedures apply to the other aircraft (two aircraft at FL360 and one at FL350).

- Maneuvering aircraft obtains the flight ID, altitude, position and ground speed transmitted by proximate ADS-B equipped non-maneuvering (reference) aircraft.

- Based on the ADS-B data from the reference aircraft, the flight crew requests clearance from ATC to conduct the appropriate ITP maneuver to the requested flight level.

  - Request will include the aircraft ID of any reference aircraft and the ITP distance

  - Request will be made using a new standard phraseology

  - Reference and maneuvering aircraft are on same track, same direction

  - Neither maneuvering nor reference aircraft are on a deviation from course or are requesting a deviation from course

  - Maneuvering aircraft will execute an altitude change of 2000 feet

- The controller verifies the ITP and reference aircraft are same track and that the maximum closing Mach differential is not exceeded.  The controller will provide standard separation between the ITP aircraft and all other aircraft that are not identified as reference aircraft for the purposes of the ITP.

- If controller determines that standard separation minima will be met with all aircraft other than the ITP reference aircraft, a clearance for climb or descent may be issued.

- After receiving the clearance to conduct the ITP, the flight crew verifies that the ITP criteria are still satisfied.  If ITP criteria are no longer satisfied, pilot shall notify controller and maintain the previously cleared level.  If ITP criteria are still satisfied the ITP climb or descent manoeuvre shall be initiated without delay.

- While performing the ITP maneuver, the flight crew monitors their speed as well as their climb or descent rate.  If the maneuver cannot be accomplished at the original cruise Mach and also meet the minimum continuous climb or descent rate of 300 fpm, the ITP  aircraft must immediately notify the controller and follow appropriate contingency procedures.

- If the ITP cannot be successfully completed once the climb or descent has been initiated, the flight crew must notify ATC and request an alternative clearance.  If unable to obtain this clearance, the flight crew must follow the appropriate contingency procedures.

- FLIGHT CREW request:

  - DM9: REQUEST CLIMB TO [altitude]

  - DM67: ITP [ITPdistanceNM BEHIND acid]

- ATC clearance:

  - UM169: ITP BEHIND [acid]

  - UM20: CLIMB TO AND MAINTAIN [altitude]

  - UM129: REPORT LEVEL [altitude]